3GPP TSG-RAN WG4 Meeting #111		R4-2408569

# Fukuoka City, Fukuoka , Japan, 20th – 24th May, 2024

Agenda item:	7.19.2.1

Source: 	Huawei, HiSilicon

Title: 	Discussion on performance requirements for TDCP for Rel-18 MIMO

Document for:	Discussion

# Introduction

The performance requirements for R18 MIMO were discussed in last meeting with agreements and issues captured in [1]. In this paper, we further provide our views on how to define TC for PDCP.

# Discussion

The latest progress on how to define TC for TDCP are summarized as follows:

Issue 2-1-1: Test metric of TDCP test cases: < Agreement> : Agreements for the test case definition: - Define the same test case for both 15kHz with FDD and and 30kHz with TDD in the same sub-clause. - Channel models: TDL-A30 - Doppler: - Two TCs with low and high Doppler - TC1: low (10) for both 15kHz and 30kHz - TC2: high - Option 1: high (300) - Option 2: 100 for 15kHz SCS, and 200 for 30kHz SCS - BW: - 10MHz for FDD and TDD - SNR: - Option 1: 20dB for TC1 - Option 2: 10dB or 20dB for TC2 - The distance between two TRSs: 1 - Report index: Bring CR for both options, make decision in the next meting - Option 1: - lower Doppler: CDP at X1 is higher than Y1 = [90] % - high Doppler: CDP at X2 is lower than Y2= [10] % - X1, X2, Y1, and Y2 can be different for TDD and FDD - Option 2: - [X1, X2] for Y2= FFS

Based on the updated TC metric and configurations, the simulation results are provided as follows:

Table I: TDCP results for 15KHz

| Channel | SNR | Values of estimated TDCP |  |  | Index of estimated TDCP |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | 10% | 50% | 90% | 10% | 50% | 90% |
| TDL-A 10 | 10 | 0.7119 | 0.8926 | 0.9565 | 12 | 9 | 6 |
|  | 20 | 0.9534 | 0.987 | 0.9951 | 7 | 3 | 0 |
| TDL-A 100 | 10 | 0.564 | 0.8531 | 0.9421 | 13 | 10 | 7 |
|  | 20 | 0.7844 | 0.9608 | 0.99 | 11 | 6 | 2 |
| TDL-A 300 | 10 | 0.3546 | 0.691 | 0.8906 | 14 | 12 | 9 |
|  | 20 | 0.419 | 0.8254 | 0.9543 | 14 | 10 | 7 |

Table II: TDCP results for 30KHz

| Channel | SNR | Values of estimated TDCP |  |  | Index of estimated TDCP |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | 10% | 50% | 90% | 10% | 50% | 90% |
| TDL-A 10 | 10 | 0.9471 | 0.8835 | 0.9582 | 12 | 9 | 6 |
|  | 20 | 0.9522 | 0.9869 | 0.9954 | 7 | 3 | 0 |
| TDL-A 200 | 10 | 0.5329 | 0.8516 | 0.9462 | 13 | 10 | 7 |
|  | 20 | 0.7924 | 0.9576 | 0.9901 | 11 | 6 | 2 |
| TDL-A 300 | 10 | 0.4615 | 0.8215 | 0.9364 | 14 | 11 | 8 |
|  | 20 | 0.6287 | 0.9304 | 0.9839 | 13 | 8 | 4 |

There are two options about how to define the test matrix. We slight prefer option 2which is to define a range of [X1, X2]. The reason is that the accuracy may only be partially verified by option 1. For instance, if the TDCP results is over estimated (i.e. smaller index) in lower doppler case, UE can still pass it. For option 2, companies commented that whether there is RAN5 impacts if the pass rate is not 90%. After check with RAN5 status, there is already precedence (V2X test about congestion control measurement A.9.1.5.1/2 with 98% pass rate), and the pass/fail limit in RAN5 was updated accordingly. Thus, considering necessary margin, the test metric for TDCP is suggested as follows:

15 KHz

- 10 Hz doppler:

10 dB SNR: The reported index shall be within [5,13] with 80% pass rate.

20 dB SNR: The reported index shall be within [0,8] with 80% pass rate.

- 300 Hz doppler:

10 dB SNR: The reported index shall be within [8,15] with 80% pass rate.

20 dB SNR: The reported index shall be within [6,15] with 80% pass rate.

30 KHz

- 10 Hz doppler:

10 dB SNR: The reported index shall be within [5,13] with 80% pass rate.

20 dB SNR: The reported index shall be within [0,8] with 80% pass rate.

- 300 Hz doppler:

10 dB SNR: The reported index shall be within [7,15] with 80% pass rate.

20 dB SNR: The reported index shall be within [3,14] with 80% pass rate.

Proposal 1: The test metric for TDCP is defined as follows:

15 KHz

- 10 Hz doppler:

10 dB SNR: The reported index shall be within [5,13] with 80% pass rate.

20 dB SNR: The reported index shall be within [0,8] with 80% pass rate.

- 300 Hz doppler:

10 dB SNR: The reported index shall be within [8,15] with 80% pass rate.

20 dB SNR: The reported index shall be within [6,15] with 80% pass rate.

30 KHz

- 10 Hz doppler:

10 dB SNR: The reported index shall be within [5,13] with 80% pass rate.

20 dB SNR: The reported index shall be within [0,8] with 80% pass rate.

- 300 Hz doppler:

10 dB SNR: The reported index shall be within [7,15] with 80% pass rate.

20 dB SNR: The reported index shall be within [3,14] with 80% pass rate.

# Conclusions

Proposal 1: The test metric for TDCP is defined as follows:

15 KHz

- 10 Hz doppler:

10 dB SNR: The reported index shall be within [5,13] with 80% pass rate.

20 dB SNR: The reported index shall be within [0,8] with 80% pass rate.

- 300 Hz doppler:

10 dB SNR: The reported index shall be within [8,15] with 80% pass rate.

20 dB SNR: The reported index shall be within [6,15] with 80% pass rate.

30 KHz

- 10 Hz doppler:

10 dB SNR: The reported index shall be within [5,13] with 80% pass rate.

20 dB SNR: The reported index shall be within [0,8] with 80% pass rate.

- 300 Hz doppler:

10 dB SNR: The reported index shall be within [7,15] with 80% pass rate.

20 dB SNR: The reported index shall be within [3,14] with 80% pass rate.

# References

[1] R4-2406503 WF on RRM requirements for NR_MIMO_evo_DL_UL

# Annex

| 15 KHz | 30 KHz |
| --- | --- |