3GPP TSG-RAN WG4 Meeting #111					R4-2408612

Fukuoka, Japan, May 20th – 24th, 2024

Agenda item:	9

Source:	Intel Corporation

Title:	Views on RAN4 Rel-18 UE feature list

Document for:	Discussion

# Introduction

Discussions on RAN4 Rel-18 UE feature list were kicked-off in RAN4 #109 (November 2023) and continued in RAN4 #110 (February 2024) and RAN4 #110bis (April 2024). The latest agreements are provided in [1]. This paper provides further views on the details of UE feature list for selected work items in Section 2.

# Rel-18 UE feature list

## NR_MG_enh2

The feature list for NR_MG_enh2 is nearly completed. In the below tables we propose to complete the feature lists for both NR and LTE features for the work item. To note that we propose to add the simultaneous reception of different numerologies between target NR 30kHz SSB and serving LTE 15kHz scheduling. This is necessary in the same way as the ones we introduced for NR to LTE measurements.

Table 1. Rel-18 NR UE features for NR_MG_enh2 WI.

| Features | Index | Feature group | Components | Prerequisite feature groups | Need for the gNB to know if the feature is supported | Applicable to the capability signalling exchange between UEs (V2X WI only)”. | Consequence if the feature is not supported by the UE | Type | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Capability interpretation for mixture of FDD/TDD and/or FR1/FR2 | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 32. NR_MG_enh2 | 32-1 | Concurrent gaps with Pre-MG in a FR | Support of multiple per-UE (or per-FR) measurement gap patterns with at least one per-UE (or per-FR) Pre-MG. Details in Clause [9.1.x.2] of TS 38.133. | 19-3-x and 19-2 x = 1 or 2 | Yes | No | UE behaviour is undefined if the network configures concurrent MGs where at least one of the gaps is a Pre-MG | Per UE | No | No | N.A |  | Optional with capability signalling |
| 32. NR_MG_enh2 | 32-2 | Support for dynamic collisions | Support RRM requirements for handling dynamic collisions between a Pre-MG and another measurement gap or Pre-MG. | 32-1 | Yes | No | UE is not expected to meet RRM requirements for dynamic collisions | Per UE | No | No | N.A |  | Optional with capability signalling |
| 32. NR_MG_enh2 | 32-3 | Concurrent gaps with NCSG in a FR | Support of multiple per-UE (or per-FR) measurement gap patterns with at least one per-UE (or per-FR) NCSG. Details in clause [9.1.y.2] of TS 38.133. | 19-1 and 19-2 | Yes | No | UE behaviour is undefined if the network configures concurrent MGs where at least one of the gaps is a NCSG | Per UE | No | No | N.A |  | Optional with capability signalling |
| 32. NR_MG_enh2 | 32-4 | Inter-RAT EUTRAN measurements without gap and outside active DL BWP | Support inter-RAT EUTRAN measurements outside active DL BWP for nogap-noncsg | 19-1b | Yes | NA | UE does not meet the requirements of inter-RAT EUTRAN measurements without gap in TS 38.133 and the UE behavior is unknown to network | Per UE | No | No | N.A |  | Optional with capability signalling |
| 32. NR_MG_enh2 | 32-5 | Inter-RAT EUTRAN measurement without gap and within active DL BWP | Support of inter-RAT EUTRAN measurements without gap when CRS is contained within UE’s active DL BWP |  | Yes | No | Measurement gap will be needed for inter-RAT EUTRAN measurements | Per UE | No | FR1 only | N.A |  | Optional with capability signalling |
| 32. NR_MG_enh2 | 32-6 | Effective measurement window for inter-RAT EUTRAN measurements | Support configuration of effective measurement window for inter-RAT EUTRAN measurements, including offset, duration and periodicity. | 32-4 or 32-5 | Yes | No | UE is not allowed to cause scheduling restriction defined in TS 38.133 for 32-6 or 32-7 | Per UE | No | No | N.A | - A bitmap for 6 effective measurement window (EMW) patterns defined in TS 38.133. - #0 and #1 are mandatory, if UE supports EMW feature. Other patterns are optional Note: If UE supports 32-4 or 32-5 and UE requires scheduling restriction, UE should support this FG | Optional with capability signalling |
| 32. NR_MG_enh2 | 32-7 | Simultaneous reception of NR data and EUTRAN CRS with different numerology | Support concurrent inter-RAT measurement on EUTRAN cell in non-DSS with CRS and PDCCH or PDSCH reception from the serving cell with a different numerology | 32-4 or 32-5 | Yes | No | scheduling restriction is applicable | Per UE | No | FR1 only | N.A |  | Optional with capability signalling |

Table 2. Rel-18 LTE UE features for NR_MG_enh2 WI.

| Features | Index | Feature group | Components | Prerequisite feature groups | Need for the gNB to know if the feature is supported | Applicable to the capability signalling exchange between UEs (V2X WI only)”. | Consequence if the feature is not supported by the UE | Type | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Capability interpretation for mixture of FDD/TDD and/or FR1/FR2 | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 32. NR_MG_enh2 | x-y | interRAT-NeedForInterruptionNR-r18 | Support of inter-RAT NR measurements without gap with or without interruption when the interRAT-NeedForGapsNR-r16 is false. Note: This feature already has a defined UE capability: ‘interRAT-NeedForInterruptionNR-r18’. The intention of adding this FG is only keep consistency between 38.822 and 36.306. | interRAT-NeedForGapsNR-r16 | Yes | NA | The UE does not support inter-RAT NR measurements without gap with or without interruption for performing inter-RAT NR measurement without gap | Per target band per BC Note: the same granularity as interRAT-NeedForGapsNR-r16 | No | No | NA | # Candidate value: “{no-gap-with-interruption, no-gap-no-interruption}” | Optional with capability signalling |
|  | [x-y] | Simultaneous reception of LTE data and NR SSB with 30kHz SCS | Support concurrent inter-RAT NR measurement with a different numerology than 15kHz and reception from LTE serving cell |  | Yes | NA | Scheduling restriction is applicable | Per UE | No | No | NA |  | Optional with capability signalling |

## NR_cov_enh2

Our views on RAN4-centric features for the NR_cov_enh2 WI [WID RP-221858] are provided below and summarized in Table 3. The updated UE feature list for Rel-18 NR coverage enhancements work item was agreed in RAN4 #110 [1]. The majority of open issues were resolved, however, there are still open issues on applicable scenarios for power boosting (PAR/MPR enhancements) features and, especially, with respect to CA scenarios applicability [2].

WF - RAN4 is still discussing the applicable scenarios. - The common understanding in RAN4 is that power boosting is only supported for single CC UL at least in Rel-18. - UL CA is not supported for this feature in Rel-18.

Our detailed views on the power boosting features applicability to UL CA scenarios are provided in the accompanying contribution [3]. Below, we propose an updated description of the feature list description in terms of applicable scenarios.

Table 3. Rel-18 NR UE features for NR_cov_enh2 WI.

| Features | Index | Feature group | Components | Prerequisite feature groups | Need for the gNB to know if the feature is supported | Applicable to the capability signalling exchange between UEs (V2X WI only)”. | Consequence if the feature is not supported by the UE | Type | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Capability interpretation for mixture of FDD/TDD and/or FR1/FR2 | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 41.NR_cov_enh2 | 41-1 | Support of ΔPPowerClass reporting mechanism | Support of ΔPPowerClass /ΔPPowerClass, CA/ΔPPowerClass, EN-DC/ΔPPowerClass, NR-DC reporting which is triggered upon uplink duty cycle exceedance or upon return to the power class after the duty cycle exceedance, as specified in TS 38.101-1 and TS 38.101-3. | No | Yes | N/A | UE does not support of report on the ΔPPowerClass | Per UE | No | FR1 only | N/A | Component 1 candidate values: - Type 1: The UE can only report ∆PPowerClass for non-CA operation Type 2: The UE can report ∆PPowerClass for non-CA operation, and the UE can also report ∆PPowerClass/ ΔPPowerClass,CA/∆PPowerClass,EN-DC/∆PPowerClass,NR-DC for CA operation. | Optional with capability signalling |
| 41.NR_cov_enh2 | 41-2 | Power boosting for DFT-s-OFDM pi/2 BPSK and QPSK transmissions without modified spectrum flatness requirement | 1. Support of UE power boosting for DFT-s-OFDM pi/2 BPSK and QPSK without modified spectrum flatness requirement for PC3 and PC2 MPR reduction, when applicable as defined in 6.2 of TS 38.101-1.The power boosting is only enabled when signalled via RCC powerBoostPi2BPSKRel18 for BPSK and powerBoostQPSKRel18 for QPSK | 1-6, 1-7 | Yes | N/A | UE cannot power boost without modified spectrum flatness requirement | Per FS | NO | FR1 only | N/A | FFS – RAN4 is still discussing the applicable scenarios The feature is applicable for the following scenarios: 1) single carrier DL/UL in FR1 bands 2) FR1 CA/DC with single UL carrier 3) FR1 CA/DC with inter-band UL CA with at least one indicated band supporting the power boosting, where a single CC is used for transmission in each power boosted uplink band. 4) FR1-FR2 CA/DC with at least one indicated FR1 band supporting the power boosting, where a single FR1 UL CC is used for transmission in uplink FR1 band. FFS for FR1 CA/DC with intra-band UL CA | Optional with capability signalling |
| 41.NR_cov_enh2 | 42-3 | Power boosting for DFT-s-OFDM pi/2 BPSK and QPSK transmissions with modified spectrum flatness requirement shaping | 1. Support of UE power boosting for DFT-s-OFDM pi/2 BPSK and QPSK with modified spectrum flatness requirement for PC3 and PC2 MPR reduction, when applicable as defined in 6.2 of TS 38.101-1. The power boosting is only enabled when signalled via RCC powerBoostPi2BPSKRel18 for BPSK and powerBoostQPSKRel18 for QPSK | 1-6, 1-7 | Yes | N/A | UE cannot power boost with modified spectrum flatness requirement | Per FS | NO | FR1 only | N/A | FFS – RAN4 is still discussing the applicable scenarios The feature is applicable for the following scenarios: 1) single carrier DL/UL in FR1 bands 2) FR1 CA/DC with single UL carrier 3) FR1 CA/DC with inter-band UL CA with at least one indicated band supporting the power boosting, where a single CC is used for transmission in each power boosted uplink band. 4) FR1-FR2 CA/DC with at least one indicated FR1 band supporting the power boosting, where a single FR1 UL CC is used for transmission in uplink FR1 band. FFS for FR1 CA/DC with intra-band UL CA | Optional with capability signalling |

## Netw_Energy_NR

In RAN4 #110 the UE feature group for inter-band SSB-less SCell activation was agreed as in the below table, and we propose to add a new intra-band NCCA feature group upon it. We propose that RAN4 introduces the UE feature group for optionally supporting SSB-less intra-band NCCA SCell activation with capability signalling and per feature set indication. This new indication should be a separate element/vector from the existing SCellwithoutSSB or SCellwithoutSSB-interband. Although per BC is enough to our views for the intra-band NCCA case however to align with legacy specs with intra-band contiguous SSB-less SCell cases, we propose a per FS indication.

Table 4. Rel-18 NR UE features Netw_Energy_NR WI.

| Features | Index | Feature group | Components | Prerequisite feature groups | Need for the gNB to know if the feature is supported | Applicable to the capability signalling exchange between UEs (V2X WI only)”. | Consequence if the feature is not supported by the UE | Type | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Capability interpretation for mixture of FDD/TDD and/or FR1/FR2 | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| # 42.Netw_Energy_NR | 42-1 | SCell without SS/PBCH block for inter-band CA | Support of SCell without SS/PBCH block for inter-band CA |  | Yes | NA | UE cannot support SCell without SS/PBCH block for inter-band CA | per FS | NA | FR1 only | NA | For each band within the BC, UE indicates if it supports the SSB-less operation when this band is the reference band and other band(s) in the BC as the SSB-less band(s). If UE indicate “support” for this band, it means all other bands within the BC can be configured as SSB-less bands. | Optional with capability signalling |
|  | 42-2 | SCell without SS/PBCH block for intra-band non-contiguous CA | Support of SCell without SS/PBCH block for intra-band non-contiguous CA |  | Yes | NA | UE does not support SCell without SS/PBCH block for intra-band non-contiguous CA | Per FS | NA | FR1 only | NA | For each carrier within the combination, UE indicates if it supports the SSB-less operation when any of the carrier can be the reference carrier and other(s) in the combination be the SSB-less carrier(s). If UE indicates ‘support’ for a combination, it means all the carriers in the combination can be configured as SSB-less carriers. | Optional with capability signalling |

# Conclusion

This paper provides our updated views on details of UE feature list for selected work items in Section 2. It is proposed to endorse the proposed feature lists in

- Table 1. Rel-18 NR UE features for NR_MG_enh2 WI.

- Table 2. Rel-18 LTE UE features for NR_MG_enh2 WI.

- Table 3. Rel-18 NR UE features for NR_cov_enh2 WI.

- Table 4. Rel-18 NR UE features Netw_Energy_NR WI.

# References

[1] R4-2406680, “Rel-18 RAN4 UE feature list for NR (version 4)”, CMCC, RAN4 #110bis, April 2024

[2] R4-2403885, “WF on coverage enhancement for part 2”, Nokia, RAN4 #110, February 2024

[3] R4-2408613, “On power boosting features applicability to DL/UL CA scenarios”, Intel, RAN4 #110bis, April 2024