3GPP TSG-RAN WG4 Meeting #111		R4-2408667

Fukuoka, Japan, May 20 – May 24, 2024

Title:	LS reply on LTM L1 intra- and inter-frequency measurements

Response to:	R2-2404014

Release:	Rel-18

Work Item:	Further NR mobility enhancements (NR_Mob_enh2-Core)

Source:	RAN4

To:	RAN WG2, RAN WG1

Contact person:	Jani-Pekka Kainulainen

Jani-Pekka.Kainulainen@nokia.com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:	None

# 1	Overall description

RAN2 send an LS in the previous meeting to RAN4 regarding the following features in feature list.

RAN2 discussed RAN1 and RAN4 feature lists associated with Rel-18 NR_Mob_enh2. RAN2 discussed wither the following L1 measurement and reporting features are mandatory to support LTM or whether L3 measurement could be used. 45-1; Intra-frequency L1 measurement and reports for L1-L2 Triggered Mobility (LTM) procedure. 45-1a; Inter-frequency L1 measurement and reports for L1-L2 Triggered Mobility (LTM) procedure. RAN2 made the following agreement: RAN2 makes no further assumptions whether L3 measurements can be used or not to trigger LTM. RAN2 would like to check the following with RAN1 and RAN4:

Question 1 : Are the above intra-frequency and inter-frequency L1 measurement and reporting features (45-1 and 45-1a) prerequisites to support intra-frequency and inter-frequency LTM, respectively?

RAN4 confirms that L3 measurements can be used to trigger LTM from the RAN4 requirement point of view.

Question 2: The above features, 45-1 and 45-1a, from RAN1 and related RAN4 features (39-1, 39-2, 39-3-1, 39-3-2, 39-3-3, 39-3-4, 39-3-5, 39-3-6) are defined per BC for both intra-frequency and inter-frequency measurements.  RAN2 would like check with RAN1/4 for which BC (e.g. BC of current serving cells, BC including current serving cells and cell to be measured or something else) these capabilities are to be considered for L1 intra-frequency and inter-frequency LTM measurements?

Regarding the question about features 45-1, 45-1a, 39-1, 39-2, 39-3-1, 39-3-2, 39-3-3, 39-3-4, 39-3-5, and 39-3-6, we understand per-BC covers the UE’s current serving cells and candidate cells.

RAN4 has made the following agreement on changing the capabilities to “per UE” instead of “per BC”:

RAN4#111 agreement: TBD

# 2	Actions

To RAN1 and RAN4:

ACTION: 	RAN4 respectfully requests RAN2 to take the provided view into account

# 3	Dates of the next TSG RAN WG4 meetings.

TSG RAN WG4 Meeting #112	19 - 23 August 2024 	Maastricht, Netherlands

TSG RAN WG4 Meeting #112bis	14 – 18 October 2024 	TBC, China