3GPP TSG-RAN WG4 Meeting #111                                                            R4-2408749

Fukuoka City, Fukuoka, Japan, 20th – 24th May, 2024

Agenda Item:	7.23.5

Source:	Ericsson

Title:	On applicability rule and declaration for mIAB requirement

Document for:	Discussion

# 1	Introduction

Great progress of selecting test cases from TS38.101-4 has been made in last RAN4 #110bis meeting. Following two open issues are left for further discussion:

Issue 1-1-1: Applicability rule for mIAB-MT to skip legacy IAB-MT requirements. - Companies to deliver feasible solutions in draft CRs for further discussion in the next meeting. Issue 1-2-2: Manufactory declaration for mIAB node for demodulation requirements. - Option 1: D.xB; Mobile IAB-node; Declaration of support of mobile feature for an IAB-node; x; x. - Other options are not precluded.

In this contribution, we discuss the issues above and provide our proposals for discussion.

# 2	Discussion

Previously in RAN4 #110meeting, companies agreed on the following way to introduce the demodulation requirement for mIAB-MT [1]:

Issue 1-1-4: How to introduce requirements for mIAB-MT demodulation? Agreement: Reuse all IAB-MT demodulation requirements except the new selected cases from 38.101-4 with similar configurations besides channel model.

Given that the test cases from TS38.101-4 have been already selected, an applicability rule is needed to indicate which requirement from clause 8 shall be skipped.

Based on the newly selected test cases from TS38.101-4 [2], we propose the following applicability rule for mIAB-MT demodulation requirement in TS38.176-1:

Note: Italic and grey for existing and agreed paragraph, highlighted for newly proposed

#### 8.2.2.1	General

##### 8.2.2.1.1	Applicability of requirements

8.2.2.1.1.1	General

Unless otherwise stated, for a IAB-MT declared to support more than 2 demodulation branches (for IAB-MT type 1-O and IAB-MT type 2-O), the performance requirement tests for 2 demodulation branches shall apply, and the mapping between connectors and demodulation branches is up to IAB-MT implementation.

The tests requiring more than [20] dB SNR level are set to N/A in the test requirements.

Performance requirement tests in Suffix B shall apply for mIAB-MT (see D.2B in table 4.6-1).

Requirements listed in the table 8.2.2.1.1.1-1 should be skipped for mIAB-MT

Table 8.2.2.1.1.1-1: Test case to be skipped for mIAB-MT

|  | Test case to be skipped |
| --- | --- |
| PDSCH | Test number 1-2 in Table 8.2.2.1.5-2 |
| PDCCH | Test number [2]in Table 8.2.2.3.5-1 |
|  | Test number [3] in Table 8.2.2.3.5-1 |

Similarly, we propose the following applicability rule for mIAB-MT demodulation requirement in TS38.176-2:

Note: Italic and grey for existing and agreed paragraph, highlighted for newly proposed

#### 8.2.2.1	General

##### 8.2.2.1.1	Applicability rule for IAB-MT

8.2.2.1.1.1	General

Unless otherwise stated, for an IAB-MT declared to support more than 2 demodulation branches (for IAB-MT type 1-O and IAB-MT type 2-O), the performance requirement tests for 2 demodulation branches shall apply, and the mapping between connectors and demodulation branches is up to IAB-MT implementation.

The tests requiring more than [20] dB SNR level are set to N/A in the test requirements.

Performance requirement tests in Suffix B shall apply for mIAB-MT (see D.6B in Table 4.6-1).

Requirements listed in the table 8.2.2.1.1.1-1 should be skipped for mIAB-MT

Table 8.2.2.1.1.1-1: Test case to be skipped for mIAB-MT

|  | Test case to be skipped |
| --- | --- |
| PDSCH | Test number 1-2 in Table 8.2.2.2.5.1-1 |
|  | Test number 1-2 in Table 8.2.2.2.5.2-1 |
|  | Test number 2-1 in Table 8.2.2.2.5.2-2 |
| PDCCH | Test number 3 in Table 8.2.2.3.5.1-1 |
|  | Test number 2 in Table 8.2.2.3.5.2-1 |

Proposal 1: Add the following part into the applicability rule for mIAB-MT in TS38.176-1:

Requirements listed in the table 8.2.2.1.1.1-1 should be skipped for mIAB-MT

Table 8.2.2.1.1.1-1: Test case to be skipped for mIAB-MT

|  | Test case to be skipped |
| --- | --- |
| PDSCH | Test number 1-2 in Table 8.2.2.1.5-2 |
| PDCCH | Test number [2]in Table 8.2.2.3.5-1 |
|  | Test number [3] in Table 8.2.2.3.5-1 |

Proposal 2: Add the following part into the applicability rule for mIAB-MT in TS38.176-2:

Requirements listed in the table 8.2.2.1.1.1-1 should be skipped for mIAB-MT

Table 8.2.2.1.1.1-1: Test case to be skipped for mIAB-MT

|  | Test case to be skipped |
| --- | --- |
| PDSCH | Test number 1-2 in Table 8.2.2.2.5.1-1 |
|  | Test number 1-2 in Table 8.2.2.2.5.2-1 |
|  | Test number 2-1 in Table 8.2.2.2.5.2-2 |
| PDCCH | Test number 3 in Table 8.2.2.3.5.1-1 |
|  | Test number 2 in Table 8.2.2.3.5.2-1 |

Regarding to the Manufactory declaration for mIAB node for demodulation requirements, we agree with option 1 listed in the approved WF [2]:

Issue 1-2-2: Manufactory declaration for mIAB node for demodulation requirements. - Option 1: D.xB; Mobile IAB-node; Declaration of support of mobile feature for an IAB-node; x; x.

Proposal 3: Consider the following as the manufactory declaration for mIAB node for demodulation requirements:

| D.xB | Mobile IAB-node | Declaration of support of mobile feature for an IAB-node | x | x |
| --- | --- | --- | --- | --- |

# 3	Summary

In this contribution, we proposed the following wording to specify the skipped test cases for mIAB-MT in TS38.176-1 and TS38.176-2. We summarized our proposals as follows:

Proposal 1: Add the following part into the applicability rule for mIAB-MT in TS38.176-1:

Requirements listed in the table 8.2.2.1.1.1-1 should be skipped for mIAB-MT

Table 8.2.2.1.1.1-1: Test case to be skipped for mIAB-MT

|  | Test case to be skipped |
| --- | --- |
| PDSCH | Test number 1-2 in Table 8.2.2.1.5-2 |
| PDCCH | Test number [2]in Table 8.2.2.3.5-1 |
|  | Test number [3] in Table 8.2.2.3.5-1 |

Proposal 2: Add the following part into the applicability rule for mIAB-MT in TS38.176-2:

Requirements listed in the table 8.2.2.1.1.1-1 should be skipped for mIAB-MT

Table 8.2.2.1.1.1-1: Test case to be skipped for mIAB-MT

|  | Test case to be skipped |
| --- | --- |
| PDSCH | Test number 1-2 in Table 8.2.2.2.5.1-1 |
|  | Test number 1-2 in Table 8.2.2.2.5.2-1 |
|  | Test number 2-1 in Table 8.2.2.2.5.2-2 |
| PDCCH | Test number 3 in Table 8.2.2.3.5.1-1 |
|  | Test number 2 in Table 8.2.2.3.5.2-1 |

Proposal 3: Consider the following as the manufactory declaration for mIAB node for demodulation requirements:

| D.xB | Mobile IAB-node | Declaration of support of mobile feature for an IAB-node | x | x |
| --- | --- | --- | --- | --- |

# References

[1]	R4-2402871: WF for [110][326] NR_mobile_IAB_demod, Ericsson, RAN4 #110, Athens, Greece

[2]	R4-2406058: Way forward on WF on [110bis][331] NR_mobile_IAB_demod, Ericsson, RAN4 #110bis, Changsha, China