3GPP TSG-RAN WG4 Meeting #111                                                            R4-2408750

Fukuoka City, Fukuoka, Japan, 20th – 24th May, 2024

Agenda Item:	7.25.1

Source:	Ericsson

Title:	On PDCCH demodulation requirements for DSS enhancement

Document for:	Discussion

# 1	Introduction

The draft CRs for introducing the PDCCH requirement have been endorsed in RAN4 #110bis meeting. Among them, however, there are some mismatches that need to be resolved to improve the quality of Spec. TS38.101-4. In this contribution, we summarized the mismatches to assist the alignment that could happen in this meeting.

# 2	Discussion

The structure of new PDCCH requirement for FDD and TDD looks stable. While there are still some mismatches that need to be discussed and aligned.

Mismatch 1:

As shown below: The ‘PDCCH configuration’ is mismatched between 2Rx and 4Rx requirement.

Table 5.3.2.1.6-1: Test parameters

| Parameter |  | Unit | Value |
| --- | --- | --- | --- |
| Duplex mode |  |  | FDD |
| Active DL BWP index |  |  | 1 |
| NR UL transmission with a 7.5 kHz shift to the LTE raster |  |  | true |
| PDCCH configuration | Start symbol |  | 1 |  |
| CRS for rate matching (Note 1) | LTE carrier centre subcarrier location |  | Same as NR carrier centre subcarrier location |
|  | LTE carrier BW | MHz | 10 |
|  | Number of antenna ports |  | 4 |
|  | v-shift |  | 0 |
| Note 1: No MBSFN is configured on LTE carrier. Note 2: NR PDCCH data REs and DMRS REs overlapped with LTE CRS are punctured at the transmitter side. |  |  |  |

Table 5.3.3.1.5-1: Test parameters

| Parameter |  | Unit | Value |
| --- | --- | --- | --- |
| Duplex mode |  |  | FDD |
| Active DL BWP index |  |  | 1 |
| NR UL transmission with a 7.5 kHz shift to the LTE raster |  |  | true |
| PDCCH configuration | Symbols with PDCCH |  | Symbol#1 and #2 |  |
| CRS for rate matching (Note 1) | LTE carrier centre subcarrier location |  | Same as NR carrier centre subcarrier location |
|  | LTE carrier BW | MHz | 10 |
|  | Number of antenna ports |  | 4 |
|  | v-shift |  | 0 |
| Note 1: No MBSFN is configured on LTE carrier. Note 2: NR PDCCH data REs and DMRS REs overlapped with LTE CRS are punctured at the transmitter side. |  |  |  |

All new TDD test parameter tables are using ‘Start symbol’ so that any new conclusion on the above will need to be correspondingly applied as well.

The following approach is used in the requirement for PDSCH CRS interference mitigation under NR-LTE coexistence scenario:

… PDCCH configuration; Symbols with PDCCH; Symbol# 2. …

Thus, we prefer to align with the above and propose the following:

Proposal 1: Align the PDCCH configuration parameter and value with clause: 5.2.2.1.18 Minimum requirement for PDSCH CRS interference mitigation under NR-LTE coexistence scenario.

Mismatch 2

Following descriptions are mismatched among different sub-clauses as shown in highlight below (part of):

##### 5.3.2.1.6	Minimum requirements for PDCCH overlapping with LTE CRS

The performance requirements are specified in Table 5.3.2.1.6-2, with the addition of test parameters in Table 5.3.2.1.6- 1. The downlink physical setup is in accordance with Annex C.3.1.

##### 5.3.2.2.6	Minimum requirements for PDCCH overlapping with LTE CRS

The parameters specified in Table 5.3.2.2.6-1 are additional prarameters for requirements with PDCCH overlapping with LTE CRS.

##### 5.3.3.1.5	Minimum requirements for PDCCH overlapping with LTE CRS

For the parameters specified in Table 5.3.3.1-1, the average probability of a missed downlink scheduling grant (Pm-dsg) shall be below the specified value in Table 5.3.3.1.5-1. The downlink physical setup is in accordance with Annex C.3.1.

We prefer to use the existing structure in the spec. to keep the consistency. Thus, we propose the following:

Proposal 2: Align the structure and description with clause 5.3.2.1.1, 5.3.2.1.2 and 5.3.2.1.3

# 3	Summary

In this contribution, we summarized the mismatches existing in the current endorsed draft CRs. Therefore, we propose to discuss and align those mismatches in this meeting. Specifically, we propose the following:

Proposal 1: Align the PDCCH configuration parameter and value with clause: 5.2.2.1.18 Minimum requirement for PDSCH CRS interference mitigation under NR-LTE coexistence scenario.

Proposal 2: Align the structure and description with clause 5.3.2.1.1, 5.3.2.1.2 and 5.3.2.1.3

# References

[1]	R4-2406019: Way forward on [110bis][333] NR_DSS_enh, Ericsson, RAN4 #110bis, Changsha, China