3GPP TSG-RAN WG4#111	R4-2408858

Fukuoka, Japan, May 20th – 24th, 2024

Agenda item:	6.1.1.2

Source: 	Qualcomm France

Title: 	Missing MSD for PC3 CA_n71B BCS4/5

Document for:	Approval

# Introduction

Missing MSD test case for PC3 CA_n71B BCS4/5 is discussed in this contribution.

- Background

BCS4/5 was added to n71B a while ago. After that, UL BW’s >20MHz were added for n71. Some TX impairments hit SCC when larger than 20MHz UL BW’s are used in BCS4/5, so MSD for those cases should be studied. We had a Tdoc in previous meeting [1] for this same topic, however there was an unfortunate bug in the settings resulting in incorrect proposal.

- Discussion

The following has been specified for n71B:

No MSD was specified for BCS 0/1/2. This is natural, because in Intra-band contiguous CA the SCC is assumed to be further from UL than PCC, which is located at normal TX-RX distance. In case SCC were closer to UL than PCC DL then MSD would need to be specified pretty much to every Intra-band combination as then PCC UL-SCC DL would be at smaller than typical TX-RX distance.

For CA_n71B with BCS0/1/2, there are no cases where only CIM3/CIM5 would cause degradation so MSD for SCC is not needed.

For BCS4/5, based on the figure above, possible PCC-SCC BW’s for BCS 4 and 5 totalling in 35MHz aggregated BW with UL >20MHz are 25+10 and 30+5 MHz.

For case PCC-SCC 25-10, the CIM5 slightly hits SCC. For case PCC-SCC 30-5, CIM5 has full hit on SCC so MSD needs to be specified.

We conducted MSD analysis for case 30MHz+5MHz:

We note that this is the first Intra-band Contiguous CA combination with single UL CC for which MSD is be specified. It was a sound addumption several years ago with max 20MHz UL BW that when SCC DL is further away from TX than PCC DL, there would never be any impairments hitting SCC DL. Because this is the first case for 1UL/2DL Intra-band CA, some general text in addition to MSD test point is needed in the specification.

Proposal 1: Add the following MSD test point for n71B:

| CA configuration | SCS (PCC/SCC) (kHz) | Aggregated channel bandwidth (PCC+SCC) | UL PCC allocation (LCRB) | ΔRIBC (dB) | Duplex mode |
| --- | --- | --- | --- | --- | --- |
| CA_n71B | 15/15 | 30MHz + 5MHz | 20 (RBSTART = 0) | 4.5 | FDD |

Proposal 2: Add the following text into 7.3A.2.1:

For specific uplink and downlink test points which are specified in Table 7.3A.2.X-Y and the reference sensitivity power level increased by ΔRIBC. The requirements apply with all downlink carriers active. Unless given by Table 7.3.2-4, the reference sensitivity requirements shall be verified with the network signaling value NS_01 (Table 6.2.3.1-1) configured.

- Conclusion

Missing MSD test case for PC3 CA_n71B BCS4/5 was discussed with the following proposals:

Proposal 1: Add the following MSD test point for PC3 n71B:

| CA configuration | SCS (PCC/SCC) (kHz) | Aggregated channel bandwidth (PCC+SCC) | UL PCC allocation (LCRB) | ΔRIBC (dB) | Duplex mode |
| --- | --- | --- | --- | --- | --- |
| CA_n71B | 15/15 | 30MHz + 5MHz | 20 (RBSTART = 0) | 4.5 | FDD |

Proposal 2: Add the following text into 7.3A.2.1:

For specific uplink and downlink test points which are specified in Table 7.3A.2.X-Y and the reference sensitivity power level increased by ΔRIBC. The requirements apply with all downlink carriers active. Unless given by Table 7.3.2-4, the reference sensitivity requirements shall be verified with the network signaling value NS_01 (Table 6.2.3.1-1) configured.

- Reference

R4-2405449, “Missing MSD for PC3 CA_n71B BCS4/5”, Qualcomm