3GPP TSG-RAN WG4 Meeting #111	R4-2409127

Fukuoka, Japan, May 20 – May 24, 2024

Source:	Nokia, Deutsche Telekom, Orange, NTT DOCOMO, INC., TELECOM ITALIA S.p.A., Telia Company, T-Mobile USA, Verizon, Vodafone

Title:	(NR_RRM_enh-Core) Discussion on Rel 16 no-gap reporting

Agenda item:	4.4

Document for:	Discussion

Introduction

This paper presents a discussion on the identified problem and proposes how to clarify the interruption behavior for “no gap” in Rel -16.

Discussion

Discussed scenarios

In this paper, the following scenarios are discussed:

- Scenario 1: the UE is connected to LTE cell with NR inter-RAT measurement objects

  - In this scenario the UE:

    - supports interRAT-NeedForGapsNR-r16,

    - is connected to an LTE eNB,

    - is configured to perform NR inter-RAT measurements.

- Scenario 2: the UE is connected to NR cell with NR measurement objects

  - In this scenario a UE

    - supports nr-NeedForGap-Reporting-r16 or interFrequencyMeas-Nogap-r16

    - is connected to NR gNB

    - is configured to perform NR measurements.

Interruption for UEs in Scenario 1 and Scenario 2

In the RAN4 #110 meeting the following tentative agreement was captured in [1]:

Issue 1-1-1: Scenario 1, LTE – NR inter-RAT measurement Tentative agreements - interRAT-NeedForGaps-r16=FALSE means that the UE support measurement without gaps - The UE may or may not cause interruption. - if UE causes interruptions when performing measurements without gaps: - Support early implementation of Rel-18 NeedForInterruption: - Optional since R17 - [FFS mandatory report of the UE capability R18 interRAT-NeedForIntrNR-r18 from Rel-18 UE if the UE reports interRAT-NeedForGaps-r16=FALSE] - FFS the UE behavior if the Rel-18 UE does not support interRAT-NeedForIntrNR-r18 Issue 1-1-2: Scenario 2, NR measurements without gaps Tentative agreements - “no-gap” as part of NeedForGapsInfoNR-r16=FALSE means that the UE support measurement without gaps - The UE may or may not cause interruption. - if UE causes interruptions when performing measurements without gaps: - Support early implementation of Rel-18 NeedForInterruption: - Optional since R17 - FFS the UE behavior if the Rel-18 UE does not support NeedForInterruptionNR-r18

As part of this agreement, it was identified that Rel-16 UEs may or may not cause interruptions when supporting Rel-16 measurements without gaps. Since RAN4 specifies minimum requirements for UE, the impact of the bullet above is that interruptions are allowed for UEs supporting NeedForGapsInfoNR-r16 and/or interRAT-NeedForGaps-r16=FALSE.

In order to have a fair deployment of the Rel-16 features for measurements without gaps, this tentative agreement needs to be captured on the specification. If this is not captured, the message for deployment will be unclear. As discussed in our previous papers [2][3] the conditions when interruptions are allowed have been always clearly indicated in the specification. Therefore, if we are now making one exception to allow interruptions for a feature from a previous release, that decision should be clearly indicated in the RRM specification.

For Scenario 1 (LTE – NR inter-RAT measurement): Clarify that a UE supporting interRAT-NeedForGaps-r16=FALSE may cause interruptions with a ratio of X %.

For Scenario 2: If needForInterruptionConfigNR-r18 is not enabled, a UE indicating “no-gap” as part of NeedForGapsInfoNR-r16 may cause interruptions with a ratio of X %.

The interruption ratio can be determined based on the minimum measurement cycle for the existing measurement delay requirements.

The measurement delay is composed by the following stages which apply for intra/interfrequency and interRAT measurements:

- Tidentify_without_index = TPSS/SSS_sync + TSSB_measurement_period

- Tidentify_with_index = TPSS/SSS_sync + TSSB_measurement_period+ TSSB_time_index

From those stages, the following tables include lower bounds, number of samples, and the calculation of the interruption ratio considering 1+1 ms interruption per measurement sample for FR1 and 0.75+0.75 ms interruption per measurement sample for FR2.

Table 1 Interruption ratio calculation for measurements without gaps in FR1

|  | FR1 (from 9.2.5 and 9.3.9 of 38.133) |  |  |
| --- | --- | --- | --- |
|  | Lower bound | # samples | Interruption ratio % |
| TPSS/SSS_sync | 600 ms | 5 | 1,67 |
| TSSB_measurement_period | 200 ms | 5 | 5 |
| TSSB_time_index | 120 ms | 3 | 5 |
| Tidentify_without_index | 800 ms | 10 | 2,5 |
| Tidentify_with_index | 920 ms | 13 | 2,83 |

Table 2 Interruption ratio calculation for measurements without gaps in FR2

|  | FR2 (from 9.2.5 and 9.3.9 of 38.133) |  |  |  |
| --- | --- | --- | --- | --- |
|  | Lower bound | 20 ms * # samples | # samples | Interruption ratio % |
| TPSS/SSS_sync | 600 ms | 600 ms | 24 | 6 |
| TSSB_measurement_period | 200 ms | 480 ms | 24 | 7,5 |
| Tidentify_without_index | 800 ms | 1080 ms | 48 | 6,6 |

Table 3 Interruption ratio calculation for inter-RAT NR measurements without gaps in FR1

|  | interRAT NR (from 8.1.2.4.21.1.1 of 36.133) |  |  |
| --- | --- | --- | --- |
|  | Lower bound | # samples | Interruption ratio % |
| TPSS/SSS_sync | 600 ms | 8 | 2,67 |
| TSSB_measurement_period | 200 ms | 8 | 8 |
| TSSB_time_index | 120 ms | 3 | 5 |
| Tidentify_without_index | 800 ms | 16 | 4 |
| Tidentify_with_index | 920 ms | 19 | 4,13 |

Table 4 Interruption ratio calculation for inter-RAT NR measurements without gaps in FR2

|  | Lower bound | 20 ms * # samples | # samples | Interruption ratio % |
| --- | --- | --- | --- | --- |
| TPSS/SSS_sync | 600 ms | 800 ms | 40 | 7,5 |
| TSSB_measurement_period | 200 ms | 800 ms | 40 | 7,5 |
| TSSB_time_index | 200 ms | 480 ms | 24 | 7,5 |
| Tidentify_without_index | 800 ms | 1080 ms | 80 | 7,5 |
| Tidentify_with_index | 1000 ms | 2080 ms | 104 | 7,5 |

From the calculation above, we can determine the maximum interruption ratio for the considered Rel-16 measurement interruption ratio as 2.5 % and 4% for NR measurement without gaps and interRAT NR measurements without gaps, respectively. The same ratio is increased to 2.83% and 4.13% when inter detection is considered. Considering that calculation on lower bounds of measurement delay, we can assume that a UE that is not collecting more samples than the ones required by 38.133 and 36.133 can perform those measurement with a interruption ratio smaller than 5% for Scenario 1 and 3% for Scenario 2.

For Scenario 1 (LTE – NR inter-RAT measurement): a UE supporting interRAT-NeedForGaps-r16=FALSE may cause interruptions with a ratio of 5 % for FR1 and 8% for FR2.

For Scenario 2: If needForInterruptionConfigNR-r18 is not enabled, a UE indicating “no-gap” as part of NeedForGapsInfoNR-r16 may cause interruptions with a ratio of 3 % in FR1 and 7 % in FR2.

Measurement gap overlapping conditions

As part of the Rel-18 discussion the following was agreed regarding SMTC overlapping with measurement gaps [5]:

Issue 1-1-3: Scaling factor definition; the scaling factor is to scale the configured (SMTC) period value towards the actual UE measurement cycle/period value - Agreements - All NFG measurements with interruptions are carried within the MG(s), when MGs are configured and SMTC partially or fully overlaps with MG(s) - Scaling factor to derive UE measurement period - Use CSSF within gap to scale the configured SMTC period value when MG is configured and SMTC partially or fully overlaps with MG - Use CSSF outside gap to scale the configured SMTC period value when MG is configured and SMTC does not overlap with MG - FFS for scaling factor when MG is not configured

As part of this agreement, it was concluded that then the SMTC partially overlap with a measurement gap, than the UE should perform the measurements within gaps. The same can be used as an approach for mitigation of interruptions from the network side fir Rel-16 measurements without gaps.

For Scenario 1: If the SMTC of the NR interRAT carrier is partially overlapping with measurement gaps, the UE shall perform NR interRAT measurements using the gaps.

For Scenario 2: If the SMTC is partially overlapping with measurement gaps, the UE shall perform the measurements using the gaps.

Early implementation of Rel-18 needForInterruption

While the early implementation is beneficial in clarifying the interruption behaviour, early implementation cannot provide a clear indication of whether the gap-less measurements will cause interruptions for Rel-18 UEs that do not support nr-NeedForInterruptionReport-r18 or interRAT-NeedForInterruptionNR-r18 signalling, since these are optional UE features.

To address this issue, it is imperative that Rel-18 UE causing interruptions when performing gap-less measurements shall support signalling indicating if interruptions are needed. By making this a mandatory requirement, we ensure consistency and eliminate any ambiguity in the UE behaviour when supporting this feature.

Early implementation of nr-NeedForInterruptionReport-r18 and interRAT-NeedForInterruptionNR-r18 has been proposed as solution for ambiguity in Rel-16 requirements.

A Rel-18 UE indicating support of nr-NeedForGap-Reporting-r16 shall also indicate if interruptions are needed.

A Rel-18 UE indicating support of interRAT-NeedForGapsNR-r16 shall also indicate if interruptions are needed. .

No new RRM test cases are defined for a Rel-18 UE supporting signalling in proposal 7 and 8.

Conclusion

The paper has presented analysis on the problems and solutions associated with Rel 16 measurements without gaps that might cause interruptions. As part of the analysis, the following proposals are made:

Proposal 1: For Scenario 1 (LTE – NR inter-RAT measurement): Clarify that a UE supporting interRAT-NeedForGaps-r16=FALSE may cause interruptions with a ratio of X %.

Proposal 2: For Scenario 2: If needForInterruptionConfigNR-r18 is not enabled, a UE indicating “no-gap” as part of NeedForGapsInfoNR-r16 may cause interruptions with a ratio of X %.

Proposal 3: For Scenario 1 (LTE – NR inter-RAT measurement): a UE supporting interRAT-NeedForGaps-r16=FALSE may cause interruptions with a ratio of 5 % for FR1 and 8% for FR2.

Proposal 4: For Scenario 2: If needForInterruptionConfigNR-r18 is not enabled, a UE indicating “no-gap” as part of NeedForGapsInfoNR-r16 may cause interruptions with a ratio of 3 % in FR1 and 7 % in FR2.

Proposal 5: For Scenario 1: If the SMTC of the NR interRAT carrier is partially overlapping with measurement gaps, the UE shall perform NR interRAT measurements using the gaps.

Proposal 6: For Scenario 2: If the SMTC is partially overlapping with measurement gaps, the UE shall perform the measurements using the gaps.

Observation 1: Early implementation of nr-NeedForInterruptionReport-r18 and interRAT-NeedForInterruptionNR-r18 has been proposed as solution for ambiguity in Rel-16 requirements.

Proposal 7: A Rel-18 UE indicating support of nr-NeedForGap-Reporting-r16 shall also indicate if interruptions are needed.

Proposal 8: A Rel-18 UE indicating support of interRAT-NeedForGapsNR-r16 shall also indicate if interruptions are needed. .

Proposal 9: No new RRM test cases are defined for a Rel-18 UE supporting signalling in proposal 7 and 8.

- References

- R4-2403559, WF on Interruption requirements for Rel-16 measurements without gaps, Nokia, Nokia Shanghai Bell

- R4-2402429, (NR_RRM_enh) Discussion on Rel-16 no-gap reporting, Nokia, Nokia Shanghai Bell, BT Plc., Vodafone, Charter Communications Inc., Deutsche Telekom, TELECOM ITALIA S.p.A., Verizon

- R4-2320716, [NR_RRM_enh-Core] Discussion on Rel 16 no-gap reporting, Nokia, Nokia Shanghai Bell, AT&T, BT plc, Vodafone

- R4-2314458, WF on R16 “no-gap” measurements, Nokia, Nokia Shanghai Bell, T-Mobile USA, Vodafone, AT&T, BT plc

- R4-2314324, WF on NR MG enhancements (Part 2), Intel Corporation