# 3GPP TSG-RAN WG4 Meeting # 111	R4-2409329

# Fukuoka, Japan, May 20 – May 24, 2024

Source: 	Huawei, HiSilicon

Title: 	Discussion on Tx requirement for Ka band NTN UE

Agenda Item:	7.16.5.1

Document for:	Approval

Introduction

In the RAN4#110bis meeting, RAN4 reached a lot of agreements for Tx requirements of Ka band VSAT referring to the WF [1].

However, there are three open issues for Tx requirements of Ka band VSAT at least.

Firstly, RAN4 need to discuss the maximum TRP for Ka band NTN VSAT type 2, 3 and 5 referring to the following agreements.

Offline agreement: For maximum TRP: - For type 2,3, 5 with electronic steering antenna (e.g. phase antenna array) - [43dBm] for further ACLR evaluation; if there is coexistence issue identified for 43dBm next meeting, then set 40dBm as maximum TRP power; - The maximum EIRP 76.2 is not changed, the antenna gain for other TRP values could further adjusted;

Secondly, there is one issue identified in the end of last meeting. Referring to the latest endorsed big CR [2], RAN4 need to further discuss how to define the VSAT spherical coverage grid.

Finally, there is an issue raised in the contribution [3] on how to meet the OFF-axis EIRP requirements for NTN VSAT type 2 and 5. The following proposal was submitted in last meeting.

Proposal: MPR for UE type 2 to meet OFF-axis EIRP requirement is: CEIL{14 – 10*LOG10(LCRB*SCS*12/1e6),1}

In this meeting, we’d like to discuss these open issues in NTN VSAT specification.

Discussion

The maximum TRP for Ka band NTN VSAT type 2, 3 and 5

Referring to the latest TR 38.863 [4], the antenna pattern for Ka band NTN VSAT is copied below.

In Rel-18, antenna pattern in section 6.4.1 of TR38.811 is reused for NTN SAN and NTN UE. Use of phased array antenna model needs further discussion. The following normalized antenna gain pattern, corresponding to a typical reflector antenna with a circular aperture, is considered. 1 for θ=0 4J1kasinθπ180kasinθπ1802 for 0<θ≤90° For UE, 4J1kasinπ2kasinπ22 for θ>90° where: - J1(x) is the Bessel function of the first kind and first order with argument ‘x’; - ais the radius of the antenna's circular aperture; - k = 2f/c is the wave number; - f is the frequency of operation; - c is the speed of light in a vacuum and  is the angle measured from the bore sight of the antenna's main beam. Note that ka equals to the number of wavelengths on the circumference of the aperture and is independent of the operating frequency. And the sin() function is in radian.

It’s observed that this is only a normalized antenna gain pattern which is not relative to specific antenna gain. In addition, the NTN UE parameters for system level simulations are cited below.

- 6a.2.2.2 NTN UE parameters NTN UE parameters are given in Table 6a.2.2.2-1. Table 6a.2.2.2-1: NTN UE characteristics for system level simulations Characteristics; Fixed VSAT; L-ESIM. Frequency band; 27GHz UL/17GHz DL; 27GHz UL/17GHz DL. Antenna type and configuration; [TBD]; [TBD]. Polarisation; Circular; Circular. Rx Antenna gain; [TBD]; [TBD]. Antenna temperature; [TBD]; [TBD]. Noise figure1; 2.5dB for GEO [2,5dB and 6dB for LEO]; 2.5dB for GEO [2,5dB and 6dB for LEO]. Tx transmit power; [TBD]; [TBD]. Tx antenna gain; [TBD]; [TBD]. Antenna elevation angle2; 25˚ and 90˚; 25˚ and 90˚. NOTE1: The NF of 4dB is used for calibration. NOTE2: 30 and 90 degree are used for calibration..

It’s observed that the [TBD]s are left for Tx transmit power and Tx antenna gain of NTN VSAT. Currently, RAN4 agreed to specify 76.2dBm EIRP as maximum output level for Ka band NTN VSAT. In my understanding, vendors can trade-off the antenna gain and maximum TRP assuming the maximum EIRP 76.2 is not changed. Thus, if 43dBm TRP is assumed and the maximum EIRP 76.2 is not changed, the coexistence studies will not be changed.

Observation 1: In Rel-18, RAN4 only assumed a normalized antenna gain pattern for NTN VSAT which is not relative to specific antenna gain.

Observation 2: The maximum EIRP 76.2dBm can guarantee the current coexistence outcomes, no matter how vendors trade-off the antenna gain and maximum TRP.

Observation 3: There is no coexistence issue with 43dBm TRP for previous ACLR evaluation under the condition that the maximum EIRP 76.2 is not changed.

Proposal 1: To specify 43dBm TRP for Ka band NTN VSAT type 2, 3, 5.

VSAT spherical coverage grid

For Ka band NTN VSAT, only partial upper Hemisphere is used for coverage grid. Thus, it’s proposed to use upper hemispherical coverage grid as term for the definition.

Proposal 2: To specify “upper hemispherical coverage grid” as term for the definition.

Figure 1 The upper hemispherical coverage grid for Ka band NTN VSAT

Figure 1 illustrate the upper hemispherical coverage grid for Ka band NTN VSAT. The normal direction for the antenna panel is toward Z+ axis. The green arrow line is the specific beam direction.

represent Horizontal coverage range. Generally, it can be 0~360 deg.

represent Vertical coverage range between the normal direction of antenna panel and beam direction. Generally, it can be 0~30, 0~45 or 0~60 deg. and so on. is the complementary angle of elevation angle.

Proposal 3: To consider figure 1 as upper hemispherical coverage grid for Ka band NTN VSAT. Additionally, Horizontal coverage range  and Vertical coverage range can be declared by manufacturers.

OFF-axis EIRP requirements for NTN VSAT type 2 and 5

Currently, RAN4 assume 50MHz configurations to derive the minimum EIRP requirements for Ka band NTN VSAT. 61dBm was specified for NTN VSAT type 3 and 70dBm was specified for NTN VSAT others types which can access to GSO network. In any case, NTN VSAT can be configured with narrower RBs, which means NTN VSAT can scale the EIRP lower than the minimum EIRP requirements. In order to address the issues raised in contribution [3], AMPR may be used to meet the additional OFF-axis EIRP requirements. However, there are two options for further discussion at least.

Proposal 4: RAN4 can further discuss the following two options by leveraging the EIRP scaling factor for narrow RB allocation.

Option 1: It’s allowed to scale the EIRP based on the allocated narrower RBs.

- 9.2.3 Configured transmitted power The NTN VSAT can configure its maximum output power. The configured NTN VSAT maximum output power PCMAX,f,c for carrier f of a serving cell c is defined as that available to the reference point of a given transmitter branch that corresponds to the reference point of the higher-layer filtered RSRP measurement as specified in TS 38.215 [11]. The configured NTN VSAT maximum output power PCMAX,f,c for carrier f of a serving cell c shall be set such that the corresponding measured peak EIRP PUMAX,f,c is within the following bounds (PUEType – Scaling factor) - TEIRP≤ PUMAX,f,c ≤ EIRPmax + TEIRP with PUEType is the NTN VSAT minimum peak EIRP as specified in sub-clause 9.2.1, EIRPmax is the applicable maximum EIRP as specified in sub-clause 9.2.1 and TEIRP is equal to 3.4 dB. The requirement is verified with the test metric of EIRP (Link=TX beam peak direction, Meas=Link angle). Scaling factor is allowed for the EIRP reduction when LCRB is configured less than/equal to 66 RBs for 60kHz SCS or 32 RBs for 120kHz SCS. The scaling factor should be calculated below. Scaling factor = 10log10((LCRB x SCS)/(66*60kHz)) while the corresponding measured total radiated power PTMAX,f,c is bounded by the maximum TRP limit TRPMAX for NTN VSAT defined in sub-clause 9.2.1: PTMAX,f,c ≤ TRPMAX + TTRP where, TTRP is specified as 3 dB. The PTMAX,f,c requirement is verified with the test metrics of TRP (Link=TX beam peak direction, Meas=TRP grid) in beam locked mode.

Option 2: It’s allowed to additionally reduce the EIRP to meet OFF-axis EIRP requirements for NTN VSAT type 2 and 5.

- 9.2.3 Configured transmitted power The NTN VSAT can configure its maximum output power. The configured NTN VSAT maximum output power PCMAX,f,c for carrier f of a serving cell c is defined as that available to the reference point of a given transmitter branch that corresponds to the reference point of the higher-layer filtered RSRP measurement as specified in TS 38.215 [11]. The configured NTN VSAT maximum output power PCMAX,f,c for carrier f of a serving cell c shall be set such that the corresponding measured peak EIRP PUMAX,f,c is within the following bounds (PUEType – A_Reduction) - TEIRP≤ PUMAX,f,c ≤ EIRPmax + TEIRP with PUEType is the NTN VSAT minimum peak EIRP as specified in sub-clause 9.2.1, EIRPmax is the applicable maximum EIRP as specified in sub-clause 9.2.1 and TEIRP is equal to 3.4 dB. The requirement is verified with the test metric of EIRP (Link=TX beam peak direction, Meas=Link angle). A_Reduction is allowed for the additional EIRP reduction to meet regional OFF-axis EIRP requirements for NTN VSAT type 2 and 5, when LCRB is configured less than/equal to 66 RBs for 60kHz SCS or 32 RBs for 120kHz SCS. The A_Reduction should be calculated below. A_Reduction = 10log10((LCRB x SCS)/(66*60kHz)) while the corresponding measured total radiated power PTMAX,f,c is bounded by the maximum TRP limit TRPMAX for NTN VSAT defined in sub-clause 9.2.1: PTMAX,f,c ≤ TRPMAX + TTRP where, TTRP is specified as 3 dB. The PTMAX,f,c requirement is verified with the test metrics of TRP (Link=TX beam peak direction, Meas=TRP grid) in beam locked mode.

Summary

Observation 1: In Rel-18, RAN4 only assumed a normalized antenna gain pattern for NTN VSAT which is not relative to specific antenna gain.

Observation 2: The maximum EIRP 76.2dBm can guarantee the current coexistence outcomes, no matter how vendors trade-off the antenna gain and maximum TRP.

Observation 3: There is no coexistence issue with 43dBm TRP for previous ACLR evaluation under the condition that the maximum EIRP 76.2 is not changed.

Proposal 1: To specify 43dBm TRP for Ka band NTN VSAT type 2, 3, 5.

Proposal 2: To specify “upper hemispherical coverage grid” as term for the definition.

Proposal 3: To consider the following as upper hemispherical coverage grid for Ka band NTN VSAT. Additionally, Horizontal coverage range  and Vertical coverage range can be declared by manufacturers.

Proposal 4: RAN4 can further discuss the following two options by leveraging the EIRP scaling factor for narrow RB allocation.

Option 1: It’s allowed to scale the EIRP based on the allocated narrower RBs.

- 9.2.3 Configured transmitted power The NTN VSAT can configure its maximum output power. The configured NTN VSAT maximum output power PCMAX,f,c for carrier f of a serving cell c is defined as that available to the reference point of a given transmitter branch that corresponds to the reference point of the higher-layer filtered RSRP measurement as specified in TS 38.215 [11]. The configured NTN VSAT maximum output power PCMAX,f,c for carrier f of a serving cell c shall be set such that the corresponding measured peak EIRP PUMAX,f,c is within the following bounds (PUEType – Scaling factor) - TEIRP≤ PUMAX,f,c ≤ EIRPmax + TEIRP with PUEType is the NTN VSAT minimum peak EIRP as specified in sub-clause 9.2.1, EIRPmax is the applicable maximum EIRP as specified in sub-clause 9.2.1 and TEIRP is equal to 3.4 dB. The requirement is verified with the test metric of EIRP (Link=TX beam peak direction, Meas=Link angle). Scaling factor is allowed for the EIRP reduction when LCRB is configured less than/equal to 66 RBs for 60kHz SCS or 32 RBs for 120kHz SCS. The scaling factor should be calculated below. Scaling factor = 10log10((LCRB x SCS)/(66*60kHz)) while the corresponding measured total radiated power PTMAX,f,c is bounded by the maximum TRP limit TRPMAX for NTN VSAT defined in sub-clause 9.2.1: PTMAX,f,c ≤ TRPMAX + TTRP where, TTRP is specified as 3 dB. The PTMAX,f,c requirement is verified with the test metrics of TRP (Link=TX beam peak direction, Meas=TRP grid) in beam locked mode.

Option 2: It’s allowed to additionally reduce the EIRP to meet OFF-axis EIRP requirements for NTN VSAT type 2 and 5.

- 9.2.3 Configured transmitted power The NTN VSAT can configure its maximum output power. The configured NTN VSAT maximum output power PCMAX,f,c for carrier f of a serving cell c is defined as that available to the reference point of a given transmitter branch that corresponds to the reference point of the higher-layer filtered RSRP measurement as specified in TS 38.215 [11]. The configured NTN VSAT maximum output power PCMAX,f,c for carrier f of a serving cell c shall be set such that the corresponding measured peak EIRP PUMAX,f,c is within the following bounds (PUEType – A_Reduction) - TEIRP≤ PUMAX,f,c ≤ EIRPmax + TEIRP with PUEType is the NTN VSAT minimum peak EIRP as specified in sub-clause 9.2.1, EIRPmax is the applicable maximum EIRP as specified in sub-clause 9.2.1 and TEIRP is equal to 3.4 dB. The requirement is verified with the test metric of EIRP (Link=TX beam peak direction, Meas=Link angle). A_Reduction is allowed for the additional EIRP reduction to meet regional OFF-axis EIRP requirements for NTN VSAT type 2 and 5, when LCRB is configured less than/equal to 66 RBs for 60kHz SCS or 32 RBs for 120kHz SCS. The A_Reduction should be calculated below. A_Reduction = 10log10((LCRB x SCS)/(66*60kHz)) while the corresponding measured total radiated power PTMAX,f,c is bounded by the maximum TRP limit TRPMAX for NTN VSAT defined in sub-clause 9.2.1: PTMAX,f,c ≤ TRPMAX + TTRP where, TTRP is specified as 3 dB. The PTMAX,f,c requirement is verified with the test metrics of TRP (Link=TX beam peak direction, Meas=TRP grid) in beam locked mode.

- References

[1] R4-2406609, WF on Rel-18 NTN UE RF requirements, ZTE, Samsung

[2] R4-2405085, draft Big CR to TS 38.101-5, Samsung

[3] R4-2405889, VSAT type 2 and 5 MPR to meet OFF-axis EIRP, Qualcomm Incorporated

[4] 3GPP TR 38.863, V18.1.0