# 3GPP TSG-RAN WG4 Meeting #111	R4-2409405

# Fukuoka City, Fukuoka , Japan, 20th – 24th May, 2024

Source: 	Huawei, HiSilicon

Title: 	Simulation assumptions for range 14.8-15.35 GHz

Agenda Item:	10.3.4.1

Document for:	Discussion

1 Introduction

Working Party 5D is seeking information on IMT parameters for the frequency bands 4 400-4 800 MHz, 7 125-8 400 MHz and 14.8-15.35 GHz [1]. In the last plenary (RAN#103) the Study on IMT parameters for 4400 to 4800 MHz, 7125 to 8400 MHz and 14800 to 15350 MHz was approved [2].

–	4 400-4 800 MHz;

–	7 125-8 400 MHz; and

–	14.8-15.35 GHz.

In the last RAN4 meeting, the way forward for the 7125-8400 MHz and 14.8-15.35 GHz was agreed in [3]

In this contribution we provide our views on the open issues from the WF [3].

2 Discussion

2.1	Deployment scenarios

The WF had the following options:

- Option 1: Follow 38.921

  - Indoor 20m ISD

  - [0.2 - 0.45]km Urban

  - [<=0.9]km sub-urban

- Option 2: 38.803 FR2 assumptions

  - Indoor

  - Dense Urban

  - Urban macro

- Other options such as smaller ISD not precluded depending on antenna array assumptions

The mid-band 7-15 GHz is expected as golden centimetre bands for future cellular networks. Urban Macro scenarios are the ones to be studied. And based on the previous experience, the ACIR required for Macro cells are more critical than that for indoor cells. It is proposed to follow TR 38.921 to study urban macro scenarios as in Table 2.1-1.

Table 2.1-1: Single operator layout for urban macro

| Parameters |  | Values | Remark |
| --- | --- | --- | --- |
| Network layout |  | hexagonal grid, 19 macro sites, 3 sectors per site with wrap around |  |
| Inter-site distance |  | 0.45 km (urban) (Note 1) 0.9 km (suburban) | Based on cell range: 0.3 km (urban) 0.6 km (suburban) |
| BS antenna height |  | 20 m (urban) 25 m (suburban) |  |
| UE location | Outdoor/indoor | Outdoor and indoor |  |
|  | Indoor UE ratio | 20% |  |
|  | Low/high penetration loss ratio | 50% low loss, 50% high loss |  |
|  | LOS/NLOS | LOS and NLOS |  |
|  | UE antenna height | Same as 3D-UMa in TR 36.873 |  |
| UE distribution (horizontal) |  | Uniform |  |
| Minimum BS - UE distance (2D) |  | 35 m |  |
| Channel model |  | UMa |  |
| Shadowing correlation |  | Between cells: 1.0 Between sites: 0.5 |  |

Proposal 1: It is proposed to follow TR 38.921 to study urban macro scenarios for the frequency range.

2.2	BS array size

The WF had the following options:

Issue 3-4 Initial views on BS array size

- Option 1: 1024 elements

- Option 2: 2048 elements

- Option 3: Other sizes

Issue 3-5 Initial views on BS sub-array architecture

  - Option 1: No sub-arrays

  - Option 2: Sub array size 2

  - Option 3: Sub array size 3

  - Option 4: Sub-array size 4

  - Option 5: Sub-array size 6

  - Option 6: Sub-array size 8

  - Option 7: Sub-array size 12

The path loss increases by approximately 20 log f. larger path loss at higher frequencies mean that higher antenna gain is important to compensate the gap if comparable cell size are maintained. As shown in the table for examples of antenna array for different frequencies. The total radiated antenna elements is 192 For 2.6 GHz/3.5 GHz and 768 (4 times of 3.5GHz) for 7 GHz/8 GHz. It is expected more than 3k antenna elements may be needed.

Table 2.2-1: Examples of antenna array

| Frequency | 2.6 GHz/3.5 GHz | 7 GHz/8 GHz | 15GHz example 1 | 15GHz example 2 |
| --- | --- | --- | --- | --- |
| Antenna configuration | 3x4x8x2 | 3x8x16x2 | 16x4x32x2 | 8x8x32x2 |
| NO. of PAs | 64T | 256T | 256T | 512T |
| NO. of AEs | 192 | 768 | 4096 | 4096 |
| Sub-array size | 3 | 3 | 16 | 8 |

Proposal 2: it is proposed to consider 4096 antenna elements for Macro BS

There are some available options to implement larger the antenna arrays. One way is to increase the TRX port, e.g. 256 or 512 which can maximize the MIMO performance and the drawback is that the number of DAC for DL and ADC for UL need be increased which results in increased power consumption and cost. The other is to use sub-array with larger number of elements. The advantage is the simplicity and the drawback is that the narrower beamwidth may impact the performance. In the real design the two ways should be compromised. TR 38.820 predicts up to 128 element antennas including the polarization diversity do 8x8 arrays, as such it seems that predicted active TRX is 64 -128 driving those antennas. For 7 GHz/8 GHz the active TRX may be 128 or 256, and for 15 GHz the potential is 256 or 512. As such sub-array size could be 8 or 16.

Figure 2.2-1: Example RDN mappings

Proposal 3: sub-array size 16 is also be considered for Macro BS

3	Summary

This paper provides some discussion on simulation assumptions for range 14.8-15.35 GHz.

Proposal 1: It is proposed to follow TR 38.921 to study urban macro scenarios for the frequency range.

Proposal 2: it is proposed to consider 4096 antenna elements for Macro BS

Proposal 3: sub-array size 16 is also be considered for Macro BS

References

[1] R4-2400333, “Parameters of terrestrial component of IMT for sharing and compatibility studies in the frequency bands 4 400-4 800 MHz, 7 125-8 400 MHz and 14.8-15.35 GHz” ITU-R WP 5D

[2] RP-240787,	New SI proposal: Study on IMT parameters for 4400 to 4800 MHz, 7125 to 8400 MHz and 14800 to 15350 MHz, 	Ericsson

[3] R4-246615, WF on IMT parameters for other frequency ranges, CATT