3GPP TSG-RAN4 Meeting # 111	R4-2409436

# Fukuoka, Japan, 20th May - 24th May 2024

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 37.941 | CR | 0052 | rev | - | Current version: | 17.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | x | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | (NR_ext_to_71GHz-Perf) CR to 37.941: 71 GHz Extension, update on missing Rx MU |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Keysight Technologies UK Ltd |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_ext_to_71GHz-Perf |  |  |  |  |  | Date: |  |  | 2024-05-13 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | There was endorsed draft CR, R4-2309857 which updated Rx MU tables in RAN4#107 (Incheon, May 2023), is not implemented in TR37.941 (only partly by other CR). Updates described below (in summary) is not implemented. There was update on EIS table in 10.2.3.4, and related MU/TT in 10.2.7 and 10.2.8 which was done by other CR. This CR updates rest of update. |  |  |  |  |  |  |  |  |
| Summary of change: |  | 10.2.3.4, comment related C1-11 is missing Modification originally in EIS table in 10.2.3 and MU/TT for EIS in 10.2.7 and 10.2.8 were done by other CR (in different format) so that these skipped. Table 10.5.4-3 MU table for FR2 to include FR2-2 values and clarify on frequency range Table 10.6.4-2 MU table for FR2 to include FR2-2 values and clarify on frequency range Table 14.2.4-1 MU table for FR2 in include FR2-2 values and clarify on frequency range |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | FR2-2 MU value derivation for some Rx testing missing |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 10.2.3.4, 10.5.4, 10.6.4, 14.2.4 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  |  | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  |  | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  |  | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

#### 10.2.3.4	MU value derivation, FR2

Table 10.2.3.4-1 captures derivation of the expanded measurement uncertainty values for OTA sensitivity measurements in CATR (Normal test conditions, FR2).

For FR2-2, UID C1-11 is used for uncertainty of RF signal generator. Power meter monitoring and generated signal level control is required for FR2-2 signal generation for better uncertainty.

Table 10.2.3.4-1: CATR MU value derivation for OTA sensitivity measurement, FR2

| UID | Uncertainty source | Uncertainty value |  |  | Distribution of the | Divisor based on | ci | Standard uncertainty ui (dB) |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | 24.25 < f < 29.5 GHz | 37 < f < 43.5 GHz | 52.6 < f ≤ 71 GHz | probability | distribution shape |  | 24.25 < f < 29.5 GHz | 37 < f < 43.5 GHz | 52.6 < f ≤ 71 GHz |
| Stage 2: BS measurement |  |  |  |  |  |  |  |  |  |  |
| B2-1a | Misalignment and pointing error of BS | 0.2 | 0.2 | 0.2 | Exp. normal | 2.00 | 1 | 0.10 | 0.10 | 0.10 |
| B2-2 | Standing wave between BS and test range antenna | 0.21 | 0.21 | 0.21 | U-shaped | 1.41 | 1 | 0.15 | 0.15 | 0.15 |
| C1-2 | Uncertainty of the RF signal generator | 0.9 | 0.9 |  | Gaussian | 1.00 | 1 | 0.90 | 0.90 |  |
| B2-3 | RF leakage & dynamic range, test range antenna cable connector terminated. | 0.01 | 0.01 | 0.01 | Gaussian | 1.00 | 1 | 0.01 | 0.01 | 0.01 |
| B2-4a | QZ ripple experienced by BS | 0.4 | 0.4 | 0.4 | Gaussian | 1.00 | 1 | 0.40 | 0.40 | 0.40 |
| B2-9 | Miscellaneous uncertainty | 0 | 0 | 0 | Gaussian | 1.00 | 1 | 0.00 | 0.00 | 0.00 |
| C1-11 | Uncertainty of the RF signal generator with power monitoring and controling by power sensor |  |  | 0.98 | Gaussian | 1.00 | 1 |  |  | 0.98 |
| Stage 1: Calibration measurement |  |  |  |  |  |  |  |  |  |  |
| C1-3 | Uncertainty of the network analyzer | 0.3 | 0.3 | 0.85 | Gaussian | 1.00 | 1 | 0.30 | 0.30 | 0.85 |
| B2-5 | Mismatch of transmit chain (i.e. between transmitting measurement antenna and BS) | 0.43 | 0.57 | 0.85 | U-shaped | 1.41 | 1 | 0.30 | 0.40 | 0.60 |
| B2-6 | Insertion loss of transmitter chain | 0.12 | 0.12 | 0.12 | Rectangular | 1.73 | 1 | 0.07 | 0.07 | 0.07 |
| B2-7 | RF leakage (SGH connector terminated & test range antenna connector cable terminated) | 0.01 | 0.01 | 0.01 | Gaussian | 1.00 | 1 | 0.01 | 0.01 | 0.01 |
| B2-8 | Influence of the calibration antenna feed cable | 0.21 | 0.29 | 0.29 | U-shaped | 1.41 | 1 | 0.15 | 0.21 | 0.21 |
| C1-4 | Uncertainty of the absolute gain of the reference antenna | 0.52 | 0.52 | 0.52 | Rectangular | 1.73 | 1 | 0.30 | 0.30 | 0.30 |
| B2-11 | Misalignment positioning system | 0 | 0 | 0 | Exp. normal | 2.00 | 1 | 0.00 | 0.00 | 0.00 |
| B2-4b | QZ ripple experienced by calibration antenna | 0.1 | 0.1 | 0.1 | Gaussian | 1.00 | 1 | 0.10 | 0.10 | 0.10 |
| B2-10 | Rotary joints | 0 | 0 | 0 | U-shaped | 1.41 | 1 | 0.00 | 0.00 | 0.00 |
| B2-1b | Misalignment and pointing error of calibration antenna | 0 | 0 | 0 | Exp. normal | 2.00 | 1 | 0.00 | 0.00 | 0.00 |
| B2-12 | Standing wave between SGH and test range antenna | 0.09 | 0.09 | 0.09 | U-shaped | 1.41 | 1 | 0.06 | 0.06 | 0.06 |
| B2-13 | Switching uncertainty | 0.1 | 0.1 | 0.1 | Rectangular | 1.73 | 1 | 0.06 | 0.06 | 0.06 |
| Combined standard uncertainty (1σ) (dB) |  |  |  |  |  |  |  | 1.15 | 1.19 | 1.55 |
| Expanded uncertainty (1.96σ – confidence interval of 95 %) (dB) |  |  |  |  |  |  |  | 2.25 | 2.33 | 3.03 |

<< Unchanged clauses skipped >>

### 10.5.4	Maximum accepted test system uncertainty

MU can be calculated as follows:

With

And

The MU for 4.2 GHz < f ≤ 6 GHz for each receiver directional requirement can be calculated as shown in tables 10.5.4-1 to 10.5.4-3 below.

Table 10.5.4-1: MU for adjacent channel selectivity, narrowband blocking, in-channel selectivity

| Test System Uncertainty | Standard uncertainty ui (dB) |  |  |  |
| --- | --- | --- | --- | --- |
|  | IAC |  | CATR |  |
|  | 3 GHz < f ≤ 4.2 GHz | 4.2 GHz < f ≤ 6 GHz | 3 GHz < f ≤ 4.2 GHz | 4.2 GHz < f ≤ 6 GHz |
| MUconductedwanted (Wanted signal level error) | 1 | 1.30 | 1 | 1.30 |
| MUconductedint (Interferer signal level error) | 1 | 1.30 | 1 | 1.30 |
| MUEIS (Combined standard uncertainty) | 0.64 | 0.77 | 0.71 | 0.83 |
| MUTestEquipment (Uncertainty of the RF signal generator) | 0.46 | 0.58 | 0.46 | 0.58 |
| MUmatching (Impedance mismatch in the transmitting chain) | 0.16 | 0.28 | 0.15 | 0.28 |
| ACLReffect (Impact of interferer leakage) | 0.4 | 0.4 | 0.4 | 0.4 |
| Combined standard uncertainty (1σ) | 1.02 | 1.19 | 1.08 | 1.24 |
| Expanded uncertainty (1.96σ - confidence interval of 95 %) | 2.00 | 2.33 | 2.12 | 2.43 |

Table 10.5.4-2: MU for in-band blocking

| Test System Uncertainty | Standard uncertainty ui (dB) |  |  |  |
| --- | --- | --- | --- | --- |
|  | IAC |  | CATR |  |
|  | 3 GHz < f ≤ 4.2 GHz | 4.2 GHz < f ≤ 6 GHz | 3 GHz < f ≤ 4.2 GHz | 4.2 GHz < f ≤ 6 GHz |
| MUconductedwanted (Wanted signal level error) | 1 | 1.30 | 1 | 1.30 |
| MUconductedint (Interferer signal level error) | 1.2 | 1.46 | 1.2 | 1.46 |
| MUEIS (Combined standard uncertainty) | 0.64 | 0.77 | 0.71 | 0.83 |
| MUTestEquipment (Uncertainty of the RF signal generator) | 0.46 | 0.58 | 0.46 | 0.58 |
| MUmatching (Impedance mismatch in the transmitting chain) | 0.16 | 0.28 | 0.15 | 0.28 |
| ACLReffect (Impact of interferer leakage) | 0.4 | 0.4 | 0.4 | 0.4 |
| Combined standard uncertainty (1σ) | 1.09 | 1.25 | 1.14 | 1.29 |
| Expanded uncertainty (1.96σ - confidence interval of 95 %) | 2.13 | 2.44 | 2.24 | 2.54 |

Substituting the variables above into the formula, the MU for each receiver directional requirement can be calculated as shown in table 10.5.4-3 below.

Table 10.5.4-3: MU for adjacent channel selectivity, in-band blocking, in-channel selectivity

| Test System Uncertainty | Standard uncertainty ui (dB) |  |  |
| --- | --- | --- | --- |
|  | 24.25 GHz < f ≤ 29.5 GHz | 37 GHz < f ≤ 43.5 GHz | 52.6 GHz < f ≤ 71 GHz |
| MUEIS (Expanded uncertainty) | 2.4 | 2.4 | 3.0 |
| MUTestEquipment (Uncertainty of the RF signal generator) | 0.9 | 0.9 | 1.0 (note) |
| MUPA (Uncertainty due to use of PA) | 0.2 | 0.2 | 0.0 (note) |
| ACLReffect (Impact of interferer leakage) | 0.4 | 0.4 | 0.4 |
| Combined standard uncertainty (1σ) | 1.74 | 1.74 | 2.03 |
| Expanded uncertainty (1.96σ - confidence interval of 95 %) | 3.40 | 3.40 | 3.99 |
| NOTE: The MUTestEquipment value for frequency range 52.6 GHz to 71 GHz is from UID C1-11 which is to use power meter and sensor for monitoring and controlling generated signal lelvel. Power monitoring and signal level control shall also be done on PA output when PA used, therefore 0.0 is used for MUPA in this frequency range. |  |  |  |

Maximum test system uncertainties derivation methodology was described in clause 5.1. The maximum accepted test system uncertainty values was derived based on test system specific values.

According to the methodology referred above, the common maximum accepted test system uncertainty values for OTA ACS, general blocking and narrowband blocking test can be derived from values captured in table 10.5.4-1, derived based on the expanded uncertainty ue (1.96σ - confidence interval of 95 %) values. The common maximum accepted test system uncertainty values are applicable for all test methods addressing OTA ACS, general blocking and narrowband blocking test requirements.

<< Unchanged clauses skipped >>

### 10.6.4	Maximum accepted test system uncertainty

MU can be calculated as follows:

With

And

Table 10.6.4-1: MU for receiver intermodulation

| Test System Uncertainty | Standard uncertainty ui (dB) |  |  |  |
| --- | --- | --- | --- | --- |
|  | IAC |  | CATR |  |
|  | 3 GHz < f ≤ 4.2 GHz | 4.2 GHz < f ≤ 6 GHz | 3 GHz < f ≤ 4.2 GHz | 4.2 GHz < f ≤ 6 GHz |
| MUconductedwanted (Wanted signal level error) | 1 | 1.30 | 1 | 1.30 |
| MUconductedmodint (Modulated interferer signal level error) | 1 | 1.30 | 1 | 1.30 |
| MUconductedCWint (CW interferer signal level error) | 0.7 | 1.08 | 0.7 | 1.09 |
| MUEIS (Combined standard uncertainty) | 0.64 | 0.77 | 0.71 | 0.83 |
| MUTestEquipment (Uncertainty of the RF signal generator) | 0.46 | 0.58 | 0.46 | 0.58 |
| MUmatching (Impedance mismatch in the transmitting chain) | 0.16 | 0.28 | 0.15 | 0.28 |
| ACLReffect (Impact of interferer leakage) | 0.4 | 0.4 | 0.4 | 0.4 |
| Combined standard uncertainty (1σ) | 1.24 | 1.57 | 1.32 | 1.62 |
| Expanded uncertainty (1.96σ - confidence interval of 95 %) | 2.43 | 3.09 | 2.60 | 3.17 |

Substituting the variables above into the formula, the MU for each receiver directional requirement can be calculated as shown in table below.

Maximum test system uncertainties derivation methodology was described in clause 5.1. The maximum accepted test system uncertainty values was derived based on test system specific values.

According to the methodology referred above, the common maximum accepted test system uncertainty values for OTA RX IMD test can be derived from values captured in table 10.6.4-1, derived based on the expanded uncertainty ue (1.96σ - confidence interval of 95 %) values. The common maximum accepted test system uncertainty values are applicable for all test methods addressing OTA RX IMD test requirements.

Table 10.6.4-2: MU for receiver intermodulation

| Test System Uncertainty | Standard uncertainty ui (dB) |  |  |
| --- | --- | --- | --- |
|  | 24.25 GHz < f ≤ 29.5 GHz | 37 GHz < f ≤ 43.5 GHz | 52.6 GHz < f ≤ 71 GHz |
| MUEIS (Expanded uncertainty) | 2.4 | 2.4 | 3.0 |
| MUTestEquipment (Uncertainty of the RF signal generator) | 0.9 | 0.9 | 1.0 (note) |
| MUCwint (CW interferer signal level error) | 0.9 | 0.9 | 1.0 (note) |
| MUPA (Uncertainty due to use of PA) | 0.2 | 0.2 | 0.0 (note) |
| ACLReffect (Impact of interferer leakage) | 0.4 | 0.4 | 0.4 |
| Combined standard uncertainty (1σ) | 1.99 | 1.99 | 2.28 |
| Expanded uncertainty (1.96σ – confidence interval of 95 %) | 3.90 | 3.90 | 4.47 |
| NOTE: The MUTestEquipment and MUCWint value for frequency range 52.6 GHz to 71 GHz is from UID C1-11 which is to use power meter and sensor for monitoring and controlling generated signal lelvel. Power monitoring and signal level control shall also be done on PA output when PA used, therefore 0.0 is used for MUPA in this frequency range. |  |  |  |

<< Unchanged clauses skipped >>

### 14.2.4	MU value derivation, FR2

It has been agreed that the MU for the out-of-band blocking requirement can be calculated as follows:

MUOOBB= MUEIS2+MUOOBint2+MUPA2   + Broadband noise effect

With

And

Substituting the variables above into the formula, the MU in FR2 for the out-of-band blocking requirement can be calculated as 3.6 dB, as shown in table 14.2.4-1 below.

Table 14.2.4-1: MU for out-of-band blocking, FR2

| Test System Uncertainty | Standard uncertainty ui (dB) |  |
| --- | --- | --- |
|  | 24.25 GHz< f ≤ 43.5 GHz | 52.6 GHz< f ≤ 71 GHz |
| MUEIS (Expanded uncertainty) | 2.4 | 3.0 |
| MUOOBint (Additional uncertainty for the OOB interferer signal) | 1.1 | 0.98 (note) |
| MUPA (Uncertainty due to use of PA) | 0.2 | 0.0 (note) |
| Broadband noise effect (Impact of interferer broadband noise) | 0.4 | 0.4 |
| Combined standard uncertainty (1σ) | 1.86 | 2.03 |
| Expanded uncertainty (1.96σ - confidence interval of 95 %) | 3.6 | 4.0 |
| NOTE; The MUOOBint value for frequency range 52.6 GHz to 71 GHz is from UID C1-11 which is to use power meter and sensor for monitoring and controlling generated signal lelvel. Power monitoring and signal level control shall also be done on PA output when PA used, therefore 0.0 is used for MUPA in this frequency range.value |  |  |

<< Unchanged clauses skipped >>