3GPP TSG-RAN WG4 Meeting #111             	R4-2409482

Fukuoka, Japan, 20th-24th May, 2024

Agenda item:	7.16.8.1

Source:	Samsung

Title:	Discussion and simulation results for NTN enhancement

Document for:	Discussion

- Introduction

In the last RAN4 meeting, test setup of demodulation requirements for NTN enhancement was discussed.  The related agreement was captured in the WF as following [1]

- General - Channel model - 1200Hz - SAN demodulation requirement - Test setup for normal PUSCH with CP-OFDM for above 10 GHz bands - MCS - MCS 2 and [MCS 12] - PTRS configuration - Not configure PTRS for test with MCS 2 and [MCS12] - Test setup for PUCCH for above 10GHz bands - UCI info - The UCI information can include both ACK/NACK and CSI part 1 for specifying PUCCH UCI BLER performance requirement. - Define PUCCH format 3/4 requirements with additional DMRS configuration both enabled and disabled. - Reuse the applicability of requirements for different configurations in section 8.1.2.3.4 of TS 38.181 - Test setup for PUCCH with DM-RS bunding for FR1 UL coverage enhancement - RB assignment - 6RBs in the middle of the test bandwidth for both 15KHz and 30KHz - PUSCH aggregation factor - N4 for 15KHz SCS - N8 for 30KHz SCS - PUSCH-TimeDomainWindowlength - 4 for 15 KHz SCS - 8 for 30 KHz SCS

In this contribution, the view on the remaining issue of test setup for demodulation requirement for NTN enhancement. Meanwhile, the ideal and impairment results are provided for requirement derivation

# 2	Test setup of BS demodulation requirement

In this part, the remaining issue of test setup for SAN demodulation requirement is discussed based on the test scope agreed in the previous meeting.

MCS

Considering the link budget limitation of NTN above 10GHz, the excepted SNR is around 10dB. Therefore, in the last meeting, the low MCS level as MCS 12 compared to MCS 16 is selected to further check the feasibility for both 1T2Rx and 1T1Rx antenna configuration. Based on our results, the SNR targeting 70% TP for MCS 12 is around 7dB, with considering the impairment margin, it shall be feasible to define PUSCH requirement with MCS 12.

Observation 1: The SNR targeting 70% TP for MCS 12 is around 7dB for 1T1R, with considering the impairment margin, it shall be feasible to define PUSCH requirement with MCS 12

Proposal 1: MCS 12 can be considered to specific the PUSCH requirement for NTN enhancement.

Test Metric

In the previous meeting, RAN4 has agreed to introduce the requirement of PUSCH requirement with repetition mapping type A for NTN enhancement above 10GHz. Regarding the test metric, the Res.BLER 1% was used for PUSCH requirement repetition mapping type A for TN scenario and NTN scenario in S-band. Since the repetition is mainly targeting ultra-high reliability. To keep the consistence, we prefer to change the test metric in the [] for specifying the requirement of PUSCH requirement with repetition mapping type A for above 10GHz.

Proposal 2: Update the test metric of PUSCH requirement with repetition mapping type A for above 10GHz as BLER% 1, where BLER is defined as residual BLER; i.e., ratio of incorrectly received transport blocks / sent transport blocks, independently of the number HARQ transmission(s) for each transport block.

# 3	Simulation Results

In this section, the ideal and impairment simulation results based agreed test parameters for SAN demodulation requirement were provided.

## 3.1	PUSCH requirement

Table 1: ideal and impairment simulation results for PUSCH requirement with CP-OFDM

| Test case | waveform | SCS& BW | MCS | Channel | Mapping Type | Length Of symbol | Number of Tx and Rx | PTRS | Number of DMRS | SNR@ 70 (ideal) | SNR@70 (impairment) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | CP-OFDM | 120KH 50MHz | 2 | NTN TDLC5-1200 | B | 10 | 1T2R | Disable | 1+1 (0,8) | -5.7 | -3.7 |
| 2 | CP-OFDM | 120KH 50MHz | 12 | NTN TDLC5-1200 | B | 10 | 1T2R | Disable | 1+1 (0,8) | 3.2 | 5.2 |
| 3 | CP-OFDM | 120KH 50MHz | 2 | NTN TDLC5-1200 | B | 10 | 1T1R | Disable | 1+1 (0,8) | -2.3 | -0.3 |
| 4 | CP-OFDM | 120KH 50MHz | 12 | NTN TDLC5-1200 | B | 10 | 1T1R | Disable | 1+1 (0,8) | 6.6 | 8.6 |

Table 2: ideal and impairment simulation results for PUSCH requirement with DFT-s-OFDM

| Test case | waveform | SCS& BW | RB | MCS | Channel | Mapping Type | Length Of symbol | Number of Tx and Rx | PTRS | Number of DMRS | SNR@ 70 (ideal) | SNR@70 (impairment) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | DFT-s-OFDM | 120KH 50MHz | 30 PRBs in the middle of test bandwidth | 2 | NTN TDLC5-3000 | B | 10 | 1T2R | Disable | 1+1 (0,8) | -5.5 | -3.5 |
| 2 | DFT-s-OFDM | 120KH 50MHz | 30 PRBs in the middle of test bandwidth | 2 | NTN TDLC5-3000 | B | 10 | 1T1R | Disable | 1+1 (0,8) | -2.3 | -0.3 |

Table 3: ideal and impairment simulation results for PUSCH requirement with repetition Type A in above 10GHz

| Test case | SCS& BW | RB | MCS (table3) | Channel | PUSCH Aggregation level | Mapping Type | Length Of symbol | Number of Tx and Rx | PTRS | Number of DMRS | SNR@BLER 1% (ideal) | SNR@BLER 1% (impairment) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 120KH 50MHz | 32 | 5 | NTN TDLC5-1200 | 2 | B | 10 | 1T2R | Disable | 1+1 (0,8) | -14.7 | -12.7 |
| 2 | 120KH 50MHz | 32 | 5 | NTN TDLC5-1200 | 2 | B | 10 | 1T1R | Disable | 1+1 (0,8) | -11.5 | -9.5 |

Table 4: ideal and impairment simulation results for PUSCH requirement with DMRS bundling

| Test case | SCS& BW | MCS (table1) | Channel | slot | Pusch-TimeDomainWindowLength | Mapping Type | Length Of symbol | Number of Tx and Rx | Number of DMRS | SNR@ 70 (ideal) | SNR@70 (impairment) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 15KHz 5MHz | 4 | NTN TDLA100-200 | 4 | 4 | A | 14 | 1T2R | 1+1 (2,11) | -6.6 | -4.6 |
| 2 | 15KHz 5MHz | 4 | NTN TDLA100-200 | 4 | 4 | B | 14 | 1T2R | 1+1 (0,8) | -6.2 | -4.2 |
| 3 | 30KHz 10MHz | 4 | NTN TDLA100-200 | 8 | 8 | A | 14 | 1T2R | 1+1 (2,11) | -9.2 | -7.2 |
| 4 | 30KHz 10MHz | 4 | NTN TDLA100-200 | 8 | 8 | B | 14 | 1T2R | 1+1 (0,8) | -9.0 | -7.0 |
| 5 | 15KHz 5MHz | 4 | NTN TDLA100-200 | 4 | 4 | A | 14 | 1T1R | 1+1 (2,11) | -2.6 | -0.6 |
| 6 | 15KHz 5MHz | 4 | NTN TDLA100-200 | 4 | 4 | B | 14 | 1T1R | 1+1 (0,8) | -2.1 | -0.1 |
| 7 | 30KHz 10MHz | 4 | NTN TDLA100-200 | 8 | 8 | A | 14 | 1T1R | 1+1 (2,11) | -5.6 | -3.6 |
| 8 | 30KHz 10MHz | 4 | NTN TDLA100-200 | 8 | 8 | B | 14 | 1T1R | 1+1 (0,8) | -5.3 | -3.3 |

## 3.2	PUCCH requirement

Table 5: ideal and impairment results for Format 0

| Test case | Format | SCS& BW | Intra-slot Frequency hopping | UCI bits | Channel | Number of RB | Number of Tx and Rx | Number of Symbol | ACK missed%1 (ideal) | ACK missed%1 (impairment) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | Format 0 | 120KH 50MHz | Disabled | 1 | NTN-TDLC5-1200 | 9 | 1T2R | 1 | 1.85 | 3.85 |
| 2 | Format 0 | 120KH 50MHz | Enabled | 1 | NTN-TDLC5-1200 | 9 | 1T2R | 2 | -0.40 | 1.6 |
| 3 | Format 0 | 120KH 50MHz | Disabled | 1 | NTN-TDLC5-1200 | 9 | 1T1R | 1 | 6.81 | 8.81 |
| 4 | Format 0 | 120KH 50MHz | Enabled | 1 | NTN-TDLC5-1200 | 9 | 1T1R | 2 | 4.39 | 6.39 |

Table 6: ideal and impairment simulation results for Format 1 (To be updated)

| Test case | Format | SCS& BW | Intra-slot Frequency hopping | UCI bits | Channel | Number of RB | Number of Tx and Rx | Number of Symbol | ACK missed%1 (ideal) | NACK to ACK %0.1 (ideal) | ACK missed%1 (impair) | NACK to ACK %0.1 (impari) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | Format 1 | 120KH 50MHz | Enabled | 2 | NTN-TDLC5-1200 | 1 | 1T2R | 14 | -8.84 | -7.26 | -6.84 | -5.26 |
| 2 | Format 1 | 120KH 50MHz | Enabled | 2 | NTN-TDLC5-1200 | 1 | 1T2R | 14 |  |  |  |  |
| 3 | Format 1 | 120KH 50MHz | Enabled | 2 | NTN-TDLC5-1200 | 1 | 1T1R | 14 | -4.34 | -2.59 | -2.34 | -0.59 |
| 4 | Format 1 | 120KH 50MHz | Enabled | 2 | NTN-TDLC5-1200 | 1 | 1T1R | 14 |  |  |  |  |

Table 7: ideal and impairment simulation results for Format 2

| Test case | Format | SCS& BW | Intra-slot Frequency hopping | UCI bits | Channel | Number of RB | Number of Tx and Rx | Number of Symbol | ACK missed%1 (ideal) | ACK missed%1 (impairment) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | Format 2 | 120KH 50MHz | Disabled | 4 | NTN-TDLC5-1200 | 4 | 1T2R | 1 | -1.31 | 0.69 |
| 2 | Format 2 | 120KH 50MHz | Disabled | 4 | NTN-TDLC5-1200 | 4 | 1T1R | 1 | 3.63 | 5.63 |

Table 8: ideal and impairment simulation results for Format 2

| Test case | Format | SCS& BW | Intra-slot Frequency hopping | UCI bits | Channel | Number of RB | Number of Tx and Rx | Number of Symbol | BLER%1 (ideal) | BLER%1 (impairment) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | Format 2 | 120KH 50MHz | Enabled | 22 | NTN-TDLC5-1200 | 9 | 1T2R | 2 | -3.35 | -1.35 |
| 2 | Format 2 | 120KH 50MHz | Enabled | 22 | NTN-TDLC5-1200 | 9 | 1T1R | 2 | 1.88 | 3.88 |

Table 9: ideal and impairment simulation results for Format 3

| Test case | Format | SCS& BW | Intra-slot Frequency hopping | UCI bits | Channel | Number of RB | Number of Tx and Rx | DMRS | Number of Symbol | 1%BLER (ideal) | 1%BLER (impairment) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | Format 3 | 120KH 50MHz | Enabled | 16 | NTN-TDLC5-1200 | 1 | 1T2R | No additional DMRS | 14 | -5.01 | -3.01 |
| 2 | Format 3 | 120KH 50MHz | Enabled | 16 | NTN-TDLC5-1200 | 1 | 1T2R | With additional DMRS | 14 | -5.39 | -3.39 |
| 3 | Format 3 | 120KH 50MHz | Enabled | 16 | NTN-TDLC5-1200 | 1 | 1T1R | No additional DMRS | 14 | -0.93 | 1.07 |
| 4 | Format 3 | 120KH 50MHz | Enabled | 16 | NTN-TDLC5-1200 | 1 | 1T1R | With additional DMRS | 14 | -0.82 | 1.18 |

Table 10: ideal and impairment simulation results for Format 4

| Test case | Format | SCS& BW | Intra-slot Frequency hopping | UCI bits | Channel | Number of RB | Number of Tx and Rx | DMRS | Number of Symbol | 1%BLER (ideal) | 1% BLER (impairment) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | Format 4 | 120KH 50MHz | Enabled | 22 | NTN-TDLC5-1200 | 1 | 1T2R | No additional DMRS | 14 | -2.88 | -0.88 |
| 2 | Format 4 | 120KH 50MHz | Enabled | 22 | NTN-TDLC5-1200 | 1 | 1T2R | With additional DMRS | 14 | -3.04 | -1.04 |
| 3 | Format 4 | 120KH 50MHz | Enabled | 22 | NTN-TDLC5-1200 | 1 | 1T1R | No additional DMRS | 14 | 1.91 | 3.91 |
| 4 | Format 4 | 120KHz 50MHz | Enabled | 22 | NTN-TDLC5-1200 | 1 | 1T1R | With additional DMRS | 14 | 2.17 | 4.17 |

## 3.3	PRACH requirement

Table 11: ideal and impairment simulation results for PRACH

| Test case | Format | SCS& BW | PRACH sequence length | Ncs | Logical sequence index | v | Number of Tx and Rx | Channel | Frequency offset | Time estimation tolerance | SNR@1 % Miss (ideal) | SNR@1 % Miss (impairment) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | C2 | 120KH | 139 | 69 | 0 | 0 | 1T2R | NTN-TDLC5-1200 | 3000 | 0.13us | -11.04 | -9.04 |
| 2 | B4 | 120KH | 139 | 69 | 0- | 0 | 1T2R | NTN-TDLC5-1200 | 3000 | 0.13us | -15.38 | -13.38 |
| 3 | C2 | 120KH | 139 | 69 | 0 | 0 | 1T1R | NTN-TDLC5-1200 | 3000 | 0.13us | -5.53 | -3.53 |
| 4 | B4 | 120KHz | 139 | 69 | 0 | 0 | 1T1R | NTN-TDLC5-1200 | 3000 | 0.13us | -9.94 | -7.94 |

# 4	Conclusion

In this contribution, the view on remaining issue of BS demodulation requirement for NTN enhancement was provided. Meanwhile, the ideal and impairment simulation results are provided for requirement derivation

Observation 1: The SNR targeting 70% TP for MCS 12 is around 7dB for 1T1R, with considering the impairment margin, it shall be feasible to define PUSCH requirement with MCS 12

Proposal 1: MCS 12 can be considered to specific the PUSCH requirement for NTN enhancement.

Proposal 2: Update the test metric of PUSCH requirement with repetition mapping type A for above 10GHz as BLER% 1, where BLER is defined as residual BLER; i.e., ratio of incorrectly received transport blocks / sent transport blocks, independently of the number HARQ transmission(s) for each transport block

# References

R4-2406024, WF on NR_NTN_enh_SAN_UE_demod