3GPP TSG-RAN WG4 Meeting #111	R4-2409629

Fukuoka, Japan, May 20 – May 24, 2024

Source:	Nokia

Title:	TDCP RRM Requirements

Agenda item:	7.19.2.1

Document for:	Discussion

Introduction

In this paper we present Nokia’s view on the remaining aspects for TDCP requirements.

The paper includes analysis on the open issues, and the updated simulation results in Appendix A.

Discussion

In the last RAN4 meeting, the following agreement was reached regarding TDCP test cases:

Issue 2-1-1: Test metric of TDCP test cases: < Agreement> : Agreements for the test case definition: - Define the same test case for both 15kHz with FDD and and 30kHz with TDD in the same sub-clause. - Channel models: TDL-A30 - Doppler: - Two TCs with low and high Doppler - TC1: low (10) for both 15kHz and 30kHz - TC2: high - Option 1: high (300) - Option 2: 100 for 15kHz SCS, and 200 for 30kHz SCS - BW: - 10MHz for FDD and TDD - SNR: - Option 1: 20dB for TC1 - Option 2: 10dB or 20dB for TC2 - The distance between two TRSs: 1 - Report index: Bring CR for both options, make decision in the next meting - Option 1: - lower Doppler: CDP at X1 is higher than Y1 = [90] % - high Doppler: CDP at X2 is lower than Y2= [10] % - X1, X2, Y1, and Y2 can be different for TDD and FDD - Option 2: - [X1, X2] for Y2= FFS

From Nokia’s previous submission in RAN4 #110bis [1], we had the simulation results for the case of 30 KHz SCS (see Appendix A). In this paper we focused on simulations using 15 KHz SCS.

Below (Table 1) are the results obtained using 15 KHz SCS.

Table 1: Results for 15 KHz SCS (Quantized amplitudes).

| Doppler | SNR | P5 | P10 | P90 | P95 |
| --- | --- | --- | --- | --- | --- |
| 10 Hz | 20 | 7 | 7 | 0 | 0 |
| 100 Hz | 10 | 13 | 13 | 7 | 6 |
|  | 20 | 10 | 9 | 2 | 1 |
| 300 Hz | 10 | 15 | 15 | 13 | 12 |
|  | 20 | 14 | 13 | 5 | 4 |

From the results in Table 1, we extract the values for 5th percentile (P5), 10th percentile (P10) , 90th percentile (P90)  and 95th percentile (P95) to demonstrate the differentiation in the TDCP reports with low and high Doppler as shown in Table 2 and Table 3 below.

Table 2: X1 and X2 values using 5th and 95th percentiles for 15 KHz SCS.

| 5th and 95th percentile |  |  |  |  |
| --- | --- | --- | --- | --- |
| Doppler | SNR | Quantized |  | Distance between X2 and X1 |
|  |  | X1 | X2 |  |
| 10 Hz | 20 | 8 | 0 | 8 |
| 100 Hz | 10 | 14 | 5 | 9 |
|  | 20 | 11 | 0 | 11 |
| 300 Hz | 10 | 15 | 11 | 4 |
|  | 20 | 15 | 3 | 12 |

Table 3: X1 and X2 values using 10th and 90th percentiles for 15 KHz SCS.

| 10th and 90th percentile |  |  |  |  |
| --- | --- | --- | --- | --- |
| Doppler | SNR | Quantized |  | Distance between X2 and X1 |
|  |  | X1 | X2 |  |
| 10 Hz | 20 | 8 | 0 | 8 |
| 100 Hz | 10 | 15 | 6 | 9 |
|  | 20 | 15 | 1 | 14 |
| 300 Hz | 10 | 15 | 12 | 3 |
|  | 20 | 15 | 4 | 11 |

For  300 Hz Doppler at 10 dB SNR, we notice that there is a very small distance between X1 and X2, independent of which testing approach is used as highlighted in Table 2 and Table 3 above. From that we can infer that 300 Hz is a better option for the TDCP verification with high doppler.

For the low doppler (10 Hz) case, the TDCP variation is very large for low SNR (10 dB), and it is significantly reduced for high SNR (20 dB). For this reason, we propose that it is better to use 10 Hz Doppler with 20 dB SNR for the test case with low Doppler.

Following the analysis, some Observations and Proposals are made:

TDCP spread for 100 Hz for 15 KHz SCS, and 200 Hz for 30 KHz SCS Doppler is about 8 quantization levels.

Adopt 300 Hz for TC2 (high Doppler case).

TDCP spread for high Doppler case is reduced with 10 dB SNR.

RAN4 to configure the SNR of test cases as

  - 20 dB for TC1

  - 10 dB for TC2.

The test design illustration in Figure 1 below depicts two approaches for setting thresholds. In scenarios a) and b), a single threshold strategy is employed, utilizing the 10th and 90th percentiles for low and high Doppler respectively.

Conversely, scenarios c) and d) take on a dual threshold approach, incorporating the 5th and 95th percentiles to establish upper and lower thresholds for low and high Doppler values.

For the UE to pass the test, it should fall in the pass region as depicted in the illustration.

| a) | b) |
| --- | --- |
| c) | d) |

Figure 1: Test Design illustration. a) and b) show the approach with a single threshold using 10th and 90th percentile for a) low and b) high Doppler. c) and d) show the approach with two thresholds using 5th and 95th percentile for c) low and d) high Doppler.

Use of 5th and 95th percentiles can result in quantization levels 0 and 15.

For the test design, define decision levels as

  - Lower Doppler: pass criteria is TCDP > X1 for 90 % of the samples.

  - High Doppler: pass criteria is TCDP < X2 for 90 % of the samples.

  - X1 is defined as quantized maximum value among the 90th percentile among all companies excluding outliers.

  - X2 is calculated as quantized minimum value among the 10th percentile among all companies excluding outliers.

  - A result is considered as outlier if it deviates from the average of the other companies by 3 or more quantization levels.

Conclusion

The following Observations and Proposals were made:

Observation 1: TDCP spread for 100 Hz for 15 KHz SCS, and 200 Hz for 30 KHz SCS Doppler is about 8 quantization levels.

Proposal 1: Adopt 300 Hz for TC2 (high Doppler case).

Observation 2: TDCP spread for high Doppler case is reduced with 10 dB SNR.

Proposal 2: RAN4 to configure the SNR of test cases as

a.	20 dB for TC1

b.	10 dB for TC2.

Observation 3: Use of 5th and 95th percentiles can result in quantization levels 0 and 15.

Proposal 3: For the test design, define decision levels as

a.	Lower Doppler: pass criteria is TCDP > X1 for 90 % of the samples.

b.	High Doppler: pass criteria is TCDP < X2 for 90 % of the samples.

c.	X1 is defined as quantized maximum value among the 90th percentile among all companies excluding outliers.

d.	X2 is calculated as quantized minimum value among the 10th percentile among all companies excluding outliers.

e.	A result is considered as outlier if it deviates from the average of the other companies by 3 or more quantization levels.

- References

- R4-2405808, TDCP RRM Requirements, Nokia, RAN4# #110bis, Changsha, China, April 15 – April 19, 2024.

- R4-2406503, WF on RRM requirements for NR_MIMO_evo_DL_UL, Samsung

- Appendix A: Simulation results

Table 4: Simulation parameters.

| Parameter | Value |
| --- | --- |
| Delay (between TRS symbols) | 1 slot |
| Channel model | TDL-A, delay spread=30ns |
| Doppler Spread | 10, 100, 300 |
| SNR | 10,20 |
| Number of averaging samples: | one shot |
| Channel BW | 10MHz |
| SCS | 15 kHz and 30 kHz |
| Reference Channel estimation | LS base CE for TRS as baseline |
| Correlation matrix and antenna configuration | 1x2 Low |

Table 5: Results for 30 KHz SCS (Unquantized amplitudes).

| Doppler | SNR | P5 | P10 | P90 | P95 |
| --- | --- | --- | --- | --- | --- |
| 10 | 20 | 0.9477 | 0.9609 | 0.9956 | 0.9966 |
| 100 | 10 | 0.6291 | 0.7019 | 0.9569 | 0.9663 |
|  | 20 | 0.9271 | 0.9474 | 0.995 | 0.9961 |
| 200 | 10 | 0.5941 | 0.6742 | 0.9544 | 0.9648 |
|  | 20 | 0.8659 | 0.9095 | 0.9933 | 0.9949 |
| 300 | 10 | 0.5386 | 0.6303 | 0.9507 | 0.9617 |
|  | 20 | 0.7684 | 0.8461 | 0.9908 | 0.9931 |

Table 6: Results for 30 KHz SCS (Quantized amplitudes).

| Doppler | SNR | P5 | P10 | P90 | P95 |
| --- | --- | --- | --- | --- | --- |
| 10 | 20 | 7 | 7 | 0 | 0 |
| 100 | 10 | 13 | 12 | 7 | 6 |
|  | 20 | 8 | 7 | 1 | 0 |
| 200 | 10 | 13 | 13 | 7 | 6 |
|  | 20 | 10 | 9 | 2 | 1 |
| 300 | 10 | 14 | 13 | 7 | 7 |
|  | 20 | 12 | 11 | 2 | 2 |

Table 7: Results for 15 KHz SCS (Unquantized amplitudes).

| Doppler | SNR | P5 | P10 | P90 | P95 |
| --- | --- | --- | --- | --- | --- |
| 10 | 20 | 0.947627 | 0.960917 | 0.99531 | 0.996378 |
| 100 | 10 | 0.596581 | 0.674678 | 0.952174 | 0.962333 |
|  | 20 | 0.86523 | 0.907938 | 0.992834 | 0.994596 |
| 300 | 10 | 0.106136 | 0.148006 | 0.64726 | 0.703444 |
|  | 20 | 0.448187 | 0.594842 | 0.976572 | 0.983549 |

Table 8: Results for 15 KHz SCS (Quantized amplitudes).

| Doppler | SNR | P5 | P10 | P90 | P95 |
| --- | --- | --- | --- | --- | --- |
| 10 | 20 | 7 | 7 | 0 | 0 |
| 100 | 10 | 13 | 13 | 7 | 6 |
|  | 20 | 10 | 9 | 2 | 1 |
| 300 | 10 | 15 | 15 | 13 | 12 |
|  | 20 | 14 | 13 | 5 | 4 |