3GPP TSG-RAN WG4 Meeting #111	R4- 2409638

Fukuoka City, Fukuoka , Japan, 20th – 24th May, 2024

Source: 	Huawei, HiSilicon

Title:	PC2 HPUE for band n28

Agenda Item:	6.19.2

Document for:	Approval

# 1	Introduction

In RAN4#110bis, a WF [1] on band n28 was discussed but not agreed due to the concerns from operators in Japan. In this paper, we share our view on how to move forward.

# 2	Discussion

### 2.1 Overview

The following issues have been discussed for band n28 during the Rel-18 WI:

- PC2 RSD for 1Tx and 2Tx

- PC2 A-MPR requirements for NS_18

- PC2 A-MPR requirements for NS_17

- Requirements for spurious emissions for UE co-existence

The last three issues were initially discussed under dual-duplexer assumption, and later considered for full-duplexer implementations.

The PC2 RSD requirements for band n28 are completed [2].

For NS_18, the PC2 A-MPR for BW=5MHz to 30MHz have been agreed and captured in WF [3].

For NS_17, it has been agreed that no A-MPR is needed for dual-duplexer implementations [4].

For full-band duplexer implementations, however, there’re still open issues regarding NS_17 and UE co-existence requirements.

Observation 1: The evaluation on PC2 for band n28 under dual-duplexer assumption has been completed for the Rel-18 WI.

### 2.2 Impact of the full-band duplexer

For NS_18, RAN4 has agreements [3] on PC2 A-MPR for all n28 CBWs up to Rel-18, which are applicable for both dual-duplexer and full-band duplexer implementations.

For NS_17, several companies have provided evaluation results and corresponding proposals [5][6][7][8]. It’s clear that a UE would need A-MPR for PC2. Among them, three companies also proposed A-MPR for PC3.

Observation 2: For NS_18, the same A-MPR requirements are applicable for both duplexer implementations. For NS_17, A-MPR is needed for PC2, FFS PC3 for full-band duplexers.

Concerns have also been raised for how to meet the requirements for spurious emissions for UE co-existence as shown below.

#### 6.5.3.2	Spurious emissions for UE co-existence

This clause specifies the requirements for NR bands for coexistence with protected bands. Unless otherwise stated, the spurious emission for UE co-existence apply for the frequency ranges that are more than FOOB (MHz) in Table 6.5.3.1-1 from the edge of the channel bandwidth.

Table 6.5.3.2-1: Requirements for spurious emissions for UE co-existence

| NR Band | Spurious emission for UE co-existence |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Protected band | Frequency range (MHz) |  |  | Maximum Level (dBm) | MBW (MHz) | NOTE |
| n28, n83 | E-UTRA Band 1, 4, 22, 32, 42, 43, 50, 51, 65, 66, 74, 75, 76 NR Band n77, n78, n100, n101 | FDL_low | - | FDL_high | -50 | 1 | 2 |
|  | E-UTRA Band 1 | FDL_low | - | FDL_high | -50 | 1 | 19, 25 |
|  | E-UTRA Band 2, 3, 5, 7, 8, 18, 19, 20, 25, 26, 27, 31, 34, 38, 39, 40, 41, 52, 71, 72, 73 NR Band n79, n105 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA Band 11, 21 | FDL_low | - | FDL_high | -50 | 1 | 19, 24 |
|  | Frequency range | 470 | - | 694 | -42 | 8 | 15, 35 |
|  | Frequency range | 470 | - | 710 | -26.2 | 6 | 34 |
|  | Frequency range | 662 | - | 694 | -26.2 | 6 | 15 |
|  | Frequency range | 758 | - | 773 | -32 | 1 | 15 |
|  | Frequency range | 773 | - | 803 | -50 | 1 |  |
|  | Frequency range | 1884.5 | - | 1915.7 | -41 | 0.3 | 8, 19 |
| NOTE 15: These requirements also apply for the frequency ranges that are less than FOOB (MHz) in Table 6.5.3.1-1 from the edge of the channel bandwidth. NOTE 34: This requirement is applicable for 5 and 10 MHz NR channel bandwidth allocated within 718-728 MHz. For carriers of 10 MHz bandwidth, this requirement applies for an uplink transmission bandwidth less than or equal to 30 RB with RBstart > 1 and RBstart < 48. NOTE 35: This requirement is applicable in the case of a 10 MHz NR carrier confined within 703 MHz and 733 MHz, otherwise the requirement of -25 dBm with a measurement bandwidth of 8 MHz applies. |  |  |  |  |  |  |  |

It can be seen that the requirement of -26.2 dBm/6MHz for 470-710MHz overlaps with that of NS_17. And NOTE 34 implies that the network scheduler should apply certain restrictions on the RB allocation. If such restrictions are followed, no A-MPR would be needed even for full-band duplexer implementation. And the applicable frequency range for NS_17 should be modified.

Proposal 1: Clarify the implication of NOTE 34 for band n28 in the requirements for spurious emissions for UE co-existence, and check whether operator’s network follows the requirement in NOTE 34.

It was pointed out by Qualcomm in [7] that the -42dBm/8MHz specified for frequencies below 694MHz is the most challenging requirement to meet.

The conclusion therefore has been that -42 dBm / 8 MHz specified below 694 MHz is the most challenging requirement to meet. For PC2 operation there are two options to handle that requirement

Option 1: With sufficiently good filter rejection the requirement can be met without specification changes

Option 2: Specify a new NS which allows A-MPR for this requirement

Observation 3: Further study is needed to confirm whether a new NS and corresponding A-MPR are needed for full-duplexer implementation to meet the requirements for spurious emissions for UE co-existence.

### 2.3 Consideration for Rel-19

Based on the above discussions, there’re open issues on supporting full-band duplexers on band n28, which cannot be completed in the timeframe of Rel-18. Additionally, new channel bandwidths were proposed for band n28 [1], which can be enabled by full-band duplexers.

Proposal 2: RAN4 to study and specify requirements for new channel bandwidths and PC2 for band n28 in Rel-19 by considering at least the following aspects:

- PC3 REFSENS for new channel bandwidths

- PC3 A-MPR for NS_17 and NS_18 using full-band duplexers

- PC2 RSD and PC2 A-MPR for both 1Tx and 2Tx

- Spurious emissions for UE co-existence

# 3	Conclusion

In this paper, we have summarized the progress on supporting PC2 on n28, and shared our views on the remaining issues.

Observation 1: The evaluation on PC2 for band n28 under dual-duplexer assumption has been completed for the Rel-18 WI.

Observation 2: For NS_18, the same A-MPR requirements are applicable for both duplexer implementations. For NS_17, A-MPR is needed for PC2, FFS PC3 for full-band duplexers.

Observation 3: Further study is needed to confirm whether a new NS and corresponding A-MPR are needed for full-duplexer implementation to meet the requirements for spurious emissions for UE co-existence.

Proposal 1: Clarify the implication of NOTE 34 for band n28 in the requirements for spurious emissions for UE co-existence, and check whether operator’s network follows the requirement in NOTE 34.

Proposal 2: RAN4 to study and specify requirements for new channel bandwidths and PC2 for band n28 in Rel-19 by considering at least the following aspects:

- PC3 REFSENS for new channel bandwidths

- PC3 A-MPR for NS_17 and NS_18 using full-band duplexers

- PC2 RSD and PC2 A-MPR for both 1Tx and 2Tx

- Spurious emissions for UE co-existence

# References

- R4-2406708 WF on band n28 full band duplexer, Apple, RAN4#110bis

- WF on FDD HPUE Basket WIs, China Unicom, RAN4#106bis-e

- R4-2310245 WF on PC2 A-MPR for band n8 and n28, Huawei, HiSilicon, Apple, China Unicom, RAN4#107

- R4-2321715 WF on HPUE for FDD bands, China Unicom, RAN4#109

- R4-2400170 On n28 with full band duplexer, Apple, RAN4#110

- R4-2404878 Remaining issues for FDD HPUE A-MPR, Huawei, HiSilicon, RAN4#110bis

- R4-2405712 PC2 A-MPR for n7 and n28, Qualcomm, RAN4#110bis

- R4-2405962 n28 NS_17 PC2, Skyworks, RAN4#110bis

- R4-2403629 WF on HPUE for FDD bands, China Unicom, RAN4#110