3GPP TSG-RAN WG4 Meeting #111	R4- 2409639

# Fukuoka City, Fukuoka , Japan, 20th – 24th May, 2024

Source: 	Huawei, HiSilicon

Title:	MSD for CA_n8-n41 with PC2 n8 UL

Agenda Item:	6.18.2

Document for:	Approval

# 1	Introduction

This paper presents the PC2 MSD analysis for CA_n8-n79 with PC2 n8 UL as per the WID in [1].  According to the existing PC3 requirements, the 5th harmonic of band n8 UL may fall into the DL of band n79.

# 2	Discussion

### 2.1 MSD for PC2 n8 UL with 1Tx

The following RFFE parameters are assumed in the analysis throughout the paper.

Table 2.1-1 Front-end component isolation and 5th order linearity parameters

| Isolation | dB | Linearity (IP5) | dBm |
| --- | --- | --- | --- |
| Duplexer Tx to H5 | 20 | Duplexer/BPF | 53 |
| Harmonic filter to H5 | 35 | Harmonic filter | 53 |
| n79 BPF filter to n8 Tx | 45 | Diplexer | 53 |
| Diplexer | 20 | Switch | 53 |
| Antenna | 10 | LNA (IIP5) | -10 |
| PCB (within main/main-to-div) | 65/70 |  |  |

Table 2.1-2 Link analysis for 5th harmonic power level calculation

| Insertion loss (dB) | 4 |  |  |
| --- | --- | --- | --- |
| PA output power (dBm) | 30 |  |  |
| Main Path | dBm | Diversity Path | dBm |
| H5 @ PA output | -15 | H5 @ antenna | -98.3 |
| H5 @ duplexer output | -35.0 | Diplexer H5 referred to antenna | -145.5 |
| H5 @ harmonic filter | -70.0 | Switch H5 referred to antenna | -245.0 |
| H5 @ switch | -69.9 | BPF H5 referred to antenna | -244.5 |
| H5 @ diplexer output/antenna | -88.3 | LNA H5 referred to antenna | -167.4 |
| LNA H5 referred to antenna | -136.2 | PCB coupling referred to antenna | -81 |
| PCB coupling referred to antenna | -76 | Total H5 | -80.9 |
| Total H5 | -75.8 |  |  |

The MSD for n79 10MHz DL channel bandwidth, as shown in Table 2.1-3, was derived using MRC with correlated interference assumed.

Table 2.1-3 MSD for n79 10MHz DL channel BW

| BW |  | Main | Diversity |
| --- | --- | --- | --- |
| 10 MHz | Noise floor (dBm) | -91.8 | -91.8 |
|  | Harmonics (dBm) | -75.8 | -80.9 |
|  | Total (dBm) | -75.6 | -80.6 |
|  | MSD per branch (dB) | 16.2 | 11.2 |
|  | MSD after MRC (dB) | 15.6 |  |

### 2.2 MSD for PC2 n8 UL with 2Tx

The MSD analysis for 2Tx are conducted in two steps. Firstly, assume only one Tx is transmitting and calculate the harmonic interference power similar to the 1Tx case, except that the PA output power is 3dB lower. Secondly, when calculating the total H5 at the main path, the total H5 at the diversity path is added to account for the effect of the 2nd PA.

Table 2.2-1 Link analysis for 5th harmonic power level calculation

| Insertion loss (dB) | 4 |  |  |
| --- | --- | --- | --- |
| PA output power (dBm) | 27 |  |  |
| The calculation below assumes only one Tx is transmitting. |  |  |  |
| Main Path | dBm | Diversity Path | dBm |
| H5 @ PA output | -18 | H5 @ antenna | -105.8 |
| H5 @ duplexer output | -41 | Diplexer H5 referred to antenna | -160.5 |
| H5 @ harmonic filter output | -76 | Switch H5 referred to antenna | -260 |
| H5 @ switch | -76 | BPF H5 referred to antenna | -259.5 |
| H5 @ diplexer output/antenna | -95.8 | LNA H5 referred to antenna | -160 |
| LNA H5 referred to antenna | -151.2 | PCB coupling referred to antenna | -84 |
| PCB coupling referred to antenna | -79 | Total H5 | -84 |
| Total H5 | -78.9 |  |  |
| For 2Tx PC2, both Tx are transmitting. Hence, the H5 on the main and diversity path are combined, and the result applies to both branches. |  |  |  |
| Total H5 | -77.7 |  |  |

The MSD for n41 10MHz DL channel bandwidth, as shown in Table 2.2-2, was derived using MRC with correlated interference assumed.

Table 2.2-2 MSD for n79 10MHz DL channel BW

| BW |  | Main | Diversity |
| --- | --- | --- | --- |
| 10 MHz | Noise floor (dBm) | -91.8 | -91.8 |
|  | Harmonics (dBm) | -77.7 | -77.7 |
|  | Total (dBm) | -77.6 | -77.6 |
|  | MSD per branch (dB) | 14.2 | 14.2 |
|  | MSD after MRC (dB) | 17.2 |  |

# 3	Conclusion

In this paper, we have presented our analysis on the MSD for CA_n8-n79 with either 1Tx PC2 or 2Tx PC2 on n8 as UL.

Proposal 1: For band n79 REFSENS exception due to H5 from n8 UL, set the MSD to [15.6] dB for 1Tx PC2 and [17.2] dB for 2Tx PC2.

# References

- RP-240123 Revised WID: Rel-18 High power UE (power class 2) for a single FR1 NR FDD band in UL of NR intra-band and inter-band CA/DC combinations with y bands downlink (y=1,2,3,4,5,6) and x bands uplink (x=1), China Unicom, RAN#103

- R4-2406711 MSD analysis for CA_n8-n41 with PC2 n8 UL, Huawei, HiSilicon, RAN4#110bis

- R4-2404175 MSD analysis for CA_n71A-n77A with n71 PC2 UL, Apple, RAN4#110bis