# 3GPP TSG-RAN WG4 Meeting #111	                                 R4-2409740

# Fukuoka, Japan, 20 – 24 May, 2024

Agenda item:	10.3.3

Source: 	CableLabs, Charter Communications, Cox Communications

Title: 	Technical parameters for the 7125 to 8400 MHz IMT band for sharing studies

Document for:	Discussion

# 1. Introduction

The 2023 World Radio Communication Conference (WRC-23, 20 November to 15 December 2023) approved agenda item 1.7 for the 2027 World Radio Communication Conference (WRC-27, Administrative Circular CA/270). ITU-R Working Party (WP) 5D is invited to conduct and compare the sharing and compatibility studies to ensure the protection of services to which the frequency band is allocated on a primary basis for the frequencies:

–	4400 – 4800 MHz;

–	7125 – 8400 MHz; and

–	14.8 – 15.35 GHz.

ITU-R WP5D sent an LS [1] to RAN4 asking for technical parameters. RAN #103 approved a SI [2] to prepare the requested technical parameters for ITU. We raised a concern in the RAN4 #110bis meeting whether macro deployment in the 7 and 15 GHz bands has sufficient coverage due to the high frequency and consequential high propagation losses [3]. In the RAN4 110bis meeting in Changsha, whether to include urban macro and sub-urban macro was discussed in an ad-hoc meeting [4] and [5], and the way forward was “Include urban and sub-urban macro, but double check coverage.” In this contribution, the simulation method and results will be presented for macro coverage estimation, along with some other considerations.

- Discussion

Figure 1. Spectrum allocation in the 7125 – 8400 MHz band in the USA.

The 7125 – 8400 MHz band is currently licensed to national mission-critical links. The FCC spectrum allocation is shown in Figure 1. This band is mainly occupied by military radar, satellites including both geostationary orbit (GSO) and non-GSO (NGSO), fixed service (FS), and earth exploration service (EES). Similarly, a large portion of this band is occupied in many European countries by NATO links. Therefore, it is not feasible to clear this band for public mobile networks (e.g., IMT-2030 or 6G), instead, this 7125 – 8400 MHz band will need to be shared. A set of low-power assumptions could make sharing more feasible and a larger portion of the band more available.

Proposal 1: The 7125 – 8400 MHz band is currently licensed to national mission-critical links. It is not feasible to clear this band for public mobile networks (e.g., IMT-2030 or 6G) in many markets, instead, this 7125 – 8400 MHz band will need to be shared. A set of low-power assumptions could make sharing more feasible and a larger portion of the band more available, such as using power class 5 (PC5) for UE.

The 7125 – 8400 MHz band will likely be considered by operators as microcells or femtocells. Outdoor pole-mount or strand-mount micro small-cell BSs and indoor BSs typically use non-AAS or a small-size array due to the cost and physical dimension limitations. Non-AAS parameters are of interest for the 7125 – 8400 MHz band.

Proposal 2: Non-AAS with omnidirectional BS antenna and small array sizes (e.g., 2×2, 2×4, or 4×4) parameters are of interest for the 7125 – 8400 MHz band for medium-range and local-area microcells.

High-power macro-cells are supposed to provide good coverage (e.g., over 95% of the area). Higher frequency signals have larger path loss, outdoor-to-indoor (O2I) loss, terrain loss, clutter attenuation, etc. Higher frequency signals typically have few multipath components. The propagation conditions are more challenging at higher frequencies. Macro-cell deployment in the 7125 – 8400 and 14800 – 15350 MHz bands may not be feasible. RAN4 will need to study the feasibility.

Proposal 3: Macro-cell deployment in the 7125 – 8400 MHz band may not be feasible due to large losses in these high frequencies. RAN4 will need to study the feasibility of: (1) what is the coverage with a 500 m UMa ISD? And (2) what is the maximum urban macro (UMa) inter-site distance (ISD) to achieve 95% coverage (> -10 dB SINR) in both DL and UL?

- Macro deployment feasibility study in the 7-GHz band

### Method 1: Monte Carlo simulation for coverage area with a constant ISD

The simulation is based on the most recent RAN4 coexistence study TR 38.858. The duplex mode is synchronized TDD. The co-channel interference (CCI) from intra-network neighbor cells is aggregated. No adjacent-channel interference (ACI) is considered. Key assumptions and steps are listed below:

- 19-site grid with wrap-around, three sectors per site.

- Center frequency 7.7625 GHz (in the middle of the band) compared with 4 GHz, channel bandwidth 100 MHz.

- 500 m UMa ISD, 23 dBm UE power, 46 dBm BS power, and multiple BS antenna array sizes are compared.

- One UE per cell with a uniformly distributed random location. 80% probability outdoor and 20% indoor. 100% network load.

- LOS probability, path loss, and O2I loss follow TR 38.858, 80% low O2I loss, and 20% high O2I loss.

- Calculate SINR in both DL and UL that aggregated co-channel interference (CCI).

- Calculate the percentage of the covered area with > -10 dB SINR with a certain ISD.

Both the DL and UL coverage results with multiple BS antenna array sizes and a constant ISD of 500 m in the 7125 – 8400 MHz band are listed in Table I. The corresponding results at 4 GHz mid-band are presented in Table II for comparison. The DL coverage is always better than the UL coverage. A 4×8 BS array at 4 GHz achieves 94.8% UL coverage, which basically meets the 95% requirement. To achieve a similar 95% UL coverage, the 7125 – 8400 MHz band will need an 8×16 BS array to compensate for the larger propagation losses due to high frequency. Note that the macro BS noise figure is 6 dB in the 7 GHz band and 5 dB at 4 GHz, and the UE noise figure is 10 dB in the 7 GHz band and 9 dB at 4 GHz

Observation 1: The 7125-8400 MHz band has large path loss and outdoor-to-indoor loss due to high frequency. The UL coverage cannot achieve 95% with a 500 m ISD and 4×8 or 8×8 BS antenna array. This band will require a large 8×16 or 16×16 BS array to maintain a 95% UL coverage.

Table I. The 7125 – 8400 MHz band coverage with 500 m ISD.

| Frequency | BS array size | BS gain (dBi) | BS conducted power (dBm) | EIRP (dBm) | ISD (m) | DL coverage | UL coverage |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 7.7625 GHz | 4×8 | 20 | 46 | 66 | 500 | 98.79% | 91.67% |
|  | 8×8 | 23 | 46 | 69 | 500 | 99.21% | 93.17% |
|  | 8×16 | 26 | 46 | 72 | 500 | 99.43% | 94.86% |
|  | 16×16 | 29 | 46 | 75 | 500 | 99.74% | 96.38% |

Table II. Coverage with 500 m ISD at 4 GHz.

|  | BS array size | BS gain (dBi) | BS conducted power (dBm) | EIRP (dBm) | ISD (m) | DL coverage | UL coverage |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 4 GHz | 4×8 | 20 | 46 | 66 | 500 | 99.51% | 94.77% |
|  | 8×8 | 23 | 46 | 69 | 500 | 99.71% | 96.10% |

### Method 2: Monte Carlo simulation for max ISD achieving 95% UL coverage

Based on Method 1 described in Section 3.B, Method 2 changes ISD to achieve a constant 95% UL coverage. The ISD is further converted to the area of the hexagon to estimate the cell coverage area reduction compared to a 500 m ISD being used at 4 GHz. The results are provided in Table III.

Observation 2: The 7125 – 8400 MHz band has large path loss and outdoor-to-indoor loss due to high frequency. The UL coverage at 4 GHz achieves almost 95% with a 500 m ISD and 4×8 BS antenna array. The 7125 – 8400 MHz band will need to reduce the ISD to 325 m (58% coverage area reduction) to maintain 95% UL coverage with a 4×8 BS antenna array, reduce the ISD to 400 m (36% coverage area reduction) to maintain 95% UL coverage with an 8×8 BS antenna array.

Table III. The max ISD in the 7125 – 8400 MHz band achieves 95% UL coverage.

|  | BS array size | BS gain (dBi) | BS conducted power (dBm) | EIRP (dBm) | ISD (m) | Hexagon area (m2) | Coverage area reduction | DL coverage | UL coverage |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 7.7625 GHz | 4×8 | 20 | 46 | 66 | 500 | 649,519 | 0.0% | 98.79% | 91.67% |
|  |  |  |  |  | 400 | 415,692 | 36.0% | 99.18% | 93.68% |
|  |  |  |  |  | 350 | 318,264 | 51.0% | 99.39% | 94.19% |
|  |  |  |  |  | 325 | 274,422 | 57.8% | 99.41% | 94.84% |
|  |  |  |  |  | 320 | 266,043 | 59.0% | 99.63% | 95.23% |
|  |  |  |  |  | 300 | 233,827 | 64.0% | 99.50% | 95.37% |
|  | 8×8 | 23 | 46 | 69 | 500 | 649,519 | 0.0% | 99.07% | 92.71% |
|  |  |  |  |  | 400 | 415,692 | 36.0% | 99.49% | 94.85% |
|  |  |  |  |  | 380 | 375,162 | 42.2% | 99.55% | 95.19% |
|  |  |  |  |  | 350 | 318,264 | 51.0% | 99.62% | 95.77% |
|  |  |  |  |  | 300 | 233,827 | 64.0% | 99.67% | 96.50% |
|  | 8×16 | 26 | 46 | 72 | 500 | 649,519 | 0.0% | 99.43% | 94.86% |
|  |  |  |  |  | 400 | 415,692 | 36.0% | 99.66% | 96.21% |
|  | 16×16 | 29 | 46 | 75 | 500 | 649,519 | 0.0% | 99.74% | 96.38% |
|  |  |  |  |  | 400 | 415,692 | 36.0% | 99.69% | 96.13% |

- Conclusions

In this contribution, terrestrial IMT parameters for the new candidate IMT operation band from 7125 – 8400 MHz are discussed with the following proposals and observations.

Proposal 1: The 7125 – 8400 MHz band is currently licensed to national mission-critical links. It is not feasible to clear this band for public mobile networks (e.g., IMT-2030 or 6G) in many markets, instead, this 7125 – 8400 MHz band will need to be shared. A set of low-power assumptions could make sharing more feasible and a larger portion of the band more available, such as using power class 5 (PC5) for UE.

Proposal 2: Non-AAS with omnidirectional BS antenna and small array sizes (e.g., 2×2, 2×4, or 4×4) parameters are of interest for the 7125 – 8400 MHz band for medium-range and local-area microcells.

Proposal 3: Macro-cell deployment in the 7125 – 8400 MHz band may not be feasible due to large losses in these high frequencies. RAN4 will need to study the feasibility of: (1) what is the coverage with a 500 m UMa ISD? And (2) what is the maximum urban macro (UMa) inter-site distance (ISD) to achieve 95% coverage (> -10 dB SINR) in both DL and UL?

Observation 1: The 7125-8400 MHz band has large path loss and outdoor-to-indoor loss due to high frequency. The UL coverage cannot achieve 95% with a 500 m ISD and 4×8 or 8×8 BS antenna array. This band will require a large 8×16 or 16×16 BS array to maintain a 95% UL coverage.

Observation 2: The 7125 – 8400 MHz band has large path loss and outdoor-to-indoor loss due to high frequency. The UL coverage at 4 GHz achieves almost 95% with a 500 m ISD and 4×8 BS antenna array. The 7125 – 8400 MHz band will need to reduce the ISD to 325 m (58% coverage area reduction) to maintain 95% UL coverage with a 4×8 BS antenna array, reduce the ISD to 400 m (36% coverage area reduction) to maintain 95% UL coverage with an 8×8 BS antenna array.

# 4. Reference

- R4-2400333, Parameters of terrestrial component of IMT for sharing and compatibility studies in the frequency bands 4 400-4 800 MHz, 7 125-8 400 MHz and 14.8-15.35 GHz, ITU-R Working Party 5D.

- RP-240765, New SI proposal: Study on IMT parameters for 4400 to 4800 MHz, 7125 to 8400 MHz and 14800 to 15350 MHz, Ericsson.

- R4-2405729, Technical parameters for the 7125 to 8400 MHz and 14800 to 15350 MHz IMT bands for sharing studies, CableLabs, Charter Communications, Cox Communications, April 2024.

- Ad-hoc minutes [110bis][130] FS_NR_IMT, Ericsson, April 2024.

- R4-2406615, WF on IMT parameters for other frequency ranges, CATT, April 2024.