3GPP TSG-RAN4 Meeting #111	R4-2409771

Fukuoka City, Fukuoka , Japan, 20th – 24th May, 2024

Agenda item:	10.4.2.2

Source:	Keysight Technologies

Title:	NTN OTA Testability Aspects

Document for:	Approval

# Introduction

This contribution briefly addresses NTN OTA testability aspects based on the Rel-19 OTA WI in [1].

# Discussion

In the last RAN4#110-bis meeting, the first meeting since the Rel-19 OTA WI [1] was approved, initial views on the NTN objective were presented and several unresolved aspects were captured in the WF [2].

Regarding the device type for NT-NTN OTA testing, the WF [2] captured the following:

Issue 3-1-1: UE type for NR-NTN Way forward: - RAN4 further check whether the test method/system can be generic for different UE type. If not, further discuss potential prioritization for different UE types (e.g., Handheld and FR1 VSAT-like device).

While the WID [1] did not capture anything specific about device types, the target DUT types for NTN OTA testing could potentially not just be limited to handheld devices but also Very Small Aperture Terminal (VSAT) like devices as stated in the WF [2]. This is a bit peculiar as VSAT or VSAT like devices were not in scope for FR1 NTN, see Rel-17 of [3].

Observation 1: VSAT or VSAT like devices were not in scope for FR1 NTN in REl-17

Proposal 1: A clarification is needed whether VSAT or VSAT like devices should be in scope of this WI.

In case such VSAT like devices are considered in scope, the typical testability device information is needed, e.g., device size, weight, typical antenna patterns. NR FR1 OTA systems are currently defined with a 30 cm quiet zone and it is not clear whether VSAT-like device types even fit into this quiet zone due to the potentially very large and highly directive antenna.

Observation 2: VSAT-like devices might not fit into the NR FR1 OTA quiet zone of 30 cm.

Proposal 2: If VSAT like devices are in scope, OEMs/Satellite Operators to provide testability device information for VSAT-like device types, e.g., device size, weight, typical antenna patterns.

Similarly, the UE Type for IoT-NTN seems to be unknown as well for now

Issue 3-1-2: UE type for IoT-NTN Way forward: - RAN4 could further discuss the target UE form factors and corresponding size, antenna design, etc., for IoT NTN OTA

Proposal 3: OEMs/Satellite Operators to provide testability device information for IoT-NTN device types, e.g., device size, weight, typical antenna patterns.

A number of suitable performance metrics were proposed by various companies in RAN4#100-bis. as captured in the WF [2]:

Issue 3-2-2: Proper performance metric for NR-NTN Way forward: Consider the following initial input as a starting point for further discussions: - Option 1: consider the following aspects - Option 1: integrated power/sensitivity within declared half sphere - Option 2: peak EIRP/EIS only - Option 3: peak EIRP/EIS + X%-tile spherical coverage within declared half sphere - Option 4: peak EIRP/EIS + Y%-tile spherical coverage from whole sphere - Option 2: consider the following aspects - Consider a general framework for each UE type, to specify performance metric for different use cases / power classes, e.g., a set of metric for Handheld, and other set for FR1 VSAT-like UE. - Consider the assumption: UE elevation angles for NTN are typically >20°, in majority of cases. - Consider TRP, TRS, EIRP, and EIS as starting point. Further discuss other performance metric based on NTN usage scenarios, e.g., directivity requirements, Antenna Gain. - Option 3: consider the following aspects - consider WRP and WRS defined in R4-2404278 as performance metrics for NTN devices - consider EIRP and EIS CDF percentile thresholds as performance metrics for NTN devices - Option 4: Conical TRP/EIRP for Tx and conical TRS/EIS for Rx could be considered as the starting point for the metric of NTN device OTA testing. FFS on the range of q and f. - Option 5: Consider the following aspects for NTN performance metric - Adopting a directional antenna pattern as the starting point for the antenna characteristics of an FR1 NTN device. - Take UIRP and UHIS as the as the starting point for the performance metric of devices using directional antenna. - Further discuss whether a smaller angle of test scan can be used or whether a EIRP/EIS CDF-like performance metric and be considered. - Option 6: other aspects are not precluded

For handheld UEs, some OTA requirements and use cases were outlined by satellite operators in [4], i.e.,

- For handheld/smartphone UE – UE may be upside-down when a call is initiated (e.g. in pocket): - If we assume only synchronization and paging typically occur when UE is upside down, we can still sacrifice some antenna performance on the bottom hemisphere, due to lower SNR requirements for the physical channels associated with these procedures. - When data transfer is occurring– e.g. PUSCH/PDSCH and the UE is pointing upwards, it is advantageous to have a higher antenna performance in the upper hemisphere to allow for a higher link margin. - This would also reduce the antenna noise temperature, meaning the system noise is less significant, which increases G/T, increasing the DL link margin, while increasing the UL margin due to higher gain towards the satellite orbital arc. - Assessing the directivity of the antenna can be done with a similar effort level to obtaining the TRP, TRS and EIS values, and would allow for more detailed link level analysis.

Given this essential input, it is proposed not to consider single-directional metrics such as peak EIRP/EIS and/or partial radiated quantities such as the spherical integration over a cone or single hemisphere as devices might not always be placed in the optimized orientation.

Proposal 4: No longer consider single-directional metrics such as peak EIRP/EIS and/or partial radiated quantities such as the spherical integration over a cone or single hemisphere

Instead, it is proposed to consider metrics that take into account a full 3D scan while not necessarily relying on equal spatial weighting of EIRP/EIS, i.e., the following candidate metrics could be considered suitable as they do take a full 3D assessment into account but allow the antenna performance in one direction/hemisphere to dominate the performance metric:

- Spherical Coverage with EIRP-CDF and EIS-CCDF at [>50%]-ile

- Weighted TRP: wdominant * PRPhemi1 + (1-wdeclared) * PRPhemi2 and Weighted TRS: 1/{[wdominant * PRPhemi1]-1+ [(1-wdeclared) * PRPhemi2)]-1} where the OEM declares one hemisphere (hemi1) to be the dominant one which is weighted higher than the opposite hemisphere (hemi2) with a weight of wdominant >0.5.

- Weighted Radiated Power, WRP, and Weighted Radiated Sensitivity, WRS, as discussed in [5], [6]

Proposal 5: Focus on full 3D assessment but allow the antenna performance in one direction/hemishpere to dominate the metric, e.g., spherical coverage with EIRP-CDF and EIS-CCDF at [>50%], weighted TRPs/TRSs, weighted radiated power/sensitivity rather than conventional TRP/TRS.

# Conclusion

The following observations and conclusions were made in this contribution.

Observation 1: VSAT or VSAT like devices were not in scope for FR1 NTN in REl-17

Observation 2: VSAT-like devices might not fit into the NR FR1 OTA quiet zone of 30 cm.

Proposal 1: A clarification is needed whether VSAT or VSAT like devices should be in scope of this WI.

Proposal 2: If VSAT like devices are in scope, OEMs/Satellite Operators to provide testability device information for VSAT-like device types, e.g., device size, weight, typical antenna patterns.

Proposal 3: OEMs/Satellite Operators to provide testability device information for IoT-NTN device types, e.g., device size, weight, typical antenna patterns.

Proposal 4: No longer consider single-directional metrics such as peak EIRP/EIS and/or partial radiated quantities such as the spherical integration over a cone or single hemisphere

Proposal 5: Focus on full 3D assessment but allow the antenna performance in one direction/hemishpere to dominate the metric, e.g., spherical coverage with EIRP-CDF and EIS-CCDF at [>50%], weighted TRPs/TRSs, weighted radiated power/sensitivity rather than conventional TRP/TRS.

# References

- RP‑240841, New WID: WI on TRP (Total Radiated Power), TRS (Total Radiated Sensitivity) and MIMO OTA (Over the Air) testing enhancement Phase 3, vivo, CAICT, RAN meeting #103, Mar 2024

- R4-2406086, Way Forward for [110bis][338] TRP_TRS_MIMO_OTA, vivo, CAICT, 3GPP TSG-RAN WG4 Meeting #110bis, April 2024

- TS 38.101-5, User Equipment (UE) radio transmission and reception; Part 5: Satellite access Radio Frequency (RF) and performance requirements

- RP-240615, Satellite Operator views on OTA requirements for NTN, Inmarsat, Viasat, RAN#103

- R4-2404278, on test methodology and radiated performance metric for FR1 NTN devices, Huawei, HiSilicon, 3GPP TSG-RAN WG4 Meeting # 110bis, April 2024

- R4-2404141, On Single-Layer UL MIMO for coherent UEs, Keysight Technologies, vivo, 3GPP TSG-RAN4 Meeting #110bis, April 2024