3GPP TSG-RAN4 Meeting #111		R4-2409784

Fukuoka, Japan, May 20th – 24th, 2024

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.133 | CR | 4606 | rev |  | Current version: | 18.5.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Draft CR on concurrent Pre-MG dynamic collision |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | MediaTek inc. |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_MG_enh2-Core |  |  |  |  |  | Date: |  |  | 2024-05-10 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | In WF R4-2406439, the agreement is: - Dynamic collision means when the occasion of Pre-MG with higher priority is involved during the gap collision, where the occasion of other MG/Pre-MG has lower priority. - With the main bullet, it includes the scenarios for higher priority Pre-MG activation/deactivation procedure colliding with other MG/Pre-MG instance within 4ms. - Further refine the wording for the UE features. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Change 1: add definition of dynamic collision Change 2: Fix some typo mistakes. Change 3: add capability condition, and improve the current writing. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The spec might not be accurate. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | Clauses 3.1, 8.19.5.1, 8.19.5.2, 8.19.5.3, 9.1.12.4 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TS 38.533 |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

------------------------------ Start of Change 1 ------------------------------

## 3.1	Definitions

For the purposes of the present document, the terms and definitions given in TR 21.905 [11] and the following apply. A term defined in the present document takes precedence over the definition of the same term, if any, in TR 21.905 [11].

1 Rx RedCap: RedCap UE for which requirements are derived assuming 1 Rx branch.

2 Rx RedCap: RedCap UE for which requirements are derived assuming 2 Rx branches.

Active DL BWP: Active DL bandwidth part as defined in TS 38.213 [3].

Blackbox Approach: Testing methodology, in which the UE internal implementation of certain specific UE functionality involved in the test, is unknown.

CD-SSB: Cell defining SSB as defined in TS 38.300 [10].

Control Resource Set: As defined in TS 38.213 [3].

DL BWP: DL bandwidth part as defined in TS 38.213 [3].

Dynamic collision: When the occasion of Pre-MG with higher priority is involved during the gap collision in concurrent measurement gaps with Pre-MG.

EN-DC: E-UTRA-NR Dual Connectivity as defined in clause 4.1.2 of TS 37.340 [17].

en-gNB: As defined in TS 37.340 [17].

FH: As defined in TS 38.214 [26].

FR1: Frequency range 1 as defined in clause 5.1 of TS 38.104 [13].

FR2: Frequency range 2 as defined in clause 5.1 of TS 38.104 [13].

gNB: as defined in TS 38.300 [10].

IBM (Independent Beam Management): As defined in TS 38.101-2 [19].

IDC solution: As described in TS 36.300 [24] and TS 38.300 [10].

LMF: as defined in TS 38.305 [22].

Master Cell Group: As defined in TS 38.331 [2].

Multi-Radio Dual Connectivity: Dual Connectivity between E-UTRA and NR nodes, or between two NR nodes, as defined in TS 37.340 [17].

NCD-SSB: Non cell defining SSB as defined in TS 38.300 [10].

ng-eNB: As defined in TS 38.300 [10].

NE-DC: NR-E-UTRA Dual Connectivity as defined in clause 4.1.3.2 of TS 37.340 [17].

NGEN-DC: NG-RAN E-UTRA-NR Dual Connectivity as defined in clause 4.1.3.1 of TS 37.340 [17].

NR-DC: NR-NR Dual Connectivity as defined in clause 4.1.3.3 of TS 37.340 [17].

Primary Cell: As defined in TS 38.331 [2].

PRS resource instance: An instance in time of a configured PRS resource as defined in TS 38.331 [2], which may or not overlap with a measurement gap occasion.

Quasi Co-Location: As defined in TS 38.214 [26].

RedCap UE: A UE with reduced capabilities as defined in clause 4.2 in TS 38.306 [14].

RLM-RS resource: A resource out of the set of resources configured for RLM by higher layer parameter RLM-RS-List [2] as defined in TS 38.213 [3].

SA operation mode: Operation mode when the UE is configured with at least PCell and not any MR-DC.

Secondary Cell: As defined in TS 38.331 [2].

Secondary Cell Group: As defined in TS 38.331 [2].

Serving Cell: As defined in TS 38.331 [2].

SMTC: An SSB-based measurement timing configuration configured by SSB-MeasurementTimingConfiguration as specified in TS 38.331 [2].

Special Cell: As defined in TS 38.331 [2].

SSB: SS/PBCH block as defined in clause 7.8.3 of TS 38.211 [6].

Timing Advance Group: As defined in TS 38.331 [2].

----------------------------- End of Change 1 -----------------------------

------------------------------ Start of Change 2 ------------------------------

### 8.19.5	Activation/deactivation delay requirements for concurrent measurement gaps with Pre-MG

The requirements in this clause apply to a UE configured with concurrent measurement gaps with Pre-MG.

#### 8.19.5.1	Activation/deactivation delay requirements for non-overlapped activation/deactivation of concurrent measurement gaps with Pre-MGs

The requirements in this clause only apply when the activation/deactivation procedures of the individual pre-configured measurement gaps do not overlap in time.

When concurrent measurement gaps with Pre-MG activation/deactivation procedure are non-overlapped upon DCI/timer-based BWP switch, upon SCell activation/deactivation or upon RRC reconfiguration, for each individual pre-configured measurement gap, the requirements defined in clauses 8.19.2, 8.19.3 and 8.19.4 apply.

#### 8.19.5.2	Activation/deactivation delay requirements for fully overlapped activation/deactivation of concurrent measurement gaps with Pre-MGs

The requirements in this clause only apply when the activation/deactivation procedures of the individual pre-configured measurement gaps fully overlap in time.

Fully overlapped activation/deactivation of pre-configured measurement gaps can occur in the following cases:

-	Both pre-configured measurement gaps are triggered by the same event.

-	Two pre-configured measurement gaps are triggered by two events of the same type at the same time.

When concurrent measurement gaps with Pre-MG are activated/deactivated simultaneously, the activation/deactivation delay equals the delay requirements defined at clause 8.19.2, 8.19.3 and 8.19.4 for DCI/timer-based BWP switch, SCell activation/deactivation or RRC reconfiguration triggered activation/deactivation, respectively, plus an additional 2ms post-processing time.

#### 8.19.5.3	Pre-configured measurement gap activation/deactivation delay when colliding with a concurrent measurement gap

When the pre-configured measurement gap activation procedure is overlapped with a concurrent measurement gap occasion and the pre-configured measurement gap has higher priority, the pre-configured gap activation shall be applied no earlier than the ending point of the Pre-MG activation procedure and 5ms after the end of the overlapping measurement gap occasion.

When the pre-configured measurement gap deactivation procedure is overlapped with one measurement gap occasion and the pre-configured measurement gap has higher priority, or when the pre-configured measurement gap activation/deactivation procedure is overlapped with one measurement gap occasion and the measurement gap has higher priority, requirements specified in 8.19.2 apply.

----------------------------- End of Change 2 -----------------------------

------------------------------ Start of Change 3 ------------------------------

#### 9.1.12.4	Collision between Pre-MG activation/deactivation and measurement gap

A measurement gap occasion and a Pre-MG activation/deactivation procedure collide when the ending point of the Pre-MG activation/deactivation procedure occurs anywhere within a time period starting 4ms before the starting point of the gap occasion and ending 4ms after the ending point of the gap occasion. The ending point of the Pre-MG activation/deactivation procedure is defined in clause 8.19.5.3.

If a UE supports the [dynamic collision] capability, then:

-  	When a collision occurs between a measurement gap occasion and a Pre-MG activation procedure, and the Pre-MG is configured with higher priority, i.e. dynamic collision scenario, the UE shall perform measurements during the measurement gap occasion and the activation of the Pre-MG is delay is defined in clause 8.19.5.3.

-  	In dynamic collision scenarios: When a collision occurs between a measurement gap occasion and a Pre-MG deactivation procedure, and the Pre-MG is configured with higher priority, the measurement gap occasion shall be dropped. The measurement gap occasion shall remain to be dropped until the ending point of the Pre-MG deactivation procedure.

When the activated Pre-MG and measurement gap satisfy the collision rule defined in clause 9.1.8.3 and the Pre-MG is configured with lower priority, the UE shall perform measurements in the occasion of the measurement gap regardless of whether it collides with the Pre-MG activation procedure or collides with the Pre-MG deactivation procedure.

The UE is expected to transmit PUCCH/PUSCH/SRS or receive PDCCH/PDSCH/TRS/CSI-RS for CQI in the corresponding NR serving cells in the slots of the configured Pre-MG that are dropped according to the requirements in clause 9.1.8.4.

----------------------------- End of Change 3 -----------------------------