3GPP TSG-RAN4 Meeting #111	   											R4-2409787

# Fukuoka, Japon, 20th of May 2024 - 24th of May 2024

Title: 	General aspects for NTN NR Phase 3

Agenda Item:	10.5.1

Source: 	THALES

Document for:	Information

# For information

As described in the Way Forward for [110bis][315] NR_NTN_Ph3 (R4-2406109, THALES), the following agreement has been made at previous RAN4 meeting:

DL coverage enhancements for FR1-NTN or FR2-NTN

Issue 1-3-1: Cell DTX

Issue 1-3-2: RF requirements enhancements for spatial domain techniques

Encourage companies to provide more inputs with beam switching delay and scenarios for deployment (switch pattern, RAN1 simulation assumption scenarios, etc.).

Consider e.g. analog/digital FR2/FR1 implementation.

This document further provides information from RAN1 scenarios for DL coverage enhancements/beam hopping and other Rel-19 aspects. Please find RAN1#116 and RAN1#116bis agreements:

RAN1#116 made the following agreements: Agreement For DL coverage study, consider the following additional reference satellite parameters scenarios for LEO600km Set1 in FR1 (i.e., S-band), referred to as Set1-1 FR1, Set1-2 FR1 and Set1-3 FR1: LEO600km Set1-1 FR1 (i.e., S-band). Maximum Bandwidth per beam; 5 MHz. SCS; 15 kHz. Beam size(Note 1); 50km. Satellite EIRP density /beam (dBW/MHz); 34. Payload Total DL power level (dBW); 31.24. Aggregated EIRP (Total) (dBW); 61.24*. Satellite Tx max Gain; 30 dBi. Maximum EIRP per Satellite beam (dBW); 41. Total number of beam footprints***; 1058. Total number of simultaneously active beams **; 106. % simultaneously active beams**; 10.02 %. *Note: EIRP limit is 61.24 dBm for the reference configuration. **Assuming 100 % Resource Block utilization within the same beam at max power. Absolute number of simultaneously active beams is up to 212 (due to limitation of RF) *** For a constellation design at 600km with low elevation angle with 30° and selected (i.e Set 1 parameters) beam size Note 1: At least this beam size is considered in this scenario, larger beam sizes maybe evaluated and reported by companies. LEO600km Set1-2 FR1 (i.e., S-band). Maximum Bandwidth per beam; 5 MHz. SCS; 15 kHz. Beam size (note 1); 50km. Satellite EIRP density /beam (dBW/MHz); 34. Payload Total DL power level (dBW); 23. Aggregated EIRP (Total) (dBW); 53*. Satellite Tx max Gain; 30 dBi. Maximum EIRP per Satellite beam (dBW); 41. Total number of beam footprints; 1058. Total number of simultaneously active beams**; 16. % simultaneously active beams**; 1.5 %. *Note: EIRP limit is 53 dBm for the reference configuration. **Absolute number of simultaneously active beams is up to 16 (due to limitation of RF) Note 1: At least this beam size is considered in this scenario, larger beam sizes maybe evaluated and reported by companies. LEO600km Set 1-3 FR1 (i.e., S-band). Maximum Bandwidth per beam; 5 MHz. SCS; 15 kHz. Beam size (note 1); 50km. Satellite EIRP density /beam (dBW/MHz); 26. Payload Total DL power level (dBW); 23.24. Aggregated EIRP (Total) (dBW); 53.24*. Satellite Tx max Gain; 30 dBi. Maximum EIRP per Satellite beam (dBW); 33. Total number of beam footprints; 1058. Total number of simultaneously active beams**; 106. % simultaneously active beams**; 10.02 %. *Note: EIRP limit is 53.24 dBm for the reference configuration. **Absolute number of simultaneously active beams is up to 212 (due to limitation of RF) Note 1: At least this beam size is considered in this scenario, larger beam sizes maybe evaluated and reported by companies. Note: RAN1 will aim to identify necessary enhancements for these scenarios in the study phase. At the end of the study phase, RAN1 will further discuss whether the potential enhancements will be specified within Rel-19 framework. Agreement For DL coverage study at system level, consider the following additional reference satellite payload parameters for LEO600km in FR2 (i.e., Ka-band): LEO600km Set1-1 FR2 (i.e., Ka-band). Maximum Bandwidth per beam; 400 MHz. SCS; 120 kHz. Beam size; TBD in next meeting. Satellite EIRP density /beam (dBW/MHz). Payload Total DL power level (dBW). Aggregated EIRP (Total) (dBW). Satellite Tx max Gain. EIRP per Satellite beam (dBW). Total number of beam footprints; 800 (note 1). Total number of simultaneously active beams; 12. % simultaneously active beams; 1.5 %. Note 1: A typical deployment scenario in FR2 should consider 800 satellites beams per a single satellite coverage area with an absolute number of simultaneously active beams equal to 16 (due to limitation of RF). Agreement Adopt the following phased array antenna parameters for LEO 600km in FR1: Satellite phased array antenna Characteristics; LEO-600. Orbit; LEO-600km. Frequency range/band; FR1/S-Band. Antenna element pattern; Table7.3-1 in TR 38.901. Horizontal/vertical 3 dB beam width of single element (degree); [65] for H [65] for V. Antenna polarization; Circular (RHCP or LHCP). Number of antenna elements; [400 elements (20 x 20)]. Equivalent satellite antenna aperture; 2m. Element maximum gain; 4 dBi. Antenna maximum gain; 30 dBi. Steering loss at 30° elevation angle; [4dB]. Agreement RAN1 to consider the following performance metrics for DL Coverage enhancement evaluation at system level: At least: - CDF of the received SINR - The dwell time and revisit time interval for each beam illumination across the coverage - Periodicity of common control channels (e.g. SSB, CORESET0/SIB1, SIB19) and corresponding coverage ratio Other metrics may be reported such as - CDF of the cell throughput - CDF of user perceived throughput (UPT) - CDF of Latency - Ratio of mean served cell throughput and offered cell throughput, denoted by 𝜌 (refer to TR36.889) For system level study based on analytical evaluation: - N1 beam footprints are in state “off” - These beam footprints are not served by any signal (no satellite service in this area) - N2 beam footprints are in state “common messages only” - These beam footprints do not have any active user traffic, and are served the necessary information for cell discovery and initial access. - Optionally, companies may consider user arrival (e.g. RACH access) in this type of cell, and should describe how this is taken into account in the analytical evaluation - N3 beam footprints are in state “active traffic” - These beam footprints have X active (e.g. VoNR) users each. - These beam footprints are also served the necessary information for cell discovery and initial access - N1 + N2 + N3 = “Total number of beam footprints “ - N1, N2, N3, X are to be reported by companies. - Resource utilization obtained under the assumptions above is to be reported by companies. - Other assumptions made in the evaluation are to be reported by companies, e.g. power sharing scheme, beam hopping scheme, etc. Agreement For NR NTN Rel-19 DL coverage evaluation, UE characteristics for handheld terminals in Table 6.1.1.1-3 in TR 38.821 can be reused, with the following: - -5.5 dBi antenna gain is assumed - at least 2Rx are considered at the UE - 4Rx can be optionally considered and reported Note: Redcap device is not considered in the scope of DL coverage study Agreement The following traffic models are considered for system level evaluation of DL coverage: - FTP3: as in Table 6.1.1.1-7 of TR 38.821: 0.5MB as packet size, 200ms as mean inter-arrival time - FTP3 IM: 0.1MB as packet size, 2s as mean inter-arrival time - VoIP can be considered in the evaluation. It is up to company report which traffic model is used among the discussed traffic models in their evaluations. - Other models may be used as well, and parameter (e.g. packet size and arrival rate) adjustment can be optionally considered and reported. Traffic type; FTP; IM; VoIP. Model; FTP model 3; FTP model 3; As defined in Rel-18 NTN CE.. Packet size; 0.5 Mbytes; 0.1 Mbytes. Mean inter-arrival time; 200 ms; 2 sec. Agreement For NR NTN Rel-19 DL coverage evaluation, Beam layout defined in Table 6.1.1.1-4 in TR 38.821 can be reused. - Using other beam layouts is not precluded, and should be reported by companies Agreement For NR NTN Rel-19 DL coverage evaluation, a value of beam steering latency equal to 0 at least if phase array antenna is assumed. Values different from 0 can be optionally reported Agreement DL coverage is evaluated at link level with the following considerations: - NGSO at LEO-600 operating in FR1 is considered in priority - Additional satellite payload parameters defined for system level evaluation are used - FFS: Antenna gain reduction due to steering loss can be considered Agreement For the evaluation of NTN downlink coverage at link level, reuse the target data rate from Rel-18 NTN Coverage enhancements: - For VoIP: AMR 4.75 kbps (TBS of 184 bits without CRC in physical layer) with 20 ms data arriving interval - For data rate service: both 3 kbps and 1Mbps can be considered - Companies can also use the data rates corresponding to the traffic types used for system level evaluations Agreement For link-level study, downlink coverage performance in NR NTN is evaluated according to the following steps. Step 1: CNR is calculated as defined in 6.1.3.1 of TR 38.821 Step 2: Required SNR of target service is evaluated by LLS Step 3: The CNR and the required SNR are compared Agreement For link-level study, for NR NTN DL coverage enhancement, the following channels/signals can be considered for evaluations: - PDSCH for VoIP - PDSCH for low data rate service - PDSCH Msg.2 - PDSCH Msg.4 - PDSCH carry SIB, e.g., SIB1, SIB 19 - PDSCH for paging - PDCCH - Broadcast PDCCH (e.g. PDCCH of Msg.2, paging) - SSB Note: RAN1 will aim to identify necessary link-level enhancements for these channels in the study phase. At the end of the study phase, RAN1 will further discuss whether the potential link-level enhancements will be specified within Rel-19 framework. Agreement For DL coverage performance evaluation, the following are assumed for all channels/signals - Channel model/Delay spread: - Channel model as in Table 6.1.2-4 of TR38.821, NTN-TDL-C (LOS) - Evaluation scenario: - Rural (LOS) - Channel estimation: Realistic estimation: - Companies are encouraged to report channel estimation method. - SCS: - 15 kHz only - UE speed: 3 km/h - Frequency drift: TBD - Frequency offset: 0.1 ppm Agreement For link budget calculation, parameters in the following table are assumed: Parameters. Carrier frequency; 2 GHz for DL (S-band). Satellite altitude; 600 km. Target elevation angle; 30° (LEO). Atmospheric loss; Equation (6.6-8) in [38.811]. Shadowing margin; 3 dB. Scintillation loss; Section 6.6.6 in [38.811] Ionospheric loss: = 2.2 dB Tropospheric loss: Table 6.6.6.2.1-1 of [38.811]. Additional loss; 0 dB. Clear sky conditions; Yes. Satellite antenna polarization; Circular polarization. Terminal type; [S band: (M, N, P) = (1,1,2)]. UE antenna gain; -5.5dBi. Free space path loss; Equation (6.6-2) in [38.811]. Polarization loss; 3dB. Outcome; CNR.

RAN1#116bis made the following agreements: Agreement For coverage evaluation of PDCCH in NR NTN, the following table is assumed: Parameter; Value. Number of UE receive chains; 2 for 2GHz. Aggregation level; 8. Payload; 40 bits. CORESET size; 2 symbols, 24 PRBs. Tx Diversity; Reported by companies. BLER; 1% BLER optional for 10% BLER. Number of SSB for broadcast PDCCH of Msg.2; Reported by companies. Other parameters; Reported by companies. Agreement For coverage evaluation of PDSCH in NR NTN, the following table is assumed: Parameter; Value. BLER; For low data rate service, w/ HARQ, 10% iBLER; w/o HARQ, 10% iBLER. For VoIP, 2% rBLER.. Waveform; CP-OFDM. Number of UE receive chains; 2 for 2GHz. HARQ configuration; Whether/How HARQ is adopted is reported by companies.. DMRS configuration; 3 DMRS symbols is used for PDSCH of Msg.2. For 3km/h: Type I, 1 or 2 DMRS symbol, no multiplexing with data. PDSCH mapping Type, the number of DMRS symbols and DMRS position(s) are reported by companies.. PRBs/TBS/MCS for data rate service; Any value of PRBs, and corresponding MCS index, reported by companies will be considered in the discussion. TBS can be calculated based on e.g. the number of PRBs, target data rate, frame structure and overhead. 24 PRBs for SIB1 and SIB19. PRBs/MCS for VoIP; Any value of PRBs reported by companies will be considered in the discussion. QPSK. PDSCH duration; 12 OS. Payload size for PDSCH of Msg.2; 72 bits. Payload size for PDSCH of SIB1; FFS. Payload size for PDSCH of Msg.4; 1040 bits. Payload size for PDSCH of SIB19; FFS. Other parameters; Reported by companies.. Agreement Antenna gain reduction due to steering loss is not considered in the link level evaluation. Note: This is aligned with the assumptions made in Rel-18 UL coverage enhancement Observation The CNRs for the satellite payload parameters Set 1-1, Set 1-2 and Set 1-3 are equal to -1.9 dB, -1.9 dB and -9.9 dB respectively. Agreement Confirm the Satellite phased-array antenna parameters for LEO 600km in FR1 defined in RAN1#116. Satellite phased array antenna Characteristics. Orbit; LEO-600km. Frequency range/band; FR1/S-Band. Antenna element pattern; Table7.3-1 in TR 38.901. Horizontal/vertical 3 dB beam width of single element (degree); 65 for H 65 for V. Antenna element spacing; 0.667 lambda. Antenna polarization; Circular (RHCP or LHCP). Number of antenna elements; 400 elements (20 x 20). Equivalent satellite antenna aperture; 2m. Element maximum gain; 4 dBi. Antenna maximum gain; 30 dBi. Steering loss at 30° elevation angle; 4 dB. Al least the above model is considered for SLS to ease the alignment between evaluation results. The model below can be optionally considered: Satellite phased array antenna Characteristics. Orbit; LEO-600km. Frequency range/band; FR1/S-Band. Antenna element pattern; TR38.820 section 7.2.4. Horizontal/vertical 3 dB beam width of single element (degree); 90 for H 90 for V. Antenna element spacing; 0.5 lambda. Antenna polarization; Circular (RHCP or LHCP). Number of antenna elements; 676 elements (26 x 26). Equivalent satellite antenna aperture; 2m. Element maximum gain; 4 dBi. Antenna maximum gain; 30 dBi (Note 1). Steering loss at 30° elevation angle; 2.5 dB. Note 1: The maximum antenna gain is determined by considering an overall array efficiency [of 50%.] R1-2401992 FL Summary #2: NR-NTN downlink coverage enhancements THALES Agreement For coverage evaluation of PDSCH in NR NTN, the following payload sizes for PDSCH are assumed: Payload; value. Payload size for PDSCH of SIB1; Option 1: 800 bits Option 2: 1280 bits. Payload size for PDSCH of SIB19; 616 bits. Note: At least the above values are simulated and reported. Other values can be considered. Note: the values above are not the TBS. Agreement For DL coverage study at system level, it is up to companies to report the following parameters for LEO600km Set1-1 FR2: Beam size. Satellite EIRP density /beam (dBW/MHz). Payload Total DL power level (dBW). Aggregated EIRP (Total) (dBW). Satellite Tx max Gain. EIRP per Satellite beam (dBW). Agreement For coverage performance evaluation of DL channels/signals before the SIB19 acquisition, the maximum Doppler frequency drift is assumed to be equal to 0.27 ppm/s based on TR 38.821.

For other information, please check RAN1 contribution R1-2401988 (3GPP TSG RAN WG1 Meeting #116bis), THALES.

# Proposals

Proposal 1. For NR NTN Rel-19 DL coverage evaluation, a value of beam steering latency equal to zero at least if SAN phase array antenna is assumed.

Proposal 2. Values different from zero beam steering latency can be optionally reported by companies.

Proposal 3. Other implementation with analog beam steering technologies are not precluded. Companies to further discuss if analog/digital antenna assumptions and for which FR1/FR2 implementations.

Proposal 4. Further discuss remaining issues from Way Forward for [110bis][315] NR_NTN_Ph3 (R4-2406109):

Issue 1-3-3: Network energy saving

Proposal 1:

- Network energy saving feature in Rel-18 can be starting point for RF impact analysis for Rel-19 NTN DL coverage enhancement objective. (P1/R4-2404869)

- No need to consider the spatial and power domain in Rel-18 NES impact on NTN RF for now. (P3/R4-2404871)

Proposal 2:

- It’s FFS whether any SAN Tx power requirements for objective 1 with power sharing between satellite beams or different satellite beam patterns/size (i.e. wide or narrow) across the satellite footprint.  (O1/R4-2405317)

UL capacity/throughput Enhancements for FR1-NTN

Issue 1-4-1: OCC

Proposals:

- Wait RAN1 reach conclusions on OCC feature before RAN4 start to evaluate the RF impact. (P2/R4-2404869)

- RAN4 waits for further progress on RAN1 work items to identify the RF requirement enhancements needed for NR NTN phase 3 for objective 2 (UL capacity/throughput Enhancements). (P2/R4-2405082)

- No RAN4 SAN RF requirements impact foreseen for Rel-19 NTN Phase-3 WI Objectives 2 (UL capacity/throughput Enhancements) (O2/R4-2405317)

Broadcast service

Issue 1-5-1: RAN4 SAN RF impact

Proposals:

- No RAN4 SAN RF requirements impact foreseen for Rel-19 NTN Phase-3 WI Objectives 3 broadcast service) (O2/R4-2405317)