3GPP TSG-RAN WG4 Meeting #111                                                                                       R4-2409847

Fukuoka, Japan, May 20 – May 24, 2024

Source: 	CATT

Title: 	Updated simulation results for SAN demodulation requirements for above 10 GHz bands

Agenda item:	7.16.8.1

Document for:	Approval

- Introduction

In RAN4#110bis meeting, the simulation assumption on demodulation requirements for NR NTN enhancements was agreed [1].

This contribution provides initial simulation results for SAN demodulation requirements for alignment.

- Discussion

Based on the agreed simulation assumptions [1], presented below are our preliminary simulation outcomes for PUSCH and PRACH concerning SAN demodulation:

2.1 Ka-band PUSCH

The simulation results for PUSCH are shown in Table2.1-1.

Table 2.1-1: Simulation results for PUSCH

| Normal PUSCH with CP-OFDM |  | CATT |  |
| --- | --- | --- | --- |
|  |  | Ideal | Impairment |
| MCS2 | 1x1 | -2.6 | -0.1 |
| MCS2 | 1x2 | -6.0 | -3.5 |
| MCS12 | 1x1 | 6.4 | 8.9 |
| MCS12 | 1x2 | 3.0 | 5.5 |
| Normal PUSCH with DFT-s-OFDM |  | CATT |  |
|  |  | Ideal | Impairment |
| MCS2 | 1x1 | -2.5 | 0.0 |
| MCS2 | 1x2 | -5.8 | -3.3 |
| PUSCH repetition Type A |  | CATT |  |
|  |  | Ideal | Impairment |
| MCS2 | 1x1 | -5.6 | -3.1 |
| MCS2 | 1x2 | -8.7 | -6.2 |

2.2 Ka-band PUCCH

The simulation results for PUCCH are shown in Table2.1-1.

| PUCCH format 0 |  |  | CATT |  |
| --- | --- | --- | --- | --- |
|  |  |  | Ideal | Impairment |
| 1 symbol | 1x1 | 1% of ACK missed detection | 2.9 | 5.4 |
| 1 symbol | 1x2 | 1% of ACK missed detection | 0.7 | 3.2 |
| 2 symbol | 1x1 | 1% of ACK missed detection | 0.7 | 3.2 |
| 2 symbol | 1x2 | 1% of ACK missed detection | -2.0 | 0.5 |
| PUCCH format 1 |  |  | CATT |  |
|  |  |  | Ideal | Impairment |
| 14 symbol | 1x1 | 0.1% of NACK to ACK | -3.2 | -0.7 |
| 14 symbol | 1x1 | 1% of ACK missed detection | -3.5 | -1.0 |
| 14 symbol | 1x2 | 0.1% of NACK to ACK | -7.2 | -4.7 |
| 14 symbol | 1x2 | 1% of ACK missed detection | -7.5 | -5.0 |
| PUCCH format 2 (ACK missed detection) |  |  | CATT |  |
|  |  |  | Ideal | Impairment |
| 1 symbol | 1x1 | 1% of ACK missed detection | 1.7 | 4.2 |
| 1 symbol | 1x2 | 1% of ACK missed detection | -2.3 | 0.2 |
| PUCCH format 2 (UCI BLER) |  |  | CATT |  |
|  |  |  | Ideal | Impairment |
| 1 symbol | 1x1 | 1% of UCI BLER | -0.2 | 2.3 |
| 1 symbol | 1x2 | 1% of UCI BLER | -4.7 | -2.2 |
| PUCCH format 3 |  |  | CATT |  |
|  |  |  | Ideal | Impairment |
| Additional DMRS | 1x1 | 1% of UCI BLER | -1.1 | 1.4 |
| Additional DMRS | 1x2 | 1% of UCI BLER | -5.0 | -2.5 |
| No additional DMRS | 1x1 | 1% of UCI BLER | 0.4 | 2.9 |
| No additional DMRS | 1x2 | 1% of UCI BLER | -4.1 | -1.6 |
| PUCCH format 4 |  |  | CATT |  |
|  |  |  | Ideal | Impairment |
| Additional DMRS | 1x1 | 1% of UCI BLER | 0.7 | 3.2 |
| Additional DMRS | 1x2 | 1% of UCI BLER | -3.9 | -1.4 |
| No additional DMRS | 1x1 | 1% of UCI BLER | 1.3 | 3.8 |
| No additional DMRS | 1x2 | 1% of UCI BLER | -2.7 | -0.2 |

2.3 Ka-band PRACH

The simulation results for PRACH missed detection are shown in Table 2.3-1.

Table 2.3-1: Simulation results for PRACH missed detection

| PRACH |  | CATT |  |
| --- | --- | --- | --- |
|  |  | Ideal | Impairment |
| B4 | 1x1 | -10.3 | -7.8 |
| B4 | 1x2 | -14.6 | -12.1 |
| C2 | 1x1 | -6.7 | -4.2 |
| C2 | 1x2 | -11.5 | -9.0 |

- Conclusion

This contribution provides our simulation results for SAN demodulation requirement for above 10GHz bands for alignment.

- References

- R4-2405146, Simulation assumption on demodulation requirements for NR NTN enhancements, Huawei, HiSilicon, RAN4#110bis