3GPP TSG-RAN WG4 Meeting #111	R4-2409867

# Fukuoka, Japan, 20th – 24th, 2024

Title: 	Simulation assumption on demodulation requirements for NR NTN enhancements

Source: 	Huawei, HiSilicon

Agenda item:	7.16.8

Document for:	Approval

Background

In this contribution, the simulation assumption on demodulation requirements for NR NTN enhancements is captured for simulation results alignment.

Simulation assumptions

UE demodulation performance requirements

PDSCH

| Parameter |  | Unit | Value |
| --- | --- | --- | --- |
| Duplex mode |  |  | FDD |
| Active DL BWP index |  |  | 1 |
| PDSCH configuration | Mapping type |  | Type A |
|  | k0 |  | 0 |
|  | Starting symbol (S) |  | 1 |
|  | Length (L) |  | 13 |
|  | PDSCH aggregation factor |  | 1 |
|  | PRB bundling type |  | Static |
|  | PRB bundling size |  | 2 |
|  | Resource allocation type |  | Type 0 |
|  | RBG size |  | Config2 |
|  | VRB-to-PRB mapping type |  | Non-interleaved |
|  | VRB-to-PRB mapping interleaver bundle size |  | N/A |
| PDSCH DMRS configuration | DMRS Type |  | Type 1 |
|  | Number of additional DMRS |  | 1 |
|  | Maximum number of OFDM symbols for DL front loaded DMRS |  | 1 |
| CSI-RS for tracking | CSI-RS periodicity | Slots | 160 for CSI-RS resource 1,2,3,4. |
|  | CSI-RS offset | Slots | 80 for CSI-RS resource 1 and 2 81 for CSI-RS resource 3 and 4 |
| PT-RS configuration |  |  | Not configured |
| Number of HARQ Processes |  |  | 16 for Test 1-1, Test 1-2 32 for Test 1-3 For Test 1-4: 4 with feedback disabled, 12 with feedback enabled in 16 HARQ processes with re-Tx disable for all HARQ processes and only transmit initial transmissions with NDI toggling. Throughput shall be measured on processes with HARQ enabled. Which 4 processes to disable are randomly select at test configuration. |
| Maximum number of HARQ transmission |  |  | 4 for Test 1-1, Test 1-2, Test 1-3 Disabled for all HARQ processes for Test 1-4 |

| Test | Bandwidth (MHz) / Subcarrier spacing (kHz) | Modulation format and code rate | Propagation condition | Correlation matrix and antenna configuration | Test metric |
| --- | --- | --- | --- | --- | --- |
| 1-1 | 200 / 120 | QPSK, 0.30(MCS4 in Table1) | NTN-TDLC5-1200 | 1x1 Low | 70% of maximum throughput |
| 1-2 | 200 / 120 | 16QAM, 0.48 (MCS13 in Table 1) | NTN-TDLC5-1200 | 1x1 Low | 70% of maximum throughput |
| 1-3 | 200 / 120 | QPSK, 0.30 | NTN-TDLC5-1200 | 1x1 Low | 70% of maximum throughput |
| 1-4 | 200 / 120 | QPSK, 0.30 | NTN-TDLC5-1200 | 1x1 Low | 70% of maximum throughput |

PDCCH

| Bandwidth (MHz) | SCS (kHz) | CORESET RB | CORESET duration | Aggregation level | CCE to REG mapping type | REG bundle size | Interleave size | Shift index | DCI format | Payload (without CRC) | Propagation Condition | Antenna configuration and correlation Matrix | Test metric |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 200 | 120 | 132 | 1 | 8 | Interleaved | 2 | 3 | 0 | 1_0 | 42 | NTN-TDLC5-1200 | 1x1 Low | 1% of Pm-dsg |
| 200 | 120 | 132 | 2 | 16 | Interleaved | 2 | 3 | 0 | 1_1 | 56 | NTN-TDLC5-1200 | 1x1 Low | 1% of Pm-dsg |

SAN demodulation performance requirements

Above 10GHz bands

PUSCH

Normal PUSCH with CP-OFDM

| Parameter |  | Value |
| --- | --- | --- |
| Transform precoding |  | Disabled |
| HARQ | Maximum number of HARQ transmissions | 4 |
|  | RV sequence | 0, 2, 3, 1 |
| DM-RS | DM-RS configuration type | 1 |
|  | DM-RS duration | single-symbol DM-RS |
|  | Additional DM-RS symbols | Pos1 |
|  | Number of DM-RS CDM group(s) without data | 2 |
|  | Ratio of PUSCH EPRE to DM-RS EPRE | -3 dB |
|  | DM-RS port(s) | {0} |
|  | DM-RS sequence generation | NID=0, nSCID =0 |
| Time domain resource | PUSCH mapping type | B |
|  | Start symbol index | 0 |
|  | Allocation length | 10 |
| Frequency domain resource | RB assignment | Full applicable test bandwidth |
|  | Frequency hopping | Disabled |
| Code block group based PUSCH transmission |  | Disabled |
| PT-RS configuration | Frequency density (KPT-RS) | Disabled |
|  | Time density (LPT-RS) | Disabled |
| Antenna configuration |  | 1x1, 1x2 |
| Propagation condition |  | NTN-TDLC5-1200 Low |
| MCS |  | MCS 2 and MCS12 in Table 1 |
| SCS |  | 120kHz |
| Channel bandwidth |  | 50MHz |
| Test metric |  | 70% of maximum throughput |

Normal PUSCH with DFT-s-OFDM

| Parameter |  | Value |
| --- | --- | --- |
| Transform precoding |  | Enabled |
| HARQ | Maximum number of HARQ transmissions | 4 |
|  | RV sequence | 0, 2, 3, 1 |
| DM-RS | DM-RS configuration type | 1 |
|  | DM-RS duration | single-symbol DM-RS |
|  | Additional DM-RS symbols | Pos1 |
|  | Number of DM-RS CDM group(s) without data | 2 |
|  | Ratio of PUSCH EPRE to DM-RS EPRE | -3 dB |
|  | DM-RS port(s) | {0} |
|  | DM-RS sequence generation | NID=0, nSCID =0 |
| Time domain resource | PUSCH mapping type | B |
|  | Start symbol index | 0 |
|  | Allocation length | 10 |
| Frequency domain resource | RB assignment | 30 PRBs in the middle of the test bandwidth |
|  | Frequency hopping | Disabled |
| Code block group based PUSCH transmission |  | Disabled |
| PT-RS configuration | Frequency density (KPT-RS) | Disabled |
|  | Time density (LPT-RS) | Disabled |
| Antenna configuration |  | 1x1, 1x2 |
| Propagation condition |  | NTN-TDLC5-1200 Low |
| MCS |  | MCS 2 in Table 1 |
| SCS |  | 120kHz |
| Channel bandwidth |  | 50MHz |
| Test metric |  | 70% of maximum throughput |

PUSCH repetition Type A

| Parameter |  | Value |
| --- | --- | --- |
| Transform precoding |  | Disabled |
| HARQ | Maximum number of HARQ transmissions | 4 |
|  | RV sequence | 0, 3, 0, 3 [Note 1] |
| DM-RS | DM-RS configuration type | 1 |
|  | DM-RS duration | single-symbol DM-RS |
|  | Additional DM-RS symbols | Pos1 |
|  | Number of DM-RS CDM group(s) without data | 2 |
|  | Ratio of PUSCH EPRE to DM-RS EPRE | -3 dB |
|  | DM-RS port(s) | 0 |
|  | DM-RS sequence generation | NID=0, nSCID =0 |
| Time domain resource | PUSCH mapping type | B |
|  | Start symbol index | 0 |
|  | Allocation length | 10 |
|  | PUSCH aggregation factor | n2 |
| Frequency domain resource | RB assignment | Full applicable test bandwidth |
|  | Frequency hopping | Disabled |
| Code block group based PUSCH transmission |  | Disabled |
| PT-RS configuration | Frequency density (KPT-RS) | Disabled |
|  | Time density (LPT-RS) | Disabled |
| Antenna configuration |  | 1x1, 1x2 |
| Propagation condition |  | NTN-TDLC5-1200 Low |
| MCS |  | MCS 5 in Table 3 |
| SCS |  | 120kHz |
| Channel bandwidth |  | 50MHz |
| Test metric [NOTE 2] |  | 1% BLER |
| NOTE 1: The effective RV sequence is {0,2,3,1} with slot aggregation NOTE 2: BLER is defined as residual BLER; i.e. ratio of incorrectly received transport blocks / sent transport blocks, independently of the number HARQ transmission(s) for each transport block. |  |  |

PUCCH

PUCCH format 0

| Parameter | Test |
| --- | --- |
| Number of UCI information bits | 1 |
| Number of PRBs | 1 |
| First PRB prior to frequency hopping | 0 |
| Intra-slot frequency hopping | Enabled |
| First PRB after frequency hopping | The largest PRB index – (Number of PRBs - 1) |
| Group and sequence hopping | neither |
| Hopping ID | 0 |
| Initial cyclic shift | 0 |
| First symbol | 12 for 2 symbols |
| Antenna configuration | 1x1, 1x2 |
| Propagation condition | NTN-TDLC5-1200 Low |
| SCS | 120kHz |
| Channel bandwidth | 50MHz |
| Test metric | 1% of DTX to ACK probability 1% of ACK missed detection probability |

PUCCH format 1

| Parameter | Test |
| --- | --- |
| Number of information bits | 2 |
| Number of PRBs | 1 |
| Number of symbols | 14 |
| First PRB prior to frequency hopping | 0 |
| Intra-slot frequency hopping | enabled |
| First PRB after frequency hopping | The largest PRB index – (nrofPRBs – 1) |
| Group and sequence hopping | neither |
| Hopping ID | 0 |
| Initial cyclic shift | 0 |
| First symbol | 0 |
| Index of orthogonal cover code (timeDomainOCC) | 0 |
| Antenna configuration | 1x1, 1x2 |
| Propagation condition | NTN-TDLC5-1200 Low |
| SCS | 120kHz |
| Channel bandwidth | 50MHz |
| Test metric | 1% of DTX to ACK probability 0.1% of NACK to ACK probability 1% of ACK missed detection probability |

PUCCH format 2 (ACK missed detection)

| Parameter | Value |
| --- | --- |
| Modulation order | QSPK |
| Starting RB location | 0 |
| Intra-slot frequency hopping | N/A |
| Number of PRBs | 4 |
| Number of symbols | 1 |
| The number of UCI information bits | 4 |
| First symbol | 13 |
| DM-RS sequence generation | NID0=0 |
| Antenna configuration | 1x1, 1x2 |
| Propagation condition | NTN-TDLC5-1200 Low |
| SCS | 120kHz |
| Channel bandwidth | 50MHz |
| Test metric | 1% of DTX to ACK probability 1% of ACK missed detection probability |

PUCCH format 2 (UCI BLER)

| Parameter | Value |
| --- | --- |
| Modulation order | QSPK |
| First PRB prior to frequency hopping | 0 |
| Intra-slot frequency hopping | enabled |
| Frist PRB after frequency hopping | The largest PRB index – (Number of PRBs – 1) |
| Number of PRBs | 9 |
| Number of symbols | 2 |
| The number of UCI information bits | 22 |
| First symbol | 12 |
| DM-RS sequence generation | NID0=0 |
| Antenna configuration | 1x1, 1x2 |
| Propagation condition | NTN-TDLC5-1200 Low |
| SCS | 120kHz |
| Channel bandwidth | 50MHz |
| Test metric | 1% of BLER |

PUCCH format 3

| Parameter | Test 1 |
| --- | --- |
| Modulation order | QPSK |
| First PRB prior to frequency hopping | 0 |
| Intra-slot frequency hopping | enabled |
| First PRB after frequency hopping | The largest PRB index – (Number of PRBs – 1) |
| Group and sequence hopping | neither |
| Hopping ID | 0 |
| Number of PRBs | 1 |
| Number of symbols | 14 |
| The number of UCI information bits | 16 |
| First symbol | 0 |
| DM-RS configuration | With and without additional DM-RS |
| Antenna configuration | 1x1, 1x2 |
| Propagation condition | NTN-TDLC5-1200 Low |
| SCS | 120kHz |
| Channel bandwidth | 50MHz |
| Test metric | 1% of BLER |

PUCCH format 4

| Parameter | Value |
| --- | --- |
| Modulation order | QPSK |
| First PRB prior to frequency hopping | 0 |
| Number of PRBs | 1 |
| Intra-slot frequency hopping | enabled |
| First PRB after frequency hopping | The largest PRB index – (Number of PRBs – 1) |
| Group and sequence hopping | neither |
| Hopping ID | 0 |
| Number of symbols | 14 |
| The number of UCI information bits | 22 |
| First symbol | 0 |
| DM-RS configuration | With and without additional DM-RS |
| Length of the orthogonal cover code | n2 |
| Index of the orthogonal cover code | n0 |
| Antenna configuration | 1x1, 1x2 |
| Propagation condition | NTN-TDLC5-1200 Low |
| SCS | 120kHz |
| Channel bandwidth | 50MHz |
| Test metric | 1% of BLER |

PRACH

| Parameter | Test |
| --- | --- |
| Burst format | B4, C2 |
| Time error tolerance | 0.13us |
| Ncs | 69 |
| Logical sequence index | 0 |
| v | 0 |
| Antenna configuration | 1x1, 1x2 |
| Propagation condition | NTN-TDLC5-1200 Low |
| Frequency offset | 3000Hz |
| SCS | 120kHz |
| Channel bandwidth | 50MHz |
| Test metric | 0.1% of DTX to ACK probability 1% of ACK missed detection probability |

PUSCH with DMRS bundling for FR1

Normal PUSCH with CP-OFDM

| Parameter |  | Value |
| --- | --- | --- |
| Transform precoding |  | Disabled |
| HARQ | Maximum number of HARQ transmissions | 4 |
|  | RV sequence [Note 1] | 0, 0, 0, 0 [Note1] |
| DM-RS | DM-RS configuration type | 1 |
|  | DM-RS duration | single-symbol DM-RS |
|  | Additional DM-RS position | pos1 |
|  | Number of DM-RS CDM group(s) without data | 2 |
|  | Ratio of PUSCH EPRE to DM-RS EPRE | -3 dB |
|  | DM-RS port | 0 |
|  | DM-RS sequence generation | NID0=0, nSCID =0 |
| Time domain resource assignment | PUSCH mapping type | A, B |
|  | Start symbol | 0 |
|  | Allocation length | 14 |
|  | PUSCH aggregation factor | n4 for 15kHz SCS n8 for 30kHz SCS |
| pusch-TimeDomainWindowLength |  | 8 for n8, 4 for n4 |
| Frequency domain resource assignment | RB assignment | 6 RBs in the middle of test bandwidth |
|  | Frequency hopping | Disabled |
| Code block group based PUSCH transmission |  | Disabled |
| Antenna configuration |  | 1x1, 1x2 |
| Propagation condition |  | NTN-TDLA100-200 Low |
| MCS |  | MCS 4 in Table 1 |
| SCS |  | 15kHz, 30kHz |
| Channel bandwidth |  | 5MHz for 15kHz SCS 10MHz for 30kHz SCS |
| Test metric |  | 70% of maximum throughput |
| Note 1: The effective RV sequence is {0, 2, 3, 1} with slot aggregation. |  |  |

Proposals

In this contribution, the simulation assumption on demodulation requirements for NR NTN enhancements is captured for simulation results alignment.

Reference

R4-2406024, WF on NR_NTN_enh_SAN_UE_demod, RAN4#110bis, Huawei, HiSilicon

R4-2402660, Topic summary for [110][320] NR_NTN_enh_SAN_UE_demod, RAN4#110, Huawei, HiSilicon

R4-2402865, WF on NR_NTN_enh_SAN_UE_demod, RAN4#110, Huawei, HiSilicon

R4-2321187, WF on NR_NTN_enh_SAN_UE_demod, RAN4#109, Qualcomm

R4-2316921, WF on NR_NTN_enh_SAN_UE_demod, RAN4#108-bis, Huawei, HiSilicon