3GPP TSG-RAN WG4 Meeting #111										       R4-2401570

Fukuoka city, Japan , May 20th – 24th, 2024

Agenda item:	10.11.5

Source: 	Qualcomm Incorporated

Title: 	WF on AI/ML

Document for:	Approval

# Introduction

This WF captures the agreements for the discussion carried out on AI/ML under the [111][133]NR_AIML_air thread.

# Agreements

## General Issues

### Agreements in main session:

Issue 1-1: Post deployment testing

Interested companies are invited to bring analysis on the feasibility of the options proposed so far.

Issue 1-4:  Inference latency handling

Agreement:

- Use the following as the starting point

  - Inference latency can be implicitly captured in the delay of corresponding procedures (e.g. reporting delay) and/or performance requirements.

  - Inference latency is part of delay of the corresponding procedure and no separate requirement will be specified for inference latency.

  - Check the above bullets based on per use case, and if other WGs define the inference latency, RAN4 can revisit the above bullets.

Issue 1-5: LCM Requirements

Agreement:

- RAN4 considers defining RRM requirements for LCM procedures based on the specific use cases (beam management and positioning), and candidates are:

- identification

- selection

- activation

- deactivation

- switching

- fallback to non-AI operation

- performance monitoring

- Others are not precluded

Issue 1-6: Legacy RRM requirements handling

Agreement:

- UE shall meet all legacy RRM requirements (non-AI/ML) even during the AI/ML operation mode.

## Testability and interoperability issues for beam management

### Agreements in main session:

Issue 2-3: Reported measurements and ground truth

Agreement:

- The ground truth for the predicted RSRP is the ideal measurement of RSRP on the predicted Tx beam

  - In RAN4, the ground truth is the approximate as the reported RSRP measurement under the certain SNR on the predicted Tx beam

    - FFS on SNR level to ensure that SNR is high enough for sufficient accuracy of reported RSRP

    - FFS on impact of multiple-AoA test setup

    - FFS on the channel condition

    - FFS on whether the ground truth will be changing or not

  - Other solutions are not precluded

### Agreements in ad-hoc session (R4-2401571)

Issue 2-5: Test setup needs

Agreement:

maximum number of set B Tx beams that test system should be able to emulate: [8-16]

maximum number of set A Tx beams that test system should be able to emulate:  [64-128]

FFS on AoAs

UE rotation during the test: FFS

UE rotation/repositioning between different tests: Yes

Issue 2-6:	Datasets for training/testing

Agreement:

- Vendors may take into account the test environment/conditions defined by RAN4 when training the UE

  - sufficient test environment/conditions should be defined to enable vendors to create the data needed for training

- FFS on proposals to augment training data to avoid overfitting of UE models to the test environment

- RAN4 to strive to make the test conditions similar to field deployment conditions

## Testability and interoperability issues for positioning accuracy enhancement

### Agreements in ad-hoc session (R4-2401571)

Issue 3-2: Requirements for case 2a

Agreement:

No requirements on LMF for positioning accuracy

FFS on RAN4 requirements for any UE reported measurements defined by other groups

Issue 3-3: Requirements for case 2b

Agreement:

No requirements on LMF for positioning accuracy

FFS on RAN4 requirements for any UE reported measurements defined by other groups

## Testability and interoperability issues for CSI compression and CSI prediction

### Agreements in ad-hoc session (R4-2401571)

Issue 4-1: Reference encoder/decoder

Agreement:

Reference encoder:

The reference encoder is used in RAN4 discussions at least for simulation alignment/requirement derivation, test decoder derivation and/or test decoder verification. It could be documented (in TR, WF, etc) or captured in the specifications as necessary.

Reference decoder:

The reference decoder is used in RAN4 discussions at least for simulation alignment/requirement derivation and/or verification of the decoder implemented by the TE. It could be documented (in TR, WF, etc) or captured in the specifications as necessary.

For option 3, for each test, a test decoder needs to be captured in the specs, a reference encoder might be needed to derive the test decoder and/or requirements and/or to validate the test decoder implementation in the TE. the same decoder might be used in multiple tests or each test could have a difference decoder.

In option 3 the Reference decoder is the test decoder.

For option 4, there might be a need to have reference encoder and/or reference decoder

### Other Agreements

#### Agreements about the parameters used in the simulations for checking the feasibility of testing options for CSI compression and prediction:

Agreement:

Parameters agreed are just for the feasibility study of testing options.

If/when RAN4 discusses requirement definition, RAN4 will define a new test decoder which may or may not reuse any of the parameters agreed in the feasibility study.

#### Agreements regarding the process for checking the feasibility of deriving a test decoder for Option 3

The following options were discussed:

Test decoder derivation procedure:

Option 1

companies bring encoder + decoder set based on agreed parameters. RAN4 chooses one of the decoders and interested companies further check if an encoder can be trained with this decoder to obtain similar performance/complexity (or other evaluation criteria)

Option 2

companies bring training data for encoder + decoder pair, interested companies train an encoder + decoder pair based on the aggregated dataset from all companies to check decoder derivation feasibility and performance/complexity (or other evaluation criteria)

RAN4 agrees to work on option 1.

Interested companies are invited to bring further feasibility analysis for Option 2

#### Simulation parameters for verifying the feasibility of aligning model among companies

System-level simulation parameters (from TR):

| Parameter |  | Value |
| --- | --- | --- |
| Duplex, Waveform |  | FDD, OFDM |
| Multiple access |  | OFDMA |
| Scenario |  | Dense Urban (Macro only) |
| Frequency Range |  | FR1 only, [2GHz, 4GHz] |
| Inter-BS distance |  | 200m |
| Channel model |  | According to TR 38.901 |
| Antenna setup and port layouts at gNB |  | Companies need to report which option(s) are used between - 32 ports: (8,8,2,1,1,2,8), (dH,dV) = (0.5, 0.8)λ |
| Antenna setup and port layouts at UE |  | 4RX: (1,2,2,1,1,1,2), (dH,dV) = (0.5, 0.5)λ for (rank 1-4) |
| BS Tx power |  | 44dBm for 20MHz |
| BS antenna height |  | 25m |
| UE antenna height & gain |  | Follow TR36.873 |
| UE receiver noise figure |  | 9dB |
| Numerology | Slot/non-slot | 14 OFDM symbol slot |
|  | SCS | Baseline: 15kHz for 2GHz; Optional: 30kHz for 4GHz |
| Simulation bandwidth |  | Baseline: 10 MHz for 15kHz Optional: 20 MHz for 30kHz |
| Frame structure |  | Slot Format 0 (all downlink) for all slots |
| MIMO scheme |  | SU-MIMO |
| MIMO layers |  | Baseline: 1 Optional: 2 |
| CSI feedback |  | Feedback assumption at least for baseline scheme - CSI feedback periodicity (full CSI feedback): 5 ms (baseline) - Scheduling delay (from CSI feedback to time to apply in scheduling): 4 ms |
| Traffic load (Resource utilization) |  | Baseline: 50% Optional: 20/70% |
| UE distribution |  | CSI compression: 80% indoor (3 km/h), 20% outdoor (30 km/h) |
| UE receiver |  | MMSE-IRC as the baseline receiver |
| Feedback assumption |  | Realistic |
| Channel estimation |  | Realistic or ideal channel estimation |

Additional parameters of the encoder/decoder and feedback based on RAN1 simulation parameters by companies in RAN1 (from Excel):

| Assumptions |  | Value |
| --- | --- | --- |
| CSI generation part | AI/ML model backbone | MLP, CNN, Transformer |
|  | Pre-processing | SVD to get channel eigenvectors |
| CSI reconstruction part | AI/ML model backbone | [MLP, CNN, Transformer] |
| Common description | Input type | Eigenvectors of channel matrix |
|  | Output type | Eigenvectors of channel matrix |
|  | Quantization /dequantization method | Scalar quantization |
| Dataset description |
|  | Train/Test | 10% of the training dataset as baseline |
|  | Ground-truth CSI quantization method (including scalar/codebook based quantization, and the parameters) | Floating point (float 32) |
| Other parameters | Laten/reporting size | [X: below 80bits, Y: 100 – 140 bits] |

| Decoder model type | MLP-Mixer |
| --- | --- |
| Embedding dimension | 64 |
| Number of mixer block | 2 |
| Size of message passed across layers | (13, 64) |
| Token mixer hidden dimension | 104 |
| Channel mixer hidden dimension | 512 |

| Model architecture parameters (convolutional) | Model type | CNN (Resnet like design) |
| --- | --- | --- |
|  | Num of Resnet blocks | 6 |
|  | Layer before resnet blocks | 2 layers (one FC layer, one Conv layer) |
|  | Layer after resnet blocks | 1 layers (one conv layer) |
|  | Layer within a resnet block | 2 Conv layer + 1 layer for combination |
|  | Kernel/Channel size within a resnet block | (3,3,128) |

| Model architecture parameters (transformer) | Model type | Transformer |
| --- | --- | --- |
|  | Embedding dimension | 64 |
|  | Number of attenuation heads in one transformer block | 8 |
|  | Number of transformer blocks | 6 |
|  | Size of message passed across layers | (13,256) |