3GPP TSG-RAN WG4 Meeting #112	R4-2412939

Maastricht, Netherlands, 19th – 23rd August, 2024

Agenda item:	8.1.1.3.3

Source:            	Google

Title:                	Discussion on SRS antenna switching requirements for 6Rx UE

Document for:	Approval

# Introduction

In RAN#103, a new WID on RF enhancements for NR FR1/FR2 and EN-DC was approved as one of Release 19 work items [1]. One of the objectives in the WID is to discuss 6Rx requirements for handheld and FWA UEs, which is shown as follows. In this contribution, we will focus on the SRS antenna switching requirements for 6Rx handheld and FWA UE.

6Rx for handheld and FWA UE - Specify the core requirements to enable 6Rx for higher frequency bands (>2.5GHz) targeting at support of handheld UE for NR FR1 single carrier scenario - Example bands: n41, n77/n78, n79, n104 - Support 4 MIMO layers at least, and study the gain and feasibility and if feasible, support 6 MIMO layers - Specify the Rx requirements including reference sensitivity requirements for support 6Rx - Note: the specified requirements can be applicable to both handheld UE and FWA devices - Specify the requirements to support SRS antenna switching including t1r6, t2r6, t3r6, t4r6 depending on UE capability - Study the issue of insertion loss imbalance across SRS ports, and if justified, specify the corresponding solution.

In RAN4#111, the SRS antenna switching requirements for 6Rx handheld and FWA UE has been discussed and some open issues are captured in the approved WF [2]. In WF, the main issue is mainly about how to specify ΔTRxSRS value for handheld and FWA UEs, and several other related issues also need further discussion. In this paper, we would like to share our view in the following.

# Topic 2: SRS antenna switching and ΔTRxSRS ## Sub-topic 2-1: General considerations for SRS antenna switching and ΔTRxSRS Issue 2-1-1: SRS antenna switching configurations Way forward: RAN4 to focus on discussing ∆TRxSRS for 1tr6 and t2r6 until RAN1 concludes work on 3t6r and 4t6r. Pending RAN1 feedback, RAN4 can consider any necessary updates to the work scope in a Revised WID for the SRS antenna configurations subject to RAN Plenary approval. Issue 2-1-2: Whether to use different ∆TRxSRS based on operating frequency Way forward: RAN4 defines different ∆TRxSRS values based on operating frequency. Further discuss whether to introduce an additional breakpoint for bands whose FUL_high is higher than the FUL_low of n104. ## Sub-topic 2-2: ΔTRxSRS values Issue 2-2-1: Proposed ∆TRxSRS values Way forward: Further discuss the following options for the ∆TRxSRS requirements. - Option 1: Consider average value of ∆TRxSRS for further discussion. Operating bands; ΔTRxSRS requirement for t1r6; ΔTRxSRS requirement for t2r6; ΔTRxSRS requirement for t1r6-t2r6. Band n41, n77, n78; 4.0 dB; 3.4 dB; 4.4 dB. Band n79, [n104]; 5.3 dB; 4.5 dB; 5.7 dB. - Option 2: Consider range of ∆TRxSRS in brackets for each case below for further discussion. Operating bands; ΔTRxSRS requirement for t1r6; ΔTRxSRS requirement for t2r6; ΔTRxSRS requirement for t1r6-t2r6. Band n41, n77, n78; [2.8 ~ 5.0] dB; [2.3 ~ 4.0] dB; [3.3 ~ 5.5] dB. Band n79, [n104]; [3.6 ~ 6.0] dB; [3.0 ~ 5.5] dB; [4.2 ~ 6.5] dB. Note: Maximum level for t1r6 corrected for OPPO proposals per OPPO. - Option 3: Other options are not precluded.

# Discussion

In Rel-15, SRS Tx switching feature is introduced to obtain more accurate channel state information (CSI) by utilizing channel reciprocity for TDD bands. The motivation to introduce SRS Tx switching feature is that Tx antenna port number is usually less than Rx antenna port number from UE implementation perspective. In other words, in order to obtain more accurate CSI for DL-MIMO transmission at the base station side, the UE can be scheduled to perform antenna switching for SRS transmission in the corresponding Rx antenna port sequentially instead of simultaneous SRS transmission on the corresponding Rx ports.

From UE capability signalling perspective, SRS Tx switching feature can be indicated by the corresponding IE srs-TxSwitch/srs-TxSwitch-v1610 with “xTyR”, e.g., 1T2R/1T4R/ 2T4R/2T2R/4T4R/1T4R-2T4R, where “x” is the capable antenna port for SRS transmission and “y” means the total number of receive antenna where SRS transmission can be performed for the target band or the target band in the band combination. In Rel-17, the xTyR has been extended to support at most “1T8R/2T8R/4T8R” in the corresponding IE srs-AntennaSwitchingBeyond4Rx-r17.

From implementation perspective, if the UE is configured to perform SRS antenna switching, SRS Tx transmission may go through different number of RFFE antenna switch components, e.g., SPXT, DPXT where X=2,3,4,…8, at each associated Rx path on PCB board, which may cause different insertion loss for each SRS Tx path. In addition, the PCB tracing loss may also be different from each SRS Tx associated Rx path. Hence, in order to accommodate both the SRS insertion loss and PCB tracing loss for different UE hardware implementation, RAN4 define ΔTRxSRS requirement within the lower bound of PCMAX equation shown as follows in TS 38.101-1 sub clause 6.2.4, where the ΔTRxSRS value means the max allowed power reduction when the UE performing SRS Tx antenna switching. According to different “xTyR” capability, the corresponding ΔTRxSRS values are defined in TS 38.101-1 sub clause 6.2.4.

PCMAX_L,f,c ≤  PCMAX,f,c  ≤  PCMAX_H,f,c with

PCMAX_L,f,c = MIN {PEMAX,c– ∆TC,c,  (PPowerClass – ΔPPowerClass + ΔPPowerBoost) – MAX(MAX(MPRc+∆MPRc, A-MPRc)+ ΔTIB,c + ∆TC,c + ∆TRxSRS, P-MPRc) }

PCMAX_H,f,c = MIN {PEMAX,c,  PPowerClass – ΔPPowerClass + ΔPPowerBoost}

In RAN4#111, SRS antenna switching for 6Rx handheld and FWA UEs has been discussed and some open issues for ΔTRxSRS requirement are captured in the WF. One of the open issues is to determine the ΔTRxSRS value for 6Rx handheld and FWA UE on the example bands. Two options for ΔTRxSRS value are agreed in the WF that the Option 1 is to consider average value of ΔTRxSRS for further discussion and the Option 2 is to consider to keep ∆TRxSRS by all companies’ proposals in the range with brackets for further discussion. In addition, some proposed ∆TRxSRS values for t1r6/t2r6/t1r6-t2r6 are divided into two groups, where the first group is n41/n77/n78 and the second group is n79/[n104]. In our view, since ΔTRxSRS value for 6Rx handheld and FWA UE may be determined by the worst insertion loss among the 6 Rx path on the PCB board, different RFFE implementation may cause different insertion loss. Hence, it is reasonable to take all proposed value into consideration to determine ΔTRxSRS value. Considering that the averaging method is most acceptable way in RAN4 to determine requirement, it is proposed to adopt the averaging of all proposed values to determine ΔTRxSRS requirement for 6Rx handheld and FWA UE in Rel-19.

Proposal 1: Considering that the averaging method is most acceptable way in RAN4 to determine requirement, it is proposed to adopt the average of all proposed values to determine ΔTRxSRS requirement for 6Rx handheld and FWA UE in Rel-19.

# Conclusion

The proposals in this contribution are summarized in the following.

Proposal 1: Considering that the averaging method is most acceptable way in RAN4 to determine requirement, it is proposed to adopt the average of all proposed values to determine ΔTRxSRS requirement for 6Rx handheld and FWA UE in Rel-19.

# Reference

- RP-240828, “New WID: UE RF enhancements for FR1/FR2 and EN-DC , Phase 4”, RAN4 chair (Huawei), RAN#103

- R4-2410751, “WF on requirements for 6Rx”, AT&T, RAN4#111

- R4-2409052, “Discussion on SRS antenna switching requirements for 6Rx UE”, Google, RAN4#111