3GPP TSG-RAN WG4 Meeting # 112                                                              R4-2411085

Maastricht, Netherlands, 19th – 23rd August, 2024

Title: 	                 A-IoT intermediate UE feasibility and requirements

Source: 	CATT

Agenda item:	8.20.3.3

Document for:	Discussion

- Introduction

This contribution provides our further analysis for AIoT intermediate UE feasibility and requirements according to the WF [1] [2].

- A-IoT intermediate UE feasibility analysis

  - Possible spectrum usage cases

According to WF[1], we only analyse the feasibility of case which R2D and CW/D2R are both in FDD UL spectrum for D2T2. SI of CW transmitting from inside topology is worse than CW transmitting from outside topology, so only analyse the worst case is sufficient. In the analysis, it is assumed that R2D and CW/D2R are TDM and CW and D2R signals are transmitted at the same time, and we only analyse feasibility of A-IoT intermediate UE SI in this paper.

For the AIoT system design, R2D and CW/D2R are transmitted in TDM mode. Then the reader interference analysis focuses on the scenario that reader receives CW and D2R signals at the same time. The analysis is the same for different spectrum usage.

A preliminary analysis similar with SBFD self-interference analysis is shown in Table 1. If the REFSENS desense is 1 dB, the required SI noise cancellation capability is up to 142 dB.

Table 1: Analysis of A-IoT intermediate UE required CW cancellation capability

| Parameters | Calculation | Value |
| --- | --- | --- |
| Power of CW PCW (dBm) |  | 23 |
| Bandwidth of D2R BWD2R (KHz) |  | [180] |
| NF (dB) |  | 9 |
| Noise power floor Pfloor (dBm/BWD2R) | Pfloor=-174dBm+10*log10(BWD2R*10^3)+NF | -112.4 |
| Receiving path capability degradation x (dB) |  | [1] |
| Total noise power Ptotal noise (dBm/BWD2R) | Ptotal noise= Pfloor+xdB | -111.4 |
| power of interference signal Pinterference (dBm/BWD2R) | Pinterference=10*log10(10^((Pfloor+xdB)/10)-10^(Pfloor/10)) | -118.3 |
| Required CW cancellation capability (dB) | Iso=PCW-Pinterference | 141.3 |

- A-IoT intermediate UE requirements analysis

It’s a little earlier to discuss intermediate UE RF requirements in SI stage before the system design in RAN1 is not complete. Some preliminary views from our side are provided in Table 2 to Table 4.

Table 2: A-IoT intermediate UE potential Tx applicable requirements

| No. | Requirement list | Requirement necessary? | Comments |
| --- | --- | --- | --- |
| 1 | maximum output power | Yes |  |
| 2 | Output power dynamics | FFS | More inputs from AIoT physical channel design is needed |
| 3 | Frequency error | Yes |  |
| 4 | Transmit modulation quality | Yes | AIoT waveform is different with NR, new requirement and test approach is needed. |
| 5 | Occupied bandwidth | Yes |  |
| 6 | Spectrum emission mask | Yes |  |
| 7 | ACLR | Yes |  |
| 8 | Transmitter spurious emissions | Yes |  |
| 9 | Transmit intermodulation | FFS | IMD scenario needs more discussion. |

Table 3: A-IoT intermediate UE potential Rx requirements

| No. | Requirement list | Requirement necessary? | Comments |
| --- | --- | --- | --- |
| 1 | Reference sensitivity power level | Yes |  |
| 2 | Maximum input level | Yes |  |
| 3 | ICS | FFS | The scenario needs more discussion. |
| 4 | ACS | Yes |  |
| 5 | In-band blocking | FFS | The scenario needs more discussion. |
| 6 | Out-of-band blocking | FFS | The scenario needs more discussion. |
| 7 | Narrow band blocking | No |  |
| 8 | Receiver intermodulation | Yes | The scenario needs more discussion. |
| 9 | Spurious emissions | Yes |  |

Table 4: A-IoT intermediate UE CW requirements

| No. | Requirement list | Requirement necessary? | Comments |
| --- | --- | --- | --- |
| 1 | Output power | yes |  |
| 2 | CW signal quality | FFS | If phase noise or emissions should be defined FFS. |

- Summary

This contribution provides our further analysis for AIoT intermediate UE feasibility and requirements. We have the following observations and proposals.

Observation 1: If SBFD SI analysis approach is used, required CW cancellation capability is up to 142 dB for 23 dBm CW power level.

Proposal 1: How CW impacts AIoT reader Rx BB DEMOD performance needs some evaluation or assumption for the feasibility study.

For the AIoT intermediate UE RF requirements, preliminary views from our side are provided in Table 2 to Table 4.

Reference

[1] R4-2410567, “WF on co-existence evaluation for ambient IoT”, CMCC, RAN4#111

[2] R4-2410597, “WF on impacts of A-IoT on RF requirements”, Huawei, RAN4#111

[3] R4-2407525, “Discussion on deployment scenarios and spectrum usage for A-IoT”, CATT, RAN4#111

[4] R4-2407524, “Discussion on RF requirements of Ambient IoT intermediate node”, CATT, RAN4#111