3GPP TSG-RAN WG4 Meeting #112	R4-2412865

Maastricht, Netherlands, Aug 19 – Aug 23, 2024

Source:	Nokia

Title:	Reply to RAN2 LS on contention based Msg3 transmission

Agenda item:	8.26.3

Document for:	Discussion

Introduction

In RAN2 #126, RAN2 has reached agreements in the Rel-19 WI related to NB-IoT over NTN (IoT_NTN_Ph3-Core) which might have impact/be impacted by RAN4 UE requirements. This was indicated by RAN2 in their LS to RAN4 (R2-2405769).

RAN2 has asked RAN4 to analyze whether it is possible for a UE in IDLE mode to skip MSG1 in the RACH procedure and still obtain timing synchronization before MSG3. In this document we provide our views on the request posed by RAN2 and suggest it to the LS Reply.

Discussion

RAN2 has sent an LS to RAN4 [1], where the following questions are asked:

Q1: Whether an RRC Idle UE with a pre-compensated TA (i.e., the one used for Msg1 transmission during random access for IoT NTN) can satisfy the required timing accuracy for Msg3 transmission without Msg1/Msg2? Q2: If the answer for Q1 is no, from RAN4 and RAN1 perspective, how the required timing accuracy for Msg3 transmission can be satisfied in this case?

In this section we analyze each of the questions based on the expected UE performance for transmit timing accuracy.

Upon close inspection, it is clear from Q1 and from RAN2 agreements, that the UE is expected to have performed timing pre-compensation (based on ephemeris and common delay). However, the UE has not acquired Network Controlled timing advance as it is in IDLE mode and this UE has skipped Msg1/Msg2 in the RACH procedure, where the network controlled component of the timing advance is set up for the first time (via the TAC in the RAR/Msg2).

In our view, the answer to Q1 depends on whether the absence of the network controlled part of timing advance might impact this UE.

Timing Requirements for Msg1 and Msg3.

In the Random Access procedure for NB-IoT, Msg1 is sent over NPRACH and Msg3 is sent over NPUSCH. And the UL Transmit timing Requirements for both is specified as showed in the box below:

From TS 36.133[2] 7.20A.2 Requirements The UE initial transmission timing error shall be less than or equal to ±Te where the timing error limit value Te is specified in Table 7.20A.2-1. This requirement applies when it is the first transmission in a DRX cycle or the first transmission in a repetition period (R>1) for NPUSCH and NPRACH, the first transmission after an uplink transmission gap in a repetition period (R>1) for NPUSCH and NPRACH transmission, or it is the transmission on PUR. The reference point for the UE initial transmit timing control requirement shall be the downlink timing of the serving NB-IoT cell minus NTA_Ref+NTAoffset+NTA,common+NTA, UE-specific×Ts. The downlink timing is defined as the time when the first detected path (in time) of the corresponding downlink frame is received from the serving NB-IoT cell. NTA_Ref for NPRACH is defined as 0. NTA_Ref+NTAoffset+NTA,common+NTA, UE-specific (in Ts units) for other channels is the difference between UE transmission timing and the Downlink timing immediately after when the last timing advance in clause 7.22A was applied. NTA_Ref for other channels is not changed until next timing advance is received. Table 7.20A.2-1: Te Timing Error Limit Downlink Bandwidth (MHz); Te_. 0.18; 97*TS. - Note 1: TS is the basic timing unit defined in TS 36.211.

First it is possible to see that the maximum allowed absolute error for UL Transmit timing NPRACH and NPUSCH is the same (97 Ts) (see green remarks on the box above).  However, the highlight on yellow above shows an important distinction between the two cases: for the NPRACH the network controlled part is always zero, whereas for the NPUSCH transmission the value of NTA_Ref is set by the network.

For terrestrial network, the value of NTA_Ref is important to establish the value of the timing advance, as the UE does not perform any pre-compensation. And therefore, the cyclic prefix of the NPRACH is designed to accommodate for long timing offsets, such that the network can track the initial preamble transmission by the UE and provide the timing advance command such that the UL transmission of all UEs are synchronized at the receiver.

For non-terrestrial network, the UEs perform pre-compensation, so the rough part of the UE transmit timing advance is already compensated by the UE. However, the network-controlled part might be necessary, if the UE pre-compensation is not enough to maintain the NPUSCH transmission timing error within the cyclic prefix at the receiver.

In fact, the maximum absolute error of 97 Ts is allowed in both directions (late or early transmission error), which means the network receiver needs to be prepared to transmit timing error between -97Ts and +97Ts: a total error accommodation of 194Ts. This is already larger than the maximum CP length for NB-IoT NPUSCH (160 Ts as stated in clause 10.1.5 in TS 36.211 [3]).  This error alone would be enough to cause interference in the NB-IoT transmission.

This issue is however mitigated by the fact that the Cyclic prefix in the NPRACH varies between 2048Ts and  24576Ts for FDD frame structure. This allows the network to accommodate for receiving the UL transmission with “poor synchronization” and correct for UE deviations from the ideal value with the Timing Advance command in RAR/Msg2.

The maximum allowed error in NB-IoT NTN allows for transmissions timing deviations larger than the NPUSCH CP. This issue is mitigated by the large accommodation of NPRACH cyclic prefix and the network-controlled timing advance component.

Besides, the network controlled timing advance might be issued for network receiver margins, and for correcting for additional errors introduced in the system. Therefore they are needed to allow some degree of freedom to the receiver.

Those two reasons indicate that the network controlled part of the timing advance is essential for NPUSCH transmissions. If the UE skips Msg1 and Msg2, the UE might transmit a Msg3 that interferes with adjacent transmissions.

Reply to RAN2 that it is not possible to initiate a NPUSCH transmission that satisfy timing requirements at the receiver with non-initiated network-controlled part of the timing advance.

Conclusion

The paper has provided our views on the LS sent by RAN2 in R2-2405769 and the following observation and proposal were made

Observation 1: The maximum allowed error in NB-IoT NTN allows for transmissions timing deviations larger than the NPUSCH CP. This issue is mitigated by the large accommodation of NPRACH cyclic prefix and the network-controlled timing advance component.

Proposal 1: Reply to RAN2 that it is not possible to initiate a NPUSCH transmission that satisfy timing requirements at the receiver with non-initiated network-controlled part of the timing advance.

- References

- R2-2405769, “LS on UL synchronization for contention based Msg3 transmission without Msg1/Msg2”, ZTE, RAN2#126, Fukuoka, Japan, May 20 – May 24, 2024.

- TS 36.133 “Evolved Universal Terrestrial Radio Access (E-UTRA); Requirements for support of radio resource management”, 3GPP, Release 18.6.0

- TS 36.211 “Evolved Universal Terrestrial Radio Access (E-UTRA); Physical channels and modulation”, 3GPP, Release 18.6.0