3GPP TSG-RAN WG4 Meeting #112 	R4-2413386

# Maastricht, Netherlands, 19th – 23rd August, 2024

Agenda Item:	6.1.1.2

Source: 	Ericsson

Title:	On remaining core issues for SL positioning and CPP

Document for:	Discussion

# Introduction

The latest agreed WF on SL positioning and CPP is in [18].

In this contribution, we discuss issues related to SL positioning and CPP core parts.

# Discussion on SL positioning

## SL PRS-RSRP and SL PRS-RSRPP measurement periods

According to the received RAN1 LS [19], the requesting and/or reporting only SL PRS-RSRP or SL PRS-RSRPP without any other SL positioning measurements is not supported. Therefore, the measurement period of these measurements is the same as specified in TS 38.133 for the corresponding mandatory measurement with which they are configured, i.e., SL RSTD, SL Rx-Tx, SL AoA, or SL RTOA.

- Proposal 1 (SL positioning): Update the measurement period definition of SL PRS-RSRP and SL PRS-RSRPP to refer to SL RSTD, SL Rx-Tx, SL AoA, and SL RTOA, depending on which of them the SL PRS-RSRP and SL PRS-RSRPP measurements are configured.

# Discussion on CPP

## Clarification on the PRS measurement period requirements for DL RSCP/DL RSCPD

In the last meeting, issues related to core requirements when the LMF does not configure the time window for the carrier phase measurement, or the time window is configured by the LMF, but the UE is not capable of performing carrier phase measurement within the configured time window were discussed. The discussions concluded by reaching agreements below [18].

Clarification on the PRS measurement period requirements for DL RSCP/DL RSCPD

Agreements:

For UE configured for CPP measurement with legacy measurement with multiple PFLs, when LMF does not configure measurement time window(s) for a PFL

  - UE performs DL RSCPD/RSCP measurement on a single PFL that is common between the reference TRP and the target TRP.

  - Existing requirements for RSTD/UE Rx-Tx without time window apply.

For UE configured for CPP measurement with legacy measurement with multiple PFLs, when UE is configured with time window and does not support FG 41-2-3:

  - UE performs DL RSCPD/RSCP measurement on the indicated PFL by the network.

  - Existing legacy requirements without time window apply for both CP measurements and legacy measurements.

For UE configured for CPP measurement with legacy measurement with multiple PFLs, when UE is configured with time window and support FG 41-2-3, but does not support FG 41-2-8/41-2-9:

  - Existing legacy requirements without time window apply for legacy measurements.

  - CPP measurement requirements apply for CP measurements for the PRS resources associated to the time window have occasions only within the measurement time window.

For UE configured for CPP measurement with legacy measurement with multiple PFLs, when UE is configured with time window and support FG 41-2-3 and FG 41-2-8/41-2-9:

  - CPP measurement requirements apply for both legacy measurements and CP measurements.

FFS: Whether and/or how to define the measurement period requirements for DL RSCPD/RSCP measurements when aperiodic time window is configured.

Based on the agreements reached in RAN2#126, signaling related to carrier phase measurement for CPP has been updated by RAN2. The updated signaling means LMF requests CPP measurements as a part of time window configuration. For reference the content of NR-DL-PRS-MeasurementTimeWindowsConfig from the latest version of TS.37.355 [2] is copied and the key changes compared to the previous signaling are highlighted in the list of observations to follow.

-- ASN1START

NR-DL-PRS-MeasurementTimeWindowsConfig-r18 ::=

SEQUENCE (SIZE(1..2)) OF

NR-DL-PRS-MeasurementTimeWindowsConfigElement-r18

NR-DL-PRS-MeasurementTimeWindowsConfigElement-r18 ::= SEQUENCE {

nr-StartSFN-TimeWindow-r18		INTEGER (0..1023),

nr-PeriodicOrOneShotTimeWindow-r18 CHOICE {

nr-PeriodicityAndSlotOffsetTimeWindow-r18

NR-DL-PRS-Periodicity-and-ResourceSetSlotOffset-r16,

nr-OneShotSlotOffsetTimeWindow-r18 CHOICE {

scs15-r18						INTEGER (0..10239),

scs30-r18						INTEGER (0..20479),

scs60-r18						INTEGER (0..40959),

scs120-r18						INTEGER (0..81919)

}

}																		OPTIONAL, -- Need ON

nr-SymbolOffsetTimeWindow-r18	INTEGER (0..13)							OPTIONAL, -- Need ON

nr-DurationTimeWindow-r18		ENUMERATED { n1, n2, n4, n6, n8, n12, n16, ... },

nr-SelectedDL-PRS-FrequencyLayerIndex-r18	INTEGER (0..nrMaxFreqLayers-1-r16)

OPTIONAL, -- Need ON

nr-SelectedDL-PRS-IndexListPerFreq-r18

SEQUENCE (SIZE (1..nrMaxTRPsPerFreq-r16)) OF

NR-SelectedDL-PRS-IndexPerTRP-r18	OPTIONAL, -- Need OP

nr-MeasurementsToPerformInTimeWindow-r18			BIT STRING { rstd (0),

ueRxTx (1),

rsrp (2),

rsrpp (3),

rscp (4),

rscpd (5)

} (SIZE(1..16)) OPTIONAL, -- Need ON

...

}

NR-SelectedDL-PRS-IndexPerTRP-r18 ::= SEQUENCE {

nr-SelectedTRP-Index-r18				INTEGER (0..nrMaxTRPsPerFreq-1-r16),

dl-SelectedPRS-ResourceSetIndexList-r18	SEQUENCE (SIZE (1..nrMaxSetsPerTrpPerFreqLayer-r16)) OF

INTEGER (0..nrMaxSetsPerTrpPerFreqLayer-1-r16)		OPTIONAL, --Need OP

...

}

-- ASN1STOP

- Observation 1: CPP measurements cannot be requested without configuring time window for CPP measurements. PFL for CPP measurement is therefore always indicated to UE by LMF.

- Observation 2: For an indicated PFL, time window for PRS measurement can either be a periodic window or one-shot window.

- Observation 3: Up to 2-time windows, indicating 2 PFLs, can be configured to UE for CPP measurements.

Based on these observations it can be concluded that the aperiodic time window for CPP measurement is not supported by RAN2 signaling designed for CPP measurements and therefore no corresponding requirement needs to be defined by RAN4. In addition, since the CPP measurements are requested by LMF as a part of the time window configuration such that the PFL on which the CPP measurements are to be performed by the UE is always indicated/configured by the LMF, there is no scenario where UE may have to identify/determine PFL for CPP measurements on its own. The agreements relating to the applicability of the core requirements depending on whether the time window for CPP measurements is configured or not, agreed in RAN4#111, therefore needs to be revisited.

## Measurement period requirements for DL RSCP/DL RSCPD with multiple PFLs configured

One of the other open issues is related to the core requirement when UE is configured with CPP measurement embedded within the request to perform the legacy measurement, such as RSCPD as a part of request for DL-TDOA measurement and RSCP as a part of request for Multi-RTT measurement. To resolve these issues some proposals were made which were agreed FFS in the last meeting. The proposals from the last meeting are listed below for reference purposes.

Measurement period requirements for DL RSCP/DL RSCPD with multiple PFLs configured

  - Option 1:

    - When multiple PFLs are configured in the assistance data and DL RSCPD is requested with RSTD, the measurement period is given by

TRSCPD with RSTD,Total=TRSCPD with RSTD,j+i=1,i≠jLTRSTD,i+ L-1*maxTeffect,i

      - where j is the index of the indicated PFL, if provided by the LFM, otherwise the PFL j is selected by UE implementation.

      - If an aperiodic time window is configured, the start of the measurement period coincides with the start of the time window.

    - When multiple PFLs are configured in the assistance data and DL RSCP is requested with UE Rx-Tx, the measurement period is given by

TRSCP with UERxTx, Total=TRSCP with UERxTx, j+i=1,i≠jLTUERxTx,i+ L-1*maxTeffect,i

      - where j is the index of the indicated PFL, if provided by the LFM, otherwise the PFL j is selected by UE implementation.

      - If an aperiodic time window is configured, the start of the measurement period coincides with the start of the time window.

  - Option 2:

    - When multiple PFLs are configured where RSCPD is supposed to be measured in PFL j, support option 1 with the following updates (taking RSCPD measurement as the example)

      - For UE supporting both FG 41-2-3 and FG 42-2-8，TRSCPD with RSTD, j is the measurement period for both RSCPD and RSTD in PFL j by taking time window into account.

      - For UE supporting FG 41-2-3 only, TRSCPD with RSTD, j=max(TRSCPD, j,TRSTD, j), where TRSCPD, j is the measurement period for RSCPD in PFL j by taking time window into account, and T RSTD, j is the legacy measurement period for RSTD in PFL j without time window.

      - For UE not supporting FG 41-2-2 or FG 41-2-8, TRSCPD with RSTD, j=TRSTD, j.

These options intend to answer the question what core requirement UE should meet when configured to perform legacy measurement on multiple PFLs and the carrier phase measurement that is limited to the indicated PFL. In our view, the UE should meet the requirements for the CPP measurements while performing measurement on the PRS resources occurring within the indicated PFL. For the legacy measurements performed on PRS resources occurring outside of the indicated PFL, legacy core requirement without time window should apply. These clarifications or in other words, the applicability rule for CPP measurement, should be enough to address the concern expressed and does not require changing the formulation to derive measurement period as proposed in the options above. To solve this issue together with the issues highlighted before, the following proposals are made.

- Proposal 2 (CPP): Aperiodic time window for CPP measurement is not supported and therefore no corresponding requirement for CPP measurement is defined.

- Proposal 3 (CPP): When UE supports DL PRS-RSCPD measurement inside measurement time window, as indicated in supportOfMeasurementsInTimeWindow, and LMF configures measurement time window(s) for RSCPD measurement:

- The existing requirements for DL RSTD measurement without time window apply to the DL RSTD measurement.

- The requirements for RSCPD measurement apply to DL RSCPD measurement for the PRS resource(s) that have occasions only within the measurement time window.

- Proposal 4 (CPP): When UE supports DL PRS-RSCPD and DL RSTD measurements inside measurement time window, as indicated in supportOfMeasurementsInTimeWindow, and LMF configures measurement time window(s) for RSCPD and RSTD measurements:

- The requirements for DL RSCPD apply to DL RSTD measurement and DL RSCPD measurement performed on the PRS resources in indicated PFL.

- The existing requirements for DL RSTD measurement without time window apply to DL RSTD measurement performed on the PRS resources of PFLs occurring outside of the PRS measurement time window.

- Proposal 5 (CPP): When UE supports DL PRS-RSCP measurement inside measurement time window, as indicated in supportOfMeasurementsInTimeWindow, and LMF configures measurement time window(s) for RSCP measurement:

- The existing requirements for UE RX-Tx measurement without time window apply to UE RX-Tx measurement.

- The requirements for RSCP measurement apply to DL RSCP measurement for the PRS resource(s) that have occasions only within the measurement time window.

- Proposal 6 (CPP): When UE supports DL PRS-RSCP and UE RX-Tx measurements inside measurement time window, as indicated in supportOfMeasurementsInTimeWindow, and LMF configures measurement time window(s) for RSCP and UE Rx-Tx measurements:

- The requirements for DL RSCP apply to UE RX-Tx measurement and DL RSCPD measurement performed on the PRS resources in indicated PFL.

- The existing requirements for UE RX-Tx measurement without time window apply to UE RX-Tx measurement performed on the PRS resources of PFLs occurring outside of the PRS measurement time window.

# Summary

The following have been proposed in the current contribution.

SL positioning:

- Proposal 1: Update the measurement period definition of SL PRS-RSRP and SL PRS-RSRPP to refer to SL RSTD, SL Rx-Tx, SL AoA, and SL RTOA, depending on which of them the SL PRS-RSRP and SL PRS-RSRPP measurements are configured.

CPP:

- Observation 1 (CPP): CPP measurements cannot be requested without configuring time window for CPP measurements. PFL for CPP measurement is therefore always indicated to UE by LMF.

- Observation 2 (CPP): For an indicated PFL, time window for PRS measurement can either be a periodic window or one-shot window.

- Observation 3 (CPP): Up to 2-time windows, indicating 2 PFLs, can be configured to UE for CPP measurements.

- Proposal 2 (CPP): Aperiodic time window for CPP measurement is not supported and therefore no corresponding requirement for CPP measurement is defined.

- Proposal 3 (CPP): When UE supports DL PRS-RSCPD measurement inside measurement time window, as indicated in supportOfMeasurementsInTimeWindow, and LMF configures measurement time window(s) for RSCPD measurement:

- The existing requirements for DL RSTD measurement without time window apply to the DL RSTD measurement.

- The requirements for RSCPD measurement apply to DL RSCPD measurement for the PRS resource(s) that have occasions only within the measurement time window.

- Proposal 4 (CPP): When UE supports DL PRS-RSCPD and DL RSTD measurements inside measurement time window, as indicated in supportOfMeasurementsInTimeWindow, and LMF configures measurement time window(s) for RSCPD and RSTD measurements:

- The requirements for DL RSCPD apply to DL RSTD measurement and DL RSCPD measurement performed on the PRS resources in indicated PFL.

- The existing requirements for DL RSTD measurement without time window apply to DL RSTD measurement performed on the PRS resources of PFLs occurring outside of the PRS measurement time window.

- Proposal 5 (CPP): When UE supports DL PRS-RSCP measurement inside measurement time window, as indicated in supportOfMeasurementsInTimeWindow, and LMF configures measurement time window(s) for RSCP measurement:

- The existing requirements for UE RX-Tx measurement without time window apply to UE RX-Tx measurement.

- The requirements for RSCP measurement apply to DL RSCP measurement for the PRS resource(s) that have occasions only within the measurement time window.

- Proposal 6 (CPP): When UE supports DL PRS-RSCP and UE RX-Tx measurements inside measurement time window, as indicated in supportOfMeasurementsInTimeWindow, and LMF configures measurement time window(s) for RSCP and UE Rx-Tx measurements:

- The requirements for DL RSCP apply to UE RX-Tx measurement and DL RSCPD measurement performed on the PRS resources in indicated PFL.

- The existing requirements for UE RX-Tx measurement without time window apply to UE RX-Tx measurement performed on the PRS resources of PFLs occurring outside of the PRS measurement time window.

# References

[1] RP-230328, Revised WID on Expanded and Improved NR Positioning, Intel Corp, March, 2023.

[2] R4-2303190, WF on Expanded and Improved NR Positioning RRM requirements, Ericsson, March, 2023.

[3] R1-2301950, Summary #1 on Measurements and reporting for SL positioning, vivo, March 2023.

[4] R1-2301951, Summary #2 on Measurements and reporting for SL positioning, vivo, March 2023.

[5] R1-2301952	Summary #3 on Measurements and reporting for SL positioning, vivo, March 2023.

[6] R4-2306351, WF on RRM requirements for NR sidelink positioning and carrier phase positioning, CATT, April 2023.

[7] R4-2310163, WF on NR Positioning – SL positioning and CPP, CATT, May 2023.

[8] R4-2310077, Simulation assumptions for SL positioning, CATT, May 2023.

[9] R4-2314343, Ad-hoc minutes on R18 NR Positioning RRM requirements, Ericsson, Aug. 2023.

[10] R4-2314354, WF on R18 NR positioning – SL positioning and Carrier Phase Positioning, CATT, Aug. 2023.

[11] R4-2314358, LS on SL positioning and CPP measurements report mapping, Aug. 2023.

[12] R4-2312724, On SL positioning, Ericsson, Aug. 2023.

[13] RP-232670, Revised WID on Expanded and Improved NR Positioning, Intel Corporation, CATT, MediaTek, Sep. 2023.

[14] R4-2317380, WF on R18 NR positioning – SL positioning and Carrier Phase Positioning, CATT, Oct. 2023.

[15] R4-2321524, WF on R18 NR positioning – SL positioning and Carrier Phase Positioning, CATT, Nov. 2023.

[16] R4-2403368, WF on R18 NR positioning – SL positioning and Carrier Phase Positioning, CATT, March 2024.

[17] R4-2406384, WF on SL positioning and carrier phase positioning, CATT, Apr. 2024.

[18] R4-2410193, WF on SL positioning and carrier phase positioning, CATT, May 2024.

[19] R4-2411003, Reply LS on SL positioning measurement, RAN1, Aug. 2024.