3GPP TSG RAN WG4 Meeting #112bis	R4-2415200

Hefei, Anhui, China, 14th – 18th October, 2024

Agenda item:			7.1

Source:	vivo

Title:	Discussion on UE capability for multi-carrier enhancements

Document for:	Discussion

# Introduction

In the last RAN2 meeting, UE capability for switching between non-dormant and dormant BWPs triggered by the new DCI format 0-3 and 1-3 was introduced. The conclusion was informed to RAN4 with reply LS [1].

RAN2 discussed the two options and concluded to down-select approach 2 because Approach 1 is a non-backward compatible change from RAN2 perspective. RAN2 introduced a field bwp-SwitchingMultiDormancyCC-DCI-0-3-And-1-3-r18 corresponding to RAN4 feature 38-9, and a field scellDormancyWithinActiveTime-DCI-0-3-And-1-3-r18 corresponding to its prerequisite feature (RAN1 feature 49-9). It is up to RAN4 whether the UE shall report the same value between RAN4 features 6-3 and 38-9.

In this contribution, we provide our views on the UE capability in terms of reporting value for BWP switch.

# Discussion

In the last RAN4 meeting, the new UE capability was captured into the TS 38.133 based on the agreed CR [2]. The change on the requirements for simultaneous DCI based BWP switch delay on multiple CCs are as follows.

UE shall finish BWP switch within the time duration TMultipleBWPswitchDelay + Y, which is defined as: TMultipleBWPswitchDelay = TBWPswitchDelay + D*(N-1) Where: - TBWPswitchDelay is the BWP switching delay on single CC defined in Table 8.6.2-1 depending on UE capability bwp-SwitchingDelay [2]. TBWPswitchDelay shall be based on the smallest SCS among SCS of all involved CCs before and after BWP switch. If the BWP switch on multiple CCs results in the change of the SCS on any CC among involved CCs, TBWPswitchDelay shall be based on the smallest SCS among all SCS values of all involved CCs. - D is the incremental delay for each additional CC involved in simultaneous BWP switch and depends on UE capability bwp-SwitchingMultiCCs-r16 [TS 38.306, 14] for switching between non-dormant BWPs, and bwp-SwitchingMultiDormancyCCs-r16 or bwp-SwitchingMultiDormancyCC-DCI-0-3-And-1-3-r18 for switching between non-dormant and dormant BWPs. - For UE which is capable of per-FR gap, and no BWP switch involves SCS change, N is the number of CCs in same FR; For UE which is not capable of per-FR gap, or the BWP switches on any CC involves SCS changing, N is the number of CCs undergoing simultaneous BWP switch. - Y=0, if the serving cell where UE receives DCI for BWP switch is same as the serving cell on which BWP switch occurs for each involved serving cell. Y equals to the length of one slot at smaller SCS of scheduling cell, scheduled cells before and scheduled cells after active BWP change, - if the serving cell where UE receives DCI for BWP switch is different from the serving cell on which BWP switch occurs for any involved serving cell. If both scheduling cell and scheduled cell are in FR2-2, Y shall follow the SCS of 120 KHz. .

The overall BWP switch delay is based BWP switching delay on single CC plus incremental delay D for each additional CC. There are two type of BWP switch delay, i.e., switching between non-dormant BWPs and switching between dormant and non-dormant BWP. The related three UE capabilities for UE to report incremental delay value for BWP switch on multiple CCs is as follows.

bwp-SwitchingMultiCCs-r16: Incremental delay for BWP switch processing on additional CCs in timer/DCI based simultaneous BWP switching on multiple CCs.

bwp-SwitchingMultiDormancyCCs-r16: Incremental delay for BWP switch processing on additional SCells in DCI based simultaneous dormant BWP switching on multiple SCells with DCI 0-1/1-1/2-6.

bwp-SwitchingMultiDormancyCC-DCI-0-3-And-1-3-r18: Incremental delay for BWP switch processing on additional SCells in DCI based simultaneous dormant BWP switching on multiple SCells with DCI 0-3/1-3.

In general, different values can be reported by the three UE capabilities. For DCI 0-3/1-3, it can trigger simultaneous BWP switch between non-dormant BWPs and between dormant and non-dormant BWP. The UE may report different values for the two types of switching. Then, the incremental delay can be different on the additional CCs related to different UE capability.

However, based on current requirements, single value D is used in the requirements. It would implicitly indicate single value D should be reported. It is not clear what requirements should apply or whether there are no requirements at all when different incremental values are reported.

From requirements perspective, it would be better to allow UE to report different values with different UE capability.

Proposal 1: UE is allowed to report different incremental value than D for the new introduced UE capability bwp-SwitchingMultiDormancyCC-DCI-0-3-And-1-3-r18 and requirements are changed accordingly.

# Summary

In this contribution, we further provided views the UE capability in terms of reporting value for BWP switch. Following proposal is present.

Proposal 1: UE is allowed to report different incremental value than D for the new introduced UE capability bwp-SwitchingMultiDormancyCC-DCI-0-3-And-1-3-r18 and requirements are changed accordingly.

- References

- R2-2405992	Reply LS on UE capability for multi-carrier enhancements

- R4-2412215	Update DCI based BWP switch delay on multiple CCs in multi-carrier enhancement, Huawei, HiSilicon