3GPP TSG-RAN WG4 Meeting #114												R4-2500042

Athens, Greece, 17 – 21 February, 2025

Agenda item:			7.14.2.1

Source:	Huawei, HiSilicon

Title:	on XR OTA tests

Document for:	Approval

# Introduction

This contribution discusses the issues listed below in WF [1] from the last meeting..

Issue 2-2-1: General views on Performance requirements for XR devices

Agreements:

  - FFS on whether Glasses and head-mounted displays should have separate performance requirements.

# Discussion

TRP and TRS performances are closely related to antenna efficiency. For example, the TRP values in free space can be estimated as maximum transmit power minus antenna efficiency with both values in decibels. In other words, the higher the antenna efficiency, the better the TRP and TRS performances.  Antenna efficiency depends on the design space available as the aim of antenna design is to match 50 ohms impedance from a transmission line to 377 ohms impedance in free space or air to ensure a smooth transfer of RF energy from a transmission line to open space. Larger antenna design space usually results in higher antenna efficiency.

Glasses has much less design space for antennas compared with head mounted displays. As a result, they have different TRP and TRS performances. If glasses and head mounted displays are grouped in the same category for TRP and TRS measurement campaign, it would be difficult for glasses to pass the TRP and TRS performance thresholds based on measurement campaign for both glasses and head mounted displays. Conversely, it would be easier for head mounted displays to meet such thresholds.

Proposal 1: have separate performance requirements for head mounted displays and glasses.

# Conclusion

This contribution presents the following proposal.

Proposal 1: have separate performance requirements for head mounted displays and glasses.

# References

- R4-2419808 WF for [113][325] TRP_TRS_Ph3