3GPP TSG-RAN WG4 Meeting #114	R4-2500048

Athens, GR, 17th – 21st February, 2025

Agenda item:	7.18.2

Source:	MediaTek Inc.

Title:	Discussion on UE requirements with 8Rx MMSE-IRC receiver

Document for:	Discussion

# Introduction

A WF for UE 8Rx MMSE-IRC receiver requirements was agreed upon at the RAN4#113 meeting [1]. In this contribution, we present our views on the outstanding issues.

- Discussion

## 2.1 General

Definition of MMSE-IRC receiver capability for 8Rx UE - Proposals - Option 1: Not to introduce new UE capability in Rel-19 or to update the existing Rel-18 UE capability definition. Instead, clearly state in the applicability rule that the new Rel-19 requirements under MU-MIMO are applicable for UE supporting SU-MIMO 8Rx receiver. - Option 2: Update the existing Rel-18 UE capability definition as a maintenance part of the R18 FR1 RF enhancement WI. - Option 3: Introduce new Rel-19 UE capabilities for MU-MIMO 8Rx receiver. And clearly state that the UE supporting ‘MU-MIMO 8Rx receiver’ are also required to support ‘SU-MIMO 8Rx receiver’ capability. - Option 4: Redefine the simplified and baseline receiver definitions in 38.101-4 section 3.1 to the following: - Baseline 8Rx Receiver: 8Rx receivers for MIMO transmissions with support of up to 8 layers with joint 8Rx MIMO detector. - Simplified 8Rx Receiver: 8Rx receivers for MIMO transmissions with support of only up to 4 layers with two joint 4Rx MIMO detectors. 8Rx UE MMSE-IRC receiver coverage - Proposals - Option 1: RAN4 to define two sets of requirements (baseline 8Rx receiver and simplified 8Rx receiver). - Option 2: Set the criteria for defining two sets of requirements as: If the span after removing a reasonable number or outliers (at least 5 results should be left) is >2.5dB, define two sets of requirements. The evaluation of the span shall be done for each configuration agreed for requirement definition. - Option 3: Postpone defining criteria for decision of one or two sets of requirements until all test cases are finalized.

From simulation results at RAN4#113, there are non-negligible performance difference between baseline and simplified receiver. Therefore, we support to define two set of requirements as what RAN4 did in Rel-18 8Rx WI. As there will have two set of requirements, we suggest updating the definition of MMSE-IRC receiver capability for 8Rx UE with Option 4.

Proposal 1: Define two set of requirements for baseline and simplified 8Rx MMSE-IRC receiver.

Proposal 2: Update the definition of MMSE-IRC receiver capability for 8Rx UE as:

- Baseline 8Rx Receiver: 8Rx receivers for MIMO transmissions with support of up to 8 layers with joint 8Rx MIMO detector.

- Simplified 8Rx Receiver: 8Rx receivers for MIMO transmissions with support of only up to 4 layers with two joint 4Rx MIMO detectors.

## 2.2 PDSCH requirements with intra-cell inter-user interference

Agreement: - Define requirements for: - Rank 2+2 - FFS: MCS17/19 - TBD: Rank 4+4: MCS13 Precoding method for the co-scheduled UE Agreement: - Rank 2+2: orthogonal - Rank 4+4 (if introduced): orthogonal DMRS Ports Agreement: - For 4+4, use different CDM group. Serving UE with ports {1000, 1001,1004, 1005} and co-scheduled UE with ports {1002, 1003, 1006, 1007} Other parameters Agreement: - For PDSCH demod requirements in intra-cell inter-user interference reuse the parameters from Rel-17 2RX/4RX for those not explicitly discussed/ captured.

For Rank 2+2, RAN4 agreed to consider MCS17 or MCS19 for defining requirements. Based on our simulation results [2], MCS17 can be used to differentiate UE implementations, making it a feasible option for defining requirements.

For Rank 4+4, the gNB must identify two UEs that exhibit very high orthogonality, meaning that the 4-layer signals are well-separated in terms of their spatial characteristics. This separation allows each UE to decode the signals without significant interference. Achieving such high orthogonality between two UEs is challenging in real-world environments because it requires optimal conditions, precise alignment, and coordination. Factors such as varying user locations, and dynamic channel conditions can make it difficult to maintain the necessary level of orthogonality. As a result, the 4+4 rank combination is deemed not very practical for deployment in typical scenarios

Proposal 3: For Rank 2+2, use MCS17 to define requirements.

Proposal 4: Do not define Rank 4+4 requirements.

## 2.3 CQI requirements with inter-cell interference

At the last meeting, RAN4 agreed to reuse the principle of CSI-RS configuration from Rel-17. However, the actual configurations are still to be determined (TBD).

CSI-IM and NZP CSI-RS configurations Agreement: - For CSI-IM and NZP CSI-RS assumption, reuse agreement from R17 2/4Rx inter-cell interference suppressing requirement for simulation, which are: - CSI-IM on serving cell overlapping with PDSCH from interference cell - NZP CSI-RS on serving cell overlapping with NZP CSI-RS from interference cell - TBD: Actual configuration to be used. - Proposals: - Option 1: FDD: First subcarrier index in the PRB used for CSI-RS (k0) - Row 5,(4) for the serving cell - Row 5,(6) for the interference cell - Other options not precluded

The current Rel-17 2/4Rx inter-cell interference requirements in Rel-17 do not actually adhere to the agreements.

For the FDD requirements in Clauses 6.2.2.1.2.3 and 6.2.3.1.2.3,

- The CSI-IM on the target cell, (kCSI-IM, lCSI-IM) = (4, 9), overlaps with the ZP CSI-RS (k0=Row5,(4) and I0 = 9) from the interference cell, instead of the PDSCH from the interference cell.

Figure 1. CSI-RS configurations in Rel-17 FDD 2/4Rx inter-cell interference requirements

There is another agreement in [2], which states that

Issue 3-1: Test configuration - Agreement: - Serving cell - Reuse assumptions from Sections 6.2.2.1.2.1, 6.2.2.2.2.1, 6.2.2.2.2.1, 6.2.3.2.2.1 in TS 38-101-4.

For TDD requirements in Clause 6.2.2.2.2.3 and 6.2.3.2.2.3,

- The ZP CSI-RS configuration, k0 = Row3,(8) and I0 = 9, is different from that in Clause 6.2.2.2.2.1 and 6.2.3.2.2.1, where k0 = Row5,(4) and I0 = 9.

- The number of CSI-RS ports (X) for NZP is 2, not 4, as specified in settings 6.2.2.2.2.1 and 6.2.3.2.2.1.

Figure 2. CSI-RS configurations in Rel-17 TDD 2/4Rx inter-cell interference requirements

Observation 1: The current CSI-RS configuration for Rel-17 2/4Rx inter-cell interference requirements did not follow agreements in Rel-17.

Therefore, we suggest applying CSI-RS configurations in Table 1 for both FDD and TDD in CQI simulations.

Table 1. CSI-RS configurations used in CQI simulations

| ZP CSI-RS configuration | CSI-RS resource Type |  | Periodic | Periodic |
| --- | --- | --- | --- | --- |
|  | Number of CSI-RS ports (X) |  | 4 | 4 |
|  | CDM Type |  | FD-CDM2 | FD-CDM2 |
|  | Density (ρ) |  | 1 | 1 |
|  | First subcarrier index in the PRB used for CSI-RS (k0) |  | Row 5,(4) | Row 5,(6) |
|  | First OFDM symbol in the PRB used for CSI-RS (l0) |  | Row 5,(9) | Row 5,(9) |
|  | CSI-RS periodicity and offset | slot | 10/1 | Same as serving cell |
| NZP CSI-RS for CSI acquisition | CSI-RS resource Type |  | Periodic | Periodic |
|  | Number of CSI-RS ports (X) |  | 2 | 1 |
|  | CDM Type |  | FD-CDM2 | noCDM |
|  | Density (ρ) |  | 1 | 1 |
|  | First subcarrier index in the PRB used for CSI-RS (k0) |  | Row 3,(6) | Row 2,(6) |
|  | First OFDM symbol in the PRB used for CSI-RS (l0) |  | Row 3,(13) | Row 2,(13) |
|  | NZP CSI-RS-timeConfig periodicity and offset | slot | 10/1 | Same as serving cell |
| CSI-IM configuration | CSI-IM resource Type |  | Periodic | Periodic |
|  | CSI-IM RE pattern |  | 0 | 0 |
|  | CSI-IM Resource Mapping (kCSI-IM,lCSI-IM) |  | (4, 9) | (6,9) |
|  | CSI-IM timeConfig periodicity and offset | slot | 10/1 | Same as serving cell |

Proposal 5: Use the proposed CSI-RS configurations in Table for FDD and TDD CQI requirements simulations.

## 2.4 Applicability rule and release independent

Test applicability to skip 2/4Rx requirements - Proposals - Option 1: Defining applicability rules to skip 2/4Rx requirements if 8Rx tests are passed. - Option 1a: For 8Rx inter-cell interference scenario and intra-cell inter-user interference scenario, for both demodulation performance tests and CQI reporting tests, prefer to define a test applicability rule to skip 2/4Rx requirements if 8Rx tests are passed, no matter same test configurations as 2RX/4RX are defined or not. - Option 1b: For UEs supporting both 8Rx and 2/4Rx, the 2/4Rx tests under inter cell interference and under intra cell inter user interference can be skipped if the corresponding 8Rx requirements are passed.

Considering 8Rx processing is more complicated than 2Rx/4Rx, we support Option 1a to save UE testing burden.

Proposal 6: For 8Rx inter-cell interference scenario and intra-cell inter-user interference scenario, for both demodulation performance tests and CQI reporting tests, prefer to define a test applicability rule to skip 2/4Rx requirements if 8Rx tests are passed, no matter same test configurations as 2RX/4RX are defined or not.

# Conclusion

In this contribution, we provide our views on UE requirements with 8Rx MMSE-IRC receiver. Our observations and proposals are captured below:

Proposal 1: Define two set of requirements for baseline and simplified 8Rx MMSE-IRC receiver.

Proposal 2: Update the definition of MMSE-IRC receiver capability for 8Rx UE as:

- Baseline 8Rx Receiver: 8Rx receivers for MIMO transmissions with support of up to 8 layers with joint 8Rx MIMO detector.

- Simplified 8Rx Receiver: 8Rx receivers for MIMO transmissions with support of only up to 4 layers with two joint 4Rx MIMO detectors.

Proposal 3: For Rank 2+2, use MCS17 to define requirements.

Proposal 4: Do not define Rank 4+4 requirements.

Proposal 5: Use the proposed CSI-RS configurations in Table for FDD and TDD CQI requirements simulations.

| ZP CSI-RS configuration | CSI-RS resource Type |  | Periodic | Periodic |
| --- | --- | --- | --- | --- |
|  | Number of CSI-RS ports (X) |  | 4 | 4 |
|  | CDM Type |  | FD-CDM2 | FD-CDM2 |
|  | Density (ρ) |  | 1 | 1 |
|  | First subcarrier index in the PRB used for CSI-RS (k0) |  | Row 5,(4) | Row 5,(6) |
|  | First OFDM symbol in the PRB used for CSI-RS (l0) |  | Row 5,(9) | Row 5,(9) |
|  | CSI-RS periodicity and offset | slot | 10/1 | Same as serving cell |
| NZP CSI-RS for CSI acquisition | CSI-RS resource Type |  | Periodic | Periodic |
|  | Number of CSI-RS ports (X) |  | 2 | 1 |
|  | CDM Type |  | FD-CDM2 | noCDM |
|  | Density (ρ) |  | 1 | 1 |
|  | First subcarrier index in the PRB used for CSI-RS (k0) |  | Row 3,(6) | Row 2,(6) |
|  | First OFDM symbol in the PRB used for CSI-RS (l0) |  | Row 3,(13) | Row 2,(13) |
|  | NZP CSI-RS-timeConfig periodicity and offset | slot | 10/1 | Same as serving cell |
| CSI-IM configuration | CSI-IM resource Type |  | Periodic | Periodic |
|  | CSI-IM RE pattern |  | 0 | 0 |
|  | CSI-IM Resource Mapping (kCSI-IM,lCSI-IM) |  | (4, 9) | (6,9) |
|  | CSI-IM timeConfig periodicity and offset | slot | 10/1 | Same as serving cell |

Proposal 6: For 8Rx inter-cell interference scenario and intra-cell inter-user interference scenario, for both demodulation performance tests and CQI reporting tests, prefer to define a test applicability rule to skip 2/4Rx requirements if 8Rx tests are passed, no matter same test configurations as 2RX/4RX are defined or not.

# Reference

- R4-2419784, Way forward for [113][322] NR_demod_Ph5_Part2_UE, Nokia

- R4-2115732, WF on CSI requirements for inter-cell interference MMSE-IRC, Ericsson

- R4-2500049, Simulations for UE demodulation requirements with 8Rx MMSE-IRC receiver, MediaTek