3GPP TSG-RAN WG4 Meeting #114                                           R4-2500161

Athens, GR, 17th – 21st February, 2025

Source: 	Huawei, HiSilicon

Title: 	Further consideration on BS RF for Rel-19 LP-WUS

Agenda Item:	7.26.3

Document for:	Approval

Introduction

BS RF requirements have been further discussed for LP-WUS and some aspects to be further analysed are captured in WF [1].

This contribution provides further analysis for the issues identified in the WF.

Discussion

Dynamic range/power boosting

The most outstanding issue for BS RF part is the requirement of dynamic range/power boosting for LP-WUS. Some progresses were made in last meeting and the WF is listed as below:

- “The LP-WUS power boosting is the difference between the average power of LP-WUS REs (which occupy certain REs within a NR transmission bandwidth configuration) and the average power over all REs (from both LP-WUS and the NR carrier containing the LP-WUS REs)” is considered as starting point for the definition.

  - FFS whether other cases can be additionally considered. E.g., single value declaration.

Although the NB-IoT-like definition would result in variable values for different CBWs, we recognize that maintaining flexibility in power declaration for each supported CBW can be reasonable in some cases. In that sense, we could accept the definition of power boosting similar to NB-IoT is used as baseline, additionally, if vendors want to declare the power boosting level with a single level for simplicity, the choice should be allowed as well. To this end, we think that a merged version could be a compromise for both sides. The definition of LP-WUS power boosting/dynamic range which accommodates both flexibility and simplicity is proposed as following:

  - The LP-WUS RB power dynamic range is the difference between the average power of LP-WUS REs (which occupy certain REs within a NR transmission bandwidth configuration) and the average power over all REs (from both LP-WUS and the NR carrier containing the LP-WUS REs). If a single power booting value is to be declared irrespective of channel bandwidth, the difference in per-RE power ratios between the average power of LP-WUS REs (within the NR transmission bandwidth configuration) and the average power NR REs (excluding LP-WUS REs) could alternatively be used.

Proposal 1: An NB-IoT-like approach to defining power boosting could serve as a baseline, offering flexibility in declaration. Additionally, the option for a single-value declaration should also be permitted, accommodating different scenarios.

Based on our previous analysis and inputs from other companies, obviously it has less benefit to support power boosting for low channel BWs as the impact to NR signal is too large, which is not the purpose of boosting power of LP-WUS in sacrifice of coverage of NR. RAN1 agreed that for 15 kHz SCS, the LP-WUR retains 11 RBs, allowing limited power boosting within a 5 MHz channel bandwidth. Adopting an NB-IoT-like definition as a baseline for LP-WUS dynamic range permits possibility to declare different boosting levels across CBWs without limiting the CBW. However, if a single per-RE power boosting value is declared, the smallest CBW supporting the boost should be declared.

Proposal 2: For an NB-IoT-like power boosting declaration, excluding small channel bandwidths is unnecessary. However, if using the alternative single-value declaration, the smallest supported channel bandwidth should be explicitly declared.

With settlement of the definition, there are still several issues around power boosting requirement to be addressed. One of them is the different considerations for de-boosting of NR transmission power. We have quite different proposals in this regard:

- Proposals

  - Proposal 1: LP-WUS power boosting does not change the total maximum transmit power of BS.

    - There is no power boosting for the case where all RBs are used by LP-WUS and 0 RBs for NR, i.e., SCS 30kHz and CBW 5MHz.

    - For the cases where there would be RB(s) allocated to NR, x dB NR power degradation is allowed.

  - Proposal 2: The average power over all REs should be fixed and when the power boost of WUS is enabled with or w/o the NR signal at the same time:

    - The WUS signal should be transmitted always with x dB above the average power.

    - There is no NR signal de-boosting in normal transmission.

    - The NR de-boosting transmission power is for the testing purpose only and whether to de-boost NR signal transmission power should be up to network implementation.

For the two proposals above regarding de-boosting of NR transmission power, proposal 1 is aligned with our understanding, while proposal 2 is more like some consideration from operation perspective rather than from requirement perspective. To make things simple and move forward, we feel it would be better to focus on the requirement itself.

Whether minimum requirement should be specified for power boosting has been discussed quite a while but not conclusion yet. It looks weird in the specification that only definition is included in the core spec without specific value. We still think that a minimum requirement is required in the core part of the specification, otherwise, we think the power boosting requirement itself is not necessary.

Proposal 3: The minimum requirement of LP-WUS power boosting should be specified. Otherwise, no need to have the power boosting requirement in the specification. Specific value can be further discussed.

Other requirements other than dynamic range

Besides the dynamic range requirement, other requirements have not been discussed in the SI and the first meeting of Rel-19 for LP-WUS. It is understandable that the introduction of LP-WUS should have minimal impact on BS implementation, as legacy BS must also accommodate this feature. In last meeting, we have the following WF:

  - Discuss on whether transmit signal quality should be defined

    - FFS whether equivalent EVM requirement could be considered,

      - E.g. NR needs to pass 64QAM EVM test with LP-WUS embedded in the NR carrier

  - FFS whether no other new RF requirements than the dynamic range or potential transmit quality requirement identified for LP-WUS

During the previous discussion, regardless whether requirements should be defined for spectrum related requirements, e.g. unwanted emissions, at least the common understanding among companies is: At least current regulation relevant spectrum requirements, e.g. SEM, SE should also be applied for LP-WUS.

Regarding transmitted signal quality, due to the inherent characteristics of low-order modulation in LP-WUS OOK signals, EVM metrics are not expected to be a bottleneck. Furthermore, frequency error can be ensured through testing with NR signals only. However, preliminary simulation results indicate that the embedded LP-WUS signal may affect the transmitted quality of the NR signal. The combined signals exhibit a slightly increased PAPR compared to NR signals alone, which could cause some performance degradation for NR, especially with higher modulation orders.

Two options are considered for the transmitter signal quality requirement:

- Option 1: Transmitted signal quality requirements should be defined for LP-WUS, but depending on further RAN1 progress on waveform design.

- Option 2: No need to introduce transmitted signal quality requirement for LP-WUS.

The choice of which option to consider for the LP-WUS EVM requirement also depends on the testing methodology. It is noted that for in-band operation, except the dynamic range requirement, no other requirements are specified for NB-IoT. Additionally, though LP-WUS draws from IEEE 802.11ba, there is also no EVM requirement for the OOK waveform there. Thus, we prefer to specifying only a dynamic range requirement, similar to NB-IoT in-band operation, leaving compliance to regulatory requirements up to implementation choices.

Specifically, transmitted signal quality of LP-WUS could be verified with 64QAM NR signal. Though the test is not directly performed for LP-WUS signal, if there is no obvious EVM degradation for NR signal with high order modulation scheme, the signal quality of LP-WUS can be ensured as well in the simultaneous transmission of both NR and LP-WUS.

Proposal 4: No specific transmit signal quality requirement is defined for LP-WUS. Instead, a higher-order EVM test for NR with embedded LP-WUS is utilized as the verification metric.

Conclusion

This contribution provides further consideration for BS RF of LP-WUS. With above analysis, we have the following proposals:

Proposal 1: An NB-IoT-like approach to defining power boosting could serve as a baseline, offering flexibility in declaration. Additionally, the option for a single-value declaration should also be permitted, accommodating different scenarios. A definition that incorporates perspectives from both sides is proposed as follows:

  - The LP-WUS RB power dynamic range is the difference between the average power of LP-WUS REs (which occupy certain REs within a NR transmission bandwidth configuration) and the average power over all REs (from both LP-WUS and the NR carrier containing the LP-WUS REs). If a single power booting value is to be declared irrespective of channel bandwidth, the difference in per-RE power ratios between the average power of LP-WUS REs (within the NR transmission bandwidth configuration) and the average power NR REs (excluding LP-WUS REs) could alternatively be used.

Proposal 2: For an NB-IoT-like power boosting declaration, excluding small channel bandwidths is unnecessary. However, if using the alternative single-value declaration, the smallest supported channel bandwidth should be explicitly declared.

Proposal 3: The minimum requirement of LP-WUS power boosting should be specified. Otherwise, no need to have the power boosting requirement in the specification. Specific value can be further discussed.

Proposal 4: No specific transmit signal quality requirement is defined for LP-WUS. Instead, a higher-order EVM test for NR with embedded LP-WUS is utilized as the verification metric.

- References

[1] R4-2419802, WF for [113][308] NR_LPWUS, Huawei, HiSilicon