3GPP TSG-RAN4 #114	R4-2500168

Athens, Greece, 17th Feb. – 21st Feb. 2025

Title:	Calculation of satellite motion by 4-th order Runge-Kutta method in the ECEF frame

Source:			Anritsu Corporation

Agenda Item:			7.10.4

Document for:			Approval

# 1.	Introduction

At the RAN4 #113 meeting in Orlando, we continued a discussion on a methodology to derive time varying doppler shift and propagation delay based on the two-body Keplerian propagator model [1]. Most of proposals from companies were based on procedures using the Newton method to numerically solve the differential equation of Keplerian while we proposed an alternative method utilizing the 4-th order Runge-Kutta (RK) method. [2]

In this contribution we share a supplemental information of the 4-th order RK method and our analysis to compare the Newton and RK methods.

Extract from [1] Sub-topic 1-1 TE emulated channel model for satellite mobility Issue 1-1-1 Time varying Doppler shift and propagation delay derivation procedure - WF - For Keplerian definition motion base method, consider the following general procedure to generate the time varying Doppler shift and propagation delay. The detail formulars for each step can be considered as reference in Appendix 1.4. (*snip*) - The proponents to use the equation of motion to derive the satellite position/velocity in Steps 1/2, as an alternative method as the Keplerian model, is encouraged to provide more detail procedure and the comparison results with the Keplerian model in the next meeting

# 2.	Discussion

  - Comparison of procedures between Newton method and 4-th order Runge-Kutta method

First, we would like to show a flow to derive doppler frequency shift and propagation delay of satellite communication signals by Newton method and 4-th order RK method. The biggest difference between these two methods is that there are cases that calculations are carried out in ECI (Earth-Centered Inertial) frame or ECEF (Earth-Centered Earth-Fixed) frame with the Newton method while only ECEF frame is applied with the RK method during its calculation flow. And most of all, 4-th order RK method has a benefit of its much light computation load in time compared to the numerical solution of the Keplerian by the Newton method while a calculation accuracy is equivalent.

Observation 1: 4-th order Runge-Kutta method has following benefits while a calculation accuracy is equivalent with Keplerian.

- calculation of doppler frequency shift and propagation delay can be carried out only in the ECEF frame

- computation load is much light in time compared to the numerical solution of the Keplerian

Equation of motion (ECI Frame)

ma=F

a=d2rdt2

Solve Kepler Eq. (ECI Frame)

M=E-e sin⁡(E)

Transform position, velocity and acceleration from (ECI Frame) to (ECEF Frame)

r→r

v→v+ω×r

a→a+2 ω×v+ω×(ω×r)

Determine SV orbit (ECI Frame)

rt, vt@(ECI Frame)

Transform

(ECI Frame) to (ECEF Frame)

{ rt, vt} @(ECEF Frame)

Eq. of motion in ECEF Frame

d2rdt2 =-μ r2 r r +2 v×ω +ω×(r×ω )

Solve Differential equation by Runge-Kutta method.  (ECEF Frame)

{ rt, vt} @(ECEF Frame)

Calculate Delay and Doppler Frequency (ECEF Frame)

Newton-Raphson method

4th order Runge-Kutta method         (See sub-clause 2.3)

See appendix C for detail

Figure 2.1-1: Comparison of procedures between the Newton method and 4-th order Runge-Kutta method

For a detailed procedures to derive the motion equation in ECEF frame, see appendix C.

Detailed calculation steps including the use of 4-th order RK method and comparison of its calculation accuracy are described from the next sub-clause.

  - Equations of motion followed by satellites in ECEF frame

As explained in detail in the appendix C, a motion followed by satellites in the ECEF frame, in other words CTRS (Conventional Terrestrial Reference System), can be expressed by the following vector equation (1) [3].

d2rdt2 =-μ  r2 rr +2  v×ω +ω×r×ω#1

After this, to express the angular velocity factor with the general expression, ω is replaced by Ω (dot omega).

d2rdt2 =-μ  r2 rr +2  v×Ω +ω×r×Ω#1'

r=x,y,z#2

r=x2+y2+z2 #3

Here Ω is the WGS-84 Earth rotation rate, Ω=0, 0, 7.2921151467×10-5 radsec., μ is the Earth gravitational parameter (μ= GM ) μ=3.986005×1014[m3/sec.2], and x, y, z are a coordinate of satellite position state vector in ECEF.  And r and v are position and velocity with vector representation.

Transforming the expression of the second-order ordinary differential equation (1’) into the form of a simultaneous equation yields the following:

dvdt=-μ  r2 rr +2 v×Ω+Ω×r×Ω#4

drdt=v#5

Observation 2: The motion equation to be solved under the condition of the ECEF frame and two-body Keplerian can be expressed as follow vector equations:

dvdt=-μ  r2 rr +2 v×Ω+Ω×(r×Ω)

drdt=v

2.3 Numerically solve the differential equation (4-th order Runge-Kutta Method) [4]

Refer initial satellite position and velocity based on SIB-19 ephemeris information.

1st step: Calculation of k(1,v) and k(1,r)  (See following explanation in this sub-clause.)

2nd step: Calculation of k(2,v) and k(2,r) using the outcome of the 1st step.

3rd step: Calculation of k(3,v) and k(3,r) using the outcome of the 2nd step.

4th step: Calculation of k(4,v) and k(4,r) using the outcome of the 3rd step.

5th step: Calculation of rn+1 and vn+1 at time tn+1. Go to 1st step and use rn+1 and vn+1 in the calculation of k(1,v) and k(1,r). Repeat till the end of measurement time.

Next, there are several methods to numerically solve the differential equation, e.g. Euler method, Predictor-Corrector method, Runge Kutta method, etc. And in this contribution, we solve the simultaneous equations (4) and (5) utilizing the 4-th order Runge-Kutta method. Calculation flow is described as follows.

Figure 2.3-1: Calculation flow by 4-th order Runge-Kutta method

First, the equation (4) in clause 2.2 can be expressed as the acceleration vector since (4) is the differential of the velocity.

dvdt=a rn ,vn  =-μ rn2  rnrn +2 vn×Ω+Ω×rn×Ω#6

Here

rn= xn ,yn ,zn #7

vn=vx, n ,vy,n ,vz,n#8

rn=xn2+yn2+zn2#9

The vector function a( rn ,vn  ) with a condition Ω=0,0,Ω can be expressed in concrete form by the following equations:

ax xn,yn,zn,vx,n , vy,n ,vz,n =-μ rn2  xnrn +2 vy,n Ω+xnΩ2 #10-1

ay xn,yn,zn,vx,n , vy,n ,vz,n =-μ rn2  ynrn -2 vx,n Ω+ynΩ2 #10-2

az xn,yn,zn,vx,n , vy,n ,vz,n =-μ rn2  znrn #10-3

Note that the condition Ω=0,0,Ω is set when using the ECEF frame such that the centre of earth's gravity is the origin, and the rotation axis is the z-axis.

Observation 3: The vector function can be expressed by the following equations:

ax xn,yn,zn,vx,n , vy,n ,vz,n =-μ rn2  xnrn +2 vy,n Ω+xnΩ2

ay xn,yn,zn,vx,n , vy,n ,vz,n =-μ rn2  ynrn -2 vx,n Ω+ynΩ2

az xn,yn,zn,vx,n , vy,n ,vz,n =-μ rn2  znrn

Next based on the 4-th order Runge-Kutta method, we can calculate from k(1,v) to k(4,v), and from k(1,r) to k(4,r) with step by step manner. In the next step 1 to 5, Δt is a step size of time. From the position and velocity vector (rn,vn) at time tn(=n Δt), equations from (11) to (16) are successively calculated to determine (rn+1,vn+1) at time tn+1. Once the (rn+1,vn+1) is obtained at step 5, then we go back to step 1 and use these values in the equations (11-1) and (11-2) to obtain the position and velocity vector at the next step time. Continuing these calculations until the end of measurement time will provide position and velocity information throughout the measurement period with the step size granularity.

Step 1: Calculate following k(1,v) and k(1,r).

k1,  v=a rn,vn  Δt#11-1

k1, r=vn Δt#11-2

If we transform equations (11-1) and (11-2) to the concrete form like (10-1), (10-2) and (10-3), vector functions above can be transformed to the following equations.

kvx=ax xn,yn,zn,vx,n , vy,n ,vz,n ×Δt=-μ rn2  xnrn +2 vy,n Ω+xnΩ2×Δt#12-1

kvy=ay xn,yn,zn,vx,n , vy,n ,vz,n ×Δt=-μ rn2  ynrn -2 vx,n Ω+ynΩ2×Δt #12-2

kvz=az xn,yn,zn,vx,n , vy,n ,vz,n ×Δt=-μ rn2  znrn ×Δt#12-3

krx=vx,n×Δt#12-4

kry=vy,n×Δt#12-5

krz=vz,n×Δt#12-6

Calculations can be carried out also from step 2 to step 5 with the same manner. For the simplicity of the expression, equations are described by the form of vector function after this.

Step 2: Calculate following k(2,v) and k(2,r) using the outcome of Step 1.

k2, v=a rn+12k1, r ,vn+12k1, v Δt#13-1

k2,  r=vn+12k1, vΔt#13-2

Step 3: Calculate following k(3,v) and k(3,r) using the outcome of Step 2.

k3, v=a rn+12k2, r ,vn+12k2, v Δt#14-1

k3,  r=vn+12k2, vΔt#14-2

Step 4: Calculate following k(4,v) and k(4,r) using the outcome of Step 3.

k4, v=a xn+k3, r ,vn+k3, v  Δt#15-1

k4,  r=vn+k3, v Δt#15-2

Step 5: Calculate following vn+1 and rn+1 using the outcome from Step 1 to Step 4.

vn+1=vn+16k1,  v+2k2,  v+2k3,  v+k4,  v#16-1

rn+1=rn+16k1,  r+2k2,  r+2k3, r+k4,  r#16-2

Go back to step 1 and continue calculation to derive position and velocity till the end of measurement period.

2.4 Initial conditions for numerical solution

Based on the definition in TS 38.508-1 [5], we use the following position and velocity of a LEO satellite and position of a UE as initial conditions. Note these values are described in ECEF frame in Table 5.6.2.1-2 of [5] and described in the ellipsoidal coordinate system in sub-clause 5.6.1 of [5].

Observation 4: Note the position and velocity of satellite are described in ECEF frame, while the position of the UE is described in the ellipsoidal coordinate system in TS 38.508-1.

Table 5.6.2.1-2: SIB19- NR NTN Ephemeris information for NGSO (LEO-600) satellites (zero Doppler conditions)

| Derivation Path: Table 5.6.3.1-1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SIB19-r17 ::= SEQUENCE { |  |  |  |
| ntn-Config-r17 SEQUENCE { |  |  |  |
| epochTime-r17 | Not present |  |  |
| ephemerisInfo-r17 CHOICE { |  |  |  |
| positionVelocity-r17 SEQUENCE { |  |  |  |
| positionX-r17 | -2550081 |  |  |
| positionY-r17 | 4151097 |  |  |
| positionZ-r17 | 2263034 |  |  |
| velocityVX-r17 | 21970 |  |  |
| velocityVY-r17 | -48164 |  |  |
| velocityVZ-r17 | 114109 |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| NOTE 1: Satellite-UE elevation angle equal to 89.61 degrees, one-way delay equal to 2.03 ms and Doppler equal to 0.013 ppm |  |  |  |

UE location (TS 38.508-1)

5.6.1 UE location UE shall determine its location during the test using any of the following means. 1. UE location for the test shall be provided to the UE via pre-configured means. During the test the UE location is not expected to change unless explicitly stated as a requirement for the test. 2. Other options such as providing UE location via AT command are not precluded. UE location provided to the UE before the start of the test is as follows for RF Tx/Rx and DEMOD test cases for both GSO and NGSO config is as follows: Longitude: 121.56076999 Latitude: 25.08439333 (NGSO satellites), 55.0 (GSO satellites) Altitude: 0

The position and velocity of the satellite described in ECEF Frame need to be converted based on the definitions in TS 38.331 [6] as EphemerisInfo field descriptions.

EphemerisInfo field descriptions

positionX, positionY, positionZ X, Y, Z coordinate of satellite position state vector in ECEF. Unit is meter. Step of 1.3 m. Actual value = field value * 1.3.

velocityVX, velocityVY, velocityVZ X, Y, Z coordinate of satellite velocity state vector in ECEF. Unit is meter/second. Step of 0.06 m/s. Actual value = field value * 0.06.

The converted satellite coordinate in ECEF Frame (X_SV, Y_SV, Z_SV):

(X_SV, Y_SV, Z_SV) = ( -3315105.300000 [m], 5396426.100000 [m], 2941944.200000 [m])

The converted satellite velocity in ECEF Frame (VX, VY, VZ):

(VX, VY, VZ) = (1318.200000 [m/s], -2889.840000 [m/s], 6846.540000 [m/s])

Also, the position of the UE needs to be converted to ECEF Frame by the following equations since the definitions of the UE location in TS 38.508-1 above are described based on the ellipsoidal coordinate system.

xyz=N+hcosϕcosλN+hcosϕsinλN1-e2+hsinϕ#(17)

Here f is geodetic latitude, l is east longitude, h is altitude from geoid, a is average earth equatorial radius, e=f(2-f)  where f is the earth’s declination, and

N=a1-e2sin2ϕ1 2

The converted UE coordinate in ECEF Frame:

(X_UE, Y_UE, Z_UE) = (-3025296.935779 [m], 4925102.789830 [m], 2687544.210048 [m])

Observation 5: Initial conditions of satellite position, velocity and UE position in ECEF frame can be as follows.

(X_SV, Y_SV, Z_SV) = ( -3315105.300000 [m], 5396426.100000 [m], 2941944.200000 [m])

(VX, VY, VZ) = (1318.200000 [m/s], -2889.840000 [m/s], 6846.540000 [m/s])

(X_UE, Y_UE, Z_UE) = (-3025296.935779 [m], 4925102.789830 [m], 2687544.210048 [m])

2.5 Comparison of propagation delay and doppler frequency shift with different calculation step sizes

Using the aforementioned equations, we calculated the propagation delay and doppler shift of the LEO-600 satellite with two different step sizes, one with Dt = 1 ms and the other with 10.24 sec. Figure 2.5-1 to 2.5-4 depict the propagation delay [ms] vs time [min.], the doppler shift [kHz] vs time [min], and those differences between step sizes.

Propagation delay [ms] vs Time[min.]

Plot with blue line: Results with Δt = 1 [msec.]
Plot with blue circle: Results with Δt = 10.24 [sec.]

Figure 2.5-1: Propagation delay of LEO-600 with two different step sizes

Figure 2.5-2: Propagation delay calculation error between two step sizes

Doppler frequency shift [kHz] vs Time[min.], (Centre Frequency = 2 [GHz])

Plot with blue line: Results with Δt = 1 [msec.]
Plot with blue circle: Results with Δt = 10.24 [sec.]

Figure 2.5-3: Doppler frequency shift of LEO-600 with two different step sizes

Figure 2.5-4: Doppler frequency shift calculation error between two step sizes

As can be seen above, we observed that the calculated results which are numerically analysed by 4-th order RK method fits quite well regardless of the step size of time (1 ms vs 10.24 sec.). Errors of the delay between 1 ms step size and 10.24 sec step size is within the range of 1.8 ps and the doppler shift is within +/- 3 mHz.

Observation 6: Calculation results of propagation delay and doppler shift utilizing the 4-th order Runge-Kutta method fit quite well regardless of the step size of time (1 ms vs 10.24 sec.).

Observation 7: Errors of the calculated delay between 1 ms step size and 10.24 sec step size is within a range of 1.8 ps, and the errors of doppler shift is within +/- 3 mHz.

2.6 Comparison between 4-th order Runge-Kutta method and Keplerian

Next, we’d like to confirm that 4-th order Runge-Kutta method and numerical solution of Keplerian do not show significant differences in terms of the calculation of satellite position and velocity.

Figure 2.6-1 and Figure 2.6-2 depict the difference of calculated satellite positions between 4-th order RK method and Keplerian either within 10 minutes of measurement time or for approximately 100 minutes period which is equivalent to the time that LEO-600 satellite circumnavigates the earth (orbital period).

Figure 2.6-1: Calculated satellite position difference between 4-th order RK method and Keplerian within 10 minutes

Figure 2.6-2: Calculated satellite position difference between 4-th order RK method and Keplerian during the orbital period

Observation 8: The calculated satellite position differences between 4-th order RK method and Keplerian are as follows.

Approximately 0.32 mm maximum within 10 minutes

Approximately 3.2 mm maximum during the orbital period

Also Figure 2.6-3 and Figure 2.6-4 depict differences of calculated satellite velocity between 4-th order RK method and Keplerian either in 10 minutes of measurement time or during the orbital period (for approx. 100 minutes).

Figure 2.6-3: Calculated satellite velocity difference between 4-th order RK method and Keplerian within 10 minutes

Figure 2.6-4: Calculated satellite velocity difference between 4-th order RK method and Keplerian during the orbital period

Observation 9: The calculated satellite velocity differences between 4-th order RK method and Keplerian are as follows.

Approximately 0.36 μm/s maximum within 10 minutes

Approximately 3.4 μm/s maximum during the orbital period

When we look at these satellite position and velocity differences, we assume these are negligible since the position and velocity information in the SIB message described in TS 38.331 [6] are rounded by 1.3 m with the position and 60 mm/s with the velocity, which are large enough compared to the current differences between 4-th order RK method and Keplerian.

Observation 10: Calculated satellite position and velocity differences between 4-th order RK method and Keplerian are ne

gligible even during the orbital period of LEO-600 (for approx. 100 minutes).

For supplemental information, comparisons of the doppler frequency shift and propagation delay calculated based on the two methos can also be found in the appendix B.

Observation 11: Difference of calculated doppler frequency shift and propagation delay between 4-th order RK method and Keplerian is less than or equal to 0.000003 Hz and 1.8 ps.

Considering the analyses above, there is no significant difference between the calculation results of 4-th order Runge-Kutta method and Keplerian. Thus, we assume that 4-th order Runge-Kutta method can also be utilized as the calculation method of the doppler frequency shift and propagation delay of the satellite.

Proposal 1: For numerical solution of satellite motion equations in the ECEF (Earth Centred Earth Fixed) coordinate system, allow the use of the 4-th order Runge-Kutta method as well as Keplerian.

- Conclusion

In this contribution we shared our explanation and analyses on a numerical solution of equations of satellite motion in the ECEF (Earth Centred Earth Fixed) coordinate system, utilizing the 4th order Runge-Kutta method.

Observation 1: 4-th order Runge-Kutta method has following benefits while a calculation accuracy is equivalent with Keplerian.

- calculation of doppler frequency shift and propagation delay can be carried out only in the ECEF frame

- computation load is light in time compared to the numerical solution of the Keplerian

Observation 2: The motion equation to be solved under the condition of the ECEF frame and two-body Keplerian can be expressed as following vector equations:

dvdt=-μ  r2 rr +2 v×Ω+Ω×(r×Ω)

drdt=v

Observation 3: The vector function can be expressed by the following equations:

ax xn,yn,zn,vx,n , vy,n ,vz,n =-μ rn2  xnrn +2 vy,n Ω+xnΩ2

ay xn,yn,zn,vx,n , vy,n ,vz,n =-μ rn2  ynrn -2 vx,n Ω+ynΩ2

az xn,yn,zn,vx,n , vy,n ,vz,n =-μ rn2  znrn

Observation 4: Note the position and velocity of satellite are described in ECEF frame, while the position of the UE is described in the ellipsoidal coordinate system in TS 38.508-1.

Observation 5: Initial conditions of satellite position, velocity and UE position in ECEF frame can be as follows.

(X_SV, Y_SV, Z_SV) = ( -3315105.300000 [m], 5396426.100000 [m], 2941944.200000 [m])

(VX, VY, VZ) = (1318.200000 [m/s], -2889.840000 [m/s], 6846.540000 [m/s])

(X_UE, Y_UE, Z_UE) = (-3025296.935779 [m], 4925102.789830 [m], 2687544.210048 [m])

Observation 6: Calculation results of propagation delay and doppler shift utilizing the 4-th order Runge-Kutta method fit quite well regardless of the step size of time (1 ms vs 10.24 sec.).

Observation 7: Errors of the calculated delay between 1 ms step size and 10.24 sec step size is within a range of 1.8 ps, and the errors of doppler shift is within +/- 3 mHz.

Observation 8: The calculated satellite position differences between 4-th order RK method and Keplerian are as follows.

Approximately 0.32 mm maximum within 10 minutes

Approximately 3.2 mm maximum during the orbital period

Observation 9: The calculated satellite velocity differences between 4-th order RK method and Keplerian are as follows.

Approximately 0.36 μm/s maximum within 10 minutes

Approximately 3.4 μm/s maximum during the orbital period

Observation 10: Calculated satellite position and velocity differences between 4-th order RK method and Keplerian are negligible even during the orbital period of LEO-600 (for approx. 100 minutes).

Observation 11: Difference of calculated doppler frequency shift and propagation delay between 4-th order RK method and Keplerian is less than or equal to 0.000003 Hz and 1.8 ps.

Proposal 1: For numerical solution of satellite motion equations in the ECEF (Earth Centred Earth Fixed) coordinate system, allow the use of the 4-th order Runge-Kutta method as well as Keplerian.

# 4.	References

[1] R4-2419786, “Way Forward for [113][322] NTN_testing_NGSO_channel_model”, Moderator (Samsung), RAN4 #113, Orlando, USA

[2] R4-2417672, “Numerical solution of equations of satellite motion under the ECEF coordinate system”, Anritsu, RAN4 #113, Orlando, USA

[3] “Mechanics”, L. D. Landau and E. M. Lifshitz, 2nd edition

[4] “Numerical Recipes in C”, William H. Press, Saul A. Teukolsky, William T. Vetterling, and Brian P. Flannery, 2nd edition

[5] TS 38.508-1, "User Equipment (UE) conformance specification; Part 1: Common test environment", v 18.4.0, 2024-09

[6] TS 38.331, “Radio Resource Control (RRC); Protocol specification”, v18.3.0, 2024-9

Appendix A: Initial position and velocity of the satellite in ECEF frame and ECI frame

For supplemental information, initial position and velocity of the satellite in ECEF frame and ECI frame which are used in the calculations are shown in Table A-1 and Table A-2.

Table A-1: Initial position and velocity of the satellite in ECEF frame

| Position, velocity | ECEF Frame | Unit |
| --- | --- | --- |
| X | -3315105.300000 | [m] |
| Y | 5396426.100000 | [m] |
| Z | 2941944.200000 | [m] |
| VX | 1318.200000 | [m/s] |
| VY | -2889.840000 | [m/s] |
| VZ | 6846.540000 | [m/s] |

Table A-2: Initial position and velocity of the satellite in ECI frame

| Keplerian orbital elements | ECI Frame (double type) | Unit |
| --- | --- | --- |
| A | 7039754.378890120 | [m] |
| e | 0.0086912356837117 | [-] |
| I | 84.15808338321723 | [deg.] |
| Ω | 118.8388944452215 | [deg.] |
| ω | 2.203828720716461 | [deg.] |
| M | 22.46638078165686 | [deg.] |

Appendix B: Supplemental information on the comparison of 4-th order Runge-Kutta method and Keplerian

Comparison of calculated doppler frequency shift and propagation delay based on the 4-th order RK method and Keplerian are depicted in the following figures from figure B-1 to figure B-4.

Doppler frequency shift: Less than or equal to 0.000003 Hz in difference

Figure B-1: Doppler frequency shift based on 4-th order RK method and Keplerian

Figure B-2: Difference of calculated doppler frequency between 4-th order RK method and Keplerian

Propagation delay: Less than 1.8 psec. in difference.

Figure B-3: Propagation delay based on 4-th order RK method and Keplerian

Figure B-4: Difference of calculated propagation delay between 4-th order RK method and Keplerian

Appendix C: Derivation of the motion equation

In this appendix, we show a way to derive the following motion equation in the ECEF frame condition based on the transformation laws of velocity and acceleration from the ECI to ECEF frame.

d2rdt2 =-μ  r2 rr +2 v×ω+ω×r×ω#C1

First, we define coordinate systems and their descriptions.

Coordinate systems and descriptions

Inertial coordinate system (ECI frame): Σ (O,xyz)

Basic vector ex , ey , ez

Position vector r=x ex+ y ey+z ez

Velocity in ECI frame: vΣ

Rotating coordinate system (ECEF Frame): Σ'(O,x'y'z')

Basic vector ex' , ey' , ez'

Position vector r'=x' ex'+ y' ey'+z' ez'

Velocity in ECEF frame: vΣ'

The rotating coordinate system origin O is assumed to coincide with the origin of the inertial coordinate system.

z’ axis of the rotating coordinate system and z axis of the inertial coordinate system is assumed to coincide.

Angular velocity vector: ω

ω=ωx ex+ωy ey+ωz ez=ωx' ex'+ωy' ey'+ωz' ez'

Time rate of change of vector

Here we consider a rotating motion on the inertial coordinate system Σ, where a vector a with constant length is represented by an angular velocity vector ω.

A head of vector a after a minim time will be displaced by

da=ωasinθdt=ω×adt#C2

Figure C-1: Rotating motion of a vector a with constant length on the inertial coordinate system

Thus, the time rate of change of vector a in the inertial coordinate system Σ is as follows.

dadtΣ=ω×a#C3

Transformation law of position vector

As mentioned above, the basic vector of inertial coordinate system Σ is ex , ey , ez, and that of the rotating coordinate system Σ' is ex' , ey' , ez'.

Figure C-2: Relationship of basic vectors on the inertia frame or on the rotating frame

From the previous equation for the time rate of change of vector, the time rate of change of the basic vector on the rotating coordinate system Σ' from the observer’s point of view on the inertial coordinate system Σ can be described as follows.

dex'dtΣ=ω×ex'#C4

dey'dtΣ=ω×ey'#C5

dez'dtΣ=ω×ez'#C6

On the other hand, the time rate of change of the basic vector on the rotating coordinate system Σ' from the observer’s point of view on the rotating coordinate system Σ' can be described as follows.

dex'dtΣ'=0#C7

dey'dtΣ'=0#C8

dez'dtΣ'=0#C9

Next the position vector r from the observer’s point of view on the inertial coordinate system Σ is as follows.

r=x ex+ y ey+z ez#C10

And the position vector r' from the observer’s point of view on the rotating coordinate system Σ' is as follows.

r'=x'ex'+ y'ey'+z'ez'#C11

Since the origin O matches between these coordinate systems, the following equation holds if the observers on each coordinate system are monitoring the same position vector.

r=r'#C12

Transformation law of velocity vector

Velocity vector can be obtained by differentiating the position vector with time. First, since the time rate of change is zero with the basic vector, velocity vΣ' of mass point from the observer’s point of view on the rotating coordinate system Σ' is expressed as follows.

vΣ'≡dr'dtΣ'=dx'dtex'+dy'dtey'+ dz'dtez'#C13

Here we define components of velocity { vx'Σ',vy'Σ',vz'Σ'} on the rotating coordinate system as follows.

vx'Σ'≡dx'dt#C14

vy'Σ'≡dy'dt#C15

vz' Σ'≡dz'dt#C16

Thus the velocity vΣ' can be expressed as follows.

vΣ'=vx'Σ'ex'+vy'Σ'ey'+vz'Σ'ez'#C17

Next, velocity v Σ  of mass point from the observer’s point of view on the inertial coordinate system Σ is expressed as follows.

v Σ≡ drdt Σ#C18

Here based on the relationship of ( r=r'), following equation holds.

v Σ= drdt Σ=dr'dtΣ= ddtx'ex'+ y'ey'+z'ez'Σ

=dx'dtex'+dy'dtey'+dz'dtez'Σ+x'dex'dtΣ+y'dey'dtΣ+z'dez'dtΣΣ              (C19)

The first term is the velocity vΣ'. And the second term can be transformed by the previously mentioned equations (C4, C5 and C6), extracted below for reference.

dex'dtΣ=ω×ex'#C4

dey'dtΣ=ω×ey'#C5

dez'dtΣ=ω×ez'#C6

In summary,

v Σ=vΣ'+ x'(ω×ex')+y'(ω×ey')+z'(ω×ez')Σ

=vΣ'+ω× x'ex'+y'ey'+z'ez'Σ

=vΣ'+ω×r'#C20

Therefore, transformation law of velocity vector is expressed by

v Σ=vΣ'+ω×r'#C21

or

v Σ=vΣ'+ω×r  　 r=r'#C22

Transformation law of acceleration

First, since the time rate of change is zero with the basic vector, acceleration aΣ' of mass point from the observer’s point of view on the rotating coordinate system Σ' is expressed as follows.

aΣ'≡dvΣ'dtΣ'=d2x 'dt2Σ'ex'+d2y 'dt2Σ'ey'+d2z 'dt2Σ'ez'

=ax'Σ'ex'+ay'Σ'ey'+az'Σ'ez'#C23

On the other hand, since the position vector is (r=r'), acceleration a Σ of mass point from the observer’s point of view on the inertial coordinate system Σ is expressed as follows.

a Σ≡dv ΣdtΣ= ddtvΣ'+ω×r'Σ=dvΣ'dtΣ+  ddt ω×r  Σ

= ddtdvx'Σ'dtex'+dvy' Σ'dtey'+dvz'Σ'dtez'Σ+  dωdtΣ×r  Σ+ω×drdtΣ Σ

=d2x 'dt2ex'+d2y 'dt2ey'+d2z 'dt2ez'Σ                                                          (A)

+dx 'dtdex'dtΣ+dy 'dtdey'dtΣ+dz 'dtdez'dtΣΣ                        (B)

+ dωdtΣ×rΣ                                                                                                     (C)

+ω×drdtΣ Σ                                                                                                    (D)

In this equation, we confirm each term from next.

Term (A):

Since the basis is that of rotating coordinate system ex' , ey' , ez', and its components are the second order differential, it is equivalent with the previously mentioned acceleration aΣ'.

aΣ'≡d2x' dt2ex'+d2y' dt2ey'+d2z 'dt2ez'Σ

=ax'Σ'ex'+ay'Σ'ey'+az'Σ'ez'#C24

Term (B):

Reusing the previous equations (C4, C5 and C6) below, the term B can be transformed into the following equation.

dex'dtΣ= ω×ex '

dey'dtΣ=ω×ey'

dez'dtΣ=ω×ez'

B=dx 'dtdex'dtΣ+dy 'dtdey'dtΣ+dz 'dtdez'dtΣΣ

=vx'Σ'dex'dtΣ+vy'Σ'dey'dtΣ+vz' Σ'dez'dtΣΣ

=vx'Σ'(ω×ex ')+vy'Σ'(ω×ey')+vz'Σ'(ω×ez')Σ

=ω×vx'Σ'ex'+vy'Σ'ey'+vz'Σ'ez'Σ

=ω×vΣ'#C25

Term (C):

Since we are assuming the case where the angular velocity vector ω does not vary in time, there is no contribution by this term.

dωdtΣ=0#C26

Term (D):

Reusing the previously described equations below,

drdt Σ=v Σ

v Σ=vΣ'+ω×r'

the term (D) can be expressed as follows.

ω×drdtΣ=ω×vΣ'+ω×r'=ω×vΣ'+ω×ω×r'#C27

Each term can be summarized as follows.

Table C-1: Summary of acceleration vector a Σ

|  | Breakdown of acceleration a Σ of mass point from the observer’s point of view on the inertial coordinate system Σ | Meanings of terms |
| --- | --- | --- |
| Term (A) | aΣ' | Acceleration |
| Term (B) | ω×vΣ' | Coriolis force |
| Term (C) | 0 | － |
| Term (D) | ω×vΣ'+ω×(ω×r') | Coriolis force + centrifugal force |

Therefore, the following equation holds as the transformation law of acceleration.

a Σ=aΣ'+2 ω×vΣ'+ω×ω×r'#C28

Transcription of Newton motion equation based on the transformation laws of velocity and acceleration from inertial coordinate system to rotating coordinate system

The motion equation on the inertial coordinate system is expressed as follows.

ma Σ=F#C29

Dividing both sides by the mass m, we obtain the equation below.

a Σ=Fm#C30

Referring to the previous equation C28 above, the following equation on the rotating coordinate system is obtained.

aΣ'+2 ω×vΣ' +ω× ω×r' =Fm#C31

Gravity term F/m with two-body Keplerian model can be expressed as follows.

Fm=-μ  r2 rr #C32

Thus, equation C31 can be converted as follows.

aΣ'=Fm-2ω×vΣ'-ω×ω×r'#C33

By changing the order of outer product, the equation C33 is expressed as follows.

aΣ'=Fm+2vΣ'×ω+ω×r'×ω#C34

Again, applying the relationship (r=r') to the equation C32, we obtain the equation below.

Fm=-μ  r2 rr =-μ  r'2r' r'#C35

Therefore, the motion equation on the rotating coordinate system is expressed as follows.

aΣ'=-μ  r'2 r'r'+2vΣ'×ω+ω×r'×ω#C36

Though we added the apostrophe (’) in the equation until now to represent the rotating coordinate system, since we can now describe position vector, velocity vector and gravity on the rotating coordinate system, we can omit the apostrophe and obtain the following equation.

d2rdt2 =a=-μ r2 r r+2 v×ω+ω×r×ω#C37

Table C-2: Summary of transformation law

|  | ECI Frame: Σ (O,xyz) | ECEF Frame: Σ'(O,x'y'z') | Transformation law |
| --- | --- | --- | --- |
| Bases | ex , ey , ez | ex' , ey' , ez' | － |
| Position | r | r' | r=r' |
| Velocity | vΣ | vΣ' | v Σ=vΣ'+ω×r' |
| Acceleration | aΣ | aΣ' | a Σ =aΣ'+2 ω×vΣ' +ω×(ω×r') |

d2rdt2 =-μ r2 r r +2 v×ω+ω×(r×ω)