3GPP TSG-RAN WG4 Meeting #114						R4-2500203

Athens, Greece, Feb 17 – 21, 2025

Source:	Nokia

Title:	Conformance testing of Expected EIRP mask requirement

Agenda item:       7.13.4.1 U6GHz BS EEIRP mask [NR_BS_RF_req_evo-Perf]

Document for:	Approval

- Introduction

TSG RAN#103 meeting approved new Rel-19 work item “NR base station (BS) RF requirement evolution for FR1/FR2 and testing” [1], where first objective is related to expected EIRP mask for upper 6 GHz.

WRC-23 has identified the frequency range 6425 – 7125 MHz which is 3GPP band n104 for IMT as in the RR Footnotes 5.457D, 5.457E and 5.457F. The IMT identification comes with some technical conditions which includes the limits on the expected equivalent isotropic radiated power (e.i.r.p.) spectral density of IMT base stations for protecting Earth-to-space fixed satellite services (FSS) as contained in ITU-R Resolution 220 (WRC-23) [2].

During RAN#106 meeting work was extended in [4].

Performance part of expected EIRP mask objective of work item [4] is as follow:

Expected EIRP mask for upper 6GHz - Specify manufacturer declarations, conformance test descriptions, test tolerances and test requirements. - Note: target completion date of this objective is March 2025.

RAN4#113 meeting discussions were concluded in agreed WF [3] and in this contribution we continue discussions for conformance testing for expected EIPR mask requirement for the upper 6 GHz.

- Discussion

In this section we provide our views and proposals for open issues and details related to the testing of OTA spatial emission requirements.

  - Test beams for EEIRP mask

Following was agreed in Way forward [3] on test beams for EEIRP mask:

Agreement: - To minimize the EEIRP difference between candidate testing beams and reference testing beam set; - [To ensure the confidence interval tight within candidate testing beams]; - For the weighting factor for test beams: - Equally weighting factor To split the CAR into the several square and the number of square starts with 20; FFS how to generate the beam within each square; - Alternative 1: to generate the beams randomly within each square; - Alternative 2: to generate the beam per squared as much as close to the center of square;

The EEIRP calculation involves calculating the average EIRP over a defined set of elevation angles (around the base station, i.e. 360° azimuth) for the full range of beamforming directions. In this contribution two Reference Sets (RefSet1 and RefSet2) of the full range of beamforming directions is shown in Figure 2.1-1 and Figure 2.1-2, with the vertical steering range set to -16° ≤ θetilt ≤ 0° and -12° ≤ θetilt ≤ 0° respectively. The number of beamforming directions for these examples are 2057 and 1573 respectively, for a 1° steering resolution (i.e. 17 x 121 and 13 x 121). These figures are from the viewpoint of the UE into the BS antenna panel.

Figure 2.1-1: Reference Set 1 (RefSet1) of beamforming directions

Figure 2.1-2: Reference Set 2 (RefSet2) of beamforming directions

Theoretically, this full range of beamforming directions are meant to be used for the EEIRP calculation, however, a representative subset of these beamforming directions can be considered as long as the overall averaging of the EEIRP is sufficiently accurate (as described in the Annex to the ITU Resolution 220). The motivation to create a small subset of beamforming directions (or Test Beams) is to reduce the test effort needed to calculate the EEIRP.

    - 2.1.1 Test Beams Sets

It is undoubtably that using the full range of beamforming directions will amount to extensive test effort as the e.i.r.p. for the entire upper hemisphere (around the base station) needs to be measured for every beamforming direction. Simplistically, halving the number of beamforming directions could potentially half the amount of effort needed to comply with the EEIRP requirement. Therefore, it is important to find a representative subset of these beamforming directions (Test Beams set) for the EEIRP calculation.

After some discussions, the following Test Beams Sets (TBSs) shown in Figure 2.1.1-1 and Figure 2.1.1-2 are evaluated against RefSet1 and RefSet2 respectively.

Figure 2.1.1-1: Test Beams Sets for RefSet1

Figure 2.1.1-2: Test Beams Sets for RefSet2

Each of the Test Beams in their respective set have equal weight.

    - 2.1.2 Antenna configurations for evaluation

These TBSs are evaluated over six different antenna configurations to show how suitable these Test Beams Sets are. The antenna parameters for these configurations are shown in Table 2.1.2-1 where Config 1 and 2 are non-sub-array antennas, whilst Config 3 to 6 contains sub-arrays.

| Parameter |  | Antenna |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Config 1 | Config 2 | Config 3 | Config 4 | Config 5 | Config 6 |
| Element | Vertical HPBW (°) | 90 | 65 | 65 | 65 | 65 | 65 |
|  | Horizontal HPBW (°) | 90 | 90 | 90 | 90 | 90 | 90 |
|  | Element gain (dBi) | 5.5 | 5.5 | 6.4 | 6.6 | 6.6 | 6.8 |
|  | Front-to-back ratio (H/V) (dB) | 30 | 30 | 30 | 30 | 30 | 30 |
| Array configuration | Number of rows | 16 | 16 | 8 | 8 | 8 | 16 |
|  | Number of columns | 8 | 8 | 16 | 8 | 16 | 16 |
|  | Sub-array | - | - | 3x1 | 2x1 | 3x1 | 2x1 |
|  | Sub-array pre-tilt: (°) | - | - | 6 | 0 | 0 | 0 |
|  | Vertical spacing (λ) | 0.5 | 0.7 | 2.1 | 1.4 | 2.1 | 1.4 |
|  | Horizontal spacing (λ) | 0.5 | 0.5 | 0.5 | 0.5 | 0.5 | 0.5 |

Table 2.1.2-1: Antenna parameters

These parameters are just some example antenna configurations for the purpose of evaluating the suitability of the Test Beams sets rather than for evaluating against the specific EEIRP values required for the different elevation ranges. In addition, the TRP for each of the configuration is set to 20 dBm/MHz and no specific antenna techniques such as sidelobe suppression are used where these techniques could potentially lower the EIRP level at various vertical and horizontal angles.

2.1.3 Evaluation of Test Beams Sets

The Test Beams sets in clause 2.1.1 are used to evaluate against the antenna configurations in clause 2.1.2. The Reference Sets illustrated in Figure 2.1-1 are based on 1° steering resolution, and the calculation of the EEIRP above the horizon (upper hemisphere) is based on 0.5° sampling resolution in order to capture the beam as accurately as possible.

The antenna pattern generated from these configurations were normalised, and just to note that the normalisation factor is only up to around 1.5 dB. The results shown in Table 2.1.3-1 to Table 2.1.3-10 is in the form of absolute difference (delta) in linear comparing the EEIRP generated from those Test Beams set to the Reference Set. This delta calculation is as shown below:

Deltabin1,…,7_linear=10EEIRPTBS_dBm -EEIRPref_dBm10    ,

where Deltabin1,…,7_linear is the linear multiple/fractional delta for elevation bin 1 to 7 (bin1 to bin7), and EEIRPTBS_dBm -EEIRPref_dBm is the absolute value of the difference between the EEIRP generated from the Test Beams set to the particular Reference Set in logarithmic scale. Just to note that the closer this delta to linear value of ‘1’, the better it represents the Reference Set.

The results benchmarking against Reference Set 1, RefSet1, is shown in Table 2.1.3-1 to Table 2.1.3-5.

| Elevation Bin | Elevation angular range | Delta from Reference Set 1 (linear) |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Test Beams Set 1: 12 Beams (3 x 4) |  |  |  |  |  |
|  |  | Config 1 | Config 2 | Config 3 | Config 4 | Config 5 | Config 6 |
| 1 | 0<q<5 | 1.40 | 2.12 | 3.22 | 2.03 | 2.96 | 2.87 |
| 2 | 5<q<10 | 1.00 | 1.13 | 1.12 | 1.11 | 1.13 | 1.05 |
| 3 | 10<q<15 | 1.22 | 1.07 | 1.15 | 1.08 | 1.03 | 1.01 |
| 4 | 15<q<20 | 1.04 | 1.05 | 1.17 | 1.17 | 1.10 | 1.04 |
| 5 | 20<q<30 | 1.02 | 1.03 | 1.17 | 1.10 | 1.03 | 1.03 |
| 6 | 30<q<60 | 1.04 | 1.06 | 1.08 | 1.07 | 1.01 | 1.19 |
| 7 | 60<q<90 | 1.02 | 1.15 | 1.03 | 1.11 | 1.13 | 1.03 |

Table 2.1.3-1: Difference of Test Beams Set 1 from RefSet1

| Elevation Bin | Elevation angular range | Delta from Reference Set 1 (linear) |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Test Beams Set 2: 12 Beams (4 x 3) |  |  |  |  |  |
|  |  | Config 1 | Config 2 | Config 3 | Config 4 | Config 5 | Config 6 |
| 1 | 0<q<5 | 1.27 | 1.52 | 2.51 | 1.49 | 2.38 | 3.25 |
| 2 | 5<q<10 | 1.04 | 1.01 | 1.14 | 1.00 | 1.15 | 1.11 |
| 3 | 10<q<15 | 1.11 | 1.00 | 1.14 | 1.01 | 1.08 | 1.06 |
| 4 | 15<q<20 | 1.00 | 1.00 | 1.02 | 1.06 | 1.00 | 1.08 |
| 5 | 20<q<30 | 1.01 | 1.00 | 1.15 | 1.12 | 1.07 | 1.08 |
| 6 | 30<q<60 | 1.05 | 1.03 | 1.00 | 1.09 | 1.08 | 1.02 |
| 7 | 60<q<90 | 1.08 | 1.05 | 1.01 | 1.08 | 1.24 | 1.13 |

Table 2.1.3-2: Difference of Test Beams Set 2 from RefSet1

| Elevation Bin | Elevation angular range | Delta from Reference Set 1 (linear) |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Test Beams Set 3: 16 Beams (4 x 4) |  |  |  |  |  |
|  |  | Config 1 | Config 2 | Config 3 | Config 4 | Config 5 | Config 6 |
| 1 | 0<q<5 | 1.27 | 1.52 | 2.51 | 1.49 | 2.38 | 3.25 |
| 2 | 5<q<10 | 1.04 | 1.01 | 1.14 | 1.01 | 1.15 | 1.11 |
| 3 | 10<q<15 | 1.11 | 1.01 | 1.14 | 1.01 | 1.07 | 1.06 |
| 4 | 15<q<20 | 1.00 | 1.01 | 1.02 | 1.06 | 1.00 | 1.08 |
| 5 | 20<q<30 | 1.00 | 1.01 | 1.14 | 1.11 | 1.07 | 1.07 |
| 6 | 30<q<60 | 1.03 | 1.01 | 1.05 | 1.09 | 1.01 | 1.02 |
| 7 | 60<q<90 | 1.01 | 1.00 | 1.03 | 1.00 | 1.02 | 1.03 |

Table 2.1.3-3: Difference of Test Beams Set 3 from RefSet1

| Elevation Bin | Elevation angular range | Delta from Reference Set 1 (linear) |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Test Beams Set 4: 20 Beams (4 x 5) |  |  |  |  |  |
|  |  | Config 1 | Config 2 | Config 3 | Config 4 | Config 5 | Config 6 |
| 1 | 0<q<5 | 1.27 | 1.52 | 2.51 | 1.49 | 2.38 | 3.25 |
| 2 | 5<q<10 | 1.04 | 1.01 | 1.14 | 1.01 | 1.14 | 1.11 |
| 3 | 10<q<15 | 1.11 | 1.01 | 1.13 | 1.00 | 1.07 | 1.06 |
| 4 | 15<q<20 | 1.01 | 1.01 | 1.02 | 1.05 | 1.01 | 1.08 |
| 5 | 20<q<30 | 1.00 | 1.02 | 1.15 | 1.11 | 1.07 | 1.08 |
| 6 | 30<q<60 | 1.02 | 1.00 | 1.11 | 1.08 | 1.02 | 1.02 |
| 7 | 60<q<90 | 1.03 | 1.00 | 1.12 | 1.00 | 1.02 | 1.03 |

Table 2.1.3-4: Difference of Test Beams Set 4 from RefSet1

| Elevation Bin | Elevation angular range | Delta from Reference Set 1 (linear) |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Test Beams Set 5: 20 Beams (5 x 4) |  |  |  |  |  |
|  |  | Config 1 | Config 2 | Config 3 | Config 4 | Config 5 | Config 6 |
| 1 | 0<q<5 | 1.22 | 1.39 | 1.66 | 1.36 | 1.60 | 2.84 |
| 2 | 5<q<10 | 1.04 | 1.02 | 1.53 | 1.02 | 1.55 | 1.09 |
| 3 | 10<q<15 | 1.08 | 1.01 | 1.11 | 1.01 | 1.07 | 1.05 |
| 4 | 15<q<20 | 1.01 | 1.00 | 1.05 | 1.05 | 1.05 | 1.08 |
| 5 | 20<q<30 | 1.00 | 1.01 | 1.09 | 1.10 | 1.05 | 1.15 |
| 6 | 30<q<60 | 1.02 | 1.02 | 1.06 | 1.09 | 1.01 | 1.12 |
| 7 | 60<q<90 | 1.01 | 1.00 | 1.04 | 1.01 | 1.10 | 1.03 |

Table 2.1.3-5: Difference of Test Beams Set 5 from RefSet1

The results benchmarking against Reference Set 2, RefSet2, is shown in Table 2.1.3-6 to Table 2.1.3-10.

| Elevation Bin | Elevation angular range | Delta from Reference Set 2 (linear) |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Test Beams Set 1: 12 Beams (3 x 4) |  |  |  |  |  |
|  |  | Config 1 | Config 2 | Config 3 | Config 4 | Config 5 | Config 6 |
| 1 | 0<q<5 | 1.25 | 1.51 | 2.74 | 1.49 | 2.61 | 3.40 |
| 2 | 5<q<10 | 1.02 | 1.01 | 1.09 | 1.01 | 1.10 | 1.12 |
| 3 | 10<q<15 | 1.16 | 1.02 | 2.13 | 1.02 | 2.34 | 1.05 |
| 4 | 15<q<20 | 1.01 | 1.03 | 1.02 | 1.02 | 1.02 | 1.06 |
| 5 | 20<q<30 | 1.00 | 1.04 | 1.12 | 1.40 | 1.10 | 1.90 |
| 6 | 30<q<60 | 1.04 | 1.00 | 1.25 | 1.08 | 1.14 | 1.04 |
| 7 | 60<q<90 | 1.03 | 1.05 | 1.05 | 1.03 | 1.00 | 1.01 |

Table 2.1.3-6: Difference of Test Beams Set 1 from RefSet2

| Elevation Bin | Elevation angular range | Delta from Reference Set 2 (linear) |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Test Beams Set 2: 12 Beams (4 x 3) |  |  |  |  |  |
|  |  | Config 1 | Config 2 | Config 3 | Config 4 | Config 5 | Config 6 |
| 1 | 0<q<5 | 1.20 | 1.35 | 1.64 | 1.33 | 1.59 | 2.76 |
| 2 | 5<q<10 | 1.03 | 1.01 | 1.25 | 1.01 | 1.23 | 1.09 |
| 3 | 10<q<15 | 1.11 | 1.00 | 1.54 | 1.00 | 1.66 | 1.04 |
| 4 | 15<q<20 | 1.01 | 1.00 | 1.02 | 1.01 | 1.01 | 1.04 |
| 5 | 20<q<30 | 1.01 | 1.00 | 1.07 | 1.29 | 1.07 | 1.89 |
| 6 | 30<q<60 | 1.05 | 1.03 | 1.14 | 1.06 | 1.03 | 1.06 |
| 7 | 60<q<90 | 1.09 | 1.02 | 1.01 | 1.05 | 1.09 | 1.11 |

Table 2.1.3-7: Difference of Test Beams Set 2 from RefSet2

| Elevation Bin | Elevation angular range | Delta from Reference Set 2 (linear) |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Test Beams Set 3: 16 Beams (4 x 4) |  |  |  |  |  |
|  |  | Config 1 | Config 2 | Config 3 | Config 4 | Config 5 | Config 6 |
| 1 | 0<q<5 | 1.20 | 1.35 | 1.64 | 1.34 | 1.59 | 2.76 |
| 2 | 5<q<10 | 1.03 | 1.01 | 1.25 | 1.01 | 1.23 | 1.10 |
| 3 | 10<q<15 | 1.10 | 1.00 | 1.54 | 1.00 | 1.66 | 1.04 |
| 4 | 15<q<20 | 1.00 | 1.01 | 1.02 | 1.00 | 1.01 | 1.04 |
| 5 | 20<q<30 | 1.00 | 1.02 | 1.07 | 1.27 | 1.07 | 1.89 |
| 6 | 30<q<60 | 1.03 | 1.02 | 1.26 | 1.06 | 1.13 | 1.06 |
| 7 | 60<q<90 | 1.02 | 1.01 | 1.05 | 1.01 | 1.12 | 1.02 |

Table 2.1.3-8: Difference of Test Beams Set 3 from RefSet2

| Elevation Bin | Elevation angular range | Delta from Reference Set 2 (linear) |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Test Beams Set 4: 20 Beams (4 x 5) |  |  |  |  |  |
|  |  | Config 1 | Config 2 | Config 3 | Config 4 | Config 5 | Config 6 |
| 1 | 0<q<5 | 1.20 | 1.35 | 1.64 | 1.34 | 1.59 | 2.76 |
| 2 | 5<q<10 | 1.03 | 1.01 | 1.25 | 1.01 | 1.23 | 1.10 |
| 3 | 10<q<15 | 1.10 | 1.00 | 1.54 | 1.00 | 1.65 | 1.04 |
| 4 | 15<q<20 | 1.00 | 1.01 | 1.02 | 1.00 | 1.01 | 1.04 |
| 5 | 20<q<30 | 1.01 | 1.02 | 1.07 | 1.27 | 1.07 | 1.90 |
| 6 | 30<q<60 | 1.02 | 1.01 | 1.27 | 1.05 | 1.10 | 1.06 |
| 7 | 60<q<90 | 1.06 | 1.02 | 1.11 | 1.01 | 1.18 | 1.04 |

Table 2.1.3-9: Difference of Test Beams Set 4 from RefSet2

| Elevation Bin | Elevation angular range | Delta from Reference Set 2 (linear) |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Test Beams Set 5: 20 Beams (5 x 4) |  |  |  |  |  |
|  |  | Config 1 | Config 2 | Config 3 | Config 4 | Config 5 | Config 6 |
| 1 | 0<q<5 | 1.17 | 1.29 | 1.49 | 1.28 | 1.46 | 2.00 |
| 2 | 5<q<10 | 1.02 | 1.02 | 1.14 | 1.02 | 1.13 | 1.01 |
| 3 | 10<q<15 | 1.08 | 1.01 | 1.43 | 1.01 | 1.50 | 1.04 |
| 4 | 15<q<20 | 1.00 | 1.00 | 1.04 | 1.01 | 1.00 | 1.07 |
| 5 | 20<q<30 | 1.01 | 1.01 | 1.07 | 1.23 | 1.07 | 1.70 |
| 6 | 30<q<60 | 1.03 | 1.02 | 1.21 | 1.05 | 1.12 | 1.05 |
| 7 | 60<q<90 | 1.02 | 1.00 | 1.05 | 1.01 | 1.12 | 1.08 |

Table 2.1.3-10: Difference of Test Beams Set 5 from RefSet2

A simple root-mean-square deviation (RMSD) metric can be used as a ‘figure of merit’ to rank the suitability of these Test Beams Sets, and the metric is shown below:

RMSDTBS_dB=10 ×log102RMSDallbins_linear_A12+⋯+RMSDallbins_linear_A626  ,

where the RMSD calculated for each antenna configuration, Ax, (over all elevation bins) is derived as

RMSDallbins_linear_Ax=2Deltabin1_linear_Ax2+⋯+Deltabin7_linear_Ax27

and, Deltabin1_linear_Ax , … , Deltabin7_linear_Ax indicating the linear delta for elevation bin 1 to 7 for the antenna configuration Ax.

| No ` | Test Beams Set | RMSD (dB) |  |
| --- | --- | --- | --- |
|  |  | RefSet1 | RefSet2 |
| 1 | 12 Beams (3 x 4) | 1.41 | 1.49 |
| 2 | 12 Beams (4 x 3) | 1.10 | 0.91 |
| 3 | 16 Beams (4 x 4) | 1.06 | 0.92 |
| 4 | 20 Beams (4 x 5) | 1.07 | 0.93 |
| 5 | 20 Beams (5 x 4) | 0.80 | 0.66 |

Table 2.1.3-11: Summary of evaluation of TBSs for both Reference Sets

Just to note again that the closer this RMSD to logaritmic (dB) value of ‘0’, the better it represents the Reference Set.

Observation 1: A metric (or a set of metrics) is needed to select which Test Beams Set is more representative of the Reference Test Beams.

Proposal 1: The root-mean-square-deviation (RMSD) metric defined below is to be used as the metric (or one of the metrics) to select which Test Beams Set is more suitable.

RMSDTBS_dB=10 ×log102RMSDallbins_linear_A12+⋯+RMSDallbins_linear_A626  ,

where the RMSD calculated for each antenna configuration, Ax, (over all elevation bins) is derived as

RMSDallbins_linear_Ax=2Deltabin1_linear_Ax2+⋯+Deltabin7_linear_Ax27

and, Deltabin1_linear_Ax , … , Deltabin7_linear_Ax indicating the linear delta for elevation bin 1 to 7 for the antenna configuration Ax.

Observation 2: For the same total number of Test Beams, higher number of vertical Beams will lead to a smaller RMSD.

Observation 3: Different Test Beams Set (which is a subset of the Reference Set) will result in different accuracies to the Reference Set (for different elevation bins). However, based on the different antenna configurations, more Test Beams does not necessarily lead to better accuracy.

Proposal 2: RAN4 to consider a suitable Test Beams Set pattern to fulfil the requirement.

Observation 4: The root mean square deviation (RMSD) can be treated as a ‘figure of merit’ and since all the considered Test Beams pattern are very similar, it would be beneficial to choose the one with the lower number of Test Beams and good performance.

Proposal 3: Test Beams Set 2: 12 Beams (4 x 3) should be chosen for the purpose of this work.

- Conclusion

In this contribution we continue discussions for conformance testing for expected EIPR mask requirement for the upper 6 GHz.

We have made following observations and proposals:

Observation 1: A metric (or a set of metrics) is needed to select which Test Beams Set is more representative of the Reference Test Beams.

Proposal 1: The root-mean-square-deviation (RMSD) metric defined below is to be used as the metric (or one of the metrics) to select which Test Beams Set is more suitable.

RMSDTBS_dB=10 ×log102RMSDallbins_linear_A12+⋯+RMSDallbins_linear_A626  ,

where the RMSD calculated for each antenna configuration, Ax, (over all elevation bins) is derived as

RMSDallbins_linear_Ax=2Deltabin1_linear_Ax2+⋯+Deltabin7_linear_Ax27

and, Deltabin1_linear_Ax , … , Deltabin7_linear_Ax indicating the linear delta for elevation bin 1 to 7 for the antenna configuration Ax.

Observation 2: For the same total number of Test Beams, higher number of vertical Beams will lead to a smaller RMSD.

Observation 3: Different Test Beams Set (which is a subset of the Reference Set) will result in different accuracies to the Reference Set (for different elevation bins). However, based on the different antenna configurations, more Test Beams does not necessarily lead to better accuracy.

Proposal 2: RAN4 to consider a suitable Test Beams Set pattern to fulfil the requirement.

Observation 4: The root mean square deviation (RMSD) can be treated as a ‘figure of merit’ and since all the considered Test Beams pattern are very similar, it would be beneficial to choose the one with the lower number of Test Beams and good performance.

Proposal 3: Test Beams Set 2: 12 Beams (4 x 3) should be chosen for the purpose of this work.

- 4. References

- RP-241284, Revised WID: NR base station (BS) RF requirement evolution for FR1/FR2 and testing, ZTE Corporation

- ITU WRC-23, Final Acts, Resolution 220

- R4-2419827,  Way Forward for [113][302] NR_BS_RF_Part1_E_EIRP, ZTE Corporation

- RP-243270, Revised WID: NR base station (BS) RF requirement evolution for FR1/FR2 and testing, ZTE Corporation, HiSilicon, Huawei, Ericsson, Nokia, NEC