3GPP TSG-RAN WG4 Meeting #114												R4-2500209

Athens, Greece, 17 – 21 February, 2025

Agenda item:			7.24.3

Source:	Huawei, HiSilicon

Title:	on OTA tests for ambient IoT devices

Document for:	Approval

# Introduction

RAN#106 approves a new work item RP-243326 [1] on solutions for ambient IoT. The RAN4 scope (copied below) contains development of OTA test methodology for device 1. This contribution addresses OTA test related issues.

- RAN4 scope:

  - Specify RF requirements for Ambient-IoT BS, device 1, and CW

    - RF requirements for Type 1-C Ambient-IoT BS

    - RF requirements for device 1

    - RF requirements for CW

  - Specify RRM core requirements for device 1, if necessary

  - Study and develop OTA test methodology for A-IoT device 1

    - Consider test methods specified in TR 38.870 as starting point. Take test system reuse, test system complexity and test time into account, when developing test methods suitable for Ambient IoT.

    - Develop the preliminary Measurement Uncertainty (MU) assessment for the test system

- Use band n8 as an example band

# Discussion

As it is not feasible to perform conducted tests for ambient IoT devices, it would be useful to develop a methodology that can be used for all device 1 related tests, viz. TRP and TRS like parameters in [2], RF and RRM and Demodulation requirements if any. Considering that there are only four meetings for core part discussion, it is important to clarify the works scope and expected outcome of OTA test methodology discussion. Different from legacy OTA WI discussion, e.g. TRP/TRS which defines the methodology and also carries out the lab measurement campaign, there is currently no available Device/TE since it is the first release in 3GPP and the corresponding design is under discussion in parallel. Thus, it is reasonable to clarify the scope of the OTA discussion as follows according to the WID, while taking TR 38.870 as baseline.

Test methodology for Ambient IoT, including

- Chamber setup

- Position of Device

- Test procedures

- Minimum range/ QZ (Quiet Zone) size/QZ calibration

- Preliminary MU assessment

Note: The performance metrics to be discussed in RF (RRM/Demodulation if any) requirements. And the test methodology takes sensitivity and backscatter power as example for discussion to develop unified test methodology.

Proposal 1: RAN4 to agree on scope of OTA test methodology discussion as:

Test methodology for Ambient IoT including

- Chamber setup

- Position of Device

- Test procedures

- Minimum range/ QZ (Quiet Zone) size/QZ calibration

- Preliminary MU assessment

Note: The performance metrics to be discussed in RF (RRM/Demodulation if any) requirements. And the test methodology takes sensitivity and backscatter power as example for discussion to develop unified test methodology.

TS 38.870 [2] includes two test environments, namely anechoic chamber and reverberation chamber. Given the four-meeting duration of this work item, it is difficult, if not impossible, to accomplish the work for both test environments. Anechoic chamber in [2] is used as a baseline setup and is widely adopted. It is therefore prudent to select anechoic chamber for ambient IoT device tests.

Proposal 2: use anechoic chamber for ambient IoT device tests.

FR1 TRP and TRS in [2] are defined over a whole spherical surface with measurement grid points at given spatial separations. However, ambient IoT devices rely on backscattering for its signals to reach the reader. In other words, such backscattering occupies hemi sphere in the direction of the reader as shown in Figure 1, where device is place at origin in XY plane facing +Z direction and reader is above XY plane in +Z direction. It would save measurement time   if the measurement grid points for transmit and receive metrics are in the same hemi sphere, namely Z>0.

Figure 1   Reference coordinate system

The following options for measurement grid points should be down selected to one. Related details such as measurement point spacing can be discussed in the subsequent meetings, which also is subject to RF core discussion about the requirements definition.

Proposal 3: select one option for measurement points for transmit and receive metrics from the following:

Option 1: Measurement grid points over hemi sphere in the direction of reader

Option 2: discrete points in the direction of reader

According to TS 38.870 [2], FR1 TRP and TRS requirements are determined from the results of measurement campaign, which requires the following:

- More than three volunteer labs, which have to be aligned in terms of its measurement variations for the same device.

- A number of commercially available devices from different manufacturers over various price ranges in the last two years.

- The results from measurement campaign are presented as CDF and a percentile on the CDF is agreed to be the TRP and TRS requirements.

However, ambient IoT devices currently available, if any, may not be sufficient in number to start a measurement campaign. In addition, availability of commercial test equipment to carry out such measurement campaign has yet to be confirmed. Finally, even if both devices and test equipment present no issue, it is almost impossible to complete the measurement campaign in four meetings.

Proposal 4: measurement campaign is not needed in this work item. Refinement in future work items based on measurement campaign is not precluded.

For TRP/TRS requirements, both free space and phantom are considered. For ambient IoT device, it can also be attached to different items.  It is difficult to account for the above scenarios in the work item albeit desirable because (a) the need to specify phantom for the device to attach, (b) it is difficult to find a single material to represent various things that ambient IoT devices could attach to. A simple solution for this work item is to test devices in free space, which is sufficient to reflect the corresponding performance. Typically foam with dielectric constant close to one is used as mounting support to emulate free space.

Proposal 5: test configuration is for a single device under free space condition.

If anechoic chamber is used for ambient IoT device tests, some of the MU terms should have similar meanings as in TR38.870. It would be convenient to reuse the MU values from TR38.870.

Proposal 6: use MU values from TR38.870 for terms with similar functionality

Similarly, the definition of quiet zone and range size could also be reused. However, the quiet zone size and the minimum range may require revision due to the smaller sizes of A-IoT tags compared to smartphones. As currently there is no commercial tags for A-IoT, RFID at UHF frequency band could be used as a reference point. Commercial RFID for various applications and ranges given in [3], [4] and [5] have maximum dimension of 12cm. It would be prudent to assume a quiet zone with smaller size to fully encompass the A-IoT tag within the quiet zone for A-IoT tag.

Observation 1: a quiet zone with smaller size (e.g. 15cm) for A-IoT tests seems to be adequate.

The quiet zone calibration procedures in TR38.870 would still apply to smaller quiet zones.

Proposal 7: reuse the quiet zone calibration procedure from TR38.870 for A-IoT tests

Taking the smaller quiet zone size of 15 cm as an example, the minimum range defined for smartphones in TR38.870 should be modified accordingly as shown in Table 1 below.

Table 1: Minimum Range Length for NR FR1 TRP-TRS OTA systems with 15cm quiet zone size

| F [GHz] | Drad [m] | RQZ+2Drad²/λ | 3D = 6RQZ | RQZ+2λ | max(RQZ+2λ,3D,RQZ+2D²/λ) |
| --- | --- | --- | --- | --- | --- |
| 0.41 | 0.15 | 0.14 | 0.45 | 1.54 | 1.54 |
| 0.60 | 0.15 | 0.17 | 0.45 | 1.08 | 1.08 |
| 0.70 | 0.15 | 0.18 | 0.45 | 0.93 | 0.93 |
| 0.80 | 0.15 | 0.20 | 0.45 | 0.83 | 0.83 |
| 1.00 | 0.15 | 0.23 | 0.45 | 0.68 | 0.68 |
| 1.20 | 0.15 | 0.24 | 0.45 | 0.58 | 0.58 |
| 1.40 | 0.14 | 0.26 | 0.45 | 0.50 | 0.50 |
| 1.60 | 0.14 | 0.28 | 0.45 | 0.45 | 0.45 |
| 1.80 | 0.14 | 0.29 | 0.45 | 0.41 | 0.45 |
| 2.00 | 0.13 | 0.30 | 0.45 | 0.38 | 0.45 |
| 2.20 | 0.13 | 0.30 | 0.45 | 0.35 | 0.45 |
| 2.40 | 0.12 | 0.31 | 0.45 | 0.33 | 0.45 |
| 2.60 | 0.12 | 0.30 | 0.45 | 0.31 | 0.45 |
| 2.80 | 0.12 | 0.32 | 0.45 | 0.29 | 0.45 |
| 3.00 | 0.11 | 0.32 | 0.45 | 0.28 | 0.45 |
| 4.00 | 0.09 | 0.29 | 0.45 | 0.23 | 0.45 |
| 5.00 | 0.07 | 0.24 | 0.45 | 0.20 | 0.45 |
| 6.00 | 0.05 | 0.18 | 0.45 | 0.18 | 0.45 |
| 7.00 | 0.03 | 0.12 | 0.45 | 0.16 | 0.45 |
| 7.13 | 0.03 | 0.10 | 0.45 | 0.16 | 0.45 |

As mentioned above, a smaller QZ size maybe applicable for Ambient IoT device, which may allow smaller range length of test system as shown in Table 1. Considering that low cost/complexity is an important characteristic for ambient IoT device, shorter range length may also serve the purposes.

Proposal 8: reuse the same principle to derive the minimum range length as defined in TR 38.870, and further discuss whether to consider a shorter range length with smaller QZ size.

Environmental requirement for A-IoT tests should be comparable TR38.870.

Proposal 9: reuse the environmental requirements from TR38.870 for A-IoT tests.

Another aspect is the energy supply during the test which is also mentioned in the study phase. Different from traditional UEs (e.g. handheld UE), Ambient IoT are low cost products and may be customized for different industry/use cases, the flexibility shall be considered for different device implementation. For instance, device operated by RF energy source may have different sizes of capacitors, which results in different charging time. What’s more, the device may also be powered by other energy sources, e.g. solar energy, in different use cases. As the energizing behaviour and performance is not in the scope the 3GPP discussion, RAN4 discussion shall focus on the communication performance and the energy supply can be up to implementation of device/TE.

Proposal 10: The device is assumed energized during the test, and the detailed energizing procedure/energy source is up to implementation, which will not be defined in RAN4 spec.

CW and reader are assumed to be at different locations during OTA tests. In order to have the same pathloss, CW and reader can be placed at the same distance away from A-IoT devices under tests, for example, on the same spherical surface  with angular separation.

Proposal 11: CW and reader are placed at the same distance away from the A-IoT device under test.

In addition, the placement of CW relative to reader may be impacted by measurement grid as well as test setup. For example, the test setups shown in Figure 7.2-1 and Figure 7.2-2 in TR 38.870 would require different considerations for CW placement. Once the measurement grid is agreed, the placement of CW can be finalised.

Observation 2: placement of CW relative to reader may be impacted by measurement grid as well as test setup.

# Conclusion

This contribution makes the following proposals.

Proposal 1: RAN4 to agree on scope of OTA test methodology discussion as:

Test methodology for Ambient IoT including

- Chamber setup

- Position of Device

- Test procedures

- Minimum range/ QZ (Quiet Zone) size/QZ calibration

- Preliminary MU assessment

Note: The performance metrics to be discussed in RF (RRM/Demodulation if any) requirements. And the test methodology takes sensitivity and backscatter power as example for discussion to develop unified test methodology.

Proposal 2: use anechoic chamber for ambient IoT device tests.

Proposal 3: select one option for measurement points for transmit and receive metrics from the following:

Option 1: Measurement grid points over hemi sphere in the direction of reader

Option 2: discrete points in the direction of reader

Proposal 4: measurement campaign is not needed in this work item. Refinement in future work items based on measurement campaign is not precluded.

Proposal 5: test configuration is for a single device under free space condition.

Proposal 6: use MU values from TR38.870 for terms with similar functionality

Proposal 7: reuse the quiet zone calibration procedure from TR38.870 for A-IoT tests

Proposal 8: reuse the same principle to derive the minimum range length as defined in TR 38.870, and further discuss whether to consider a smaller range length with smaller QZ size.

Proposal 9: reuse the environmental requirements from TR38.870 for A-IoT tests.

Proposal 10: The device is assumed energized during the test, and the detailed energizing procedure/energy source is up to implementation, which will not be defined in RAN4 spec.

Proposal 11: CW and reader are placed at the same distance away from the A-IoT device under test.

Observation 1: a quiet zone with smaller size (e.g. 15cm) for A-IoT tests seems to be adequate.

Observation 2: placement of CW relative to reader may be impacted by measurement grid as well as test setup.

# References

- RP-243326 Solutions for Ambient IoT (Internet of Things) in NR

- TS 38.870 v18.3 Enhanced Over-the-Air (OTA) test methods for NR FR1 Total Radiated Power (TRP) and Total Radiated Sensitivity (TRS)

- Commercial RFID of various sizes and ranges: https://www.encstore.com

- Commercial RFID of various sizes and ranges: https://fongwahrfid.com

- Commercial RFID of various sizes and ranges: https://www.synometrix.com