3GPP TSG-RAN WG4 Meeting # 114					R4-2500210

Athens Greece, 17th -21st February 2025

Agenda item:			7.11.1

Source:	Intelsat, CHTTL

Title:	Ku Band: Narrow Band Services with 15 kHz Sub-Carrier Spacing

Document for:	Discussion and Decisions

# Introduction

In this contribution, we present the case for narrow band services and the need for 15 kHz Sub-Carrier Spacing (SCS).

Currently several Satellite operators are providing services to various customers using narrowband beams. These services and application involve providing capacity for Narrow Band applications such as IoT for telemetry, voice, texting, and firmware upgrades. Operationally, narrow bandwidths are also used for terminals on standby and paging.

The network involves terminals with a substantial number of devices in a wider coverage area. Majority of these mass devices are low-cost entry level devices designed with simple technology for the consumer market. Specifically, these low-cost devices are power limited and low form-factor antenna aperture resulting in narrowband carrier services.

Currently these services are provided with proprietary technology and the aim is to migrate these services to 5G-NR Services.

Additionally, we provide calculations to illustrate the benefits of the 15 kHz SCS for various services, applications, and for satellites constellation types.

The use cases and the examples are for FR1-NTN Numerology.

# Use Cases

Application involves providing capacity for Narrow band applications such as IoT for telemetry, voice, texting, and firmware upgrades. The network involves terminals with a substantial number of devices in a wider coverage area. See following Figure 1.

Figure 1 Narrow band services over 0.5 Degree Beams

This application requires the smallest carriers with the least amount of signalling overhead to close the link budget with acceptable throughput. The 10 MHz carriers are the smallest carriers available for 5G NTN and 15 kHz subcarrier spacing offers least signalling overhead and hence meets the requirement and will be recommended for usage.

The following is one such network with 1500 subscribers spread out within a 500 square kilometre area.  1 in every 10 subscribers use 50 kbps data traffic and additionally 1 in every 10 subscribers make a VoIP call using their phone.

Please note that this illustration is an example of a live service provided over a proprietary and non 5G-NR satellite network.

Observation 1: Narrowband 10 MHz beams are used to provide connectivity to multitude of devices within the Satellite coverage area.

# Throughput and Timing Calculation

Table 1 illustrates the 10 MHz channel bandwidth resource block, signal bandwidth, and downlink/uplink data rates for 15kHz and 30kHz SCS.  The data rate calculation assumes 16-QAM modulation, coding rate of 553/1024, 18% downlink overhead, and 14% uplink overhead.  These assumptions are reasonable for GEO or LEO/MEO satellite systems [1].  Table 1 reveals that the data rate at 30kHz SCS is about 8% lower compared to the data rate at 15kHz SCS which is significant for narrow band services where every bit of data is valued.

For narrow band services that we are providing with 10 MHz channel bandwidth, due to multitude of terminals in the field and services that we are providing, it is essential to utilise these narrow bandwidths as efficiently as possible and effectively ‘pump’ as much data as possible to and from the devices.

The following Table 1, illustrates the DL and UL throughput of a 10 MHz bandwidth channel with 15 and 30 kHz SCS, respectively.

Table 1: FR1 10MHz Channel: Resource Block, Signal Bandwidth, and Data Rate

| 10 MHz Channel Bandwidth |  |  |  |  |
| --- | --- | --- | --- | --- |
| SCS (kHz) | Resource Block | Signal Bandwidth (MHz) | Downlink Data Rate (Mbps) | Uplink Data Rate (Mbps) |
| 15 | 52 | 9.375 | 15.5 | 16.2 |
| 30 | 24 | 8.670 | 14.3 | 15.0 |

Furthermore, as we have demonstrated previously [2] the small SCS such as 15 kHz, offers substantially more UL Timing Alignment flexibility due to long Cyclic Prefix (CP) : this in practice means that for cheaper devices with poor Global Navigation Satellite Systems (GNSS) and entry-level devices with poorer UL Synch capability, with 15 kHz SCS can complete the UL Timing synchronisation and connect with passing LEO or MEO satellites.

Figure 2 illustrates graphically the cyclic prefix threshold, timing error and margin duration for 15kHz and 30 kHz numerologies.  It is evident that the cyclic prefix varies significantly between 15kHz and 30kHz.  Specifically, the timing margin for 15kHz SCS (1400nsec) is more than three time larger compared to the timing margin for 30kHz SCS (456nsec).  The timing margin is the timing difference between the cyclic prefix minus the NTN timing error.  Larger timing margin enables cheaper cost-effective devices with poor GNSS capability in a dynamic NGSO based satellite system with large propagation delay and timing-varying Doppler Shift.

Figure 2: Cyclic prefix duration, NTN timing error, and margin

Observation 2: The 15 kHz SCS allow us to use the narrow bandwidth carriers more efficiently and pump more data in the DL and UL directions.

Observation 3: The 15 kHz SCS allows greater UL Timing Margin than the 30 kHz SCS

# VoIP and Data Calculations

In the above scenario, see Table 2, 15 kHz subcarrier spacing will make up to 1Mbps additional bandwidth available when compared to using 30 kHz subcarrier spacing. NOTE: The additional 1 Mbps in each DL and UL direction will allow us to support from 20-40 more VoIP calls with the same capacity depending on the type of voice codec used, see following Table 3

Table 2 Narrow Band Services

| Parameter | Value |
| --- | --- |
| Number of Terminals | 1500 |
| Peak Aggregate DL Data throughput | 7.5 Mbps (1 in every 10 users use 50kbps) |
| Peak Aggregate UL Data throughput | 7.5 Mbps (1 in every 10 users use 50kbps) |
| Peak Number of simultaneous Voice calls | 150 (1 in every 10 sites at a time – in addition to data traffic) |
| DL FWD Carrier | 10 MHz |
| UL RTN Carrier | 10 MHz |
| Max DL Throughput | 15 Mbps (see next section) |
| Max UL Throughput | 15 Mbps (see next section) |
| Satellite Type | GEO with 0.5-degree beam |

Table 3 Bandwidth Required Per VoIP Calls with Different Voice Codecs

| CODEC | Throughput/link in Kbps/call |
| --- | --- |
| G.729 | 24 |
| Opus | 48 |

NOTE:
1- Opus CODEC is used by WhatsApp and other applications that support voice calls. 
2- Opus CODEC requires approximately 48 Kbps/voice call/each direction. Essentially Opus CODEC requires approximately 96 kbps aggregate for a bidirectional call.

3- The throughput values shown also include network and IP overheads etc.

The CODECs shown in Table 3 are examples used in the priority satellite system.  Other more efficient voice CODEC systems with lower throughput are available in the market.  However, the additional 1 Mbps for 15kHz SCS using more efficient CODEC will allow us to support even more VoIP calls with the same capacity.

The capacity assumes a 0.5-degree beam using a GEO satellite with 1500 terminals (subscribers). In the example provided is based on an agricultural application.

Assuming the full 10MHz bandwidth was used for VoIP Services, Table 4 shows the number of additional VoIP calls supported by 15kHz.

Table 4 Number of Additional VoIP Calls Supported by 15kHz

| CODEC | Throughput per CODEC (kbps) | SCS (kHz) | Number of Downlink VoIP calls | Number of Uplink VoIP calls | Number of additional VoIP calls supported by 15 kHz total |
| --- | --- | --- | --- | --- | --- |
| G.729 | 24 | 15 | 645 | 675 | 100 |
|  | 24 | 30 | 595 | 625 |  |
| Opus | 48 | 15 | 322 | 337 | 50 |
|  | 48 | 30 | 297 | 312 |  |

# Discussions

With 15 kHz Sub Carrier spacing we gain approximately 1 Mbps per each direction on a 10 MHz carrier. This will allow for an additional 50 Voice calls using Opus CODEC or 100 Voice calls when using G.729 CODEC. More efficient voice CODECs enables even more additional Voice calls for 15kHz SCS.  For operators providing narrow band services, this additional voice capacity is significant.

Furthermore, 15kHz SCS and its longer UL Timing Margin, allows for the mass deployment of cheaper low-cost devices in the satellite coverage area.

Observation 4: On a narrow 10 MHz bandwidth carrier channel, 15 kHz SCS allows for additional 50 Voice calls using Opus CODEC or additional 100 Voice calls when using G.729 CODEC.

# Conclusion and Proposals

Satellite operators rely heavily on narrow band services to provide multitude of services, such as VoIP to their customers and it is essential to be able to use the beams’ capacity as efficiently as possible.

In this contribution, we provide the following Observations

Observation 1: Narrowband 10 MHz beams are used to provide connectivity to multitude of devices within the Satellite coverage area.

Observation 2: The 15 kHz SCS allow us to use the narrow bandwidth carriers more efficiently and pump more data in the DL and UL directions.

Observation 3: The 15 kHz SCS allows greater UL Timing Margin than the 30 kHz SCS

Observation 4: On a narrow 10 MHz bandwidth carrier channel, 15 kHz SCS allows for additional 50 Voice calls using Opus CODEC or additional 100 Voice calls when using G.729 CODEC.

We can draw the following conclusions

- 15 kHz SCS allows for more efficient use of the narrow bandwidth

- 15 kHz SCS provides higher DL and UL data throughout than 30 kHz SCS

- For entry-level low-cost devices with simpler technologies, 15 kHz SCS allows for longer Uplink Timing alignment to NGSO Satellites

- For VoIP services, 15 kHz SCS for a 10 MHz carrier, allows 50 additional voice calls using Opus CODEC or additional 100 voice calls when using G.729 CODEC

In this contribution we have demonstrated that for narrow band services, 15 kHz SCS allows for more efficient use of the satellite’s narrow beam capacity.

Furthermore, the current FR1 FDD chipset is based on 15 kHz SCS Numerology

Proposal: We propose, to continue to study the 15 kHz SCS option for the Ku Band services, along with the 30 kHz option.

# References

[1] RP-2418029, Ku Band Link Budget Analysis, Intelsat, RAN#113, Orlando, US, 18th-22nd November 2024.

[2] RP-2418030, Ku Band: Analysis of the Timing Alignment and Cyclic Prefix, Intelsat, RAN#113, Orlando, US, 18th-22nd November 2024.