3GPP TSG-RAN WG4 Meeting #114					R4-2500211

Athens Greece, 17th -21st February 2025

Agenda item:			7.11.3

Source:	Intelsat, Thales, Sharp, Gatehouse Satcom, JSAT

Title:	Ku Band: Band Numbering, ARFCN and GSCN Calculations

Document for:	Discussion and Decisions

…

# Introduction

Following the Ku Band WID update in RAN Plenay#106 in Madrid and the agreement to proceed with two Numerologies, i.e. FR1-NTN and FR2-NTN, we present detailed RF Calculations to define Ku band’s fundamental band parameters.

The goal of this contribution is to define for FR1-NTN and FR2-NTN Ku band numerology, the band number; channel bandwidth; Absolute Radio Frequency Channel Number (ARFCN) and channel raster; and Global Synchronization Frequency Channel Number (GSCN) and synchronization raster. Furthermore, the ARFCN and GSCN calculations, provide the maximum flexibility in the utilisation of the Ku Band spectrum.

The calculations presented in this contribution focuses on the FR1-NTN and FR2-NTN Channel Bandwidths and Sub-Carrier Spacing (SCS) agreed in the Ku Band WID[1].

The Ku Band numbering in this document is shown as tentative and the final band numbering is left to RAN4 working group to define.

The Appendix introduces the key concepts of channel raster/ARFCN and synchronization raster/GSCN with examples.

# Ku Band Numerology FR1-NTN

In the following section: the band number; channel bandwidth; ARFCN and channel raster; and GSCN and synchronization raster are defined for FR1-NTN Ku band numerology.

## Band number.

Table 1 illustrates the band numbers for FR1-NTN Ku band numerology as defined in [1] based on Proposal 2.  Currently three unique band numbers are defined, namely: N25W, N25X, and N25Y.  If necessary, we can redefine the band number as desired later.

Table 1: Band definitions for Ku band using FR1-NTN numerology.

| Satellite operating band | Uplink (UL) operating band SAN receive / UE transmit FUL,low – FUL,high | Downlink (DL) operating band SAN transmit / UE receive FDL,low – FDL,high | Duplex mode |
| --- | --- | --- | --- |
| N25W1,2 | 13750 MHz - 14000 MHz | 10700 MHz - 12750 MHz | FDD |
| N25X1,2 | 14000 MHz - 14500 MHz | 10700 MHz - 12750 MHz | FDD |
| N25Y3 | 12700 MHz - 13500 MHz | 10700 MHz - 12750 MHz | FDD |
| NOTE 1: This band is applicable for the countries and or regions subject to or referring to CEPT ECC Decision(05)01 and ECC Decision (13)01. For countries and or regions referring to or subject to FCC 47 CFR part 25 the downlink is limited to 10700 - 12700 MHz. NOTE 2: The UL range 13750 – 14000 is subject to different minimum antenna size and power restrictions for GSO and NGSO in some countries and or regions (FCC 47 C.F.R. § 2.106 US356, ITU-R 5.502) NOTE 3: This band is applicable in the countries and or regions referring to or subject to FCC 47 CFR part 25. For the countries and or regions subject to or referring to CEPT ECC Decision(05)01 and ECC Decision (13)01 [Ka context] the uplink is limited to 12750 - 13500 MHz. NOTE 4: The applicability of bands in the US is pending FCC rulemaking on terrestrial allocation from 12700 – 13500 MHz. |  |  |  |

Observation 1: FR1-NTN band definition has three unique bands.

## Channel Bandwidth.

Table 2 illustrates the channel bandwidth for FR1-NTN Ku band numerology.  The 30 kHz SCS is currently the only subcarrier spacing defined for FR1-NTN Ku band.

Table 2: FR1-NTN channel bandwidth.

| FR1-NTN band / SCS / Channel bandwidth |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Satellite operating band | SCS kHz | 10 MHz | 20 MHz | 25 MHz | 35 MHz | 50 MHz | 70 MHz | 100 MHz |
| N25W | 30 | Yes | Yes | Yes | Yes | Yes | Yes | Yes |
| N25X | 30 | Yes | Yes | Yes | Yes | Yes | Yes | Yes |
| N25Y | 30 | Yes | Yes | Yes | Yes | Yes | Yes | Yes |

Observation 2: FR1-NTN channel bandwidth are 10, 20, 25, 35, 50, 70, and 100 MHz with 30kHz SCS.

## ARFCN and Channel Raster

The Absolute Radio Frequency Channel Number (ARFCN) and channel raster are defined in Appendix 6.1. The ARFCN defines a set of RF reference frequencies used in signalling to identify the position of RF channels on the downlink and uplink frequencies.  For Ku band, the granularity of the global frequency raster (DFGlobal) is 15kHz.

The channel raster defines a subset of RF reference frequencies that can be used to identify the RF channel position in the uplink and downlink.  For 30kHz SCS, the channel raster (DFRaster) is defined as follows: DFRaster = 2 x DFGlobal. In this case, every 2nd AFRCN within the Ku band are applicable for the channel raster and the step size of the channel raster is 2.  Table 3 illustrates the FR1-NTN ARFCN per Ku operating band.  For 30 kHz SCS, DFRaster of 30 kHz should be chosen with step size given as 2 (bolded).

Table 3: FR1-NTN ARFCN per Ku operating band.

| Satellite operating band | ΔFRaster (kHz) | Uplink range of NREF (First - <Step size>- Last) | Downlink range of NREF (First - <Step size>- Last) |
| --- | --- | --- | --- |
| N25W | 15 | 1316667 – <1> – 1333333 | 1113334 – <1> – 1249999 |
|  | 30 | 1316667 – <2> – 1333333 | 1113334 – <2> – 1249998 |
|  | 60 | 1316667 – <4> – 1333331 | 1113334 – <4> – 1249998 |
|  | 120 | 1316667 – <8> – 1333331 | 1113334 – <8> – 1249998 |
| N25X | 15 | 1333334 – <1> – 1366666 | 1113334 – <1> – 1249999 |
|  | 30 | 1333334 – <2> – 1366666 | 1113334 – <2> – 1249998 |
|  | 60 | 1333334 – <4> – 1366666 | 1113334 – <4> – 1249998 |
|  | 120 | 1333334 – <8> – 1366662 | 1113334 – <8> – 1249998 |
| N25Y | 15 | 1250000 – <1> – 1283333 | 1113334 – <1> – 1249999 |
|  | 30 | 1250000 – <2> – 1283332 | 1113334 – <2> – 1249998 |
|  | 60 | 1250000 – <4> – 1283332 | 1113334 – <4> – 1249998 |
|  | 120 | 1250000 – <8> – 1283328 | 1113334 – <8> – 1249998 |

Proposal 1: We propose the FR1-NTN ARFCN entries per Ku operating band with channel raster of 30 kHz and step size of 2 as illustrated in Table 3.

## GSCN and Synchronization Raster

The Global Synchronization Frequency Channel Number (GSCN) and synchronization raster are generally defined in Appendix 6.2.  The synchronization raster indicates the frequency position of the Synchronization Signal Block (SSB) that can be used by the UE on the downlink for system acquisition.  The methods of calculating the GSCN range, namely GSCNmin and GSCNmax, are described in TR 38.817-01, subclause 4.3.1.5 [2]. Specifically :

- Based on the minimum bandwidth of 10MHz and 24 RB for 30kHz SCS, calculate the guard band.  The guard-band is calculated as 665kHz.

- Calculate fmin and fmax using the guard-band.

- Assuming the step size, calculate synchronization raster position, Nmin and Nmax, followed by GSCNmin and GSCNmax using fmin and fmax.

- The step size was chosen is 1.

Figure 1 illustrates the relationship between GSCN, guard-band, and step size for 30 kHz SCS and N25W satellite operating band.  The bandwidth between each SSB is 1.44 MHz, resulting in significant overlap between adjacent SSBs.

Figure 1: Relationship between GSCN, guard-band, and step size for 30kHz SCS and N25W NTN band number.

Table 4 illustrates the FR1-NTN GSCN entries for the different Ku band numbers.  The SSB block pattern for FR1-NTN Ku-band is Case C.

Table 4: FR1-NTN Applicable GSCN entries per operating band.

| Satellite operating band | SS Block SCS | SS Block Pattern | Range of GSCN (First – <Step size> – Last) |
| --- | --- | --- | --- |
| N25W | 30 kHz | Case C | 12850 – <1> – 14266 |
| N25X | 30 kHz | Case C | 12850 – <1> – 14266 |
| N25Y | 30 kHz | Case C | 12850 – <1> – 14266 |

Proposal 2: We propose the FR1-NTN GSCN entries per Ku operating band with step size of 1 as illustrated in Table 4.

# Ku Band Numerology FR2-NTN

In the following section: the band number; channel bandwidth; ARFCN and channel raster; and GSCN and synchronization raster are defined for FR2-NTN Ku band numerology.

## Band number.

Table 5 illustrates the band numbers for FR2-NTN Ku band numerology as defined in [1] based on Proposal 2.  Currently three unique band numbers are defined, namely: N50W, N50X, and N50Y.  If necessary, we can redefine the band number as desired later.

Table 5: Band definitions for Ku band using FR2-NTN numerology.

| Satellite operating band | Uplink (UL) operating band SAN receive / UE transmit FUL,low – FUL,high | Downlink (DL) operating band SAN transmit / UE receive FDL,low – FDL,high | Duplex mode |
| --- | --- | --- | --- |
| N50W1,2 | 13750 MHz - 14000 MHz | 10700 MHz - 12750 MHz | FDD |
| N50X1,2 | 14000 MHz - 14500 MHz | 10700 MHz - 12750 MHz | FDD |
| N50Y3 | 12700 MHz - 13500 MHz | 10700 MHz - 12750 MHz | FDD |
| NOTE 1: This band is applicable for the countries and or regions subject to or referring to CEPT ECC Decision(05)01 and ECC Decision (13)01. For countries and or regions referring to or subject to FCC 47 CFR part 25 the downlink is limited to 10700 - 12700 MHz. NOTE 2: The UL range 13750 – 14000 is subject to different minimum antenna size and power restrictions for GSO and NGSO in some countries and or regions (FCC 47 C.F.R. § 2.106 US356, ITU-R 5.502) NOTE 3: This band is applicable in the countries and or regions referring to or subject to FCC 47 CFR part 25. For the countries and or regions subject to or referring to CEPT ECC Decision(05)01 and ECC Decision (13)01 [Ka context] the uplink is limited to 12750 - 13500 MHz. NOTE 4: The applicability of bands in the US is pending FCC rulemaking on terrestrial allocation from 12700 – 13500 MHz. |  |  |  |

Observation 3: FR2-NTN band definition has three unique bands.

## Channel Bandwidth.

Table 6 illustrates the channel bandwidth for FR2-NTN Ku band numerology.  The 120 kHz SCS is currently the only subcarrier spacing defined for FR2-NTN Ku band.

Table 6: FR2-NTN channel bandwidth.

| FR2-NTN band / SCS / Channel bandwidth |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Satellite operating band | SCS kHz | 50 MHz | 100 MHz | 200 MHz | 400 MHz |
| N50W | 120 | Yes | Yes | Yes | Yes |
| N50X | 120 | Yes | Yes | Yes | Yes |
| N50Y | 120 | Yes | Yes | Yes | Yes |

Observation 4: FR2-NTN channel bandwidth are 50, 100, 200 and 400 MHz with 120kHz SCS.

## ARFCN and Channel Raster

The Absolute Radio Frequency Channel Number (ARFCN) and channel raster are defined in Appendix 6.1.  The ARFCN defines a set of RF reference frequencies used in signalling to identify the position of RF channels on the downlink and uplink frequencies.  For Ku band, the granularity of the global frequency raster (DFGlobal) is 15kHz.

For 120 kHz SCS, the channel raster (DFRaster) is defined as follows: DFRaster = 8 x DFGlobal. In this case, every 8th  AFRCN within the Ku band are applicable for the channel raster and the step size of the channel raster is 8.  Table 8 illustrates the FR2-NTN ARFCN per Ku operating band.  For 120 kHz SCS, DFRaster of 120 kHz should be chosen with step size of 8 (bolded).

Table 7: FR2-NTN ARFCN per downlink Ku operating band.

| Satellite operating band | ΔFRaster (kHz) | Uplink range of NREF (First - <Step size>- Last) | Downlink range of NREF (First - <Step size>- Last) |
| --- | --- | --- | --- |
| N50W | 15 | 1316667 – <1> – 1333333 | 1113334 – <1> – 1249999 |
|  | 30 | 1316667 – <2> – 1333333 | 1113334 – <2> – 1249998 |
|  | 60 | 1316667 – <4> – 1333331 | 1113334 – <4> – 1249998 |
|  | 120 | 1316667 – <8> – 1333331 | 1113334 – <8> – 1249998 |
| N50X | 15 | 1333334 – <1> – 1366666 | 1113334 – <1> – 1249999 |
|  | 30 | 1333334 – <2> – 1366666 | 1113334 – <2> – 1249998 |
|  | 60 | 1333334 – <4> – 1366666 | 1113334 – <4> – 1249998 |
|  | 120 | 1333334 – <8> – 1366662 | 1113334 – <8> – 1249998 |
| N50Y | 15 | 1250000 – <1> – 1283333 | 1113334 – <1> – 1249999 |
|  | 30 | 1250000 – <2> – 1283332 | 1113334 – <2> – 1249998 |
|  | 60 | 1250000 – <4> – 1283332 | 1113334 – <4> – 1249998 |
|  | 120 | 1250000 – <8> – 1283328 | 1113334 – <8> – 1249998 |

Proposal 3: We  propose the FR2-NTN ARFCN entries per Ku operating band with channel raster of 120 kHz and step size of 8 as illustrated in Table 7.

## GSCN and Synchronization Raster

The Global Synchronization Frequency Channel Number (GSCN) and synchronization raster are generally defined in Appendix 6.2.  The synchronization raster indicates the frequency position of the Synchronization Signal Block (SSB) that can be used by the UE on the downlink for system acquisition.  The methods of calculating the GSCN range, namely GSCNmin and GSCNmax, are described in TR 38.817-01, subclause 4.3.1.5 [2]. Specifically :

- Based on the minimum bandwidth of 50MHz and 32 RB for 120kHz SCS, calculate the guard band.  The guard-band is calculated as 1.9 GHz.

- Calculate fmin and fmax using the guard-band.

- Assuming the step size, calculate synchronization raster position, Nmin and Nmax, followed by GSCNmin and GSCNmax using fmin and fmax.

- The step size was chosen is 12.

Figure 2 illustrates the relationship between GSCN, guard-band, and step size for 120 kHz SCS and N50W NTN band number.  With step size of 12, the bandwidth between each SSB is 17.28 MHz, resulting in some overlap between adjacent SSB.

Figure 2: Relationship between GSCN, guard-band, and step size for 120kHz SCS and N50W NTN band number.

Table 8 illustrates the FR2-NTN GSCN entries for the different Ku band numbers.  The SSB block pattern for FR2-NTN Ku-band is Case D.

Table 8: FR2-NTN Applicable SS raster entries per operating band.

| Satellite operating band | SS Block SCS | SS Block Pattern | Range of GSCN (First – <Step size> – Last) |
| --- | --- | --- | --- |
| N50W | 120 kHz | Case D | 12864 – <12> – 14256 |
| N50X | 120 kHz | Case D | 12864 – <12> – 14256 |
| N50Y | 120 kHz | Case D | 12864 – <12> – 14256 |

Proposal 4: We  propose the FR2-NTN GSCN entries per Ku operating band with step size of 12 as illustrated in Table 8.

# Conclusion and Proposals

In this contribution, we have defined for FR1-NTN and FR2-NTN Ku band numerology, the band number; channel bandwidth; Absolute Radio Frequency Channel Number (ARFCN) and channel raster; and Global Synchronization Frequency Channel Number (GSCN) and synchronization raster.

From our detailed calculations we have the following observations:

Observation 1: FR1-NTN band definition has three unique bands.

Observation 2: FR1-NTN channel bandwidth are 10, 20, 25, 35, 50, 70, and 100 MHz with 30kHz SCS.

Observation 3: FR2-NTN band definition has three unique bands.

Observation 4: FR2-NTN channel bandwidth are 50, 100, 200 and 400 MHz with 120kHz SCS.

The proposals that can be drawn from our calculations are as follows:

Proposal 1: We propose the FR1-NTN ARFCN entries per Ku operating band with channel raster of 30 kHz and step size of 2 as illustrated in Table 3.

| Satellite operating band | ΔFRaster (kHz) | Uplink range of NREF (First - <Step size>- Last) | Downlink range of NREF (First - <Step size>- Last) |
| --- | --- | --- | --- |
| N25W | 30 | 1316667 – <2> – 1333333 | 1113334 – <2> – 1249998 |
| N25X | 30 | 1333334 – <2> – 1366666 | 1113334 – <2> – 1249998 |
| N25Y | 30 | 1250000 – <2> – 1283332 | 1113334 – <2> – 1249998 |

Proposal 2: We propose the FR1-NTN GSCN entries per Ku operating band with step size of 1 as illustrated in Table 4.

| Satellite operating band | SS Block SCS | SS Block Pattern | Range of GSCN (First – <Step size> – Last) |
| --- | --- | --- | --- |
| N25W | 30 kHz | Case C | 12850 – <1> – 14266 |
| N25X | 30 kHz | Case C | 12850 – <1> – 14266 |
| N25Y | 30 kHz | Case C | 12850 – <1> – 14266 |

Proposal 3: We  propose the FR2-NTN ARFCN entries per Ku operating band with channel raster of 120 kHz and step size of 8 as illustrated in Table 7.

| Satellite operating band | ΔFRaster (kHz) | Uplink range of NREF (First - <Step size>- Last) | Downlink range of NREF (First - <Step size>- Last) |
| --- | --- | --- | --- |
| N50W | 120 | 1316667 – <8> – 1333331 | 1113334 – <8> – 1249998 |
| N50X | 120 | 1333334 – <8> – 1366662 | 1113334 – <8> – 1249998 |
| N50Y | 120 | 1250000 – <8> – 1283328 | 1113334 – <8> – 1249998 |

Proposal 4: We  propose the FR2-NTN GSCN entries per Ku operating band with step size of 12 as illustrated in Table 8.

| Satellite operating band | SS Block SCS | SS Block Pattern | Range of GSCN (First – <Step size> – Last) |
| --- | --- | --- | --- |
| N50W | 120 kHz | Case D | 12864 – <12> – 14256 |
| N50X | 120 kHz | Case D | 12864 – <12> – 14256 |
| N50Y | 120 kHz | Case D | 12864 – <12> – 14256 |

# References

- RP-2500223, Minimization of the number of Ku bands, Eutelsat Group, Athens, Greece, February 2024.

- TR 38.817-01, subclause 4.3.1.5.

# Appendix

## Channel Raster and ARFCN

The global frequency raster (FREF) defines a set of RF reference frequencies for 5G.  The RF reference frequency is essential for signalling to identify the position of RF channels, the position of the Synchronization Signal block (SSB) and other elements.  As shown in Table 9, the global frequency raster is defined for all frequencies from 0 to 100 GHz.  The granularity of the global frequency raster is DFGlobal.

Table 9: ARFCN parameters for the global frequency raster.

| Range of frequencies (MHz) | DFGlobal (kHz) | FREF-Offs (MHz) | NREF-Offs | Range of NREF |
| --- | --- | --- | --- | --- |
| 0 – 3000 | 5 | 0 | 0 | 0 -599999 |
| 3000 – 24250 | 15 | 3000 | 600000 | 600000 – 2016666 |
| 24250 - 100000 | 60 | 24250.08 | 2016667 | 2016667 - 3279165 |

The global frequency raster in MHz is defined by the following equation:

FREF = FREF-Offs + DFGlobal (NREF – NREF-Offs)   [E.1]

where FREF-Offs and NREF-Offs are given in Table 9, and NREF is the Absolute Radio Frequency Channel Number (ARFCN) - code that specifies a pair of reference frequencies used for transmission and reception (see TS 38.104 Table 5.4.2.1-1).  In a FDD system one ARFCN number is required for downlink and another for uplink because the downlink and uplink frequencies are different.  Conversely, the ARFCN can be calculated using E.1 given the global frequency raster:

NREF = (FREF -FREF-Offs)/DFGlobal + NREF-Offs   [E.2]

As shown in Table 9, the granularity of the global frequency raster at Ku Band is 15kHz.  The channel raster defines a subset of RF reference frequencies that can be used to identify the RF channel position in the uplink and downlink. For each operating band, a subset of frequencies from the global frequency raster are applicable for that Band and forms a channel raster with a granularity DFRaster, which may be equal to or larger than DFGlobal.

As an example, Table 10 provides the channel frequency raster and ARFCN for Ku Band #1a downlink ranging from 10700 - 12750 MHz where the spacing between each channel raster frequency is 15kHz.  There are 136,667 unique ARFCN values inside this band.

Table 10: Example of global frequency raster and ARFCN at Ku Band downlink (10700 - 12750 MHz)

| Channel raster in MHz | ARFCN (NREF) |
| --- | --- |
| 10700.01 | 1113334 |
| 10700.03 | 1113335 |
| 10700.04 | 1113336 |
| 10700.06 | 1113337 |
| 10700.07 | 1113338 |
| 10700.09 | 1113339 |
| 10700.10 | 1113340 |
| 10700.12 | 1113341 |
| 10700.13 | 1113342 |
| 10700.15 | 1113343 |
| 10700.16 | 1113344 |
| 10700.18 | 1113345 |
| 10700.19 | 1113346 |
| 10700.21 | 1113347 |
| 10700.22 | 1113348 |
| *** | *** |
| 12749.81 | 1249987 |
| 12749.82 | 1249988 |
| 12749.84 | 1249989 |
| 12749.85 | 1249990 |
| 12749.87 | 1249991 |
| 12749.88 | 1249992 |
| 12749.90 | 1249993 |
| 12749.91 | 1249994 |
| 12749.93 | 1249995 |
| 12749.94 | 1249996 |
| 12749.96 | 1249997 |
| 12749.97 | 1249998 |
| 12749.99 | 1249999 |
| 12750.00 | 1250000 |

## Synchronization Raster and GSCN

The synchronization raster indicates the frequency positions of the synchronization block that can be used by the UE for system acquisition.  As shown in Table 11 (see TS 138.104 Table 5.4.3.1-1), the global synchronization raster is defined for all frequencies from 0 to 100 GHz.  The frequency position of the Synchronization Signal block (SSB) is defined by SSREF with the corresponding Global Synchronization Channel Number (GSCN). The GSCN identifies the center frequency for the SSB.  The parameters defining the SSREF and GSCN for all the frequency ranges are provided in Table 11.  The SS block frequency position spacing for Ku Band is 1.44 MHz.  The granularity of synchronization raster is wider than that of global frequency raster.

Table 11: GSCN parameters for the global frequency raster.

| Range of frequencies (MHz) | SS block frequency position SSREF | GSCN | Range of GSCN |
| --- | --- | --- | --- |
| 0 – 3000 | N * 1200 kHz + M * 50 kHz, N = 1:2499, M ϵ {1,3,5} | 3N + (M-3)/2 | 2 – 7498 |
| 3000 – 24250 | 3000 MHz + N * 1.44 MHz, N = 0:14756 | 7499 + N | 7499 – 22255 |
| 24250 – 100000 | 24250.08 MHz + N * 17.28 MHz, N = 0:4383 | 22256 + N | 22256 – 26639 |

As an example, Table 12 provides the index N, SS block frequency position (SSREF), and GSCN for Ku Band #1a downlink ranging from 10700 - 12750 MHz assuming step size of two.  The GSCN considers the guard band at the beginning and end of the band.  There are 711 unique GSCN values inside this band.

Table 12: Example of sync raster and GSCN at Ku Band downlink (10700 - 12750 MHz)

| N | SS block frequency (MHz) | GSCN |
| --- | --- | --- |
| 5349 | 10702.56 | 12848 |
| 5351 | 10705.44 | 12850 |
| 5353 | 10708.32 | 12852 |
| 5355 | 10711.20 | 12854 |
| 5357 | 10714.08 | 12856 |
| 5359 | 10716.96 | 12858 |
| 5361 | 10719.84 | 12860 |
| 5363 | 10722.72 | 12862 |
| 5365 | 10725.60 | 12864 |
| 5367 | 10728.48 | 12866 |
| 5369 | 10731.36 | 12868 |
| 5371 | 10734.24 | 12870 |
| 5373 | 10737.12 | 12872 |
| 5375 | 10740.00 | 12874 |
| 5377 | 10742.88 | 12876 |
| *** | *** | *** |
| 6743 | 12709.92 | 14242 |
| 6745 | 12712.80 | 14244 |
| 6747 | 12715.68 | 14246 |
| 6749 | 12718.56 | 14248 |
| 6751 | 12721.44 | 14250 |
| 6753 | 12724.32 | 14252 |
| 6755 | 12727.20 | 14254 |
| 6757 | 12730.08 | 14256 |
| 6759 | 12732.96 | 14258 |
| 6761 | 12735.84 | 14260 |
| 6763 | 12738.72 | 14262 |
| 6765 | 12741.60 | 14264 |
| 6767 | 12744.48 | 14266 |
| 6769 | 12747.36 | 14268 |