3GPP TSG-RAN WG4 Meeting #114                                   	R4-2500223

# Athens, Greece, 17th Feb 2025 – 21st Feb 2025

Agenda item:			7.11.1 Genera aspects

Source:	Eutelsat Group

Title:	Minimization of the number of Ku bands

Document for:	Discussion

# Introduction

Following the decision in [1] to define Ku bands for both FR1-NTN numerology and FR2-NTN numerology it is now more important to minimize the number of Ku bands since the number will double because of the decision to define bands for both numerologies. This paper proposes ways to minimize the number of bands by defining bands with the maximum range then using notes to define exceptions.

# Discussion

## Current WF for Ku band definitions

The WF in [2] for band definition is:

Define as a first Ku band for Region 1 (CEPT countries): 10.7 – 12.75 GHz (S-to-E) and 14.0 - 14.5 GHz (E-to-S).

The WF in [3] for band definition is:

Consider the following two bands for the NR NTN Ku band for the studied regulations (i.e. Taiwan, Japan, Australia) in the region 3:

- Band nX: downlink 10.70 – 12.75 GHz (Space-to-Earth) and uplink 14 – 14.5 GHz (Earth-to-Space)

- Band nY: downlink 10.70 – 12.75 GHz (Space-to-Earth) and uplink 13.75 – 14 GHz (Earth-to-Space) with priority for nX.

Agree the band 10.7-12.7 GHz (S-to-E) and 14.0-14.5 GHz (E-to-S) for North America.

The WF for band definitions is given in [4] as:

Table X: Band definitions for Ku band

| Satellite operating band | Uplink (UL) operating band SAN receive / UE transmit FUL,low – FUL,high | Downlink (DL) operating band SAN transmit / UE receive FDL,low – FDL,high | Duplex mode |
| --- | --- | --- | --- |
| nX(Note 1) | 14000 MHz - 14500 MHz | 10700 MHz - 12750 MHz | FDD |
| nY(Note 2) | 14000 MHz - 14500 MHz | 10700 MHz - 12700 MHz | FDD |
| NOTE 1: This band is applicable in the countries and or regions subject to or referring to CEPT ECC Decision(05)01 and ECC Decision (13)01. NOTE 2: This band is applicable in the countries and or regions referring to or subject to FCC 47 CFR part 25. |  |  |  |

.

Were these approaches continued we could get the following:

Table X: Band definitions for Ku band

| Satellite operating band | Uplink (UL) operating band SAN receive / UE transmit FUL,low – FUL,high | Downlink (DL) operating band SAN transmit / UE receive FDL,low – FDL,high | Duplex mode |
| --- | --- | --- | --- |
| nX(Note 1) | 14000 MHz - 14500 MHz | 10700 MHz - 12750 MHz | FDD |
| nY(Note 2) | 14000 MHz - 14500 MHz | 10700 MHz - 12700 MHz | FDD |
| nZ(Note 1) | 13750 MHz - 14000 MHz | 10700 MHz - 12750 MHz | FDD |
| nA(Note 2) | 13750 MHz - 14000 MHz | 10700 MHz - 12700 MHz | FDD |
| nB(Note 1) | 12750 MHz - 13500 MHz | 10700 MHz - 12750 MHz | FDD |
| nC(Note 2) | 12700 MHz - 13500 MHz | 10700 MHz - 12700 MHz | FDD |
| NOTE 1: This band is applicable in the countries and or regions subject to or referring to CEPT ECC Decision(05)01 and ECC Decision (13)01. NOTE 2: This band is applicable in the countries and or regions referring to or subject to FCC 47 CFR part 25. |  |  |  |

These six bands would then double to allow for both numerologies giving 12 bands in total. This is an unacceptable number of new bands as it would bloat the specifications.

## Minimum number of Ku bands

Table 2.2.1 proposes a minimum set of Ku bands for FR1 numerology using the principles set out in the introduction.

Table 2.2-1: Band definitions for Ku band using FR1-NTN numerology

| Satellite operating band | Uplink (UL) operating band SAN receive / UE transmit FUL,low – FUL,high | Downlink (DL) operating band SAN transmit / UE receive FDL,low – FDL,high | Duplex mode |
| --- | --- | --- | --- |
| N24W1,2 | 13750 MHz - 14500 MHz | 10700 MHz - 12750 MHz | FDD |
| N24X3 | 12700 MHz - 13500 MHz | 10700 MHz - 12750 MHz | FDD |
| NOTE 1: This band is applicable for the countries and or regions subject to or referring to CEPT ECC Decision(05)01 and ECC Decision (13)01. For countries and or regions referring to or subject to FCC 47 CFR part 25 the downlink is limited to 10700 - 12700 MHz. NOTE 2: The UL range 13750 – 14000 is optional and when supported, is subject to different minimum antenna size and power restrictions for GSO and NGSO in some countries and or regions (FCC 47 C.F.R. § 2.106 US356, ITU-R 5.502) NOTE 3: This band is applicable in the countries and or regions referring to or subject to FCC 47 CFR part 25. For the countries and or regions subject to or referring to CEPT ECC Decision(05)01 and ECC Decision (13)01 [Ka context] the uplink is limited to 12750 - 13500 MHz. NOTE 4: The applicability of bands in the US is pending FCC rulemaking on terrestrial allocation from 12700 – 13500 MHz. |  |  |  |

Table 2.2.2 proposes a minimum set of Ku bands for FR1 numerology using the principles set out in the introduction.

Table 2.2-2: Band definitions for Ku band using FR2-NTN numerology

| Satellite operating band | Uplink (UL) operating band SAN receive / UE transmit FUL,low – FUL,high | Downlink (DL) operating band SAN transmit / UE receive FDL,low – FDL,high | Duplex mode |
| --- | --- | --- | --- |
| N50W1,2 | 13750 MHz - 14500 MHz | 10700 MHz - 12750 MHz | FDD |
| N50X3 | 12700 MHz - 13500 MHz | 10700 MHz - 12750 MHz | FDD |
| NOTE 1: This band is applicable for the countries and or regions subject to or referring to CEPT ECC Decision(05)01 and ECC Decision (13)01. For countries and or regions referring to or subject to FCC 47 CFR part 25 the downlink is limited to 10700 - 12700 MHz. NOTE 2: The UL range 13750 – 14000 is optional and when supported, is subject to different minimum antenna size and power restrictions for GSO and NGSO in some countries and or regions (FCC 47 C.F.R. § 2.106 US356, ITU-R 5.502) NOTE 3: This band is applicable in the countries and or regions referring to or subject to FCC 47 CFR part 25. For the countries and or regions subject to or referring to CEPT ECC Decision(05)01 and ECC Decision (13)01 [Ka context] the uplink is limited to 12750 - 13500 MHz. NOTE 4: The applicability of bands in the US is pending FCC rulemaking on terrestrial allocation from 12700 – 13500 MHz. |  |  |  |

Band n24W or n50W are defined for the entire uplink block B and using the wider downlink range defined in Note 1 for countries and or regions that follow the European regulations with a narrower 10.7 – 12.7 GHz downlink for countries and or regions following the FCC regulations. This approach works well for countries and or regions that implement the entire uplink B in a single band definition.

Note 2 then indicates that the lower part of the block from 13.75 – 14 GHz is optional, and that in some countries and or regions, different requirements due to antenna size may apply.

Band n24X or n50X are defined for the uplink block A and using the wider downlink range defined in Note 3 for countries and or regions that follow the FCC regulations with a narrower 12.75 – 13.5 GHz downlink for countries and or regions following the European regulations.

## Alternative number of Ku bands

If it is not acceptable to have the entire uplink block B defined as one band with exceptions for support and requirements, Table 2.3-1 proposes an alternate definition for FR1-NTN that splits the uplink block into two bands.

Table 2.3-1: Band definitions for Ku band using FR1-NTN numerology

| Satellite operating band | Uplink (UL) operating band SAN receive / UE transmit FUL,low – FUL,high | Downlink (DL) operating band SAN transmit / UE receive FDL,low – FDL,high | Duplex mode |
| --- | --- | --- | --- |
| N24W1,2 | 13750 MHz - 14000 MHz | 10700 MHz - 12750 MHz | FDD |
| N24X1,2 | 14000 MHz - 14500 MHz | 10700 MHz - 12750 MHz | FDD |
| N24Y3 | 12700 MHz - 13500 MHz | 10700 MHz - 12750 MHz | FDD |
| NOTE 1: This band is applicable for the countries and or regions subject to or referring to CEPT ECC Decisions. For countries and or regions referring to or subject to FCC 47 CFR part 25 the downlink is limited to 10700 - 12700 MHz. NOTE 2: The UL range 13750 – 14000 is subject to different minimum antenna size and power restrictions for GSO and NGSO in some countries and or regions (FCC 47 C.F.R. § 2.106 US356, ITU-R 5.502) NOTE 3: This band is applicable in the countries and or regions referring to or subject to FCC 47 CFR part 25. For the countries and or regions subject to or referring to CEPT ECC Decisions the uplink is limited to 12750 - 13500 MHz. NOTE 4: The applicability of bands in the US is pending FCC rulemaking on terrestrial allocation from 12700 – 13500 MHz. |  |  |  |

Table 2.3-2 proposes an alternate definition for FR2-NTN that splits the uplink block not two bands.

Table 2.3-2: Band definitions for Ku band using FR2-NTN numerology

| Satellite operating band | Uplink (UL) operating band SAN receive / UE transmit FUL,low – FUL,high | Downlink (DL) operating band SAN transmit / UE receive FDL,low – FDL,high | Duplex mode |
| --- | --- | --- | --- |
| N50W1,2 | 13750 MHz - 14000 MHz | 10700 MHz - 12750 MHz | FDD |
| N50X1,2 | 14000 MHz - 14500 MHz | 10700 MHz - 12750 MHz | FDD |
| N50Y3 | 12700 MHz - 13500 MHz | 10700 MHz - 12750 MHz | FDD |
| NOTE 1: This band is applicable for the countries and or regions subject to or referring to CEPT ECC Decisions. For countries and or regions referring to or subject to FCC 47 CFR part 25 the downlink is limited to 10700 - 12700 MHz. NOTE 2: The UL range 13750 – 14000 is subject to different minimum antenna size and power restrictions for GSO and NGSO in some countries and or regions (FCC 47 C.F.R. § 2.106 US356, ITU-R 5.502) NOTE 3: This band is applicable in the countries and or regions referring to or subject to FCC 47 CFR part 25. For the countries and or regions subject to or referring to CEPT ECC Decisions the uplink is limited to 12750 - 13500 MHz. NOTE 4: The applicability of bands in the US is pending FCC rulemaking on terrestrial allocation from 12700 – 13500 MHz. |  |  |  |

With this definition, Note 2 becomes simpler as there is no optional element. However, this approach means that for operators deploying the entire uplink block B, inter-band CA may be required for some configurations.

# Conclusions

Proposal 1: Define a minimal set of Mu bands for FR1-NTN numerology and FR2-NTN numerology as:

Table 2.2-1: Band definitions for Ku band using FR1-NTN numerology

| Satellite operating band | Uplink (UL) operating band SAN receive / UE transmit FUL,low – FUL,high | Downlink (DL) operating band SAN transmit / UE receive FDL,low – FDL,high | Duplex mode |
| --- | --- | --- | --- |
| N24W1,2 | 13750 MHz - 14500 MHz | 10700 MHz - 12750 MHz | FDD |
| N24X3 | 12700 MHz - 13500 MHz | 10700 MHz - 12750 MHz | FDD |
| NOTE 1: This band is applicable for the countries and or regions subject to or referring to CEPT ECC Decisions. For countries and or regions referring to or subject to FCC 47 CFR part 25 the downlink is limited to 10700 - 12700 MHz. NOTE 2: The UL range 13750 – 14000 is subject to different minimum antenna size and power restrictions for GSO and NGSO in some countries and or regions (FCC 47 C.F.R. § 2.106 US356, ITU-R 5.502) NOTE 3: This band is applicable in the countries and or regions referring to or subject to FCC 47 CFR part 25. For the countries and or regions subject to or referring to CEPT ECC Decisions the uplink is limited to 12750 - 13500 MHz. NOTE 4: The applicability of bands in the US is pending FCC rulemaking on terrestrial allocation from 12700 – 13500 MHz. |  |  |  |

Table 2.2-2: Band definitions for Ku band using FR2-NTN numerology

| Satellite operating band | Uplink (UL) operating band SAN receive / UE transmit FUL,low – FUL,high | Downlink (DL) operating band SAN transmit / UE receive FDL,low – FDL,high | Duplex mode |
| --- | --- | --- | --- |
| N50W1,2 | 13750 MHz - 14500 MHz | 10700 MHz - 12750 MHz | FDD |
| N50X3 | 12700 MHz - 13500 MHz | 10700 MHz - 12750 MHz | FDD |
| NOTE 1: This band is applicable for the countries and or regions subject to or referring to CEPT ECC Decisions. For countries and or regions referring to or subject to FCC 47 CFR part 25 the downlink is limited to 10700 - 12700 MHz. NOTE 2: The UL range 13750 – 14000 is subject to different minimum antenna size and power restrictions for GSO and NGSO in some countries and or regions (FCC 47 C.F.R. § 2.106 US356, ITU-R 5.502) NOTE 3: This band is applicable in the countries and or regions referring to or subject to FCC 47 CFR part 25. For the countries and or regions subject to or referring to CEPT ECC Decisions the uplink is limited to 12750 - 13500 MHz. NOTE 4: The applicability of bands in the US is pending FCC rulemaking on terrestrial allocation from 12700 – 13500 MHz. |  |  |  |

Proposal 2: Alternatively, define a minimal set of Mu bands for FR1-NTN numerology and FR2-NTN numerology as:

, Table 2.3-1: Band definitions for Ku band using FR1-NTN numerology

| Satellite operating band | Uplink (UL) operating band SAN receive / UE transmit FUL,low – FUL,high | Downlink (DL) operating band SAN transmit / UE receive FDL,low – FDL,high | Duplex mode |
| --- | --- | --- | --- |
| N24W1,2 | 13750 MHz - 14000 MHz | 10700 MHz - 12750 MHz | FDD |
| N24X1,2 | 14000 MHz - 14500 MHz | 10700 MHz - 12750 MHz | FDD |
| N25Y3 | 12700 MHz - 13500 MHz | 10700 MHz - 12750 MHz | FDD |
| NOTE 1: This band is applicable for the countries and or regions subject to or referring to CEPT ECC Decisions. For countries and or regions referring to or subject to FCC 47 CFR part 25 the downlink is limited to 10700 - 12700 MHz. NOTE 2: The UL range 13750 – 14000 is subject to different minimum antenna size and power restrictions for GSO and NGSO in some countries and or regions (FCC 47 C.F.R. § 2.106 US356, ITU-R 5.502) NOTE 3: This band is applicable in the countries and or regions referring to or subject to FCC 47 CFR part 25. For the countries and or regions subject to or referring to CEPT ECC Decisions the uplink is limited to 12750 - 13500 MHz. NOTE 4: The applicability of bands in the US is pending FCC rulemaking on terrestrial allocation from 12700 – 13500 MHz. |  |  |  |

Table 2.3-2: Band definitions for Ku band using FR2-NTN numerology

| Satellite operating band | Uplink (UL) operating band SAN receive / UE transmit FUL,low – FUL,high | Downlink (DL) operating band SAN transmit / UE receive FDL,low – FDL,high | Duplex mode |
| --- | --- | --- | --- |
| N50W1,2 | 13750 MHz - 14000 MHz | 10700 MHz - 12750 MHz | FDD |
| N50X1,2 | 14000 MHz - 14500 MHz | 10700 MHz - 12750 MHz | FDD |
| N50Y3 | 12700 MHz - 13500 MHz | 10700 MHz - 12750 MHz | FDD |
| NOTE 1: This band is applicable for the countries and or regions subject to or referring to CEPT ECC Decisions. For countries and or regions referring to or subject to FCC 47 CFR part 25 the downlink is limited to 10700 - 12700 MHz. NOTE 2: The UL range 13750 – 14000 is subject to different minimum antenna size and power restrictions for GSO and NGSO in some countries and or regions (FCC 47 C.F.R. § 2.106 US356, ITU-R 5.502) NOTE 3: This band is applicable in the countries and or regions referring to or subject to FCC 47 CFR part 25. For the countries and or regions subject to or referring to CEPT ECC Decisions the uplink is limited to 12750 - 13500 MHz. NOTE 4: The applicability of bands in the US is pending FCC rulemaking on terrestrial allocation from 12700 – 13500 MHz. |  |  |  |

# References

[1] RP-243314, “Revised Ku band WID” (Eutelsat Group) RAN #106 Dec 2024

[2] R4-2413519 “WF for [112][314] NR NTN Ku Band General” (Eutelsat Group) RAN4 #112 Aug 2024

[3] R4-2416569 “WF for [112bis][313] NR NTN Ku Band General” (Eutelsat Group) RAN4 #112bis Sep 2024

[4] R4-2419801 “WF for [113][313] NR NTN Ku Band General” (Eutelsat Group) RAN4 #113 Nov 2024