# 3GPP TSG-RAN WG4 Meeting #114	R4-2500235

# Athens, GR, 17th – 21st February, 2025

Agenda item:	5.21.3

Source:	Apple

Title:	On R18 NTN RRM core and performance requirement maintenance

Document for:	Discussion

# Introduction

In last RAN4#113 meeting, the WF for eNTN has been agreed in [1] and moderator has captured all open issues in the topic summary [2]. There are still some opens issues for further discussion, and in this contribution, we continue the discussion on remaining RRM performance requirements for eNTN.

In addition, there is one in-coming LS from RAN2[3], and we also discuss the potential reply LS to RAN2 in this paper.

# Discussion on Core part maintenance

### Issue 5-2-S: Soft’ Satellite switch

FFS: - A UE that supports soft satellite switching shall mandatorily support parallelMeasurementWithoutRestriction-r17.

How to optimize the scheduling restriction is not fully concluded yet as highlighted in yellow above. In previous meeting, some companies proposed to use PDD measurement/report to enhance the scheduling restriction during the soft satellite switch. However, RAN2 had following agreements that,

3. We don’t introduce the support for configuring and reporting measurements on the target cell (from target satellite) in the soft switch case 4. RAN2 understands that the NW is not expected to configure PDD reporting between serving and target satellites involved in the satellite switch

In addition, when to start target satellite synchronization is up to UE implementation, even though network can somehow know the time location of received target SAT’s SSB occasion at UE, network may still need to apply scheduling restriction on all possible SSB occasions, therefore PDD reporting benefit is very limited. So RAN4 shall not consider PDD reporting between serving and target satellites involved in the satellite switch. Regarding the scheduling restriction itself, if UE is not capable of parallelMeasurementWithoutRestriction-r17, network can still configure hard satellite switching for simplicity, and it can always be alternative for these special cases. From UE capability perspective, our understanding is: If UE indicates to support soft satellite switching without PCI change, it would be typical for UE also to indicate to support parallelMeasurementWithoutRestriction-r17 as well. And in this case, then no scheduling restriction will be applied during the time period of [t-serviceStart, t-Service] during the soft satellite switching.

Proposal 1: to enhance scheduling restriction in soft satellite switching, consider modifying the capability rule such that: if UE indicates to support soft satellite switching without PCI change, it also means UE supports parallelMeasurementWithoutRestriction-r17.

# Discussion on LS[3] from RAN2

RAN4 received one LS[3] from RAN2, and the text is duplicated below,

# 1. Overall Description: For soft satellite switching with re-synchronization, to ensure non-colliding SSB with same PCI at the UE side, RAN2 has discussed one possible solution to introduce a temporarily shifted SSB pattern during [t-ServiceStart, t-Service]. To be specific, the target satellite transmits SSB during [t-ServiceStart, t-Service] which is not overlapping with SSB from the source satellite. At t-Service, the new serving satellite SSB pattern (e.g., SSB offset) is restored to the same as the previous source satellite. RAN2 wonders whether the pre-acquired DL synchronization started between [t-ServiceStart, t-Service] with target satellite can be assumed as maintained if the SSB pattern (e.g., SSB offset) is changed (determined by ssb-TimeOffset) at t-Service. # 2. Actions: To RAN4, RAN1 ACTION: RAN2 kindly asks RAN4 and RAN1 whether the pre-acquired DL synchronization started between [t-ServiceStart, t-Service] with target satellite can be assumed as maintained if the SSB pattern (e.g., SSB offset) is changed (determined by ssb-TimeOffset) at t-Service.

As shown in the following figure, the ssb-TimeOffset is to avoid the colliding between source satellite SSB and target satellite SSB during [t-ServiceStart, t-Service], and then after t-Service, the new serving satellite SSB offset is restored to the same as the previous source satellite. That means, the SSB offset restoring will only offset back the SSB absolute timing by a fixed time value (e.g., the ssb-TimeOffset of target satellite during [t-ServiceStart, t-Service]).

Figure 1. illustration of soft satellite switching with SSB offset reset

In RAN4 requirement for soft satellite switching as following, the Tsearch is the time required to search the target NR SAN cell assuming the target cell is not already known when the handover command is received by the UE. Cell search can determine the coarse timing information for symbol/slot boundary for the target satellite and it shall be maintained even after t-Service. The only difference is UE can determine the SSB location on time domain by using this coarse timing and restored SSB offset (it implies 0 t-Service after in our view).

Additionally, we used maximum function for the requirement to consider whether UE can complete the timing acquisition before t-Service. Thus, the only thing we need to clarify in RAN4 is: if UE does not complete the cell searching when t-service is expired, we need to clarify in the current definition of Tfirst_SSB.

The current definition of Tfirst_SSB is the time to the end of the first complete SSB burst of target satellite, the location of which is determined by the periodicity and location of SSB of the source satellite, the ssb-TimeOffset and the difference between propagation delay of the serving satellite and the target satellite counted from [SSB-TimeOffset reference point as defined in 38.331 [2]] to UE.

If the ssb-TimeOffset is restored after the t-Service and UE does not complete the cell searching when t-service is expired, that actually means no ssb-TimeOffset shall be applied to the SSB timing for target satellite, and therefore it could be:

- is the time to the end of the first complete SSB burst of target satellite, the location of which is determined by the periodicity and location of SSB of the source satellite, the ssb-TimeOffset and the difference between propagation delay of the serving satellite and the target satellite counted from [SSB-TimeOffset reference point as defined in 38.331 [2]] to UE; or

- is the time to the end of the first complete SSB burst of target satellite in the SMTC window.

Proposal 2: to answer RAN2’s question in the LS[3]:

- the pre-acquired DL synchronization started between [t-ServiceStart, t-Service] with target satellite can be assumed as maintained if the SSB pattern (e.g., SSB offset) is changed (determined by ssb-TimeOffset) at t-Service.

Proposal 3: if UE does not complete the cell searching when t-service is expired, RAN4 to discuss whether the clarification is needed for Tfirst_SSB, e.g., Tfirst_SSB can be revised to either:

- is the time to the end of the first complete SSB burst of target satellite, the location of which is determined by the periodicity and location of SSB of the source satellite, the ssb-TimeOffset and the difference between propagation delay of the serving satellite and the target satellite counted from [SSB-TimeOffset reference point as defined in 38.331 [2]] to UE; or

- is the time to the end of the first complete SSB burst of target satellite in the SMTC window.

# Conclusion

In this contribution, we discuss the NR-NTN core requirement maintenance.

Proposal 1: to enhance scheduling restriction in soft satellite switching, consider modifying the capability rule such that: if UE indicates to support soft satellite switching without PCI change, it also means UE supports parallelMeasurementWithoutRestriction-r17.

Proposal 2: to answer RAN2’s question in the LS[3]:

- the pre-acquired DL synchronization started between [t-ServiceStart, t-Service] with target satellite can be assumed as maintained if the SSB pattern (e.g., SSB offset) is changed (determined by ssb-TimeOffset) at t-Service.

Proposal 3: if UE does not complete the cell searching when t-service is expired, RAN4 to discuss whether the clarification is needed for Tfirst_SSB, e.g., Tfirst_SSB can be revised to either:

- is the time to the end of the first complete SSB burst of target satellite, the location of which is determined by the periodicity and location of SSB of the source satellite, the ssb-TimeOffset and the difference between propagation delay of the serving satellite and the target satellite counted from [SSB-TimeOffset reference point as defined in 38.331 [2]] to UE; or

- is the time to the end of the first complete SSB burst of target satellite in the SMTC window.

# References

[1] R4-2418265, Topic summary for [113][207] NR_NTN_enh, Qualcomm Incorporated, RAN4 #113

[2] R4-2420235, WF on RRM requirements for NR_NTN_enh, Qualcomm Incorporated, RAN4#113

[3] R2-2410978, LS on soft satellite switch with re-sync, Huawei, RAN2#128