# 3GPP TSG-RAN WG4 Meeting #114	R4-2500240

# Athens, GR, 17th – 21st February, 2025

Agenda item:	7.29.4.2

Source:	Apple

Title:	On R19 other NTN RRM requirements

Document for:	Discussion

# Introduction

The WF on RRM requirements for Rel-19 NR NTN phase3 [1] has been agreed in RAN4#112, and the open issues have been summarized in the topic summary for [113][231] NR_NTN_Ph3_Part2 [2].

In this contribution, we continue the discussion on the other RRM requirements for R19 NTN.

# RRM for downlink coverage enhancements

Issue 1-1: SSB periodicity enhancement

Agreement (online): For SSB periodicity up to 160ms: - No further discussion on the impact on initial access from the perspective of RRM requirement - Discuss whether to Modify the default values for SSB and SMTC periodicities for the following if the UE is not provided with SMTC configuration or measurement object on the corresponding frequency by NW - RRC Re-establishment: TSMTC,i assumption - RRC connection release with redirection to NR: Trs assumption - Handover, Conditional handover, RACH-less HO and satellite switch with re-sync: Trs assumption - Further discuss the details on how to update the requirement. - Further discuss the following for SSB periodicity > 20ms: - Cell Reselection: for the SSB-periodicity of 160ms, whether the values for Tmeasure_NR_xxx_enh need to be updated for DRX Cycles equal to 0.32 s and other DRX cycles - Paging interruption in RRC Idle/Inactive: whether/how to adjust the requirements when SSB periods =160 ms in NGSO scenarios - RRC re-establishment: RRC re-establishment requirements for Serving cell SSB Ês/Iot < -8dB - Handover, Conditional handover, RACH-less HO and satellite switch with re-sync - Further discuss whether and how to update the requirement Agreement (online): For SSB periodicity larger than 160ms if supported by other WGs: - Whether RAN4 will define the RRM requirements for SSB periodicity larger than 160ms is depending on whether it is supported by WGs.

In last RAN1 and RAN2 meetings, some agreements for DL coverage have been made as followings:

Agreement in RAN1 #118bis For NR NTN, support extended periodicity of the half frames with SS/PBCH blocks assumed by UE during initial access. - The maximum of the additional default value (apart from the existing 20ms value) is at least 160 ms. - FFS: whether 320ms can be supported as the maximum of the additional default value instead of or in addition to 160ms Agreement in RAN2 #127bis 1. If it turns out that there is a need to bar UEs not supporting DL-CE, Rel-19 UEs not supporting DL-CE can be barred from accessing a cell operating with DL-CE using the existing NTN bar bit, in the same way as pre-Rel19 NTN UEs (this is an extension of the previous agreement to include also Rel-19 UE not supporting DL-CE) 2. If it turns out that there is a need to bar UEs not supporting DL-CE, then we need to introduce a barring mechanism to control access of UEs supporting Rel19 NTN DL-CE. FFS on the details. (This also implies that UEs supporting Rel19 NTN DL-CE will not consider the existing NTN barring bit) 3. (also depending on the details of the RAN1 solution) we can further consider methods to allow UEs not supporting DL CE to down-prioritize or prevent re-selection to cells operating with DL CE.

The SSB periodicity extension for DL coverage enhancement can probably impact UE initial access procedure compared with the legacy assumption of 20ms. RAN4 may need to double check if any RRM requirement shall be clarified when the SSB periodicity is changed to a value different from 20ms.

In the current initial access assumption for cell selection, it was defined in RAN1 that,

In the current NTN network (R17, and R18), by default NTN UE will try the initial access to network assuming SSB periodicity as 20ms, however, if the R19 NTN network changes the SSB periodicity to be more than 20ms, it will cause the initial access failure for the R17/R18 NTN UEs. Even for the R19 NTN UE, it has no clear idea if the network behavior is following R17/R18 or R19, and the UE behavior is unclear for implementation during the initial access.

Besides the initial access, there are other UE behaviors requiring assumption of the by-default SSB periodicity in the previous NTN networks (R17/R18). Some examples as followings:

    - For RACH based HO, CHO,  RACH-less HO and satellite switch with re-sync, if the UE is not provided SMTC configuration or measurement object on this frequency, the requirement in this clause is applied with Trs=5ms assuming the SSB transmission periodicity is 5ms. There is no requirement if the SSB transmission periodicity is not 5ms.

    - For RRC Re-establishment for Satellite Access, the definition of TSMTC,i is the periodicity of the SMTC occasion configured for the inter-frequency carrier i. If it is not configured, the UE may assume that the target SSB periodicity is no larger than 20 ms.

    - For RRC connection release with redirection to NR, if the UE is not provided with SMTC configuration or measurement object for the frequency which is also configured for the RRC connection release with redirection then: the requirement in this clause is applied with Trs = 20 ms if the SSB transmission periodicity is not larger than 20 ms; otherwise, there is no requirement if the SSB transmission periodicity is larger than 20ms.

With the SSB periodicity change(e.g., change to a SSB periodicity > 20ms), the above corresponding requirement needs to be modified.

Proposal 1: With the SSB periodicity change for DL coverage enhancement, the following assumptions in RRM requirements need to be modified:

    - Trs assumption for RACH based HO, CHO, RACH-less HO and satellite switch with re-sync, if the UE is not provided SMTC configuration or measurement object on this frequency.

    - TSMTC,i assumption for RRC Re-establishment for Satellite Access, if it is not configured.

    - Trs assumption for RRC connection release with redirection to NR, if the UE is not provided with SMTC configuration or measurement object for the frequency which is also configured for the RRC connection release with redirection.

Regarding the RRC re-establishment requirements for Serving cell SSB Ês/Iot < -8dB, we have one note in the requirement to say: The UE is not required to successfully identify a cell on any NR frequency layer when TSMTC,i > 20 ms and serving cell SSB Ês/Iot < -8 dB. We understand this note was based on an assumption that initial access SSB periodicity is 20ms and SMTC will be configured in line with the SSB periodicity. However, the requirement can still work if SMTC is not configured and RAN4 clarifies the new by-default SSB periodicity for re-establishment. We slightly prefer to not change the RRC re-establishment requirements for SAN.

Proposal 2: for SSB periodicity > 20ms, slightly prefer to not change the RRC re-establishment requirements for SAN.

In RAN#106 meeting, the WID revision of R19 NTN has been agreed in RP-243300 as following:

Only the 160ms SSB periodicity is considered for this R19 NTN DL coverage enhancement, and therefore RAN4 doesn’t need to have any requirement for SSB periodicity greater than 160ms.

Proposal 3: NOT define the RRM requirements for SSB periodicity larger than 160ms.

In addition, we identified a new issue about scheduling restriction with DL coverage enhancement. In the current TN FR1 scheduling restriction for L1 or L3 measurement, it has two main kinds:

- TDD based scheduling restriction because UE cannot do UL transmission when UE is operation on DL for neighbor cell measurement.

- Other scheduling restriction applies because UE cannot do e.g., mixed numerology Rx or Tx when UE is performing SSB or CSI-RS measurement.

R17 NTN has new scheduling restriction if UE does’t support parallelMeasurementWithoutRestriction, when UE is performing SS-RSRP/RSRQ/SINR measurement on a neighbor cell served by a different satellite in LEO.

The above scheduling restriction happens during the SSB measurement occasions of the target cells, e.g., UE is doing SSB measurement but cannot receive the data/control or transmit data/control/RACH to the serving satellite. However, in this special case for DL coverage enhancement with Tx beam hopping, even on other time period outside the configured SSB occasions, NW may be still not possible to schedule or receive any channel/signal to or from UE, as shown in following figure 1. To the example UE in cell 2, outside the cell 2 SSB occasions, before T2 and after T2+40ms, NW hopped Tx beams to other cells (cell 1/3/4), and therefore serving satellite cannot transmit/receive anything to or from UE in cell 2. Therefore, we need to reconsider the scheduling restriction applicability in this scenario of DL coverage enhancement.

Figure 1. cell 2 UE(s) may not transmit or receive signaling from satellite when beam hopped away

If the RO periodicity configuration from serving cell is not greater than the SSB periodicity (less than 160ms in the example figure 2), the scheduling restriction applies to UE RACH and RACH related channels transmission outside the time period of serving Tx beam availability, time period of serving Tx beam availability means, e.g., the time period between the beginning of slot containing available SSB (i.e., T2 in example figure 2) and (original SSB periodicity + T2). Thus, the scheduling restriction for RACH could be: If DL coverage enhancement or satellite beam hopping is enabled, the UE is not expected to transmit PRACH on symbols outside the serving satellite Tx/Rx beam availability durations.

Figure 2. example for RO in SSB periodicity

Proposal 4: RAN4 to reconsider the scheduling restriction applicability for DL coverage enhancement, e.g., scheduling restriction for RACH transmission if serving satellite hopped away the beam to not cover the footprint of target UE.

# Conclusion

In this contribution, we discuss the RRM requirement scope for WI of Non-Terrestrial Networks (NTN) for NR Phase 3.

Proposal 1: With the SSB periodicity change for DL coverage enhancement, the following assumptions in RRM requirements need to be modified:

    - Trs assumption for RACH based HO, CHO, RACH-less HO and satellite switch with re-sync, if the UE is not provided SMTC configuration or measurement object on this frequency.

    - TSMTC,i assumption for RRC Re-establishment for Satellite Access, if it is not configured.

    - Trs assumption for RRC connection release with redirection to NR, if the UE is not provided with SMTC configuration or measurement object for the frequency which is also configured for the RRC connection release with redirection.

Proposal 2: for SSB periodicity > 20ms, slightly prefer to not change the RRC re-establishment requirements for SAN.

Proposal 3: NOT define the RRM requirements for SSB periodicity larger than 160ms.

Proposal 4: RAN4 to reconsider the scheduling restriction applicability for DL coverage enhancement, e.g., scheduling restriction for RACH transmission if serving satellite hopped away the beam to not cover the footprint of target UE.

# References

[1] R4-2420105, WF on RRM requirements for NR_NTN_Ph3_Part2, Qualcomm, RAN4#113

[2] R4-2418289, Topic summary for [113][231] NR_NTN_Ph3_Part2, Qualcomm, RAN4#113