3GPP TSG-RAN WG4 Meeting #114		R4-2500264

Athens, GR, 17th – 21st February, 2025

Agenda item:	7.16.2

Source:	Apple, Qualcomm

Title:	Draft TP to TR on SCM study - 5.1 CDL approach

Release:	Rel-19

Document for:	Discussion

# 1. Introduction

In RAN4#113 spatial channel model for demodulation requirements was discussed and way forward [1] was approved, where the work split for TR was agreed. In this contribution we share initial draft for TP on Section 5.1 CDL Approach.   This contribution intends to capture the structure of the section without many details in the sub-sections with the purpose of sharing our views on organization, contents of the section and harmonizing views.

# 2. TP

## 5.1	CDL Approach

<Introduction>

The details of the CDL based approach are provided in this section with the relevant parameter configurations.

### 5.1.1	Antenna Panel Configuration

<This section will have the parameters for BS and UE antenna panel configuration>

Table 5.1.1-1: BS and UE Panel parameters

| Parameter | Value |
| --- | --- |
| LCS UE | α = 180°, β=0°, γ = 0° |
| LCS gNodeB | α = 0°, β=10°, γ = 0° |
| GCS UE | Height = 1.5 m; Azimuth = 0; X Coordinate = 100 m |
| GCS gNodeB | Height = 25 m; Azimuth = 0; X Coordinate = 0 m |
| Antenna Panel Placement | YZ Plane |

### 5.1.2	Antenna Array Virtualization

The antenna panel is composed by antenna elements arranged in columns and rows, single or dual polarized, and can be split in subarrays. One subarray is a unique set of co-polarized antenna elements mapped exclusively to one antenna port through a virtualizer (set of complex values).

The antenna panel configuration is defined as (M,N,P,Ms,Ns), according to the description below:

M is the number of antenna elements per panel with the same polarization in each column (vertical size)

N is the number of antenna elements per panel with the same polarization in each row (horizontal size)

P is the number of polarizations

Ms is the number of antenna elements per subarray with the same polarization in each column (vertical size)

Ns is the number of antenna elements per subarray with the same polarization in each row (horizontal size)

BS AAV Configurations:

- Option 1: with Antenna Array Virtualisation (AAV) using subarray configuration

(M,N,P,Ms,Ns)  = (8,8,2,8,4) for 4Tx CSI-RS Ports (option X)

(M,N,P,Ms,Ns)  = (8,8,2,8,2) for 8Tx CSI-RS Ports (option X)

(M,N,P,Ms,Ns)  = (8,2,2,8,1) for 4Tx CSI-RS Ports (option Y)

(M,N,P,Ms,Ns)  = (8,4,2,8,1) for 8Tx CSI-RS Ports (option Y)

- Option 2: with AAV without subarray (fully connected)

(M,N,P,Ms,Ns) = (8,8,2,8,8) for 8Tx and 4Tx CSI-RS Ports

- Option 3: with AAV with the following configuration:

(M,N,P,Ms,Ns) = (1,4,2,1,1) for 8Tx CSI-RS Ports

(M,N,P,Ms,Ns) = (1,2,2,1,1) for 4Tx CSI-RS Ports

UE AAV Configuration

(M,N,P,Ms,Ns) = (1,2,2,1,1) for 4 RX

<Include the figures for illustration>

### 5.1.3	Antenna Element Configuration

<This section provides parameters for antenna element configuration>

Table 5.1.3-1: BS and UE Antenna element parameters

| Parameter | Value |
| --- | --- |
| BS Antenna Polarization | (+45, -45) |
| BS Radiation Pattern | defined in TR38.901 Table 7.3-1. |
| UE Antenna Polarization | (0, 90) |
| UE Antenna Radiation Pattern | Omnidirectional |

### 5.1.4	Channel Models

#### 5.1.4.1	Parameters for Angle Scaling

The parameters for scaling the angles of arrival and departure are provided here.

Table 5.1.4.1-1: Angle scaling parameters

| AS | ASmodel / ASdesired [deg] |  |  |  |
| --- | --- | --- | --- | --- |
|  | ASD | ASA | ZSD | ZSA |
| Model | 39.0949 | 71.1175 | 4.0666 | 10.4245 |
| Desired | 25.7620 | 74.1138 | 4.8978 | 18.2050 |

#### 5.1.4.2	Parameters for Randomness Reduction

Subsequently, the AOD angles are coupled to AOA angles within a cluster n. Instead of random procedure, the coupling is performed using the fixed coupling pattern specified in Table 7.2-6.  The same fixed coupling pattern is applied for all clusters n.

Table 5.1.4.2-1: Fixed coupling pattern of ray angles to be applied for each cluster

|  | m |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 6 | 12 | 5 | 10 | 8 | 11 | 16 | 14 | 18 | 9 | 20 | 4 | 2 | 15 | 7 | 13 | 19 | 17 | 3 | 1 |
|  | 20 | 9 | 12 | 1 | 13 | 18 | 10 | 4 | 8 | 2 | 6 | 14 | 11 | 19 | 7 | 3 | 17 | 5 | 15 | 16 |
|  | 2 | 16 | 3 | 11 | 18 | 9 | 5 | 17 | 4 | 19 | 15 | 20 | 13 | 7 | 10 | 1 | 8 | 12 | 6 | 14 |
|  | 15 | 18 | 13 | 1 | 12 | 9 | 6 | 7 | 5 | 3 | 2 | 8 | 14 | 17 | 19 | 16 | 11 | 20 | 10 | 4 |

The set of fixed initial phases can be same for all clusters, i.e. Φn,mθθ=Φmθθ ∀ n=1,…,N etc. for all four polarization combinations. These 20×4 initial phase values can be specified either by a table of values or by setting a random number generator and a fixed seed number. The distribution of scalar initial phases Φn,m is uniform within -π,π. Its purpose is to enable generation of different fading sequences on different uses of the model, but still maintaining the power angular distribution of the model. The scalar initial phases can be fixed (or removed) if completely deterministic process, i.e. exactly same fading sequences at each model use, is aimed at.

Table 5.1.4.2-2: Fixed initial phases for 2x2 polarization matrices

| m | Φmθθ [rad] | Φmϕθ [rad] | Φmθϕ [rad] | Φmϕϕ [rad] |
| --- | --- | --- | --- | --- |
| 1 | 1.7609 | -0.6928 | -1.6230 | -0.6037 |
| 2 | -2.5356 | -2.3124 | 2.7775 | 2.8660 |
| 3 | 0.4725 | -2.7660 | -1.6664 | -0.9226 |
| 4 | 2.0181 | -3.0448 | -2.8713 | -2.0798 |
| 5 | 0.9369 | 1.4560 | 0.9283 | -0.3084 |
| 6 | 0.2954 | -1.2798 | 1.5375 | -1.9544 |
| 7 | 1.1735 | -1.9886 | -0.8263 | 0.7893 |
| 8 | 1.7607 | -2.6319 | 2.6979 | 1.7324 |
| 9 | -0.0830 | -0.4030 | -0.3344 | -1.2167 |
| 10 | 0.0535 | 0.0677 | 1.9957 | 1.8525 |
| 11 | 0.9068 | -0.7627 | 1.9577 | 0.2062 |
| 12 | -0.9379 | 2.7583 | 2.3621 | 0.3151 |
| 13 | 0.7695 | 0.5469 | -1.8363 | -1.2488 |
| 14 | -0.1827 | -1.6934 | 2.1634 | -1.9179 |
| 15 | -1.7221 | -2.0690 | -1.7111 | -0.4040 |
| 16 | -1.1869 | 2.6602 | -0.4385 | -1.9804 |
| 17 | 2.5439 | 3.0143 | -0.3841 | -2.4434 |
| 18 | -1.5201 | -0.5735 | 0.5962 | -1.4941 |
| 19 | 0.6462 | 1.3271 | -1.7483 | -2.4038 |
| 20 | -1.2775 | -1.1386 | -0.4765 | 0.0494 |

#### 5.1.4.3	Channel Models

- Option A1: CDL1 [CDL (TR 38.753) with Table 7.2.1-8 in TR 38.827]

Table 5.1.4.3-1: Channel model parameters for CDL1 at 3.5 GHz

| Cluster # | Absolute Delay [ns] | Power in [dB] | AOD in [°] | AOA in [°] | ZOD in [°] | ZOA in [°] |
| --- | --- | --- | --- | --- | --- | --- |
| 1 | 0 | -4.4215 | -37.4195 | -96.4031 | 96.7645 | 90 |
| 2 | 76.6135 | -1.25 | -21.7362 | 118.7405 | 98.4506 | 90 |
| 3 | 80.9935 | -3.4684 | -21.7362 | 118.7405 | 98.4506 | 90 |
| 4 | 85.0085 | -5.2294 | -21.7362 | 118.7405 | 98.4506 | 90 |
| 5 | 79.424 | -2.5215 | -33.5316 | -124.0196 | 100.8594 | 90 |
| 6 | 232.359 | 0 | -6.5142 | 171.2639 | 99.1732 | 90 |
| 7 | 235.352 | -2.2185 | -6.5142 | 171.2639 | 99.1732 | 90 |
| 8 | 239.44 | -3.9794 | -6.5142 | 171.2639 | 99.1732 | 90 |
| 9 | 240.316 | -7.4215 | 41.4581 | 51.4188 | 106.3995 | 90 |
| 10 | 289.6275 | -7.1215 | -49.2149 | 62.9864 | 94.4761 | 90 |
| 11 | 299.7745 | -10.7215 | 46.1367 | -41.2744 | 107.4834 | 90 |
| 12 | 340.764 | -11.1215 | -70.697 | 42.5606 | 92.3083 | 90 |
| 13 | 448.4025 | -5.1215 | -43.1524 | 64.6538 | 104.5929 | 90 |
| 14 | 477.5295 | -6.8215 | -49.0831 | -62.7423 | 105.1951 | 90 |
| 15 | 792.196 | -8.7215 | -58.4403 | 78.6184 | 91.7061 | 90 |
| 16 | 989.3325 | -13.2215 | 60.9633 | 25.6781 | 105.1951 | 90 |
| 17 | 1554.4985 | -13.9215 | 58.6569 | -23.4063 | 93.9944 | 90 |
| 18 | 1679.1095 | -13.9215 | 51.8037 | -2.3553 | 91.8265 | 90 |
| 19 | 2003.923 | -15.8215 | -73.86 | -20.5926 | 90.7426 | 90 |
| 20 | 2046.8105 | -17.1215 | 54.0442 | 3.7933 | 108.2061 | 90 |
| 21 | 2301.8725 | -16.0215 | 54.7691 | -0.4794 | 91.7061 | 90 |
| 22 | 2422.651 | -15.7215 | 63.5332 | -5.5859 | 91.5856 | 90 |
| 23 | 2570.5855 | -21.6215 | 72.0338 | -29.1381 | 106.3995 | 90 |
| 24 | 3158.0895 | -22.8215 | -88.2912 | 28.7003 | 109.5309 | 90 |
| Per-Cluster Parameters |  |  |  |  |  |  |
| Parameter | CASD in [°] | CASA in [°] | CZSD in [°] | CZSA in [°] | XPR in [dB] |  |
| Value | 1.3179 | 15.632 | 3.6131 | 0 | 7 |  |

#### Option A2: CDL2 [CDL (TR 38.753) with Table TBD]

Table 5.1.4.3-2: Channel model parameters for CDL2

| Cluster # | Absolute Delay [ns] |  | Power in [dB] |  | AOD in [°] |  | AOA in [°] |  | ZOD in [°] |  | ZOA in [°] |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 0 |  | -4.4 |  | -46.6 |  | -101 |  | 97.2 |  | 87.6 |
| 2 | 0.2099 |  | -1.2 |  | -22.8 |  | 120 |  | 98.6 |  | 72.1 |
| 3 | 0.2219 |  | -3.5 |  | -22.8 |  | 120 |  | 98.6 |  | 72.1 |
| 4 | 0.2329 |  | -5.2 |  | -22.8 |  | 120 |  | 98.6 |  | 72.1 |
| 5 | 0.2176 |  | -2.5 |  | -40.7 |  | -127.5 |  | 100.6 |  | 70.1 |
| 6 | 0.6366 |  | 0 |  | 0.3 |  | 170.4 |  | 99.2 |  | 75.3 |
| 7 | 0.6448 |  | -2.2 |  | 0.3 |  | 170.4 |  | 99.2 |  | 75.3 |
| 8 | 0.6560 |  | -3.9 |  | 0.3 |  | 170.4 |  | 99.2 |  | 75.3 |
| 9 | 0.6584 |  | -7.4 |  | 73.1 |  | 55.4 |  | 105.2 |  | 67.4 |
| 10 | 0.7935 |  | -7.1 |  | -64.5 |  | 66.5 |  | 95.3 |  | 63.8 |
| 11 | 0.8213 |  | -10.7 |  | 80.2 |  | -48.1 |  | 106.1 |  | 71.4 |
| 12 | 0.9336 |  | -11.1 |  | -97.1 |  | 46.9 |  | 93.5 |  | 60.5 |
| 13 | 1.2285 |  | -5.1 |  | -55.3 |  | 68.1 |  | 103.7 |  | 90.6 |
| 14 | 1.3083 |  | -6.8 |  | -64.3 |  | -68.7 |  | 104.2 |  | 60.1 |
| 15 | 2.1704 |  | -8.7 |  | -78.5 |  | 81.5 |  | 93.0 |  | 61.0 |
| 16 | 2.7105 |  | -13.2 |  | 102.7 |  | 30.7 |  | 104.2 |  | 100.7 |
| 17 | 4.2589 |  | -13.9 |  | 99.2 |  | -16.4 |  | 94.9 |  | 62.3 |
| 18 | 4.6003 |  | -13.9 |  | 88.8 |  | 3.8 |  | 93.1 |  | 66.7 |
| 19 | 5.4902 |  | -15.8 |  | -101.9 |  | -13.7 |  | 92.2 |  | 52.9 |
| 20 | 5.6077 |  | -17.1 |  | 92.2 |  | 9.7 |  | 106.7 |  | 61.8 |
| 21 | 6.3065 |  | -16 |  | 93.3 |  | 5.6 |  | 93.0 |  | 51.9 |
| 22 | 6.6374 |  | -15.7 |  | 106.6 |  | 0.7 |  | 92.9 |  | 61.7 |
| 23 | 7.0427 |  | -21.6 |  | 119.5 |  | -21.9 |  | 105.2 |  | 58 |
| 24 | 8.6523 |  | -22.8 |  | -123.8 |  | 33.6 |  | 107.8 |  | 57 |
| Per-Cluster Parameters |  |  |  |  |  |  |  |  |  |  |  |
| Parameter |  | cASD in [°] |  | cASA in [°] |  | cZSD in [°] |  | cZSA in [°] |  | XPR in [dB] |  |
| Value |  | 2 |  | 15 |  | 3 |  | 7 |  | 7 |  |

# 3. Conclusion

In this paper, we provided a basic TP for section 5.1 to TR 38.753

# Reference

- R4-2419782, “Way Forward for [113][320] NR_SCM”, Nokia.