3GPP TSG-RAN WG4 Meeting #114		R4-2500268

Athens, GR, 17th – 21st February, 2025

Agenda item:	7.22.3.1

Source:	Apple

Title:	On RRM Requirements for UE initiated beam management

Release:	Rel-19

Document for:	Discussion

# 1. Introduction

In RAN4#113 the RRM requirements for UE initiated beam management was discussed and WF [1] was approved. In this contribution we present our views on RRM core requirements for UE initiated beam management.

# 2. Discussion

In [1] the following agreements were made for RRM core requirements with UE initiated beam management:

Issue 1-1: How to define Event triggered measurement reporting delay? < Agreement> - Start point of Event triggered measurement reporting delay: reuse the same definition in L3 event trigger report which is the event is met over the air. - Ending point is the point when the UE transmits a first PUCCH triggered by the event. Issue 1-2: How to define measurement period (evaluation time / measurement time) in Event triggered measurement reporting delay? < Agreement> Agreement: For specifying the RAN4 delay requirement for event 2 - Measure both the current beam and new beam after the starting point, FFS the details. - Note: this does not imply to define the delay requirement based on the sum of the current beam measurement period and new beam measurement period, which is pending on further RAN1 discussion. - Start the RAN4 RRM discussion for FR2. - Send LS to RAN1 to ask whether FR1 is covered as well. FFS the exact wording in the LS. Issue 1-3: Whether RAN4 to specify requirements for case 1 or cover both case1 and case2? Case 1: no time window based Case 2: with time window based, M is configurable < Agreement> Agreement: It is RAN4 common understanding that no additional filtering applies for case 2 compared to case 1, if RAN4 will define requirement for case 2.

Issue 1-3: Whether RAN4 to specify requirements for case 1 or cover both case1 and case2?

Case 1: no time window based

Case 2: with time window based, M is configurable

From RAN1#117:

Based on above RAN1 agreement the time window-based event triggering is subject to UE capability and the basic feature is or UE to trigger report when L1-RSRP of the new beam becomes a threshold value better than the current beam.

- Time window based even triggering is not basic feature and subject to UE capability.

It is not feasible to define core requirement for reporting delay with time window based triggering.

- It is not feasible to define core requirement for reporting delay with time window based triggering

Hence, we propose that the measurement delay requirement for UE initiating beam reporting shall only be introduced for the basic feature without time window based evaluation.

Measurement reporting delay requirement for UE initiated beam report is defined without time window based evaluation.

Issue 1-4: Reporting delay requirements

<Way forward >

- Option 1: (Qualcomm)

  - Tgap + TL1-RSRP-measure; TL1-RSRP-measure in Issue 1-2; The measurement reporting delay excludes a delay which is caused by no UL resources being available for UE to send the measurement report on; In test cases, set Tgap = 0;

- Option 2: (Nokia)

  - The maximum delay requirement for the first PUCCH channel is determined in relation to the reception time instant of the RS that triggered the UEIBM event.

- Option 3: (Xiaomi)

  - If multiple beams will be measured, total measurement time will be maximum measurement period between multiple beams.

- Option 4: (Samsung)

  - Need further RAN1 progress for RSs relationship between current beam and new beams.

- Option 5: (CMCC)

  - The event triggered measurement reporting delay shall be less than TL1-RSRP_Measurement_Period or TL1-RSRP_Measurement_Period + TSSB_time_index. The measurement reporting delay excludes a delay which is caused by no UL resources being available for UE to send the measurement report on

- Option 6: (Apple)

  - The total reporting delay is Tfirst-RS-NB + TL1-RSRP-NB + TPUCCH, Add TPucch in reporting delay

- Option 7: ( MediaTek)

  - The total reporting delay is TL1-RSRP_measure + Tfirst UL channel, TL1-RSRP_measure is base on SSB or CSI-RS

  - Event evaluation period for a SSB or CSI-RS is same as their respective measurement period

- Option 8: (Ericsson)

  - Event triggered measurement reporting delay for SSB is TL1-RSRP_event triggered_measurement_Period_SSB (ms) + TTime to first UL channel and event triggered measurement delay for CSI-RS is TL1-RSRP_event triggered_measurement_Period_CSI-RS (ms) + TTime to first UL channel

  - RAN4 to discuss components and criteria for defining TTime to first UL channel.

RAN4 received LS from RAN1 on the following –

In RAN1#119, regarding UE-initiated/event-driven beam management for Rel-19 NR MIMO Ph5, the following agreement on the evaluation periodicity for determining Event-2 instance was reached:

Agreement

Regarding for the evaluation periodicity for determining Event-2 instance [at least when DRX is not configured], at least Alt-1 is supported for the single-CC beam reporting (for case that the CSI report configuration and RS for the current beam and new beam(s) are in the same CC).

- Alt-1: The periodicity of the current beam RS should be the same as that of the new beam RS(s).

  - The evaluation periodicity is the same as the periodicity of the current and new beam RS(s).

- Alt-2: The periodicity of the current beam RS can be different from that of the new beam RS(s)

  - Alt-2_1: The evaluation periodicity is the same as the periodicity of the current beam RS.

  - Alt-2_2: The evaluation periodicity is the same as periodicity of the new beam RS.

  - Alt-2_3: The evaluation periodicity is the same as shortest periodicity of the current beam RS and new beam RS(s).

  - Alt-2_4: The evaluation periodicity is the maximum of {X ms, shortest periodicity of the current beam RS and new beam RS(s)}.

  - Alt-2_5: The evaluation periodicity is the same as largest periodicity of the current beam RS and new beam RS(s).

Note: There is the same periodicity for the new beam RS(s).

FFS: Down-selection among above Alt(s) for cross-CC beam reporting.

From agreement in RAN4#114, the measurement period shall include both the current beam and the new beam measurement. RAN1 agreement for single CC beam reporting currently only supports the same periodicity of the current beam and the new beam.

RAN4 only define requirements for same periodicity of new beam and current beam, until further agreement on support of different periodicity in RAN1

The L1-RSRP measurement period for SSB and CSI-RS can be used as the starting point for the measurement period for current beam and new beam. The current L1-RSRP measurement period already considers the required resource sharing factors to account for overlap with MG, SMTC or other resources.

Figure 1: Measurement reporting delay for UEIBM

Components of the measurement reporting delay for UEIBM are –

- Measurement period

- Time to PUCCH

Components of measurement reporting delay for UEIBM are measurement period, time to PUCCH.

Use L1-RSRP measurement period for SSB and CSI-RS as starting point for measurement period for current beam and new beam.

The measurement period for UEIBM would be max of measurement period for current beam and new beam – max{ TL1-RSRP_currBeam , TL1-RSRP_newBeam }

In FR1 and FR2 when current beam and new beam are not adjacent or overlapping:

TL1-RSRP_currBeam = Mc*N*Pc*TcurrBeam

TL1-RSRP_newBeam = MN*N*PN*TnewBeam

In FR2 when current beam and new beam are adjacent or overlapping:

TL1-RSRP_currBeam = Mc*N*2*Pc*TcurrBeam

TL1-RSRP_newBeam = MN*N*2*PN*TnewBeam

Where M is the number of samples =1 if  timeRestrictionForChannelMeasurement is configured, 3 otherwise

N: RX beam sweeping factor in FR2

P: Resource sharing factor for the beam (SSB-intra-cell, SSB-inter-cell, CSI-RS)

TcurrBeam , TnewBeam periodicity of the current and new beams (both are equal)

Measurement period for UEIBM is defined as max{ TL1-RSRP_currBeam , TL1-RSRP_newBeam }
In FR1/ FR2 with resources not adjacent or overlapping: max{ Mc*N*Pc*TcurrBeam , MN*N*PN*TnewBeam }
In FR2 with resources adjacent or overlapping: max{ Mc*N*2*Pc*TcurrBeam , MN*N*2*PN*TnewBeam }

The time to PUCCH is time to first PUCCH after measurement delay or TPUCCH – periodicity of PUCCH resource for UEIBM.

Time to PUCCH can be defined as time to first PUCCH resource or periodicity of PUCCH resource for UEIBM - Tfirst-PUCCH or TPUCCH.

Measurement reporting delay for UEIBM is defined as max{ TL1-RSRP_currBeam , TL1-RSRP_newBeam } + Tfirst-PUCCH or TPUCCH.

Issue 1-7: CSI-RS based L1-RSRP for inter-cell BM

<Way forward >

- Option 1: (Qualcomm, Nokia, Samsung, MediaTek)

  - RAN4 should not introduce in Rel-19 CSI-RS based L1-RSRP measurements for inter-cell beam management as long as RAN1 has not identified a clear use case.

- Option 2: (CMCC)

  - For reporting delay for CSI-RS based UE-initiated/event-driven beam reporting, it is proposed to define L1 measurement reporting delay requirements, and FFS whether legacy CSI-RS based L1 measurement delay requirements can be reused.

- Option 3: (Apple, Huawei)

  - For UE perspective, for CSI-RS based BM, there is no differentiation for intra-cell and inter-cell. CSI-RS based L1-RSRP measurement requirements applies to both intra-cell and inter-cell without differentiation.

- Option 4: (ZTE)

  - RAN4 needs to identify whether additional clarifications on the following points are necessary:

    - Known condition for the NSC, especially for the time offset assumption between serving cell and NSC;

    - The prerequisite on L3 measurement on NSC.

    - RAN4 to use principles of SSB based L1-RSRP for inter-cell CSI-RS based L1-RSRP measurements.

- Option 5: (Ericsson)

  - RAN4 to define CSI-RS based L1-RSRP measurements requirements for inter-cell mTRP scenario and inter-cell beam management in FR2

  - RAN4 to use principles of sharing agreed for SSB based L1-RSRP for inter-cell CSI-RS based L1-RSRP measurements

The CSI_RS resources configured for new beam measurement cannot be identified as from serving cell or cell with different PCI. For event triggered reporting based on CSI-RS, the requirements are defined for CSI-RS without specifying inter-cell or intra-cell. The measurement delay can be based on the legacy CSI-RS based L1-RSRP measurement as proposed in proposal #1.

- CSI-RS based new beam measurement cannot be distinguished between inter-cell and intra-cell.

For UE perspective, for CSI-RS based BM, there is no differentiation for intra-cell and inter-cell. CSI-RS based L1-RSRP measurement requirements applies to both intra-cell and inter-cell without differentiation.

Issue 1-8: Impact of beam on known/unknown conditions for TCI states for TCI states switching based on UEIBM

<Way forward >

- Option 1: (Qualcomm, Nokia, Samsung, Apple, Huawei)

  - Reuse known/unknown conditions for Rel-17 unified TCI states when defining requirements for TCI-switching based on UEIBM

- Option 2: (vivo)

  - Regarding the impact to know conditions of TCI state activation from event-triggered L1 reporting, RAN4 waits further RAN1/2 conclusions.

- Option 3: (ZTE)

  - Need to define the condition of known cell for UEIBM. The definition in ICBM can be starting point.

The TCI state known/unknown condition is relevant only when TCI state switch is activated or indicated for PDCCH/ PDSCH ; PUSCH/PUCCH/SRS in case of joint TCI state. When the UE receives TCI state switch (MAC-CE or DCI based) to a TCI state, the known/unknown condition would be determined by the same criteria as legacy. The L1-RSRP report should have been received within 1280 ms of the TCI state switch command, and it doesn’t matter if the L1-RSRP report was based on UE triggered beam report or legacy measurement report.

- No impact of UE triggered beam report to known/unknown conditions for TCI state.

- The known/unknown condition would be determined by the same criteria as legacy.

Reuse known/unknown conditions for Rel-17 unified TCI states when defining requirements for TCI-switching based on UEIBM

Issue 1-9: Whether to specify new unified TCI switching delay requirements?

<Way forward >

- Option 1: (Qualcomm, Nokia, Samsung (For known conditions for TCI states), Apple, ZTE, Ericsson)

  - The existing requirements for TCI state switch delay should be reused for UE-initiated / event-triggered beam management.

- Option 2: (Samsung)

  - No unknown case is applied if the TCI states switching is based on UEIBM.

- Option 3: (ZTE)

  - Need to decide whether to define L1-RSRP requirements for unknown cell

After the UE sends event triggered beam report, the network could send a TCI state switch command to the new beam. The existing TCI state switching delay requirements for unified TCI state or legacy DL active TCI state would still be appliable.

The existing requirements for TCI state switch delay should be reused for UE-initiated / event-triggered beam management.

Issue 1-10: Whether to enhance beam sweeping in event triggered L1 beam reporting?

<Way forward >

- Option 1: (Nokia)

  - Specify RAN4 requirements for a UE supporting Rel-18 multi-Rx fast beam sweeping and configured with UEIBM reporting.

- Option 2: (Xiaomi, Samsung, Apple, Ericsson, MediaTek)

  - Reuse current beam sweeping factor for UE/event triggered beam reporting

Faster Tx and Rx beam refinement is not in the scope of this WI. Keeping in mind the objectives of this WI, RAN4 shall define requirements associated with UE/ event triggered beam reporting with legacy assumptions for beam sweeping.

- Faster RX, TX beam refinement is not in scope of this WI.

Reuse current beam sweeping factor for UE/event triggered beam reporting.

Issue 1-11: Capabilities for Support of Event Triggering and Reporting Criteria?

<Way forward >

- Option 1: (Nokia)

  - RAN4 to define UE capabilities in the context of UE initiated beam reporting towards the later part of the work item.

- Option 2: (Apple)

  - Capabilities shall be defined by RAN1, to be discussed later if any RAN4 specific UE capabilities need to be added.

- Option 3: (Ericsson)

  - Define UE capability for total number of events supported for UE-initiated beam management across single and multiple TRP.

- Option 4: (MediaTek)

  - Define a capability for supported number of UE-initiated/event-driven beam reporting for intra-frequency.

For RAN1 led WIs and features developed RAN1 would typically define associated capabilities. It would be premature to define any UE capabilities related to supported number of UE-initiated/event-driven beam reporting in RAN4.

Capabilities shall be defined by RAN1, to be discussed later if any RAN4 specific UE capabilities need to be added.

Issue 1-12: Compatibility of capabilities with LTM

<Way forward >

- Option 1: (Nokia)

  - RAN4 to define UE capabilities in the context of UE initiated beam reporting towards the later part of the work item.

- Option 2: (Samsung, Apple)

  - Do not combine capabilities across different L1 measurements.

- Option 3: (Ericsson)

  - RAN4 to wait for further progress in RAN1/2 for the UE capability discussions and compatibility between MIMO and mobility.

- Option 4: (MediaTek)

  - Events configured for mobility and MIMO should be considered as different events even when they are configured for the same beams and the same cell with the same threshold.

  - Define the single capability for total number of L1 events shared by mobility L1 events and MIMO L1 events on intra-frequency.

In parallel WI in R19 for L1 based mobility, UE initiated beam reporting for neighbor cell is being discussed. It is very early in the WI stage to discuss UE capabilities. The RRM requirements for the respective WIs should proceed independently and while UE features are discussed, revisit discussion on compatibility of capabilities with LTM.

Discuss later in the WI on compatibility of capabilities with LTM.

Issue 1-13: Whether to define new accuracy requirements by Enhancement for UE-initiated/event-driven beam management?

<Way forward >

- Option 1: (Qualcomm, Nokia, Xiaomi, Apple, MediaTek)

  - The existing accuracy requirements for L1 measurements can be applied. Not to define new accuracy requirements.

- Option 2: (Samsung)

  - Focus on core requirements firstly, FFS on accuracy requirements.

- Option 3: (CMCC)

  - For SSB based beam report, Rel-18 LTM accuracy requirements can be used as baseline.

  - For CSI-RS based beam report, FFS whether to use legacy accuracy requirements which only cover serving cell, or to define new accuracy requirements taking neighbour cell measurement into account.

- Option 4: (ZTE)

  - Discuss the accuracy for the two trigger-event solutions respectively.

    - For the basic trigger-event detection, tight the existing L1-RSRP accuracy requirement;

    - For the optional trigger-event detection, resue the existing L1-RSRP accuracy requirement.

The L1-RSRP measurement is the same as legacy whether it is for beam report configured or based on UE/event triggered beam report. With the assumption that same one-shot measurement for RSRP is employed for new beams as legacy L1-RSRP measurement, the accuracy requirements should remain the same. Hence there is no need to define new accuracy requirements for L1-RSRP measurements for UEIBM.

Legacy accuracy requirements for L1-RSRP shall be applicable for UEIBM.  No new accuracy requirements are necessary.

# 3. Conclusion

In this paper, we provide our views on impact to RRM requirements from UE initiated beam management. Our observations and proposals are captured below:

- Time window based even triggering is not basic feature and subject to UE capability.

- It is not feasible to define core requirement for reporting delay with time window based triggering

- Measurement reporting delay requirement for UE initiated beam report is defined without time window based evaluation.

Reporting Delay Requirements

RAN4 only define requirements for same periodicity of new beam and current beam, until further agreement on support of different periodicity in RAN1

Components of measurement reporting delay for UEIBM are measurement period, time to PUCCH.

Use L1-RSRP measurement period for SSB and CSI-RS as starting point for measurement period for current beam and new beam.

Measurement period for UEIBM is defined as max{ TL1-RSRP_currBeam , TL1-RSRP_newBeam }
In FR1/ FR2 with resources not adjacent or overlapping: max{ Mc*N*Pc*TcurrBeam , MN*N*PN*TnewBeam }
In FR2 with resources adjacent or overlapping: max{ Mc*N*2*Pc*TcurrBeam , MN*N*2*PN*TnewBeam }

Time to PUCCH can be defined as time to first PUCCH resource or periodicity of PUCCH resource for UEIBM - Tfirst-PUCCH or TPUCCH.

Measurement reporting delay for UEIBM is defined as max{ TL1-RSRP_currBeam , TL1-RSRP_newBeam } + Tfirst-PUCCH or TPUCCH.

CSI-RS Based measurements

- CSI-RS based new beam measurement cannot be distinguished between inter-cell and intra-cell.

For UE perspective, for CSI-RS based BM, there is no differentiation for intra-cell and inter-cell. CSI-RS based L1-RSRP measurement requirements applies to both intra-cell and inter-cell without differentiation.

TCI State/ Known condition

- No impact of UE triggered beam report to known/unknown conditions for TCI state.

- The known/unknown condition would be determined by the same criteria as legacy.

Reuse known/unknown conditions for Rel-17 unified TCI states when defining requirements for TCI-switching based on UEIBM

The existing requirements for TCI state switch delay should be reused for UE-initiated / event-triggered beam management.

Enhancements to RX beam sweeping

- Faster RX, TX beam refinement is not in scope of this WI.

Reuse current beam sweeping factor for UE/event triggered beam reporting.

Capabilities

Capabilities shall be defined by RAN1, to be discussed later if any RAN4 specific UE capabilities need to be added.

Discuss later in the WI on compatibility of capabilities with LTM.

Accuracy Requirements

Legacy accuracy requirements for L1-RSRP shall be applicable for UEIBM.  No new accuracy requirements are necessary.

# Reference

- R4-2420096, “WF on RRM requirements for NR_MIMO_Ph5_Part1”, Samsung.

- R1-2410914, “LS to RAN4 on event-instance evaluation periodicity for UE-initiated/event-driven beam management”, RAN1