# 3GPP TSG-RAN WG4 Meeting #114	R4-2500274

Athens, GR, 17th – 21st February, 2025

Agenda Item:	7.19.3

Source: 	OPPO

Title: 	RRM core requirement for beam management

Document for:	Approval

- Discussion

In 3GPP RAN #102[1], for A/ML based beam management, it is agreed to provide specification support for the core requirements, especially for AI/ML LCM procedures e.g., for performance monitoring. Besides, specify performance requirements and test cases for AI/ML LCM procedures (including performance monitoring) and UE features enabled by UE-sided models are also agreed in the AI/ML WID, including specify necessary performance requirements and tests (including metrics), and specify necessary test cases and performance requirements for LCM procedure, including performance monitoring.

4.1 Objective of SI or Core part WI or Testing part WI Provide specification support for the following aspects: … - Beam management - DL Tx beam prediction for both UE-sided model and NW-sided model, encompassing [RAN1/RAN2]: … - Core requirements for the above two use cases for AI/ML LCM procedures and UE features [RAN4]: - Specify necessary RAN4 core requirements for the above two use cases. - Specify necessary RAN4 core requirements for LCM procedures including performance monitoring. 4.2 Objective of Performance part WI - For Beam Management and Positioning Accuracy enhancement use cases, specify performance requirements and test cases for AI/ML LCM procedures (including performance monitoring) and UE features enabled by UE-sided models - Specify necessary performance requirements and tests (including metrics) for the above-mentioned use cases - Specify necessary test cases and performance requirements for LCM procedure, including performance monitoring.

In this contribution, we will continue to discuss the RRM core requirements for beam management.

- Discussion

KPI:

In RAN1#116 meeting, 4 options regarding the content in the BM report of inference results have been agreed, as listed below,

Agreement For UE-sided model, at least for BM-Case1, for content in the report of inference results, support - Opt 1: Beam information on predicted Top K beam(s) among a set of beams - Opt 2: Beam information on predicted Top K beam(s) among a set of beams and RSRP of predicted Top K beam(s) among a set of beams - At least K=1 and more, FFS on max value - FFS on beam information - FFS on the definition of predicted Top K beam(s) - FFS on definition of reported RSRP when applicable - FFS on other information in the report with potential down selection among the following options - Opt 3: Beam information on predicted Top K beam(s) among a set of beams and probability information of predicted Top K beam(s) among a set of beams - FFS on the quantization method of probability information - Probability information is the probability of the beam to be the Top 1 or Top K beam - Opt 4: Beam information on predicted Top K beam(s) among a set of beams, RSRP of predicted Top K beam(s) among a set of beams, and confidence information of the RSRP - FFS on definition of reported RSRP - FFS on the definition and quantization method of confidence information - Other options are not precluded. where the set of beams is Set A, i.e., the beams for UE prediction.

Based on the agreements, metrics for BM model tests may depend on RAN1’s final decision, e.g.,

- if Beam information on predicted Top K beam(s) among a set of beams is the only reported content, then Beam prediction accuracy could be a test KPI accordingly.

- if both Beam information on predicted Top K beam(s) among a set of beams and RSRP of predicted Top K beam(s) among a set of beams are reported by the UE, then both Beam prediction accuracy and RSRP accuracy should be considered when defining RAN4 test KPIs

Proposal 1: Regarding the BM tests, the selection of metrics depends on the progress in RAN1, e.g.,

- if Beam information on predicted Top K beam(s) among a set of beams is the only reported content, then Beam prediction accuracy could be an test KPI accordingly

- if both Beam information on predicted Top K beam(s) among a set of beams and RSRP of predicted Top K beam(s) among a set of beams are reported by the UE, then both Beam prediction accuracy and RSRP accuracy should be considered when defining RAN4 test KPIs

In RAN4 R19 tests, for both BM-Case1 and BM-Case2, three options have been agreed in RAN4 R18, i.e.,

Option 1: RSRP accuracy

Option 2: Beam prediction accuracy

Top-1 (%) : the percentage of “the Top-1 strongest beam is Top-1 predicted beam”

Top-K/1 (%) : the percentage of “the Top-1 strongest beam is one of the Top-K predicted beams”

Top-1/K (%) : the percentage of “the Top-1 predicted beam is one of the Top-K strongest beams”

Option 3: The successful rate for the correct prediction which is considered as maximum RSRP among 		top-K predicted beams is larger than the RSRP of the strongest beam – x dB,

For option1, different companies still have different understandings regarding the definition of RSRP accuracy.

According to R18 TR 38.843, “Average L1-RSRP difference of Top-1 predicted beam” is utilized as the beam prediction accuracy related KPI in RAN1, and defined as “the difference between the ideal L1-RSRP of Top-1 predicted beam and the ideal L1-RSRP of the Top-1 genie-aided beam”.

Also in R18 TR 38.843, we could find another related definition that “for AI/ML models, which provide L1-RSRP as the model output, the accuracy of predicted L1-RSRP is to be evaluated. Companies optionally report average (absolute value)/CDF of the predicted L1-RSRP difference, where the predicted L1-RSRP difference is defined as the difference between the predicted L1-RSRP of Top-1 predicted beam and the ideal L1-RSRP of the same beam”.

In RAN4#112bis meeting, RAN4 agreed “If the absolute RSRP is agreed to be metric for the beam management prediction, the absolute RSRP accuracy for AI/ML based beam prediction = predicted L1-RSRP of beam index i – groundtruth of L1-RSRP of beam index i. The index i may be any beam index [in top-K beams based on groundtruth L1-RSRP]”.

However, from our understanding, the definition of absolute RSRP accuracy may not truly reflect the effectiveness of beam prediction in some cases. For example, if beam i is not the best choice, yet an AI/ML model designates it as the optimal beam. Despite achieving a seemingly perfect absolute RSRP accuracy score by accurately predicting the RSRP of beam i, this could lead to sub-optimal outcomes that deviate from expectations.

Option 3 may circumvent the aforementioned issues. For instance, as long as the quality of the predicted top-K beams is not significantly worse than that of the strongest beam, it can ensure that the selected beam functions properly. In the textual description, we have added some modification to Option 3, as stated below:

-  Option 3(revised):  The successful rate for the correct prediction which is considered as maximum RSRP ground truth L1-RSRP among top-K predicted beams is larger than the RSRP ground truth L1-RSRP of the strongest beam – x dB,

Observation 1: Absolute RSRP accuracy could be utilized as metric for L1-RSRP reporting.

Observation 2: At least for beam ID + L1-RSRP reporting, absolute RSRP accuracy may not truly reflect the effectiveness of beam prediction in some cases.

- For example, if beam i is not the best choice, yet an AI/ML model designates it as the optimal beam. Despite achieving a seemingly perfect absolute RSRP accuracy score by accurately predicting the RSRP of beam i, this could lead to sub-optimal outcomes that deviate from expectations.

Proposal 2: Revised option3 could be utilized as BM performance metric for beam ID + L1-RSRP reporting.

- Option 3(revised):  The successful rate for the correct prediction which is considered as maximum ground truth L1-RSRP among top-K predicted beams is larger than the ground truth L1-RSRP of the strongest beam – x dB. FFS the value of x.

Performance monitoring:

UE can monitor and estimate the performance of AI/ML based BM model/functionality through identified KPIs according to the discussion and conclusions in RAN1. Besides, stability of the performance monitoring and decision-making mechanism should be considered to mitigate the impact of random effects on monitoring outcomes, includes: (1) obtaining a consistent monitoring result by considering multiple evaluating samples within an evaluation window, (2) assessing whether model monitoring should be handled at the UE level or the cell level.

Proposal 3: Stability of performance monitoring and decision-making mechanism should be considered to mitigate the impact of random effects on monitoring outcomes.

Impact of measurement error on BM performance evaluation:

A simulation assumption for beam prediction was agreed as a starting point for the evaluation in RAN4#112bis meeting. Currently, the assumption is to create a measurement error modeling through link-level simulations. And then hope to apply this link-level measurement error model to system-level simulation assumptions, ultimately completing the evaluation of the impact of measurement error on BM performance requirements.

The current discussion in RAN4 and preliminary simulation assumptions that made in precious meetings are beneficial to the progress of research, but still some issues need further clarification and discussion. For example, 1) considering that the testing environment assumes TDL or CDL channel modeling, it needs to be clarified whether the impact of measurement error obtained from the SLS UMA channel (measuring error impact added from a CDL channel evaluation) can be directly applied to the final OTA testing environment. Besides, 2) differences in implementation (e.g., BS antenna configurations, BS antenna element radiation pattern, Beamforming characteristic of the BS pattern) among companies may result in inconsistent evaluation results. How to align the evaluation needs to be considered.

Observation 3: Regarding the impact of measurement error on BM performance evaluation, the following aspects should be further considered

- Whether the impact of measurement error obtained from the SLS UMA channel (measuring error added from a CDL channel evaluation) can be directly applied to the OTA testing environment

- Differences in implementation (e.g., BS antenna configurations, BS antenna element radiation pattern, Beamforming characteristic of the BS pattern) among companies may result in inconsistent evaluation results

- Conclusions

In this contribution, we have discussed the RRM core requirements for beam management and got the following proposals:

Proposal 1: Regarding the BM tests, the selection of metrics depends on the progress in RAN1, e.g.,

- if Beam information on predicted Top K beam(s) among a set of beams is the only reported content, then Beam prediction accuracy could be an test KPI accordingly

- if both Beam information on predicted Top K beam(s) among a set of beams and RSRP of predicted Top K beam(s) among a set of beams are reported by the UE, then both Beam prediction accuracy and RSRP accuracy should be considered when defining RAN4 test KPIs

Observation 1: Absolute RSRP accuracy could be utilized as metric for L1-RSRP reporting.

Observation 2: At least for beam ID + L1-RSRP reporting, absolute RSRP accuracy may not truly reflect the effectiveness of beam prediction in some cases.

- For example, if beam i is not the best choice, yet an AI/ML model designates it as the optimal beam. Despite achieving a seemingly perfect absolute RSRP accuracy score by accurately predicting the RSRP of beam i, this could lead to sub-optimal outcomes that deviate from expectations.

Proposal 2: Revised option3 could be utilized as BM performance metric for beam ID + L1-RSRP reporting.

- Option 3(revised):  The successful rate for the correct prediction which is considered as maximum ground truth L1-RSRP among top-K predicted beams is larger than the ground truth L1-RSRP of the strongest beam – x dB. FFS the value of x.

Proposal 3: Stability of performance monitoring and decision-making mechanism should be considered to mitigate the impact of random effects on monitoring outcomes.

Observation 3: Regarding the impact of measurement error on BM performance evaluation, the following aspects should be further considered

- Whether the impact of measurement error obtained from the SLS UMA channel (measuring error added from a CDL channel evaluation) can be directly applied to the OTA testing environment

- Differences in implementation (e.g., BS antenna configurations, BS antenna element radiation pattern, Beamforming characteristic of the BS pattern) among companies may result in inconsistent evaluation results

- References

- RP-234039, New WID on Artificial Intelligence (AI)/Machine Learning (ML) for NR Air Interface