3GPP TSG-RAN WG4 Meeting #114	R4-2500283

Athens, Greece,  17th February – 21st February 2025

Agenda item:	4.2.1

Source:	Apple

Title:	(NR_newRAT-Core) On NS_05 reevaluation

WI/SI:	NR_newRAT-Core

Release:	Rel-15

Document for:	Approval

# 1	Introduction

The WF [1] captures the state

# 2  Discussion

##### 2.1 Simulation assumptions

The following assumptions and requirements for the simulations are applied:

- Power Class 3 & 2

- NR Waveform

- Calibration: 1dB MPR: DFT-s-OFDM QPSK 20MHz, 100RB0

- Carrier Leakage: 28dBc

- Image: 28dBc

- CIM3: 60dBc

- CIM5: 70dBc

Emission requirements:

- BW = 5MHz

    - 1884.5-1910.0 MHz: -30 dBm/MHz

    - 1910.0-1915.7 MHz: -25 dBm/MHz

- BW = 10/15/20MHz

    - 1884.5-1906.6 MHz: -30 dBm/MHz

    - 1906.6-1915.7 MHz: -25 dBm/MHz

The carrier arrangements from WF [1] have been analyzed:

##### 2.2 NS_05 simulation results for PC3

In this section the simulation results for DFT-s-OFDM and CP-OFDM with QPSK modulation are provided. The results cover 5MHz, 10MHz, 15MHz and 20MHz channels at various positions.

| CBW in MHz | DFT-s-OFDM | CP-OFDM |
| --- | --- | --- |
| 5 |  |  |
|  | Fc = 1927.5MHz does not require A-MPR | Fc = 1927.5MHz does not require A-MPR |
| 10 |  |  |
|  | Fc = 1930MHz does not require A-MPR |  |
|  | Fc = 1935MHz does not require A-MPR | Fc = 1935MHz does not require A-MPR |
| 15 |  |  |
| 20 |  |  |

The simulation results suggest minor adjustments to starting point from WF [1]:

Table 1: Proposed RB regions for PC3

| NS_05 | Fc MHz | Region A |  | Region B |  |
| --- | --- | --- | --- | --- | --- |
|  |  | LCRB | RBstart | LCRB | RBstart |
| 5M |  | N/A | N/A | N/A | N/A |
| 10M | < 1930.7 | ≥ 1.35*RBstart + 4.32MHz/12/scs | >=0 | <= 2.16MHz/12/scs | <= 1.8MHz/12/scs |
| 10M |  | N/A | N/A | N/A | N/A |
| 15M | 1927.5≦fc＜1932.5 | >= RBstart*1.20 + 4.68MHz/12/scs | >=0 | <= 3.96MHz/12/scs | <= 3.6MHz/12/scs |
| 15M | 1932.5≦fc＜1938.2 | ≥ 10.80/12/scs | ≤ 0.54/12/scs | <= 2.16MHz/12/scs | <= 2.16MHz/12/scs |
| 20M | < 1945.7 | >= RBstart*1.35 + 3.42MHz/12/scs; | >=0 | <= 5.76MHz/12/scs; <= RBstart*1.35 + 3.42MHz/12/scs; | <= 5.04MHz/12/scs |

Further adjustments to the RB regions could be made for 20MHz CBW. The A1 RB region of the starting point cuts into the region A2 which means that certain RB allocations get unnecessary high A-MPR allowance. A small fix can solve this issue:

| Starting Point | Adjusted RB region |
| --- | --- |

The fix can be implemented as shown below. As a small bonus the region definition looks a bit simpler.

| NS_05 | Fc MHz | Region A1 |  | Region A2 |  |
| --- | --- | --- | --- | --- | --- |
|  |  | LCRB | RBstart | LCRB | Rbstart |
| 20M | < 1945.7 | >= Rbstart*1.35 + 3.42MHz/12/scs; > 5.76MHz/12/scs; | >=0 | <= 5.76MHz/12/scs; | <= 5.04MHz/12/scs |

Finally, we would like to propose to use the table format which is more often utilised nowadays.

Table 2: A-MPR for Rel-19 NS_05

| Channel Bandwidth, MHz | Carrier Center Frequency, Fc, MHz | Regions |  | A-MPR |
| --- | --- | --- | --- | --- |
|  |  | RBstart*12*SCS MHz | LCRB*12*SCS MHz |  |
| 10 MHz | < 1930.7 | >=0 | ≥ 1.35* RBstart+ 4.32 | A1 |
|  |  | <= 1.8 | <= 2.16 | A2 |
| 15 MHz | 1927.5≦fc＜1932.5 | >=0 | >= RBstart*1.20 + 4.68 | A3 |
|  |  | <= 3.6 | <= 3.96 | A4 |
|  | 1932.5≦fc＜1938.2 | ≤ 0.54 | ≥ 10.80 | A5 |
|  |  | <= 2.16 | <= 2.16 | A5 |
| 20MHz | < 1945.7 | >=0 | >= RBstart*1.35 + 3.42 > 5.76 | A6 |
|  |  | <= 5.04 | <= 5.76 | A7 |

Additional to the simulations we also conducted measurements to check certain edge cases. Comparing our results with the starting point we found that the proposed A-MPR is acceptable. Restructuring the values into the currently used table format we obtain Table 3.

Table 3: A-MPR for Rel-19 NS_05

| Modulation/Waveform |  | A1 | A2 | A3 | A4 | A5 | A6 | A7 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Outer/ Inner | Outer/ Inner | Outer/ Inner | Outer/ Inner | Outer/ Inner | Outer/ Inner | Outer/ Inner |
| DFT-s-OFDM | PI/2 BPSK | 4 | 3 | 4.5 | 3 | 3 | 5 | 3.5 |
|  | QPSK | 5 | 4 | 5.5 | 4 | 3.5 | 5.5 | 4 |
|  | 16 QAM | 5 | 4 | 6 | 4.5 | 4 | 6 | 4.5 |
|  | 64 QAM | 5.5 | 4.5 | 6 | 5 | 4 | 6 | 5 |
|  | 256 QAM | 5.5 | 4.5 | 6 | 5 | 4 | 6.5 | 5 |
| CP-OFDM | QPSK | 6 | 5 | 7 | 5.5 | 5 | 7.5 | 6 |
|  | 16 QAM | 6 | 5 | 7 | 5.5 | 5 | 7.5 | 6 |
|  | 64 QAM | 6 | 5 | 7 | 5.5 | 5 | 7.5 | 6 |
|  | 256 QAM | 6 | 5 | 7 | 5.5 | 5 | 7.5 | 6 |

Proposal 1: Use Table 2 and Table 3 to specify the Rel-19 RB regions and the Rel-19 A-MPR for NS_05 for PC3

##### 2.3 On NS_05U

The network signalling NS_05U includes the UTRA protection as an additional emission requirement. The UTRA protection is specified via NS_100 and only specifies A-MPR for outer allocations. Given the RB allocation shapes of NS_05 it might be challenging to introduce additional RB regions which only occupy the outer RB allocations which are not covered by the other RB regions.

It is therefore proposed to copy the NS_100 and place its A-MPR into the A-MPR table as A8. This is shown in table 4. The final A-MPR for outer RB allocations shall be calculated as max(MPR, A-MPR, A8).

Table 3: A-MPR for Rel-19 NS_05U

| Modulation/Waveform |  | A1 | A2 | A3 | A4 | A5 | A6 | A7 | A8 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Outer/ Inner | Outer/ Inner | Outer/ Inner | Outer/ Inner | Outer/ Inner | Outer/ Inner | Outer/ Inner | Outer |
| DFT-s-OFDM | PI/2 BPSK | 4 | 3 | 4.5 | 3 | 3 | 5 | 3.5 | 2 |
|  | QPSK | 5 | 4 | 5.5 | 4 | 3.5 | 5.5 | 4 | 2 |
|  | 16 QAM | 5 | 4 | 6 | 4.5 | 4 | 6 | 4.5 | 2.5 |
|  | 64 QAM | 5.5 | 4.5 | 6 | 5 | 4 | 6 | 5 | ≤3 |
|  | 256 QAM | 5.5 | 4.5 | 6 | 5 | 4 | 6.5 | 5 | 4.5 |
| CP-OFDM | QPSK | 6 | 5 | 7 | 5.5 | 5 | 7.5 | 6 | 4 |
|  | 16 QAM | 6 | 5 | 7 | 5.5 | 5 | 7.5 | 6 | 4 |
|  | 64 QAM | 6 | 5 | 7 | 5.5 | 5 | 7.5 | 6 | 4 |
|  | 256 QAM | 6 | 5 | 7 | 5.5 | 5 | 7.5 | 6 | 6.5 |

Proposal 2: For NS_05U it is proposed to copy the NS_100 and place its A-MPR into the A-MPR table as A8. This is show in table 4. The final A-MPR for outer RB allocations shall be calculated as max(MPR, A-MPR, A8).

##### 2.4 NS_05 simulation results for PC2

In this section the simulation results for DFT-s-OFDM and CP-OFDM with QPSK modulation are provided. The results cover 5MHz, 10MHz, 15MHz and 20MHz channels at various positions.

| CBW in MHz | DFT-s-OFDM | CP-OFDM |
| --- | --- | --- |
| 5 |  |  |
|  | Fc = 1927.5MHz does not require A-MPR | Fc = 1927.5MHz does not require A-MPR |
| 10 |  |  |
|  | Fc = 1935MHz does not require A-MPR | Fc = 1935MHz does not require A-MPR |
| 15 |  |  |
| 20 |  |  |

PC2 RB regions require modifications due to the higher PC2 output power compared to PC3. As PC2 seems to be second priority it should be considered to focus the discussion on PC3 and continue PC2 in the next meeting.

Proposal 3: PC2 RB regions require modifications due to the higher PC2 output power compared to PC3. As PC2 seems to be second priority it should be considered to focus the discussion on PC3 and continue PC2 in the next meeting.

# 4	Conclusions

This contribution provides simulation results and proposals for NS_05 and NS_05U. The following proposals are made:

Proposal 1: Use Table 2 and Table 3 to specify the Rel-19 RB regions and the Rel-19 A-MPR for NS_05 for PC3

Table 2: A-MPR for Rel-19 NS_05

| Channel Bandwidth, MHz | Carrier Center Frequency, Fc, MHz | Regions |  | A-MPR |
| --- | --- | --- | --- | --- |
|  |  | RBstart*12*SCS MHz | LCRB*12*SCS MHz |  |
| 10 MHz | < 1930.7 | >=0 | ≥ 1.35* RBstart+ 4.32 | A1 |
|  |  | <= 1.8 | <= 2.16 | A2 |
| 15 MHz | 1927.5≦fc＜1932.5 | >=0 | >= RBstart*1.20 + 4.68 | A3 |
|  |  | <= 3.6 | <= 3.96 | A4 |
|  | 1932.5≦fc＜1938.2 | ≤ 0.54 | ≥ 10.80 | A5 |
|  |  | <= 2.16 | <= 2.16 | A5 |
| 20MHz | < 1945.7 | >=0 | >= RBstart*1.35 + 3.42 > 5.76 | A6 |
|  |  | <= 5.04 | <= 5.76 | A7 |

Table 3: A-MPR for Rel-19 NS_05

| Modulation/Waveform |  | A1 | A2 | A3 | A4 | A5 | A6 | A7 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Outer/ Inner | Outer/ Inner | Outer/ Inner | Outer/ Inner | Outer/ Inner | Outer/ Inner | Outer/ Inner |
| DFT-s-OFDM | PI/2 BPSK | 4 | 3 | 4.5 | 3 | 3 | 5 | 3.5 |
|  | QPSK | 5 | 4 | 5.5 | 4 | 3.5 | 5.5 | 4 |
|  | 16 QAM | 5 | 4 | 6 | 4.5 | 4 | 6 | 4.5 |
|  | 64 QAM | 5.5 | 4.5 | 6 | 5 | 4 | 6 | 5 |
|  | 256 QAM | 5.5 | 4.5 | 6 | 5 | 4 | 6.5 | 5 |
| CP-OFDM | QPSK | 6 | 5 | 7 | 5.5 | 5 | 7.5 | 6 |
|  | 16 QAM | 6 | 5 | 7 | 5.5 | 5 | 7.5 | 6 |
|  | 64 QAM | 6 | 5 | 7 | 5.5 | 5 | 7.5 | 6 |
|  | 256 QAM | 6 | 5 | 7 | 5.5 | 5 | 7.5 | 6 |

Proposal 2: For NS_05U it is proposed to copy the NS_100 and place its A-MPR into the A-MPR table as A8. This is show in table 4. The final A-MPR for outer RB allocations shall be calculated as max(MPR, A-MPR, A8).

Proposal 3: PC2 RB regions require modifications due to the higher PC2 output power compared to PC3. As PC2 seems to be second priority it should be considered to focus the discussion on PC3 and continue PC2 in the next meeting.

# 4	References

- R4-2420481, “WF on PHS protection update”, KDDI, OPPO, 3GPP TSG-RAN WG4 Meeting #113