3GPP RAN WG4 Meeting #114																	R4-2500292

Athens, Greece, 17th – 21st February 2025

Agenda item:	6.20.1

Source:	Apple

Title:	Emission requirements for the UE operating in 1626.5-1660.5MHz and 1668-1675MHz UL frequency ranges

WI/SI:	NR_NTN_combinedLband-Core

Release:	Rel-19

Document for:	Decision

# 1	Introduction

After the RAN#86 meeting, the Rel-17 WI was approved that aimed at enabling the 5G/NR radio access technology for non-terrestrial satellite deployments [1]. One of the WI objectives was to define and introduce the corresponding bands into the 3GPP specifications. The outcome of these discussions was two NTN bands – L-band and S-band – which were added as bands n255 and n256, respectively, to the Rel-17 core specifications. However, already at that time it was noted that there exist other satellite deployments. To limit the overall workload in Rel-17, it was decided by RAN WG4 to focus on L- and S-bands with the common understanding that other potential NTN bands should follow the "normal" process of submitting a new spectrum WI. For instance, after the RAN#98 meeting a new Rel-18 NTN spectrum WI was agreed with the objective of adding support for a new NTN band (band n254) where the UL is on the L-band and the DL is on the S-band [2].

After the RAN#104 meeting another spectrum WI was agreed to introduce new bands operating in the L-band frequency range [3]. In this discussion paper we present a short overview of the existing ETSI requirements that govern UE operation in the frequency ranges corresponding to the considered L-bands.

# 2	Regulatory requirements for L-bands

## 2.1	Background and ETSI L-band frequency arrangements

One of the key aspects for a correct introduction of a band to the 3GPP core specifications is a complete set of regulatory requirements from the regions, where the considered band is going to be used. And since satellite bands are intended to be deployed on a large geographical scale, it becomes especially crucial and important to reflect the corresponding requirements so that a band can be activated in desired geographical areas.

The satellite L-bands are regulated by a number of documents including ETSI requirements, such as ETSI EN 301441 and EN 301681. It should be noted that ETSI EN 301441 covers the case when the L-band has only UL allocation at 1.6GHz, while the DL part is in the S-band at 2.5GHz; this combination has been introduced as 3GPP NTN band n254. On the contrary to it, ETSI EN 301681 covers the case when both DL and UL are in the L-band frequency range (more precisely 1.5GHz for the DL block and 1.6GHz for the UL block).  The table below from the corresponding ETSI document presents frequency block arrangements, for which additional ETSI specific requirements exist. As can be seen from the table these frequency ranges, sub-bands 1 and 2, corresponding to the considered 3GPP NTN bands.

Table 2.1-1: ETSI EN 301681 frequency block arrangements.

## 2.2	Emission requirements for L-bands (in- and out-of-band)

Referring to sub-clause 4.2.2 in ETSI EN 301681, the following in-band emission requirements are defined for a device operating in either sub-band 1 or sub-band 2. It should be noted that there are two sets of requirements which have slightly different "shape" of the resulting emission mask. Nevertheless, these emission masks are not the same as the existing NR general mask adopted for NTN.

Figure 2.2-1: ETSI in-band emission masks for a device operating in 1626.5-1660.5MHz and 1668-1675MHz

In addition to the in-band emission requirements, ETSI EN 301681 also defines out-of-band emission requirements as presented below in Figure 2.2-2 (refer to sub-clause 4.2.1 in the ETSI EN 301681 for a complete technical description). So, while the table on the left-hand side is applicable to a device capable of operating in 1626.5-1660.5MHz frequency range, the second table is followed by a device implementing 1668-1675MHz or both sub-bands.

Figure 2.2-2: ETSI out-of-band emission requirements for a device operating in 1626.5-1660.5MHz and 1668-1675MHz

As commented earlier in the background section, the complete definition of the 3GPP band should capture the requirements from all regions where it is intended to be used. And since ETSI requirements are adopted not only in the EU/CEPT area, but also in quite many Region 1 and Region 3 countries, we suggest capturing them in the 3GPP specifications following existing procedures (i.e. sub-clauses capturing additional emission requirements). In fact, this what 3GPP already did while capturing ETSI EN requirements for the NTN band n254.

Proposal:	Capture existing in-band and out-of-band emission requirements for L-bands as defined by ETSI EN 301681.

# 3	Conclusions

In this discussion paper we have presented our technical input on the existing ETSI regulations for a device operating UL in the 1.6GHz frequency range. More precisely, there are additional ETSI in- band out-of-band emission requirements for the 1626.5-1660.5MHz and 1668-1675MHz UL blocks. Since these blocks correspond to the considered 3GPP NTN bands, we suggest capturing them accordingly.

Proposal:	Capture existing in-band and out-of-band emission requirements for L-bands as defined by ETSI EN 301681.

# 4	References

- RP-193234, "Solutions for NR to support non-terrestrial networks (NTN)", Thales

- RP-223431, "New WID on Introduction of the satellite L-/S-band", Globalstar, Apple

RP-241689, "Revised WID on New NR NTN bands to support Extended L-band and combined MSS L-band and Extended L-band ranges ", Inmarsat, Viasat