3GPP RAN WG4 Meeting #114																	R4-2500297

Athens, Greece, 17th – 21st February 2025

Agenda item:	7.10.3.1

Source:	Apple, Globalstar

Title:	Further considerations on the system related aspects of the NTN 3MHz channel

WI/SI:	NR_IoT_NTN_req_test_enh

Release:	Rel-19

Document for:	Decision

# 1	Introduction

After the RAN#86 meeting, the Rel-17 WI was approved that aimed at enabling the 5G/NR radio access technology for non-terrestrial satellite deployments [1]. One of the WI objectives was to define and introduce the corresponding bands into the 3GPP specifications. The outcome of these discussions was two NTN bands – L-band and S-band – which were added as bands n255 and n256, respectively, to the Rel-17 core specifications. And since that time the smallest channel bandwidth was 5MHz, all NTN bands have had only 5MHz as the smallest bandwidth.

After the RAN#104 meeting a new WI was agreed [2], one of the aims of which is to enable the 3MHz channel for the NTN bands. Based on the WF document from the previous meetings, it was not concluded whether the 3MHz channel should be optional or mandatory [3]. In addition to that, it should be clarified further which range of the 3MHz channel sync raster entries should be adopted.

# 2	System related aspects for the 3MHz channel

## 2.1	Mandatory versus optional 3MHz channel bandwidth

During the previous meetings RAN WG4 contemplated a lot an open issue of whether the 3MHz channel bandwidth should be optional or mandatory [4]-[10]. And if it is mandatory, then starting from which releases, for which NTN bands, etc. Firstly, our view is that a new channel bandwidth is usually always optional in the release when it is introduced, so from that perspective the 3MHz channel bandwidth is optional in Rel-19.  As for later releases, 3GPP can contemplate further whether the 3MHz channel is required to be mandatory and for which NTN bands.

Observation 1a:	Following the existing 3GPP principles, a new channel bandwidth is always optional in the release when it is introduced, i.e. the 3MHz channel bandwidth is optional for NTN in Rel-19.

Observation 1b:	The 3MHz channel bandwidth can be considered as mandatory for future releases for certain NTN bands.

Whether the 3MHz channel bandwidth should be mandatory in later releases requires further discussions based on further input from satellite operators on whether it is needed. Our general view is that is not practical to mandate the 3MHz channel bandwidth for all NTN bands (the ones we have, are going to have and will have) as it will just increase the development and testing efforts, which is maybe not even needed for some NTN bands. The benefit of supporting the 3MHz channel is to be able to support irregular spectrum blocks and to ensure lower power back-off if the NTN band has strict emission requirements. However, if neither aspect is an issue for the given NTN band, then it is not clear why the 3MHz channel would be considered.

Observation 1c:	Mandating the 3MHz channel bandwidth for all NTN bands might be counter-productive because not all NTN bands may need it.

Observation 1d:	3MHz channel bandwidth is useful for the spectrum blocks of the corresponding size and/or to ensure lower power back-off if the NTN band has strict emission requirements; otherwise, it is not clear why it would be deployed.

Furthermore, mandating the 3MHz channel for the NTN band will require a UE to implement the 3MHz sync raster search, which will impact negatively overall search time as a UE does not know a priori whether the 3MHz channel is deployed by an operator or not.  Thus, even if the 3MHz channel becomes mandatory, it should be on the per-band basis, not for all the NTN bands, to avoid negative impact on performance for other satellite operators. As an example, if the given NTN band can be used in different countries and if there is a request from operator A to support the 3MHz channel, it will impact negatively this band search time in another region, which maybe does not even intend to deploy this channel bandwidth.

Observation 1e:	Since the 3MHz sync raster entries are not the same as the 5MHz raster, mandating the 3MHz channel for all NTN bands will increase the search time irrespective of the fact whether operators will deploy it.

Observation 1f:	For global NTN bands, mandating the 3MHz channel will increase cell search time in all countries/regions, even in the ones where the 3MHz channel is not used.

Proposal 1a:	3MHz channel bandwidth for NTN is optional by default.

Proposal 1b:	3MHz channel bandwidth for NTN can be mandatory on the per-band basis (based on further discussions and input from all satellite operators using a given band).

Another open issue from the previous meeting discussions is which IE should be used to indicate support for this channel bandwidth. Our understanding is that there are no compelling reasons to introduce new signalling, i.e. the existing IE can be used for that.

Proposal 1c:	Existing IE is used to signal whether the 3MHz channel bandwidth is supported for the given NTN band.

## 2.2	Sync raster entries

When the 3MHz channel bandwidth was initial introduced for the terrestrial bands, the corresponding sync raster entries were defined for frequencies up to 1GHz. If the 3MHz channel bandwidth becomes available for the NTN bands, then our understanding is that the 3MHz sync raster entries should be defined for frequencies at least up to 2.5GHz (which corresponds to the DL block of the NTN band n254). Furthermore, if the 3MHz sync raster is going to be extended up to 2.5GHz, then 3GPP can consider extending this range already up to 3GHz making it aligned with the TS 38.101-1 specification.  Table 2.2-1 and 2.2-2 below show the sync raster extensions for the 2.5GHz and 3GHz upper bounds, respectively.

Proposal 2a:	3MHz sync raster entries are defined for frequencies at least up to 2.5GHz to cover existing NTN L- and S-bands.

Proposal 2b:	3GPP can consider defining the 3MHz sync raster entries for frequencies up to 3GHz.

Table 2.2-1: Exemplary GSCN definition for frequencies up to 2.5GHz.

| Range of frequencies (MHz) | SS block frequency position SSREF | GSCN | Range of GSCN |
| --- | --- | --- | --- |
| 0 – 2500 | N * 600 kHz + M * 50 kHz + 300 kHz, N = 1:4165, M ϵ {1,3,5} | 26638+3N + (M-3)/2 | 26640 – 39134 |

Table 2.2-2: Exemplary GSCN definition for frequencies up to 3GHz.

| Range of frequencies (MHz) | SS block frequency position SSREF | GSCN | Range of GSCN |
| --- | --- | --- | --- |
| 0 – 3000 | N * 600 kHz + M * 50 kHz + 300 kHz, N = 1:4999, M ϵ {1,3,5} | 26638+3N + (M-3)/2 | 26640 – 41636 |

# 3	Conclusions

In this discussion paper we have presented our further views on the open issues for the 3MHz channel bandwidth system parameters. As explained in our paper, our view is that 3MHz channel bandwidth should be optional in Rel-19 and remain optional by default in future releases. It can be considered as a mandatory channel bandwidth, on a per-band basis, accounting for the feedback from the satellite operators to avoid unnecessary increase in the cell search time.

Proposal 1a:	3MHz channel bandwidth for NTN is optional by default.

Proposal 1b:	3MHz channel bandwidth for NTN can be mandatory on the per-band basis (based on further discussions and input from all satellite operators using a given band).

Proposal 1c:	Existing IE is used to signal whether the 3MHz channel bandwidth is supported for the given NTN band.

Proposal 2a:	3MHz sync raster entries are defined for frequencies at least up to 2.5GHz to cover existing NTN L- and S-bands.

Proposal 2b:	3GPP can consider defining the 3MHz sync raster entries for frequencies up to 3GHz.

# 4	References

- RP-193234, "Solutions for NR to support non-terrestrial networks (NTN)", Thales

- RP-240857, "New WID: Enhanced requirements and test methodology for NR and IoT NTN"

R4-2419789, "Way Forward for [113][311] NR_IoT_NTN_less_than_5MHz_UERF", Xiaomi

- R4-2417812, "Discussion on system parameter for NTN support spectrum less than 5 MHz", CATT

- R4-2417934, "Discussion on system parameters for NTN less than 5MHz", Xiaomi

- R4-2418190, "Discussion on system parameters for less than 5MHz in FR1-NTN bands", vivo

- R4-2418256, "Discussion on less than 5 MHz NTN System/UE aspects", Nokia

- R4-2418361, "Discussion on system parameters for NTN less than 5MHz", ZTE

- R4-2418891, "Discussion on 3MHz channel bandwidth for NR-NTN", China Telecom

- R4-2419337, "System parameters for 3MHz BW in NTN", Sony