3GPP TSG-RAN WG4 Meeting #114	R4-2500310

Athens, Greece,  17th February – 21st February 2025

Agenda item:	7.9.2

Source:	Apple

Title:	On the introduction of 7MHz CBW

WI/SI:	NR_FR1_7MHz_BW-Core

Release:	Rel-19

Document for:	Discussion

# 1	Introduction

The WI [1] has the goal to introduce 7MHz channel bandwidth for n26 and n5. For the initial discussion this contribution focuses on selected objectives from the WID. Before any further analysis can be made it is important to specify the spectrum utilization. Another important objective is to determine the impact on network signaling label so that the A-MPR work can be started.

# 2  Discussion

##### 2.1 Spectrum utilisation of 7MHz CBW

Maximising spectrum utilisation is an important aspect for 5G as it allows to optimise throughput. Through the efforts made on spectrum utilisation the NR channels feature more RBs compared to LTE at 15kHz SCS. When introducing the new 7MHz NR channel it is paramount to optimise spectrum utilisation. Another factor of equal importance is the impact on output power. If spectrum utilisation is overdrawn, then considerable back-off might be needed to meet SEM requirements. It is critical to select the right amount of total RBs which leave enough guard band. The guard band should be higher than the 5MHz guard but lower than the 10MHz guard. Setting NRB to 36 maximizes spectrum utilization to around 0.93% and ensures a minimum guard band of 252.5kHz.

Proposal 1: For 7MHz CBW set the NRB to 36 for 15kHz SCS.

##### 2.2 Spectral emission mask modification

The SEM from clause 6.5.2.2 of TS 38.101-1 needs to be updated. It is proposed to add the channel to the row below 10MHz. The modification to Table 6.5.2.2-1 from TS 38.101-1 is provided on Table 1.

Table 1: Modification of SEM to introduce 7MHz channel

| ΔfOOB (MHz) | Channel bandwidth (MHz) / Spectrum emission limit (dBm) |  |  |  | Measurement bandwidth |
| --- | --- | --- | --- | --- | --- |
|  | 3 | 5 | 7, 10, 15, 20, 25, 30, 35, 40, 45 | 50, 60, 70, 80, 90, 100 |  |
| ± 0-1 | -13 | -13 | -13 |  | 1 % of channel BW |
| ± 0-1 |  |  |  | -24 | 30 kHz |
| ± 1-5 | -10 | -10 | -10 |  | 1 MHz |
| ± 5-6 | -25 | -13 |  |  |  |
| ± 6-10 |  | -25 |  |  |  |
| ± 5-BWChannel |  |  | -13 |  |  |
| ± BWChannel-(BWChannel+5) |  |  | -25 |  |  |

Proposal 2: Introduce 7MHz channel to SEM requirements as proposed in Table 1.

##### 2.2 Network signalling and A-MPR

Whenever adding a new channel to bands featuring network signalling labels an update to A-MPR is typically required. A shortened version of Table 6.2.3.1-1A from TS 38.101-1 is provided below. It has been reduced to only contain the target bands n5 and n26.

Table 2: Mapping of network signalling label

| NR band | Value of additionalSpectrumEmission |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 |
| n5 | NS_01 | NS_100 |  |  |  |  |  | Reserved |
| n26 | NS_01 | NS_100 | NS_12 | NS_13 | NS_14 | NS_15 |  | Reserved |

It can be observed that n5 features NS_100. This network signalling label is dedicated to the protection of UTRA systems. It provides A-MPR allowance for outer allocations and is bandwidth independent. Since 5MHz channel and 10MHz channel are covered by NS_100 it is safe to assume that NS_100 will be suited for 7MHz channel as well. Therefore, we do not expect any requirements work or n5 for NS_100.

Observation: Band n5 features NS_100. This network signalling label provides A-MPR allowance for outer allocations and is bandwidth independent. Since 5MHz channel and 10MHz channel are covered by NS_100 it is safe to assume that NS_100 will be suited for 7MHz channel as well. Therefore, we do not expect any requirements work or n5 for NS_100.

Band n26 features NS_100 which should not require modification as discussed above. However, it also has four other labels which require careful analysis. RAN4 needs to discuss which labels require the introduction of 7MHz channels.

Proposal 3: Discuss which network signalling labels require the introduction of 7MHz channel to enable the work on A-MPR.

# 4	Conclusions

This contribution provides initial insight and discussion points for the introduction of 7MHz channel. The following observations and proposals are made:

Proposal 1: For 7MHz CBW set the NRB to 36 for 15kHz SCS.

Proposal 2: Introduce 7MHz channel to SEM requirements as proposed in Table 1.

Observation: Band n5 features NS_100. This network signalling label provides A-MPR allowance for outer allocations and is bandwidth independent. Since 5MHz channel and 10MHz channel are covered by NS_100 it is safe to assume that NS_100 will be suited for 7MHz channel as well. Therefore, we do not expect any requirements work or n5 for NS_100.

Proposal 3: Discuss which network signalling labels require the introduction of 7MHz channel to enable the work on A-MPR.

# 4	References

- RP-243323, “New WID on 7 MHz Channel Bandwidth for n26 and n5”, China Telecom, T-Mobile USA, 3GPP TSG RAN Meeting #106