3GPP RAN WG4 Meeting #114		R4-2500324

Athens, Greece, February 17th – 21st, 2025

Agenda item:	10

Source:	Apple, TELUS, T-Mobile USA, Skyworks Solutions Inc., Telstra, BT plc, Vodafone

Title:	Motivation and proposal on UL Tx switching for FR1 intra-band NC UL CA in Rel-20

WI/SI:

Release:	Rel-20

Document for:	Information

# 1	Introduction

UL Tx switching was first introduced in 3GPP Rel-16 as a new feature to enhance inter-band EN-DC, inter-band UL CA, and SUL operations, where the switching is either between (1Tx + 1Tx) and (0Tx + 2Tx) or (1Tx + 0Tx) and (0Tx + 2Tx) for two UL carriers in two corresponding bands [1]. The feature provides the benefits of coverage improvement, UL throughput enhancement, and latency reduction. Considering that the same feature can very well be applied to FR1 intra-band non-contiguous UL CA where UE is typically limited with two Tx paths for simultaneous transmission, in this contribution, we provide the motivation on introducing the UL Tx switching feature for FR1 intra-band non-contiguous UL CA as part of the Rel-20 RAN4 objectives.

# Discussion

## 2.1	Motivation for UL Tx switching for FR1 intra-band NC UL CA

Similar to UL Tx switching for inter-band UL CA, the UL Tx switching for intra-band non-contiguous UL CA allows UE to dynamically switch between the two non-contiguous UL carriers without the RRC reconfiguration process. The switching can be based on the channel conditions of the two carriers where one carrier may have better propagation condition than the other due to frequency selective fading.

Observation 1: UL Tx switching for intra-band non-contiguous UL CA allows UE to dynamically switch between the two non-contiguous UL carriers without the RRC reconfiguration process.

On the other hand, the UL Tx switching avoids the potential high MPR/A-MPR for simultaneous non-contiguous UL CA transmission due to clustered out-of-band emissions caused by 2UL inter-modulations (IMD) as conceptually illustrated in Figure 2.1-1. As a result, the UL coverage can be improved yet without compromising UL throughput performance as compared to the non-switched UL CA which will be further elaborated below.

Figure 2.1-1 Potential high MPR for intra-band non-contiguous UL CA due to IMD emissions

Observation 2: The UL Tx switching avoids the potential high MPR/A-MPR for simultaneous non-contiguous UL CA transmission due to clustered out-of-band emissions caused by 2UL inter-modulations.

Table 2.1-1 summarizes the PC3 and PC2 MPR requirements for intra-band non-contiguous UL CA based on dualPA implementation to fulfill the general spurious emission requirement at -30 dBm/MHz. It can be seen that the required MPR is at least 9 dB which implies UL coverage range would be severely impacted.

| PC3 |  | PC2 |  | PC1.5 |  |
| --- | --- | --- | --- | --- | --- |
| B (MHz) | MPR (dB) | B (MHz) | MPR (dB) | B (MHz) | MPR (dB) |
| 0 ≤ B < 1.08 | 15.0 | 0 ≤ B < 1.44 | 15.5 | TBD | TBC |
| 1.08 ≤ B < 2.16 | 14.5 | 1.44 ≤ B < 2.88 | 15.0 | TBD | TBC |
| 2.16 ≤ B < 3.24 | 13.5 | 2.88 ≤ B < 5.76 | 14.0 | TBD | TBC |
| 3.24 ≤ B < 5.04 | 12.5 | 5.76 ≤ B < 10.8 | 12.0 | TBD | TBC |
| 5.04≤ B < 10.08 | 11.5 | 10.8 ≤ B < 23.04 | 10.5 | TBD | TBC |
| 10.08 ≤ B < 16.38 | 10.5 | 23.04 ≤ B | 9.0 | TBD | TBC |
| 16.38 ≤ B < 21.78 | 10.0 |  |  | TBD | TBC |
| 21.78 ≤ B | 9.0 |  |  | TBD | TBC |
| NOTE: B is the effective aggregated bandwidth in MHz |  |  |  |  |  |

Table 2.1-1 MPR requirements for intra-band NC UL CA with dualPA

For simultaneous intra-band non-contiguous UL CA transmission, due to the UE limitation on number of Tx path at two, only non-MIMO operation can be supported for each carrier, as is shown in Figure 2.1-2. However, with UL Tx switching, each UL carrier can leverage the two Tx paths to support UL MIMO as conceptually illustrated in Figure 2.1-3, as a result, the UL throughput performance for UL Tx switching is not compromised as compared to non-switched UL CA.

Figure 2.1-2 Only non-MIMO operation can be supported for intra-band non-contiguous UL CA

Figure 2.1-3 UL MIMO can be supported with UL Tx switching

Observation 3: UL Tx switching does not compromise UL throughput performance as compared to UL CA as each switched carrier can support UL MIMO while UL CA can only support non-MIMO operation for each carrier due to the limitation of UL Tx path for simultaneous transmission.

Another potential benefit for intra-band non-contiguous UL CA with UL Tx switching is that only one frequency synthesizer (1 LO) is required to support the operation while for simultaneous transmission, it would require two frequency synthesizers (2 LOs) to support the operation when the frequency span between the two carriers is wider than 200 MHz.

Observation 4: For intra-band non-contiguous UL CA with UL Tx switching, only one frequency synthesizer (1 LO) is required to support the operation, while for simultaneous transmission it would require two frequency synthesizers (2 LOs) to support the operation when the frequency span between the two carriers is wider than 200 MHz.

## 2.2	Rel-20 RAN4 objectives for Intra-CA UL Tx switching

As the proposed feature/objective is on a smaller scale, we can take the same approach as in the past to include the UL Tx switching feature in the Rel-20 RAN4 UE RF enhancement work item (Phase 5) with the objective as shown below,

- Introduce UL Tx switching feature for FR1 intra-band non-contiguous UL CA, focusing on the collocated scenario and 2Tx to 2Tx switching first.

  - Specify the requirement of switching period.

  - Introduce new feature/capability signaling if necessary.

# 3	Conclusion

In this contribution, we provide the motivation on introducing the UL Tx switching feature for FR1 intra-band non-contiguous UL CA as part of the Rel-20 RAN4 UR RF enhancement work item (Phase 5) objectives for consideration.

Observation 1: UL Tx switching for intra-band non-contiguous UL CA allows UE to dynamically switch between the two non-contiguous UL carriers without the RRC reconfiguration process.

Observation 2: The UL Tx switching avoids the potential high MPR/A-MPR for simultaneous non-contiguous UL CA transmission due to clustered out-of-band emissions caused by 2UL inter-modulations.

Observation 3: UL Tx switching does not compromise UL throughput performance as compared to UL CA as each switched carrier can support UL MIMO while UL CA can only support non-MIMO operation for each carrier due to the limitation of UL Tx path for simultaneous transmission.

Observation 4: For intra-band non-contiguous UL CA with UL Tx switching, only one frequency synthesizer (1 LO) is required to support the operation, while for simultaneous transmission it would require two frequency synthesizers (2 LOs) to support the operation when the frequency span between the two carriers is wider than 200 MHz.

The following Rel-20 RAN4 objectives for UL Tx switching for FR1 intra-band non-contiguous UL CA are proposed for consideration:

- Introduce UL Tx switching feature for FR1 intra-band non-contiguous UL CA.

  - Specify the requirement of switching period.

  - Introduce new feature/capability signaling if necessary.

# 4	Reference

RP-192282, "WID revision: RF requirements for NR frequency range 1 (FR1)", Huawei, HiSilicon, China Telecom, 3GPP TSG RAN Meeting #84, Newport Beach, USA, September 16th – 20th, 2019

# Appendix I: Example RAN4 CR draft

#### 6.3A.3.2	Transmit ON/OFF time mask for intra-band non-contiguous CA

##### 6.3A.3.2.1	General

For intra-band non-contiguous carrier aggregation, the general output power ON/OFF time mask specified in clause 6.3.3.1 is applicable for each component carrier during the ON power period and the transient periods. The OFF period as specified in clause 6.3.3.1 shall only be applicable for each component carrier when all the component carriers are OFF.

##### 6.3A.3.2.2	Time mask for switching between two uplink carriers

In addition to the requirements in 6.3A.3.2.1 and the maximum output power requirement specified in Table 6.2A.1.2-1 with uplink assigned to two carriers, the switching time mask specified in this clause is applicable for an intra-band non-contiguous UL CA configuration when the capability uplinkTxSwitchingPeriod is present, and is only applicable for uplink switching mechanisms specified in clause 6.1.6 of TS 38.214 [10], where both NR UL carriers are capable of two transmit antenna connectors. The UE shall support switching between the two uplink carriers both with two-layer transmission following the scheduling commands and rank adaptation.

The switching period described in Figure 6.3A.3.2.2-1 are located in SCell uplink carrier, and the length of uplink switching period X is less than the value indicated by UE capability uplinkTxSwitchingPeriod.

When switching from one carrier to another, if there is no uplink transmission scheduled or configured on the switch-from carrier for at least the duration of the switching period (X µs) before the point in time the UE is scheduled or configured to start the transmission on the switch-to carrier, the switching period is fully contained in the time period between the end of the transmission on the switch-from carrier and the start of the transmission on the switch-to carrier.

Figure 6.3A.3.2.2-1: Time mask for switching between PCell UL carrier and SCell UL carrier

# Appendix II: Draft new WID