3GPP TSG-RAN WG4 Meeting #114	R4-2500339

Athens, Greece, 17th Feb 2025 – 21st Feb 2025

Agenda item:	7.20.3

Source:	MediaTek Inc.

Title:	Discussion on Testing issues for CSI compression two-sided models

Document for:	Discussion

# Introduction

In RAN4#113, RAN4 concluded that simulation results in RAN4#113 are aligned enough to proceed in two study tracks to further check the feasibility of testing for AI/ML CSI compression [1][2]. In the first track encoder is trained with pretrained decoders from other companies using companies own dataset. In the second track encoder and decoder are trained with mixed dataset from contributing companies. In this contribution, we provide our simulation results based on the agreed data generation assumptions and AI/ML model.

# Discussion

For training, validation, and testing data, we follow RAN4’s agreement to reuse system level parameters in TR38.843. Here, we consider the baseline case and ideal channel estimation.

Table 1. Parameters for system level simulation

| Parameter |  | Value |
| --- | --- | --- |
| Duplex, Waveform |  | FDD, OFDM |
| Multiple access |  | OFDMA |
| Scenario |  | Dense Urban (Macro only) |
| Frequency Range |  | FR1 only, [2GHz, 4GHz] |
| Inter-BS distance |  | 200m |
| Channel model |  | According to TR 38.901 |
| Antenna setup and port layouts at gNB |  | Companies need to report which option(s) are used between - 32 ports: (8,8,2,1,1,2,8), (dH,dV) = (0.5, 0.8)λ |
| Antenna setup and port layouts at UE |  | 4RX: (1,2,2,1,1,1,2), (dH,dV) = (0.5, 0.5)λ for (rank 1-4) |
| BS Tx power |  | 44dBm for 20MHz |
| BS antenna height |  | 25m |
| UE antenna height & gain |  | Follow TR36.873 |
| UE receiver noise figure |  | 9dB |
| Numerology | Slot/non-slot | 14 OFDM symbol slot |
|  | SCS | Baseline: 15kHz for 2GHz; |
| Simulation bandwidth |  | Baseline: 10 MHz for 15kHz |
| Frame structure |  | Slot Format 0 (all downlink) for all slots |
| MIMO scheme |  | SU-MIMO |
| MIMO layers |  | Baseline: 1 |
| CSI feedback |  | Feedback assumption at least for baseline scheme -CSI feedback periodicity (full CSI feedback): 5 ms (baseline) -Scheduling delay (from CSI feedback to time to apply in scheduling): 4 ms |
| Traffic load (Resource utilization) |  | Baseline: 50% Optional: 20/70% |
| UE distribution |  | CSI compression: 80% indoor (3 km/h), 20% outdoor (30 km/h) |
| UE receiver |  | MMSE-IRC as the baseline receiver |
| Feedback assumption |  | Realistic |
| Channel estimation |  | Realistic or ideal channel estimation |

The detailed structures of ResCNN encoder and decoder for two-sided CSI compression AI/ML model are shown in Figure 1 and 2.

| Figure 1. Detailed model diagrams of encoder | Figure 2. Detailed model diagrams of decoder |
| --- | --- |

We generate training, validation and testing data based on assumptions in Table 1 and use the above encoder and decoder pair for training with the main hyperparameters listed in Table 2. We also use ideal channel estimation.

Table 2. Hyperparameters used for ResCNN model.

| Hyperparameters | MediaTek dataset | Mixed dataset |
| --- | --- | --- |
| Number of samples in training dataset (80%) | 480k | 560k |
| Number of samples in validation dataset (10%) | 60k | 70k |
| Number of samples in testing dataset (10%) | 60k | 70k |
| Quantization (for quantized models) | (1/8, 3/8, 5/8, 7/8) | (1/8, 3/8, 5/8, 7/8) |
| Optimizer | Adam | Adam |
| Learning rate | 0.0001 | 0.0001 |
| Batch size | 256 | 256 |
| Max number of epochs | 150 | 150 |
| Stopping criteria | No validation loss improvement over 25 epochs | No validation loss improvement over 25 epochs |
| Loss function | 1 - SGCS | 1 - SGCS |
| Drop rate | No drop out | No drop out |

## 2.1 Track 1 (training of encoder using other companies pre-trained decoders)

In RAN4#113 companies were encouraged to train encoder, using other companies pretrained decoders, using own dataset, and report the CDF of the SGCS across sub-bands and input samples. We provide CDFs of SGCS for ResCNN, quantized and/or non-quantized depending on provided decoders, with MediaTek dataset. Also, for dataset comparison purposes we have trained encoders using datasets matching provided decoders where available.

We summarize sub-band averaged SGCS for ResCNN encoder trained with pretrained CATT decoder with MediaTek and CATT datasets in Table 3 (only quantized decoder model from CATT available). Furthermore, in Figure 3 we provide CDFs of sub-band averaged SGCS and in Figure 4 we provide CDFs per sub-band of SGCS for more statistical details.

Table 3. SGCS after training encoder with MediaTek, CATT and mixed datasets using the pretrained CATT decoder

|  | Overhead (bits) | MediaTek dataset | CATT dataset | Mixed dataset |
| --- | --- | --- | --- | --- |
|  |  | SGCS | SGCS | SGCS |
| ResCNN model 2-bit quantized | 64 bits | 0.516 | 0.713 | 0.494 |
| ResCNN model Float32 | 32*32 bits | N/A | N/A | N/A |

Figure 3. CDFs of SGCS of CATT decoder with MediaTek and CATT datasets.

Figure 4. CDFs of SGCS per sub-bands of CATT decoder with MediaTek and CATT datasets.

We summarize sub-band averaged SGCS for ResCNN encoder trained with pretrained Samsung decoder with MediaTek and Mixed datasets in Table 4. Furthermore, in Figure 5 we provide CDFs of sub-band averaged SGCS and in Figure 6 we provide CDFs per sub-band of SGCS for more statistical details.

Table 4. SGCS after training encoder with MediaTek and mixed datasets using the pretrained Samsung decoder

|  | Overhead (bits) | MediaTek dataset | Samsung dataset | Mixed dataset |
| --- | --- | --- | --- | --- |
|  |  | SGCS | SGCS | SGCS |
| ResCNN model 2-bit quantized | 64 bits | 0.715 | N/A | 0.640 |
| ResCNN model Float32 | 32*32 bits | 0.761 | N/A | 0.691 |

Figure 5. CDFs of SGCS of Samsung decoder with MediaTek and Mixed datasets.

Figure 6. CDFs of SGCS per sub-bands of Samsung decoder with MediaTek and Mixed datasets.

We were not able to get reasonable performance results with the decoder model shared by Huawei. As an alternative, we have chosen shared decoder model and dataset from Nokia. We summarize sub-band averaged SGCS for ResCNN encoder trained with pretrained Nokia decoder with MediaTek and Nokia datasets in Table 5. Furthermore, in Figure 7 we provide CDFs of sub-band averaged SGCS and in Figure 8 we provide CDFs per sub-band of SGCS for more statistical details.

Table 5. SGCS after training encoder with MediaTek, Nokia and mixed datasets using the pretrained Nokia decoder

|  | Overhead (bits) | MediaTek dataset | Nokia dataset | Mixed dataset |
| --- | --- | --- | --- | --- |
|  |  | SGCS | SGCS | SGCS |
| ResCNN model 2-bit quantized | 64 bits | 0.716 | 0.705 | 0.630 |
| ResCNN model Float32 | 32*32 bits | 0.769 | 0.757 | 0.691 |

Figure 7. CDFs of SGCS of Nokia decoder with MediaTek and Nokia datasets.

Figure 8. CDFs of SGCS per sub-bands of Nokia decoder with MediaTek and Nokia datasets.

As a further alternative, we also used shared decoder model and dataset from Qualcomm. We summarize sub-band averaged SGCS for ResCNN encoder trained with pretrained Qualcomm decoder with MediaTek and Qualcomm datasets in Table 6 (only quantized decoder model from Qualcomm available). Furthermore, in Figure 9 we provide CDFs of sub-band averaged SGCS and in Figure 10 we provide CDFs per sub-bands of SGCS for more statistical details.

Table 6. SGCS after training encoder with MediaTek, Qualcomm and mixed datasets using the pretrained Qualcomm decoder

|  | Overhead (bits) | MediaTek dataset | Qualcomm dataset | Mixed dataset |
| --- | --- | --- | --- | --- |
|  |  | SGCS | SGCS | SGCS |
| ResCNN model 2-bit quantized | 64 bits | 0.452 | 0.711 | 0.460 |
| ResCNN model Float32 | 32*32 bits | N/A | N/A | N/A |

Figure 9. CDFs of SGCS of Qualcomm decoder with MediaTek and Qualcomm datasets.

Figure 10. CDFs of SGCS per sub-bands of Qualcomm decoder with MediaTek and Qualcomm datasets.

We summarize sub-band averaged SGCS for ResCNN encoder trained with pretrained decoders with MediaTek or other datasets in Table 7, collected from Tables 3-6. In the last row of the table there is Reference corresponding to MediaTek Encoder and Decoder combination.

Table 7. Collection of SGCS for pretrained decoders with different datasets

|  | MediaTek Encoder with pretrained Decoder ResCNN model 2-bit quantized |  |  | MediaTek Encoder with pretrained Decoder ResCNN model Float32 |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Decoder Company | MediaTek dataset | Decoder Company dataset | Mixed dataset | MediaTek dataset | Decoder Company dataset | Mixed dataset |
|  | SGCS | SGCS | SGCS | SGCS | SGCS | SGCS |
| CATT | 0.516 | 0.713 | 0.494 | N/A | N/A | N/A |
| Samsung | 0.715 | N/A | 0.640 | 0.761 | N/A | 0.691 |
| Nokia | 0.716 | 0.705 | 0.630 | 0.769 | 0.757 | 0.691 |
| Qualcomm | 0.452 | 0.711 | 0.460 | N/A | N/A | N/A |
| Reference | 0.721 | 0.721 | 0.694 | 0.774 | 0.774 | 0.745 |

From these results we can see that SGCS results after training encoders with own MediaTek dataset using pretrained decoders can in most cases closely match reference SGCS results where Encoder and Decoder are trained together.  However, we can also observe that, for some shared pretrained models, expected SGCS results can only be achieved if the same dataset used for the decoder training is also used for the encoder training indicating that some datasets in some way potentially differ significantly from the datasets of other companies.

Observation #1: SGCS results with pretrained decoder can in most cases closely match reference SGCS results where Encoder and Decoder are trained together.

Observation #2: Dataset can have significant impact on SGCS results for some shared pretrained decoder models.

Proposal #1: We propose to study how to guarantee the validity of shared company datasets to ensure that a shared company decoder is not trained on invalid data and that a mixed dataset created of individual company datasets does not potentially include invalid data.

## 2.2 Track 2 (training of encoder-decoder pairs using mixed dataset)

We provide our results for test set performance in Table 8 with MediaTek and Mixed datasets where both encoder and decoder models are trained by MediaTek. The SGCS of AI CSI feedback and eTypeII CSI feedback are both relative to ideal CSI. These results show that the SGCS of eTypeII and quantized ResCNN are similar.

Table 8. SGCS of eTypeII and ResCNN (Quantized and non-quantized) with MediaTek and Mixed datasets. Both encoder and decoder models are trained by MediaTek.

|  | Overhead (bits) | MediaTek dataset | Mixed dataset |
| --- | --- | --- | --- |
|  |  | SGCS | SGCS |
| eTypeII PC1 | 62 bits | 0.719 | - |
| ResCNN model 2-bit quantized | 64 bits | 0.721 | 0.694 |
| ResCNN model Float32 | 32*32 bits | 0.774 | 0.745 |

Observation #3: For the case of MediaTek dataset, the SGCS of quantized ResCNN model is 0.721 with 64 bits feedback and SGCS of unquantized ResCNN model is 0.774. Also, the SGCS of eTypeII PC1 with 62 bits feedback is 0.719.

Observation #4: For the case of Mixed dataset, the SGCS of quantized ResCNN model is 0.694 with 64 bits feedback and SGCS of unquantized ResCNN model is 0.745.

In RAN4#113 companies were encouraged to use mixed dataset from contributing companies and report the CDF of the SGCS across sub-bands and input samples. Also, If possible, report CDF for eTypeII. We provide CDFs of SGCS for ResCNN (Quantized and non-quantized) and eTypeII with MediaTek and Mixed datasets in Figure 11, except eTypeII is excluded with Mixed dataset.

Figure 11. CDFs of SGCS of MediaTek encoder and decoder with MediaTek and Mixed datasets.

Observation #5: Using MediaTek and Mixed datasets, the comparison of ResCNN CDFs of SGCS indicate that the performance is almost on par.

Furthermore, in RAN4#113 companies were encouraged to report the CDF of the SGCS per sub-bands. Also, If possible, report CDF for eTypeII. We provide CDFs per sub-band of SGCS for ResCNN (Quantized and non-quantized) and eTypeII with MediaTek and Mixed datasets in Figure 12, except eTypeII is excluded with Mixed dataset.

Figure 12. CDFs of SGCS per sub-bands of MediaTek encoder and decoder with MediaTek and Mixed datasets.

Observation #6: Using MediaTek and Mixed datasets, the comparison of SGCS between sub-bands indicate that the performance is slightly reduced on band edge sub-bands.

## 2.3 Model architecture/sharing issues

Decoder input interface

Encoder output interface

Inverse

Sigmoid

Demapping

Mapping

Sigmoid

Dequantizer

Quantizer

Figure 13. Encoder and Decoder interface layer options. For the quantized models, all blocks could potentially be included. For unquantized models, only the Sigmoid and Inverse Sigmoid blocks could potentially be included.

We have studied shared Encoders and Decoders for RAN4#113 meeting, and we make some observations of these models. To help with the discussions related to the encoder-decoder interface, we have in Figure 13 shown the encoder and decoder functionalities which affect the statistics of the data seen at the encoder-decoder interface.

Observation #7: Data dimensions and types of model interfaces are not aligned among shared models.

Proposal #2: We propose to define Encoder Input and Decoder Output as data dimension (n, 2, 13, 32) with data type float32 where n is the dynamic batch size.

Proposal #3: We propose to define Encoder Output and Decoder Input as data dimension (n, 32) with data type float32.

Proposal #4: We propose not to define any additional inputs, e.g., control parameters.

Observation #8: Interface between unquantized decoder output and unquantized encoder input is not aligned among shared models.

Proposal #5: We propose to not include sigmoid and inverse sigmoid for the unquantized models.

Observation #9: Interface between quantized decoder output and quantized encoder input is not aligned among shared models.

Proposal #6: We propose to include sigmoid and inverse sigmoid in the quantization and dequantization stage, respectively, for the quantized models.

Proposal #7: We propose to define quantized values as (1/8, 3/8, 5/8, 7/8).

Proposal #8: We propose to include Quantizer to shared models.

Proposal #9: We propose to not include the mapping of quantized values to 2 bits and the demapping from 2 bits to quantized values in the shared models as this is unnecessary functionality when evaluating the performance.

# Conclusion

In this contribution, simulation results for alignment are provided. Considering ResCNN AI/ML model, we have the following observations:

Observation #1: SGCS results with pretrained decoder can in most cases closely match reference SGCS results where Encoder and Decoder are trained together.

Observation #2: Dataset can have significant impact on SGCS results for some shared pretrained decoder models.

Proposal #1: We propose to study how to guarantee the validity of shared company datasets to ensure that a shared company decoder is not trained on invalid data and that a mixed dataset created of individual company datasets does not potentially include invalid data.

Observation #3: For the case of MediaTek, the SGCS of quantized ResCNN model is 0.716 with 64 bits feedback and SGCS or unquantized ResCNN model is 0.774. Also, the SGCS of eTypeII PC1 with 62 bits feedback is 0.719.

Observation #4: For the case of Mixed dataset, the SGCS of quantized ResCNN model is 0.700 with 64 bits feedback and SGCS or unquantized ResCNN model is 0.745.

Observation #5: Using MediaTek and Mixed datasets, the comparison of ResCNN and eTypeII CDFs of SGCS indicate that the performance is almost on par.

Observation #6: Using MediaTek and Mixed datasets, the comparison of SGCS between sub-bands indicate that the performance is slightly reduced on band edge sub-bands.

Observation #7: Data dimensions and types of model interfaces are not aligned among shared models.

Proposal #2: We propose to define Encoder Input and Decoder Output as data dimension (n, 2, 13, 32) with data type float32 where n is the dynamic batch size.

Proposal #3: We propose to define Encoder Output and Decoder Input as data dimension (n, 32) with data type float32.

Proposal #4: We propose not to define any additional inputs, e.g., control parameters.

Observation #8: Interface between unquantized decoder output and unquantized encoder input is not aligned among shared models.

Proposal #5: We propose to not include sigmoid and inverse sigmoid for the unquantized models.

Observation #9: Interface between quantized decoder output and quantized encoder input is not aligned among shared models.

Proposal #6: We propose to include sigmoid and inverse sigmoid in the quantization and dequantization stage, respectively, for the quantized models.

Proposal #7: We propose to define quantized values as (1/8, 3/8, 5/8, 7/8).

Proposal #8: We propose to include Quantizer to shared models.

Proposal #9: We propose to not include the mapping of quantized values to 2 bits and the demapping from 2 bits to quantized values in the shared models as this is unnecessary functionality when evaluating the performance.

# Reference

- R4-2420513, WF on AI/ML air interface part 1, Qualcomm inc.

- R4-2420336, WF on AI/ML air interface part 2, Ericsson