# 3GPP TSG-RAN WG4 Meeting #114	                                R4-2500343

Athens, GR, 17th – 21st February, 2025

Source:                Tejas Networks Pvt. Ltd.

Title:                     Discussion on Potentially new requirements for SBFD operation

Agenda Item:       7.23.2.1

Document for:     Discussion

# Introduction

This contribution presents our views on the potential new requirements for SBFD operation. In the RAN4 #112bis meeting, RAN4 agreed to further investigate the duration and characteristics of the transient period between SBFD and non-SBFD operation, as outlined in the following agreement [1].

Way Forward: - FFS location of transient period between SBFD and non-SBFD: - Option 1: No need to discuss and decide whether the transmitter transient period shall be located within the SBFD symbol or not. - Option 2: Place the transient period outside of both SBFD and non-SBFD slots. - Option 3: The location of the transient period between SBFD and non-SBFD shall be within the SBFD symbol).

Further in RAN 113 meeting, RAN4 agreed to further investigate the location of transient period between SBFD and non-SBFD, as outlined in the following agreement [2].

Way Forward: - FFS whether the transmitter transient period shall be located within the SBFD symbol or not: - FFS it is an implementation issue and no RAN4 standard impact

# Discussion

## Location of transient period between SBFD and non-SBFD

Figure 1: SBFD configuration

Figure 1 shows a SBFD configuration. The possible switching cases may be [DL-SBFD], [SBFD-DL], [NON SBFD UL - SBFD UL] and [SBFD UL - NON SBFD UL].

In SBFD systems, the transient time should align with the existing TDD switching mechanisms to ensure a consistent total TX switching time budget for transitions between complete UL/DL and SBFD RBs. For DL-SBFD UL and SBFD-DL transitions, the existing TDD switching time mechanisms, such as guard symbols and TA_offset, are sufficient since the power amplifier (PA) remains on during these transitions. Furthermore, as UE operates in half-duplex mode, the transition from non-SBFD DL to SBFD UL only involves PA switching time, with no additional requirements imposed on the BS.

Observation 1: The location of the transmitter transient period is primarily an implementation-specific issue, influenced by factors like hardware capabilities, system timing mechanisms, and self-interference management techniques. Variability across vendors and deployment scenarios makes a standardized approach impractical.

Observation 2: The transient period’s location does not directly impact system-level performance if key performance indicators (ACLR, REFSENS, in-band blocking, etc.) are maintained. Current mechanisms such as guard symbols and TA_offset offer sufficient flexibility to accommodate transient periods in different deployment scenarios without requiring standardization.

Proposal 1: Recognize the placement of the transmitter transient period as an implementation-specific decision, with no direct impact on RAN4 standards.

Proposal 2: Prioritize defining and enforcing system-level performance requirements (e.g., ACLR, REFSENS, in-band blocking) to ensure proper operation during transient periods, independent of their location.

# Conclusion

In this contribution, we summarised the potentially new requirements for SBFD operation with the following proposals and observations.

Observation 1: The location of the transmitter transient period is primarily an implementation-specific issue, influenced by factors like hardware capabilities, system timing mechanisms, and self-interference management techniques. Variability across vendors and deployment scenarios makes a standardized approach impractical.

Observation 2: The transient period’s location does not directly impact system-level performance if key performance indicators (ACLR, REFSENS, in-band blocking, etc.) are maintained. Current mechanisms such as guard symbols and TA_offset offer sufficient flexibility to accommodate transient periods in different deployment scenarios without requiring standardization.

Proposal 1: Recognize the placement of the transmitter transient period as an implementation-specific decision, with no direct impact on RAN4 standards.

Proposal 2: Prioritize defining and enforcing system-level performance requirements (e.g., ACLR, REFSENS, in-band blocking) to ensure proper operation during transient periods, independent of their location.

# Reference

- R4-2416584: Way forward for [112bis][306] NR_duplex_evo_General.

- R4-2419907, Way Forward for [113][306] NR_duplex_evo_General, Samsung