3GPP TSG-RAN WG4 Meeting #114										    R4-2500388

Athens, Greece, 17th - 21th February 2025

Agenda Item:	7.25.2.1

Source:	MediaTek Inc.

Title:	Discussion on RRM requirements for OD-SSB for network energy saving

WID/SID:        Netw_Energy_NR_enh-Core

Document for:	Discussion

# 1  Introduction

The remaining issues on On-Demand SSB SCell operation [1] are discussed in this paper.

# 2  On-Demand SSB SCell operation (Objective#1 )

Issue 1-2-2: OD-SSB based deactivated SCell measurement – CSSF in fast mode measurement

Whether to apply the legacy CSSF in fast mode measurement was discussed [1].

- Option 1: The same CSSF as legacy

- Option 2:

  - OD-SSB based L3 measurement is prioritized over legacy L3 measurement, if cell identification is needed for OD-SSB-based L3 measurements.

    - RRM requirements on PSCC and other SCCs are not applicable during the time-window.

  - Legacy L3 measurement on other SCC are not impacted, if OD-SSB based cell identification is not needed.

Other option is not precluded.

Option 2 prioritizes OD-SSB when cell identification is needed for OD-SSB (i.e. CSSF=1) to speed up the OD-SSB measurement.

First, this approach may compromise the mobility performance concerning PSCC and SCC.

Second, it is unclear how the UE would determine whether the cell identification is needed for OD-SSB. To our understanding, in case 1, for the 2nd OD-SSB/fast mode the cell identification is not required if the time span between the 2nd OD-SSB and the OD-SSB deactivation is within 5sec, as the legacy known cell condition. However it is not clear if this is the only case or if this case should be optimized.

Proposal 1: For OD-SSB based deactivated SCell measurement, the legacy CSSF applies in fast mode measurement.

Issue 1-2-3: OD-SSB based deactivated SCell measurement – Procedure

Sub-issue 2: What’s the expected minimum interval(T2, stage 4) between multiple times OD-SSB activation/deactivation(reconfiguration)?

The minimum interval(T2, stage 4) between multiple OD-SSB transmissions was discussed to limit the UE measurement complexity.

However, it appears that the network has no intention to trigger frequent OD-SSB transmissions from network power saving perspective. Therefore, it may not be necessary to define T2.

Second, to limit the UE measurement complexity, the concern is that the two fast mode should not be too close to each other. Therefore, the T2’ should be defined as the minimum interval between the two fast mode (i.e. the end of 1st fast mode and the start of the 2nd fast mode), instead of between OD-SSB transmissions.

Proposal 2: For multiple OD-SSB transmission, not to define the minimum interval (T2, stage 4) between OD-SSB transmissions. Instead, the minimum interval between fast mode windows (T2’) can be defined.

Sub-issue 5: What’s the UE behaviour when OD-SSB further activation/reconfiguration after T2?

By defining the minimum interval between fast mode windows (i.e. T2’), the UE measurement behaviour on the 2nd/future OD-SSB will be clear, i.e. the UE performs fast mode measurement after T2’ (in 5), as illustrated below.

Proposal 3: For multiple OD-SSB transmission, UE performs fast mode measurement after the minimum interval between fast mode windows (T2’), if the OD-SSB transmission is available.

# Conclusion

Proposal 1: For OD-SSB based deactivated SCell measurement, the legacy CSSF applies in fast mode measurement.

Proposal 2: For multiple OD-SSB transmission, not to define the minimum interval (T2, stage 4) between OD-SSB transmissions. Instead, the minimum interval between fast mode windows (T2’) can be defined.

Proposal 3: For multiple OD-SSB transmission, UE performs fast mode measurement after the minimum interval between fast mode windows (T2’), if the OD-SSB transmission is available.

# References

- R4-2420098, WF on RRM requirements for Netw_Energy_NR_enh_Part1, Ericsson, Nov. 2024.