3GPP TSG-RAN WG4 Meeting #114	R4-2500405

Athens, Greece, 17th - 21nd February 2025

Title: 	NR_LBCA_Sw Introduction

Agenda Item:	7.8.2

Source: 	Nokia

Document for:	Approval

# Introduction

In previous RAN meeting work item for NR_LBCA_Sw was approved. This contribution discusses about UE requirements and related capabilities. Work item objectives as listed WF [1] are as following.

# Discussion

Operators possess 10 to 20 times more spectrum in mid bands compared to low bands. The former is advantageous for use in proximity to cell sites, whereas the latter is more beneficial at greater distances. Low-band spectrum supports substantial traffic volumes in both urban (indoor) and rural areas, resulting in congestion that significantly degrades customer experience [2]. Low-band SDL bands are underutilized due to their close frequency proximity to other low bands, complicating their aggregation for CA. Conversely, SDL low bands may be located far from another low-band carrier, rendering the UE antenna design impractical. To address these practical challenges in UE implementation, a switched Low-Band to Low-Band Carrier Aggregation operation is proposed.

## Switched LB CA feature introduction

The switched LB CA operation, as stipulated by the WID, is based on a semi-static switching pattern configured through RRC. The switching pattern alternates between two modes:

- FDD

  - UL and DL

- SDL

  - DL only

At UE these two modes are enabled in following domains:

- Antenna

  - Antenna is tuned from one carrier frequency to another. Assumption is a switch-based aperture tuning

- RFFE

  - RF path selection from antenna to TRX

- TRX

  - RF Rx selection and/or

  - RF Rx chain configuration for given carrier frequency

Figure 1. Concept of switched LB CA feature [2]

## Potential implementations

Figure 2. Possible implementation options for switched LB CA. A) Rx chains are shared in RFIC for two carriers, B) Dedicated Rx chains for each carrier in TRX but switched in RFFE

For simplicity, a 2+2 DL MIMO operation is considered. One Rx chain, either primary or diversity in the TRX, comprises a LNA, mixer, analog channel filter, ADC. The Local Oscillator is shared between the primary and diversity receivers. RFFE domain may or may not include an LNA for each Rx chain. Detailed antenna interface, including antenna tuning, is not depicted here.

Option A presumes that the Rx RF chain is shared between the FDD carrier and the SDL carrier; therefore, the configuration for each carrier, as well as the frequency and time synchronization, need to be established for each cycle. This will result in a period during which the receiver cannot properly demodulate the received signal.

Option B presumes that the Rx RF chains in the TRX are separate, and routing to the antenna is switched in the RFFE domain. In this way, any configuration for the Rx RF chain can be pre-programmed before carrier switching occasion, allowing reception to start at the desired point in time.

Time and frequency synchronization for the SDL carrier may be supported by an FDD carrier or may use its own synchronization signals.

Observation 1. UE HW implementation may have an impact to UE capability to receive DL signal

## UE timing considerations

Depending on the implementation, RF solution may have various DL switching time capabilities, alternating DL reception between two carriers in a manner which resembles TDD operation on different carriers. The SDL carrier needs to transmit essential signals for maintaining the radio link, such as HARQ on the FDD carrier. Consequently, the FDD-SDL switching pattern must be sufficiently short.

### DL Timing

Considering for example downlink reference measurement channel for TDD which has:

- TDD slot configuration DDDSU

- Special slot configuration 10D+2G+2U

A typical DL interruption length for UL reconfigurations with 15kHz SCS is one slot in 38.133. However, this long interruption with a short switching pattern will degrade UE ability receive on SDL carrier.

Observation 2. A long gap or delay due to DL carrier switching will degrade UE ability to receive on SDL carrier when carrier switching pattern is short

There are shorter than one slot DL interruptions allowed in 38.133 at UE switching between two uplink carriers.

“When dynamic switching between two uplink carriers is conducted, UE is allowed to cause DL interruption of X OFDM symbols in NR downlink carrier(s) as indicated by uplinkTxSwitching-DL-Interruption [38.331]. The DL interruption starts from the first OFDM symbol which fully or partially overlaps with the UL switching period located in either NR carrier 1 or carrier 2 as indicated in RRC signalling [38.331].”

Example of allowed interruption lengths can be found from 38.133, Table 8.2.2.2.10-1.

Figure 3. 38.133, Table 8.2.2.2.10-1.

Reflecting figure 3 to this work item, an analogous requirement would be 2 symbol interruption on DL with shortest UL switching time of 35us. However, there are CA combinations listed in 38.101-1 that do not allow a DL interruption for DL at UL switching as HW capability of a UE is assumed to enable this.

Transmit to receive and receive to transition time for a UE not capable of full-duplex communication after last transmitted UL symbols and last received DL symbols respectively are defined in 38.211 as below.

Figure 3. 38.211 transition times. 25600xTc = 13us

Observation 3. Current specification allows gaps in the DL signal from 13us and two symbols up to one slot.

### UL timing

General on/off time mask is defined in 38.101-1 as following.

Figure 3 General UE on/off time mask

On power requirement start at the start slot boundary and end also at end slot boundary. Off power shall be met at start slot boundary -10us and end slot boundary +10us.

### Timing at UE

#### Two separate receivers

Considering a UE implementation like in figure 2 option B in which there are two receivers available for CA_n5-n29. According to WID both carriers are co-located, synchronized and are in single TAG. Timing advance for uplink is signalled by gNB by

TTA = NTA + NTA,offset., where NTA,offset is fixed, 25600xTc = 13us

UE transitioning from FDD to SDL turns off transmitter by TTA before end of DL reception. RF switch point is at the slot boundary and considering separate RF rx chains there is time both sides to configure SDL Rx chain to receive PDCCH on first FDD symbol. CP period for RF switches and antenna tuners for state change is enough. Same applies for transitioning from SDL to FDD.

Unlike TDD operation there is no DL-UL gap in FDD operation. When UL switch from SDL to FDD, UL transmission may overlap with SDL unless the first symbols are muted or UL transmission delayed by the time of TTA. This is illustrated by gray area in figure 4.

Figure 4. Timing at the UE with two receivers considering semi-static switching pattern and phase aligned downlink frames.

Observation 4. By assuming a UE is having two dedicated receiver chains it is possible to switch between to LB carriers without interruption or gap on downlink.

Observation 5. Unlike TDD operation there is no DL-UL gap in FDD operation When UL switch from SDL to FDD, UL transmission may overlap with SDL unless the first symbols are muted or UL transmission delayed by the time of TTA.

Observation 6. Switching from SDL to FDD operation general UL on/off time mask with 10us transition time before slot boundary need to be considered for any UL interruption or gap.

#### Shared receiver for FDD and SDL

Considering shared receivers there is no possibility to configure receivers prior to switch point so there is a need to allow some time for this by interrupting DL for both FDD and SDL. Exact amount is to be discussed.

Observation 7. For a shared receivers in TRX there is a need to interrupt DL prior to switch points to allow RF Rx chains to be reconfigured

Proposal 1. Only separate RF Rx chain implementation should be considered for switched LB CA operation.

Proposal 2. No DL interruption allowed for switched LB CA operation.

# Conclusion

Considering the motivation and objective of the work item addressing low band congestion and improving user experience we propose:

Proposal 1. Only separate RF Rx chain implementation should be considered for switched LB CA operation.

Proposal 2. No DL interruption allowed for switched LB CA operation.

# 4	References

[1] RP-243317 NR_LBCA_Sw WID_cl, RAN#106

[2] RP-24253 Motivation for a new work item on low band carrier aggregation via switching in Rel-19, RAN#106