# 3GPP TSG-RAN WG4 Meeting #114                                                                R4-2500412

Athens, Greece, 17-21 February, 2025

Title: 	Remaining system parameters issues for supporting 3MHz CBW in NTN bands

Source: 	Huawei, HiSilicon

Agenda item:	7.10.3.1

Document for:	Discussion

# Introduction

RAN4 113 meeting the WF[1] was agreed, and some system parameters are remained as FFS in this document, those open issues will be addressed.

# Discussion

Issue 1-1: Whether 3MHz channel bandwidth for NR-NTN in FR1-NTN bands is mandatory or optional? - Option 1: Optional - Option 2: Mandatory

As the nature of NTN communication is based on low spectrum allocations, having 3MHz mandatory on the bands that it supports would be beneficial for operators, moreover, in the last meeting at least one operator raised this issue and asked to having this feature mandatory. To avoid any issues for the legacy UE, such mandatory feature should be considered from Rel19 onward. On the other hand, it is understandable if some manufacturers prefer the feature to be optional to have more flexibility

Observation 1: one option could be 3MHz mandatory for n255, n256, and n254 from Rel-19 onward, as there is request from an operator. Though it is understandable if some manufacturers prefer the feature to be optional to have more flexibility

Issue 1-2: UE capability Signalling - FFS on re-use of existing rel-18 signaling parameters

In Rel-18 UE capabilities support3MHz-ChannelBW-Symmetric-r18 and support3MHz-ChannelBW-Asymmetric-r18 were introduced to show that the UE supports 3MHz CBW in symmetrical and asymmetrical (3MHz in UL) configurations. These UE capabilities are for single carrier operations , excluding RedCap. Therefore they can be used in the NTN communication, too.

Proposal 1: Reuse the Rel-18 UE capabilities support3MHz-ChannelBW-Symmetric-r18 and support3MHz-ChannelBW-Asymmetric-r18 for NTN communications in Rel-19 onward.

Issue 1-3: If 3MHz is a mandatory feature, the UE behaviour to support 3MHz Channel bandwidth - WF: - FFS on UE behaviours if 3MHz CHBW is mandatory for certain band. - Reception of 12 PRB PBCH based on RB-level puncturing; - Short RACH preamble formats with 15kHz SCS, and long PRACH formats with 1.25kHz SCS; - Reception of 15 PRB CORESET0;

To support 3MHz , even optional, the above mentioned bullet points have to be considered as they are based on the Rel-18 3MHz design on TN. It should be noted that in contrast to TN, on NTN no 12 PRB CORESET0 is needed, because it was dedicated for a special case for band n100.

Proposal 2:  To support 3MHz, even optional if not mandatory, the following should be considered per band:

    - Reception of 12 PRB PBCH based on RB-level puncturing;

    - Short RACH preamble formats with 15kHz SCS, and long PRACH formats with 1.25kHz SCS;

    - Reception of 15 PRB CORESET0;

# Conclusions

Observation 1: one option could be 3MHz mandatory for n255, n256, and n254 from Rel-19 onward, as there is request from an operator. Though it is understandable if some manufacturers prefer the feature to be optional to have more flexibility

Proposal 1: Reuse the Rel-18 UE capabilities support3MHz-ChannelBW-Symmetric-r18 and support3MHz-ChannelBW-Asymmetric-r18 for NTN communications in Rel-19 onward.

Proposal 2:  To support 3MHz, even optional if not mandatory, the following should be considered per band:

    - Reception of 12 PRB PBCH based on RB-level puncturing;

    - Short RACH preamble formats with 15kHz SCS, and long PRACH formats with 1.25kHz SCS;

    - Reception of 15 PRB CORESET0;

# Reference

[1]	“R4-2419789, Way Forward for [113][311] NR_IoT_NTN_less_than_5MHz_UERF, Xiaomi.”

[2]	“R4-2416255, A-MPR evaluations of NS_03N, NS_N04N, NS_05N and NS_24 for 3MHz CBW for NTN communication, Huawei, HiSilicon.”

[3]	“R4-2419156, Remaining RF issues for supporting 3MHz CBW in NTN bands, Huawei, Hisilicon.”